| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0177 |
| --- | --- |

| Title: | AHG5: Merge index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method for binarising the merge_idx syntax element.  The proposed binarisation removes a transition from arithmetic to bypass coding, resulting in the merge_idx syntax element making exclusive use of arithmetic coding. This change resulted in 0.0%, 0.0%, 0.0% in AI Main, AI HE10, RA Main and RA HE10 configs. Results were 0.0%, 0.1%, 0.0% in LDB Main and 0.0%, 0.0%, 0.3% in LDB HE10 and -0.1%, -0.1% and 0.0% in LDP Main, -0.1%, 0.0%, -0.1% in LDP HE10.

# Problem Statement

In HM-7.0, the binarisation for merge_idx is a truncated unary bin string, using arithmetic coding for the first bin and bypass coding for subsequent bins.  One contexts are used, allocated as shown in Table 1.  This binarisation results from adoption of JCTVC-H0251 [1], which introduced the bypass coding and removed three contexts.

Table 1. merge_idx binarisation.

| binIdx | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| ctxIdx | 0 | B | B | B |

A drawback of this approach is that coding efficiency in LDP configurations is impaired by the use of bypass coding for merge_idx values greater than 0.

# Description of the proposed change

The proposed binarisation makes exclusive use of arithmetic coding for the merge_idx syntax element.  This binarisation is illustrated in .

Table 2. merge_idx binarisation.

| binIdx | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| ctxIdx | 0 | 0 | 0 | 0 |

# Experiment Results

Results are presented in Table 3 below. Simulations were performed under the common conditions [2]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Context initialization values were not updated in this simulation.

Table 3. Simulation results in HM-7.0.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class E | -0.2% | 0.0% | -0.4% | -0.1% | -0.3% | 0.3% |
| Class F | -0.1% | -0.3% | 0.1% | 0.1% | 0.1% | 0.6% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.3% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.0% | 0.0% | -0.1% | -0.1% | -0.3% |
| Class C | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | 0.0% |
| Class D | -0.1% | 0.2% | 0.0% | -0.1% | -0.4% | -0.1% |
| Class E | -0.2% | -0.6% | -0.1% | -0.2% | 0.2% | 0.1% |
| Class F | -0.1% | -0.4% | 0.2% | 0.0% | 0.5% | 0.1% |
| Overall | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

# Working Draft changes

| Table 9-4 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| merge_idx | Error: Reference source not found | 0 | 0 | 0na | 0na | 0na | na |
|  |  | 1 | 0 | 0na | 0na | 0na | na |

# Conclusion

This contribution presents a binarisation for merge_idx with a transition from arithmetic to bypass coding removed and coding efficiency results in luma of -0.1% in LDP Main and LDP HE10 configurations.

# References

- O. Bici, J. Lainema, K. Ugur, “Context reduction for merge index coding”, Joint Collaborative Team on Video Coding, JCTVC-H0251, Feb 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-I1100, May 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).