| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0178 |
| --- | --- |

| Title: | AHG5: On SAO syntax elements coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V.Kolesnikov 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: Tel: Email: | +61-2-9805-2844 {chris.rosewarne, vladimir.kolesnikov}@cisra.canon.com.au +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia (CiSRA), Canon Inc. |  |  |

_____________________________

# Abstract

A method of encoding sao_merge_left_flag is presented. The method reportedly allows to simplify implementation and to reduce the number of used contexts. In the common test conditions the method reportedly shows the following coding impact: 0.0%, 0.0%, -0.1% in AI, -0.1%, -0.2%, -0.2% in RA,        -0.1%, -0.2%, -0.3% in LD-B, and -0.1%, -0.4%, -0.3%  in LD-P configurations.

A method of encoding sao_type_idx syntax element is being presented. The method reportedly allows to reduce the number of arithmetic contexts and to improve throughput without compromising the coding efficiency. In the common test conditions the method reportedly shows the following coding impact: 0.0%, 0.0%, -0.1% in AI, 0.0%, 0.0%, -0.1% in RA, 0.0%, 0.0%, 0.1% in LD-B, 0.0%, 0.1%, -0.1% in LD-P configurations.

A method of grouping of LCU SAO syntax elements is being presented. The method reportedly allows to simplify implementation and to improve the throughput.

# Problem Statement

In HM-7.0 [1] SAO parameters are being coded on LCU level. The SAO parameter block for an LCU can either indicate a reference to a set of SAO parameters from left/top neighbour LCUs which is indicated by binary flags sao_merge_left_flag, sao_merge_up_flag respectively. If both sao_merge_left_flag, sao_merge_up_flag are zero (indicating that the referencing is not used) then SAO parameter values are begin encoded explicitly for the current LCU.

In HM-7.0 sao_merge_left_flag syntax element is encoded using a separate context model for every channel thus using 3 context models in total.

In HM-7.0 the sao_type_idx syntax element is arithmetically encoded using unary binarisation. HM-7.0 sao_type_idx coding scheme is shown in Table 1.1:

Table 1.1. sao_type_idx binarisation and context allocation in HM-7.0

| Sao_type_idx Value | Bin Idx 0 | Bin Idx 1 | Bin Idx 2 | Bin Idx 3 | Bin Idx 4 | Bin Idx 5 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |
| 4 | 1 | 1 | 1 | 1 | 0 |  |
| 5 | 1 | 1 | 1 | 1 | 1 | 0 |
| Context index id: | 0 | 1 | 1 | 1 | 1 | 1 |

HM-7.0 stream layout for SAO syntax elements is presented on Figure. 1.2

Fig. 1.2. SAO syntax elements stream layout.

In the HM-7.0 all syntax elements are coded separately for each channel.

We propose an alternative binarisation scheme, context assignment scheme and stream layout for SAO syntax elements. We assert that these alternatives allow to reduce the number of used contexts, to simplify implementation and to increase CABAC throughput without compromising the coding efficiency.

# Proposed Solution

## Context reduction for sao_merge_left_flag

We assert that the number of context models used for sao_type_left_flag coding can be reduced without compromising coding efficiency. We propose to use a single context model for encoding sao_merge_left_flag in all channels thus reducing the number of used context models from 3 to 1.

## Fixing unary binarisation for sao_type_idx

Unary coding is intended to be used when the range of encoded value magnitudes is unknown. This is not the case with sao_type_idx [1]. Thus we assert that utilizing truncated unary coding as presented in Table 2.1 is more optimal than the unary coding currently used in HM-7.0. The truncated unary coding scheme is presented in the Table 2.1:

Table 2.1. Truncated Unary Coding for sao_type_idx

| Sao_type_idx Value | Bin Idx 0 | Bin Idx 1 | Bin Idx 2 | Bin Idx 3 | Bin Idx 4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |
| 1 | 1 | 0 |  |  |  |
| 2 | 1 | 1 | 0 |  |  |
| 3 | 1 | 1 | 1 | 0 |  |
| 4 | 1 | 1 | 1 | 1 | 0 |
| 5 | 1 | 1 | 1 | 1 | 1 |
| Context index id: | 0 | 1 | 1 | 1 | 1 |

## Alternative “1+3” binarisation for_sao_type_idx

However, we further assert that the truncated unary binarisation shown above can be further optimized. We propose a scheme shown in the Table 2.2 below.

Table 2.2. The proposed sao_type_idx binarisation

| Sao_type_idx Value | Bin Idx 0 | Bin Idx 1 | Bin Idx 2 | Bin Idx 3 |
| --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |
| 1 | 1 | 0 | 0 | 0 |
| 2 | 1 | 1 | 0 | 0 |
| 3 | 1 | 0 | 1 | 0 |
| 4 | 1 | 1 | 1 | 0 |
| 5 | 1 | 0 | 0 | 1 |
| Context index id: | 0 | n/a (bypass) | n/a (bypass) | n/a (bypass) |

In our tests the case of value “0” has shown probability of more than 0.9 so in the proposed scheme the value “0” is encoded with a single arithmetically coded bin using one context model. All other values are encoded with an arithmetically coded bin followed by a three-bit fixed length bypass code. With the new scheme only one context model is used (instead of 2 in HM-7.0). Moreover, the worst case of number of arithmetically encoded bins per sao_type_idx value is reduced from 6 (in HM-7.0) to 1.

After decoding the first bin only one condition check is required in order to determine the length and bypass-decode a code-word suffix. Thus we assert that the proposed binarisation for the bypass coded portion allows a simplification of implementation logic.

## SAO syntax element stream layout

Based on the modifications to the SAO syntax element coding proposed above we propose an alternative SAO parameter syntax element stream layout. The proposed layout reportedly allows to simplify implementation logics and to improve the throughput.

Fig. 2.3 Proposed SAO syntax element layout

The encoding of sao_merge_left_flag in HM-7.0 depends on a check of whether the current LCU is on the leftmost position in frame/tile. With the layout from Fig. 1.2 utilized by the current HM the “leftmost” condition needs to be checked three times (once per channel). We propose to encode sao_merge_left_flag syntax elements for all channels adjacently as shown on Fig. 2.3. In this case the “leftmost” condition needs to be checked only once per LCU.

Similarly, the encoding of sao_merge_up_flag in HM-7.0 depends on a check of whether the current LCU is on the topmost position in the frame/tile. With the layout from Fig. 1.2 utilized by the current HM the “topmost” condition needs to be checked three times. We propose to encode sao_merge_up_flag syntax elements for all channels adjacently as shown on Fig. 2.3. In this case the “topmost” condition needs to be checked only once per LCU.

With the binarisation scheme proposed in chapter 2.3 the sao_type_idx syntax elements have an arithmetically encoded prefix and bypass-coded suffix. We propose to group the arithmetic portions of sao_type_idx elements for all channels and to group the bypass portions of sao_type_idx elements for all channels as shown on Fig. 2.3. We assert that this allows a simple implementation which can read all sao_type_idx bypass portions at once thus improving the throughput.

We also assert that syntax element grouping as proposed above can be beneficial for implementations which use context model caching and other entropy coding optimization techniques because the number of context model switches is reduced compared to HM-7.0.

# Experiment Results

Results presented in Table 3.1 below were gathered for the configuration proposed in the chapter 2.4 and refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 3.1. Experimental Results (normal QP, LCU size = 64)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 98% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.4% | 0.0% | -0.3% | -0.1% |
| Class B | 0.0% | -0.1% | -0.4% | 0.0% | -0.2% | -0.2% |
| Class C | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class D | -0.2% | -0.1% | -0.2% | -0.1% | -0.2% | -0.4% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.1% |
| Class D | -0.1% | 0.2% | 0.7% | -0.1% | -0.3% | -0.4% |
| Class E | -0.1% | -0.2% | -0.3% | -0.1% | -0.7% | -0.1% |
| Class F | -0.1% | 0.0% | -0.1% | 0.1% | -0.2% | 0.5% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | 0.0% | -0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.8% | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.3% |
| Class D | 0.0% | 0.3% | 0.3% | -0.1% | 0.1% | -1.0% |
| Class E | 0.0% | -0.3% | -0.1% | -0.2% | 0.2% | 0.2% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | 0.8% | 0.4% |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | 0.2% | -0.2% |
|  | 0.0% | -0.1% | -0.2% | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Working Draft Text

7.3.4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_param( xCtb, yCtb, cIdx ) |  |
| all_sao_merge_flags(xCtb, yCtb) |  |
| all_sao_type_idx(xCtb, yCtb) |  |
| all_sao_offset(xCtb, yCtb) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

7.3.4.1 Sample adaptive offset parameter syntax (remove the table)

7.3.4.1 Sample adaptive offset merge flags syntax

| sao_merge_flags( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_merge_left_flag[cIdx] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( ry > 0) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] && !sao_merge_left_flag[cIdx]) |  |
| sao_merge_up_flag[cIdx] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.4.2 Sample adaptive offset type index syntax

| all_sao_type_idx(rx, ry){ | Descriptor |
| --- | --- |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( !sao_merge_up_flag[cIdx] && !sao_merge_left_flag[cIdx] ) { |  |
| sao_type_idx_prefix[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if(sao_type_idx_prefix[ cIdx ][ rx ][ ry ] && !sao_merge_up_flag[cIdx] && !sao_merge_left_flag[cIdx] ) { |  |
| sao_type_idx_suffix[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |
| } |  |

7.3.4.3 Sample adaptive offset syntax

| all_sao_offset( rx, ry){ | Descriptor |
| --- | --- |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( !sao_merge_up_flag[cIdx] && !sao_merge_left_flag[cIdx] ) { |  |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if(SaoTypeIdx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if(SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.4.1 Sample adaptive offset parameter semantics

sao_merge_left_flag equal to 1 specifies that the syntax elements sao_type_idx_prefix, sao_type_idx_suffix, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the left coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding tree block. When sao_merge_left_flag is not present, it is inferred to be equal to 0.

sao_merge_up_flag equal to 1 specifies that the syntax elements sao_type_idx_prefix, sao_type_idx_suffix, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the above coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the above coding tree block. When sao_merge_up_flag is not present, it is inferred to be equal to 0.

sao_type_idx[ asdascIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7 9Error! Reference source not found. of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

–	If sao_merge_left_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry  ].

–	Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

–	Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to 0.

sao_type_idx_prefix[ cIdx ][ rx ][ ry ] indicates the prefix of the offset type of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_prefix[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_prefix[ cIdx ][ rx ][ ry − 1 ].

Otherwise, sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is set equal to 0.

sao_type_idx_suffix[ cIdx ][ rx ][ ry ] indicates the suffix of the offset type of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_suffix[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_suffix[ cIdx ][ rx ][ ry − 1 ].

Otherwise, sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is set equal to 0.

The offset type of current coding tree block at position rx and ry for the colour component cIdx SaoTypeIdx is derived as follows:

- If sao_type_idx_prefix is equal to 0, SaoTypeIdx is set equal to 0.

- Otherwise SaoTypeIdx is set equal to sao_type_idx_suffix plus 1.

Table 9 7 – Values of variable initValue for sao_type_idx_prefix ctxIdx

| Initialization value | Sao_type_idx_prefix ctxIdx |
| --- | --- |
|  | 0 |
| initValue | 61 |

Table 9 34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| sao_type_idx | 0 | U | 1 | Table 9-7 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 1 | Table 9-7 | 2 |
|  | 2 |  | 1 | Table 9-7 | 4 |
| sao_type_idx_prefix | 0 | FL, cMax = 1 | 0 | Table 9-7 | 0 |
|  | 1 |  | 0 | Table 9-7 | 1 |
|  | 2 |  | 0 | Table 9-7 | 2 |
| sao_type_idx_suffix | 0 | FL, cMax = 3 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |

Table 9 39 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| sao_type_idx | Table 9-7 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 0 | 1 | 1 | 1 | 1 |
|  |  | 4 | 0 | 1 | 1 | 1 | 1 |
| sao_type_idx_prefix | Table 9-7 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |

# Conclusion

## Context reduction for sao_merge_left_flag

Presented is an approach to simplify implementation by reducing the number of context models for sao_merge_left_flag syntax element. Results were presented using HM-7.0.

## Alternative “1+3” binarisation for_sao_type_idx

Presented is an approach to improve the throughput and simplify implementation by using an alternative binarisation of the sao_type_idx syntax element. Results were presented using HM-7.0.

## SAO syntax element stream layout

Presented is an approach to improve the throughput and simplify implementation by using an alternative SAO elements stream layout. Results were presented using HM-7.0.

# References

[1] B. Bross, et al., “High efficiency video coding (HEVC) text specification draft 7”, JCT-VC document JCTVC-I1003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012

# Patent rights declaration(s)

CiSRA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).