| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0179 |
| --- | --- |

| Title: | On sao_merge_left_flag for effective Mx1 CTB coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tomoyuki Yamamoto Yukinobu Yasugi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents a coding method of SAO to solve the low coding efficiency problem when CTB size is 32x32 and 16x16. The proposed method enlarges SAO unit size to reduce SAO parameters overhead. Specifically, the method codes M sequential CTB (Mx1 CTB) as one SAO unit. Because the height of SAO unit is still one CTB, it doesn’t require the picture level optimization and can be used in realtime encoding. The syntax and semantics of sao_left_merge_flag is changed so that, in the SAO unit, sao_left_merge_flag is not coded and inferred to be 1 (forces left merge). The parameter M, the horizontal size of the SAO unit, is codes in PPS. On common condition (CTB size is 64x64 and M is 2), it is reported that average BD-rate gain is 0.1 % to 0.3 %. When CTB size is 32x32 and M is 8, it is reported that that average BD-rate gain is 0.4 % to 1.2 %. Encoder only implementation is also tested. In encoder only case, no gain was observed when CTB size is 64x64 and 0.3 % to 0.8 % BD-rate gain was observed when CTB size is 32x32.

# Introduction

The SAO technique performs good coding gain when CTB size is 64x64. However, the coding gain significantly reduces when CTB size is 32x32 and coding loss is observed when CTB size is 16x16. This smaller CTB block problem has not been solved by the smaller number of offsets technique such as one offset per CTB. The basic reason is relatively large SAO parameter overhead. Therefore the straightforward technique is to enlarge SAO unit size so that it is larger than CTB. HM7 uses one syntax mode “LCU based syntax”, which has sao_left_merge_flag and sao_upper_merge_flag. Those flags can be used to enlarge SAO unit virtually. However, the merge flag’s overhead is large. To reduce this overhead, the inferring technique is needed. In this proposal, we apply only horizontal merge, sao_left_merge_flag, considering LCU based syntax *1 and real time encoding.

*1: If MxN unit is applied, which needed picture level optimization, APS based syntax is more appropriate. Now that we have only LCU based syntax, Mx1 is more appropriate.

# Proposal

In the proposal, the SAO unit size can be changed from one CTB to Mx1 CTB. The parameter M, the horizontal size of the SAO unit, is codes in PPS. This change is achieved with the syntax and semantics change of sao_left_merge_flag. The proposal is illustrated in Fig. 1. As shown, the SAO unit consists of sequential CTB, whose width and height is sao_unit_width and 1 respectively. In the first CTB in a SAO unit, sao_merge_left_flag is coded, while in the other CTBs inside the SAO unit, sao_merge_left_flag is not coded and inferred to equal to 1. The starting coordinate value of the SAO unit is aligned with sao_unit_width x CTB width.

Fig. 1 The proposed method (example of M = 4)

# WD text

The change of WD text is shown as follows.

.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| diff_cu_qp_delta_depth | ue(v) |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| dependent_slice_enabled_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if (sample_adaptive_offset_enabled_flag) { |  |
| sao_unit_width_minus1 | ue(v) |
| } |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_header_extension_present_flag | u(1) |
| slice_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

sao_unit_width_minus1 specifies the width of SAO unit size. In the first CTB in a SAO unit, sao_merge_left_flag is coded, while in the other CTBs inside the SAO unit, sao_merge_left_flag is not coded.

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| headOfSaoUnit = ((rx % (sao_unit_width_minus1 + 1) * CtbSize) == 0) |  |
| if( leftCtbInSlice && leftCtbInTile && headOfSaoUnit) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Fig. 1: Proposal change of WD

7.4.4.1	Sample adaptive offset parameter semantics

sao_merge_left_flag equal to 1 specifies that the syntax elements sao_type_idx, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the left coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding tree block. When sao_merge_left_flag is not present, it is inferred as follows. When headOfSaoUnit is not true and leftCtbInSlice is true and leftCtbInTile is true, it is inferred to be equal to 1, otherwise (headOfSaoUnit is true or leftCtbInSlice is true or leftCtbInTile is true), it is inferred to be equal to 0.

# Simulation results

The simulation results of the CTB size 64x64, 32x32 are summarized in Table 1. In the proposed method produces, the BDrate is -0.1 % to -0.3 % when CTB size is 64x64 and -0.4 % to -1.2 % when CTB size is 32x32. On the other hand the encoder only method, which is developed for comparison, doesn’t achieve gain at CTB 64x64 and lower gain at CTB 32x32. The difference of gain between proposal and encoder only is 0.4 % when CTB size is 32x32.

Table 1: Summary: Comparison of proposal and encoder only

|  |  | AIMA | RAMA | LBMA | AI10 | RA10 | LB10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64x64 CTB | proposal (M=2) | -0.1 | -0.2 | -0.1 | -0.1 | -0.2 | -0.3 |
|  | enconly (M=2) | 0.0 | 0.2 | 0.4 | 0.0 |  | -0.2 |
| 32x32 CTB | proposal (M=8) | -0.4 | -0.7 | -0.8 | -0.5 | -0.6 | -1.2 |
|  | enconly (M=8) | -0.3 | -0.3 | -0.4 | -0.4 |  | -0.8 |

The common condition’s results are shown Table 2 (--SAOUnitWidth=2) and 32x32 case’s results are shown Table 3 (--SAOUnitWidth=8, --MaxCUWidth=32, --MaxCUHeight=3, --MaxPartitionDepth=3).

Table 2: Results of CTB64 (M=2, 128x64) on common condition (compared to HM70)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.6% | -0.8% | -0.1% | -0.4% | -0.5% |
| Class B | -0.1% | -0.7% | -1.0% | -0.1% | -0.6% | -1.0% |
| Class C | -0.1% | -0.3% | -0.3% | -0.1% | -0.5% | -0.6% |
| Class D | -0.1% | -0.5% | -0.6% | -0.1% | -0.3% | -0.4% |
| Class E | -0.1% | -1.1% | -1.5% | -0.1% | -0.6% | -0.8% |
| Overall | -0.1% | -0.6% | -0.8% | -0.1% | -0.5% | -0.7% |
|  | -0.1% | -0.6% | -0.8% | -0.1% | -0.5% | -0.7% |
| Class F | -0.1% | -0.4% | -0.5% | -0.2% | -1.1% | -1.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -1.1% | -1.2% | -0.2% | -0.9% | -1.4% |
| Class B | -0.2% | -1.0% | -1.0% | -0.2% | -0.8% | -1.1% |
| Class C | -0.1% | -0.3% | -0.3% | -0.1% | -0.6% | -0.8% |
| Class D | -0.2% | -0.1% | -0.3% | -0.1% | -0.4% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.6% | -0.7% | -0.2% | -0.7% | -1.0% |
|  | -0.2% | -0.6% | -0.7% | -0.2% | -0.7% | -1.0% |
| Class F | -0.4% | -0.6% | -0.8% | -0.4% | -1.0% | -1.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -2.1% | -2.6% | -0.4% | -1.6% | -2.1% |
| Class C | 0.0% | -0.8% | -1.2% | -0.2% | -1.4% | -1.7% |
| Class D | -0.1% | -0.4% | -0.3% | -0.2% | -1.0% | -1.3% |
| Class E | -0.1% | -2.6% | -3.5% | -0.4% | -1.7% | -2.1% |
| Overall | -0.1% | -1.4% | -1.8% | -0.3% | -1.4% | -1.8% |
|  | -0.1% | -1.3% | -1.7% | -0.3% | -1.4% | -1.7% |
| Class F | -0.6% | -2.0% | -2.3% | -0.4% | -1.5% | -2.1% |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 3: Results of CTB32 (M=8, 256x32) (compared to HM70)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -2.4% | -2.5% | -0.5% | -0.7% | -1.3% |
| Class B | -0.4% | -2.3% | -3.2% | -0.4% | -0.9% | -1.6% |
| Class C | -0.3% | -2.0% | -2.4% | -0.4% | -1.0% | -1.5% |
| Class D | -0.3% | -1.6% | -2.1% | -0.4% | -0.4% | -0.6% |
| Class E | -0.6% | -2.8% | -3.3% | -0.7% | -0.7% | -0.7% |
| Overall | -0.4% | -2.2% | -2.7% | -0.5% | -0.7% | -1.2% |
|  | -0.4% | -2.1% | -2.6% | -0.5% | -0.7% | -1.2% |
| Class F | -0.3% | -2.9% | -3.2% | -0.8% | -2.6% | -3.5% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 107% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.5% | -3.3% | -3.2% | -0.9% | -1.7% | -2.3% |
| Class B | -0.7% | -3.4% | -3.6% | -0.8% | -1.6% | -2.2% |
| Class C | -0.2% | -1.9% | -2.4% | -0.4% | -1.3% | -1.7% |
| Class D | -0.2% | -1.1% | -1.4% | -0.1% | -0.9% | -1.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.7% | -2.5% | -2.7% | -0.6% | -1.4% | -1.9% |
|  | -0.7% | -2.4% | -2.7% | -0.6% | -1.4% | -1.8% |
| Class F | -1.2% | -3.0% | -3.9% | -1.3% | -3.0% | -4.4% |
| Enc Time[%] | 111% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.3% | -5.3% | -6.4% | -1.1% | -2.5% | -3.1% |
| Class C | -0.4% | -4.1% | -5.5% | -0.9% | -2.8% | -3.7% |
| Class D | -0.2% | -2.6% | -4.1% | -0.6% | -2.0% | -2.5% |
| Class E | -1.5% | -6.7% | -7.5% | -2.4% | -2.0% | -1.8% |
| Overall | -0.8% | -4.6% | -5.8% | -1.2% | -2.3% | -2.9% |
|  | -0.8% | -4.6% | -5.7% | -1.1% | -2.4% | -3.0% |
| Class F | -2.3% | -6.0% | -9.5% | -2.7% | -5.1% | -7.9% |
| Enc Time[%] | 109% |  |  | 99% |  |  |
| Dec Time[%] | 88% |  |  | 99% |  |  |

Note: Enc time and Dec time is not accurate due to cluster computing.

# Conclusion

This contribution presents a coding method of SAO. The proposed method code Mx1 CTB as one SAO unit to solve the low coding efficiency problem when CTB size is 32x32 and 16x16. The average BD-rate gain is 0.1 to 0.3 % at 64x64 CTB case and 0.4 to 1.2 % at 32x32 CTB case. It is recommended to adopt this method into HEVC draft 8 and HM8.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).