| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0180_r1 |
| --- | --- |

| Title: | Zero merge candidate simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Houqiang Li (USTC) Haitao Yang (Huawei) | Tel: Email: | +86 755 28976972 haitao.yang@huawei.com |
| Source: | University of Science and Technology of China Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents a modification on the construction of the zero merge candidates for B slices. Specifically, bi-directional zero merge candidates are replaced by uni-directional zero merge candidates in B slices. It is asserted that the complexity of motion compensation can be reduced with the proposed modification, while the BD-Rate change is reported to be 0.0% under common test condition.

# Introduction

This contribution represents a simplified design of zero merge candidates. In the simplified design, uni-directional zero merge candidates are always used for both P slices and B slices. With the simplified method, the complexity of motion compensation is reduced for B slices when zero merge candidate is selected. The impact on the Y BD-Rate is about 0.0% on average and the proposed algorithm has no significant impact on the encoding\decoding time.

# Solution

Uni-directional (list 0 prediction) zero merge candidate is always used for both P slices and B slices in the proposed algorithm. Detailed results are provided in Section 3 and the text modifications are provided in Section 5.

# Experimental results

The proposed two methods are implemented in HM-7.0. The common test conditions specified in [1] are followed.

The detailed results can be found in JCTVC-J0180_ZeroMrgSimp_r1.xlsm.

Table 1 Y BD-Rate of simplified zero merge candidate

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.0% |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  | -0.1% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc. Time | 100% | 100% | 100% | 100% |
| Dec. Time | 99% | 100% | 99% | 100% |

# Conclusion

When using the proposed simplified zero merge candidate, the complexity of motion compensation can be decreased when zero merge candidate is selected. The proposed simplification does not have significant impact on both the BD-Rate and the encoding/decoding time.

# Text Modification

The text modifications are based on JCTVC-I1003_d4 [2].

8.5.2.1.4 Derivation process for zero motion vector merging candidates

The variable numRefIdx is derived as follows.

- If slice_type is equal to P, numRefIdx is set to num_ref_idx_l0_active_minus1 + 1

- Otherwise (slice_type is equal to B), numRefIdx is set to min(num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1)

……

    - If (slice_type is equal to P), tThe candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-1)

refIdxL1zeroCandm = −1	(8-2)

predFlagL0zeroCandm = 1	(8-3)

predFlagL1zeroCandm = 0	(8-4)

mvL0zeroCandm[ 0 ] = 0	(8-5)

mvL0zeroCandm[ 1 ] = 0	(8-6)

mvL1zeroCandm[ 0 ] = 0	(8-7)

mvL1zeroCandm[ 1 ] = 0	(8-8)

numMergeCand = numMergeCand + 1	(8-9)

    - Otherwise (slice_type is equal to B), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-10)

refIdxL1zeroCandm = zeroIdx	(8-11)

predFlagL0zeroCandm = 1	(8-12)

predFlagL1zeroCandm = 1	(8-13)

mvL0zeroCandm[ 0 ] = 0	(8-14)

mvL0zeroCandm[ 1 ] = 0	(8-15)

mvL1zeroCandm[ 0 ] = 0	(8-16)

mvL1zeroCandm[ 1 ] = 0	(8-17)

numMergeCand = numMergeCand + 1	(8-18)

……

# References

- JCTVC-I1100, “Common HM test conditions and software reference configurations,” Geneva, CH, April – May 2011.

- JCTVC-I1003, “High efficiency video coding (HEVC) text specification draft 7,” Geneva, CH, April – May 2011.

# Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).