| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0181 |
| --- | --- |

| Title: | AHG6: On deblocking filter parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuo Lu Ohji Nakagami Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Shuo.Lu@jp.sony.com ohji.nakagami@jp.sony.com Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to increase the adaptive capability of deblocking filter by expanding the effective value range of variable β. Proposed solutions aim at expanding the value range for high QP to enable stronger filtering. It is reported that better visual quality can be achieved. The proposed solution does not change the core process of current deblocking filter and has no influence on common test conditions.

# Problem statement

In the 9th Geneva meeting, tc table in deblocking filter was modified by using higher clipping values in the part related to higher QPs. The motivation was to enable stronger filtering at higher QP. However, for some sequences, obvious blocky artifacts can still be observed. Sequence “riverbed” is shown as below (@LP-Main, QP37). With LoopFilterBetaOffset_div2 = 13 and LoopFilterTcOffset_div2 = 13, the maximum thresholds values, β = 64, tc = 24, is selected. Figure 1-2 shows all filtering applied. Green lines mark where weak filter are applied, and red lines mark where strong filters are applied. The other places are not filtered. It is observed that most of block noises lie on boundaries with no filter or weak filter. As example, the yellow part highlighted in Figure 1-2, Because of the limit of β, still weak filter is selected and visible blocky noises remain as shown in Figure 1-1.

Figure 1-1: HM7.0, LP-Main, QP37,

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13, Cropped from Frame 13th

Figure 1-2: filtering applied to riverbed, LP-Main, QP=37, Frame num=13

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13

# Solution

Two solutions are proposed to increase the amount of strong filter through expanding the value range of β.

## Proposal-1: Modification on β table

During deblocking filter process, the following conditions are checked to make filtering decision (Please refer to [1] for details and definitions of d/p0/d0/p3/q3).

(s1)Filtering on a line of samples only takes place when d is less than β.

Based on the result of (s1), the following conditions need to be checked further to determine the filtering type, strong filter or weak filter.

(s2)Strong filter will be chosen if the following conditions all hold:

d is less than ( β >> 2 ); | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 );

| p0 – q0 | is less than ( 5*tc + 1 ) >> 1

It can be observed that the value of β determines whether filtering is applied or not. To choose strong fitler, β and tc is working together to make the decision. As pointed out in JCTVC-I0071, strong filter can easily be selected with higher β. Therefore, for high QP, higher β should also be set.

Table 2-1 shows current values of β and tc in HM7.0. It is proposed to replace the β values with Table2-2. The changes are highlighted in red.

The value is determined by the principle that the changing rate of β should have similar trend with the changing rate of tc to reflect quantization change. The β-QP relation with proposed β is shown in Figure 2-1.

Since the changes lie at QP bigger than 41, there is no impact on common test conditions.

Table 2-1: threshold variables β and tc in HM7.0

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |  |  |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | - | - |  |  |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 22 | 24 |  |  |  |

Table 2-2: Proposed threshold variables

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |  |  |  |
| β | 38 | 40 | 42 | 44 | 52 | 60 | 68 | 76 | 84 | 92 | 100 | 116 | 132 | 148 |  |  |  |  |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 22 | 24 |  |  |  |

Figure 2-1: β-QP relation of HM7.0 and proposed new table

Figure 2-2: β/4 and tc in HM7.0

Figure 2-3: proposed β/4 and tc

## Proposal-2: Adaptive control on deblocking filter parameters

In HM7, β and tc is determined by the average quantization parameter QP of the adjacent two blocks and offset. The effect of the offset is to address bigger threshold values from higher QP than current one to increase the amount of filtering, while to address smaller threshold values from lower QP than current one to decrease the amount of filtering. But the problem is adjustment is limited only within the values linking to current QPs.

Instead of adjusting QP position, it is proposed to adjust the threshold values directly by adding the offsets to default thresholds, which was recommended in the last Geneva meeting.

### Fixed granularity

Regarding to capability, for high QP, larger value range of β and tc can be selected than current table-based way.

Regarding to flexibility, different adjusting-steps, beta_step_log2 and tc_step_log2, are suggested for different range of QP to achieve fine grain adjustment for low QP and coarse grain adjustment for high QP. QP range is divided based on the values of tc and β as shown in Table 2-2. And beta_step_log2 and tc_step_log2 are shown in Table 2-3 accordingly.

β = Max(0, β [Q] + (beta_offset_div2 << beta_step_log2))

, where β[Q] is β value specified as Table 2-1 with the input Q.

tc = Max(0, tc [Q+ 2*( bS − 1)] + (tc_offset_div2 << tc_step_log2))

, where tc[Q] is tc value specified as Table 2-1 with the input Q.

The available value range can be expanded to be [0, 168] for β, [0, 76] for tc,

Table 2-3: The beta_step_log2 and tc_step_log2

|  | Q∊[0,16) | Q∊[16,27) | Q∊[27,37) | Q∊[37,53] |
| --- | --- | --- | --- | --- |
| beta_step_log2 | 0 | 1 | 2 | 3 |
| tc_step_log2 | 0 | 0 | 1 | 2 |

Figure 2-4: fine grain adjustment for low QP and coarse grain adjustment for high QP

### Encoder selectable granularity

This method is basically same as Method3 in JCTVC-I0071. Fixed granularity adjusting can extend the thresholds’ range for high QP. However, for low QP，in order to keep precision, the value range is shorted with current offset values. Therefore, by adding syntax beta_step_log2 and tc_step_log2 to control the granularity, it enables encoder to select granularity for all QPs depending on video content. Relatively large beta_step_log2 and tc_step_log2 enable coarse grain adjustment to achieve big threshold values if stronger filter is desired, while small beta_step_log2 and tc_step_log2 enable fine grain adjustment if the video content is sensitive for filtering.

beta_step_log2 shall be in the range of 0 to 3, and tc_step_log2 shall be in the range of 0 to 2. The available value range can be expanded to be [0, 168] for β, [0, 76] for tc, which is same as 2.2.1, Set beta_step_log2 = 2, tc_step_log2 = 0 as default values.

β = Max(0, β [Q] + (beta_offset << beta_step_log2))

, where β[Q] is β value specified as Table 2-1 with the input Q.

tc = Max(0, tc [Q+ 2*( bS − 1)] + (tc_offset << tc_step_log2))

, where tc[Q] is tc value specified as Table 2-1 with the input Q.

Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| beta_offset | se(v) |
| tc_offset | se(v) |
| beta_step_log2 | ue(v) |
| tc_step_log2 | ue(v) |
| } |  |
| } |  |
| … |  |

beta_offset and tc_offset specify the default deblocking parameter offsets for β and tC (divided by 2beta_step_log2 and 2tc_step_log2) that are applied for pictures referring to the picture parameter set unless the default deblocking parameter offsets are overriden by the deblocking parameter offsets present in the slice header for pictures referring to the picture parameter set.

beta_step_log2 and tc_step_log2 specify the granularity for deblocking parameter offsets beta_offset and tc_offset that are applied for pictures referring to the picture parameter set unless the default deblocking parameter are overriden by the deblocking parameter present in the slice header for pictures referring to the picture parameter set. beta_step_log2 shall be in the range of 0 to 3, and tc_step_log2 shall be in the range of 0 to 2.

Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| beta_offset | se(v) |
| tc_offset | se(v) |
| beta_step_log2 | ue(v) |
| tc_step_log2 | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |

beta_offset and tc_offset specify the deblocking parameter offsets for β and tC (divided by 2beta_step_log2 and 2tc_step_log2) for the current slice.

beta_step_log2 and tc_step_log2 specify the granularity for deblocking parameter offsets beta_offset and tc_offset for the current slice. beta_step_log2 shall be in the range of 0 to 3, and tc_step_log2 shall be in the range of 0 to 2.

# Results

## Objective Results:

The changes of proposed table lie at QP bigger than 41, so there is no impact on common test conditions. Since for the high QPs part, the default value of β is modified, test results on QP 37/42/47/51 in Table 3-1show that there is up -0.1% and -0.7% for LP gain with the new default value, which means the new values have no negative impact on high QPs.

Table 3-1: BD-rate with new table for high QPs = 37, 42, 47, 51vs. HM7.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.4% | 0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 150% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.7% | -1.1% | -0.1% | 0.1% | 0.8% |
| Class B | -0.1% | -0.1% | 0.4% | 0.0% | 0.1% | 0.0% |
| Class C | -0.1% | 0.0% | 0.2% | -0.1% | 0.5% | 0.4% |
| Class D | -0.1% | 0.1% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Overall | -0.1% | -0.2% | -0.1% | 0.0% | 0.2% | 0.3% |
|  | -0.1% | -0.2% | -0.3% | 0.0% | 0.2% | 0.3% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 105% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.7% | 0.3% | -0.1% | -1.1% | -1.0% |
| Class C | -0.1% | 0.5% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | -3.1% |
| Class E | -0.2% | -0.1% | 0.2% | 0.0% | -1.2% | -0.9% |
| Class F | -0.1% | -1.6% | -0.6% | -0.1% | -0.9% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | -0.1% | -0.6% | -1.0% |
|  | 0.0% | -0.2% | -0.5% | -0.1% | -0.7% | -1.0% |
| Enc Time[%] | 103% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 109% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | 0.0% | 0.7% | -0.1% | 0.3% | 1.9% |
| Class C | -0.7% | 0.2% | 0.0% | -0.3% | -1.4% | 0.4% |
| Class D | -0.8% | -0.1% | -1.9% | -0.7% | -0.7% | 1.2% |
| Class E | -0.9% | 0.3% | -0.8% | -0.4% | -0.9% | -0.1% |
| Class F | -0.3% | 0.0% | -1.5% | -0.3% | -0.8% | 0.1% |
| Overall | -0.7% | 0.1% | -0.6% | -0.3% | -0.6% | 0.8% |
|  | -0.7% | 0.2% | -0.5% | -0.3% | -0.4% | 0.6% |
| Enc Time[%] | 104% |  |  | 98% |  |  |
| Dec Time[%] | 106% |  |  | 103% |  |  |

The results of sequence riverbed/DucksTakeOff/ChinaSpeed are generated as representative.

For subjective result, block artifacts in the areas where filtering applied changed from weak filter to strong filter are reduced as expected.

The objective results are shown in Table 3-1 to Table 3-3 with QP32/37/41 respectively. It shows that proposed method can improve visual quality while improve or keep the objective evaluations.

Table 3-1: objective coding result@LB-Main, QP32

| Sequence | QP | Condition | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Riverbed | 32 | HM7.0 offset13 | 5852.75 | 34.62 | 39.03 | 41.43 |
|  |  | Proposal-1 offset13 | 5850.69 | 34.57 | 39.03 | 41.44 |
|  |  | Proposal-2-FIX offset13 | 5850.13 | 34.61 | 39.03 | 41.43 |
| DucksTakeOff | 32 | HM7.0 offset13 | 18569.44 | 30.29 | 32.63 | 36.60 |
|  |  | Proposal-1 offset13 | 18686.73 | 30.12 | 32.63 | 36.59 |
|  |  | Proposal-2-FIX offset13 | 18593.41 | 30.24 | 32.63 | 36.60 |
| ChinaSpeed | 32 | HM7.0 offset13 | 1491.10 | 34.61 | 39.90 | 38.97 |
|  |  | Proposal-1 offset13 | 1556.41 | 33.94 | 39.93 | 38.97 |
|  |  | Proposal-2-FIX offset13 | 1509.63 | 34.41 | 39.87 | 39.02 |

Table 3-2: objective coding result@LB-Main, QP37

| Sequence | QP | Condition | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Riverbed | 37 | HM7.0 offset13 | 2851.39 | 31.71 | 38.13 | 40.90 |
|  |  | Proposal-1 offset13 | 2846.61 | 31.70 | 38.13 | 40.90 |
|  |  | Proposal-2-FIX offset13 | 2849.35 | 31.70 | 38.13 | 40.89 |
| DucksTakeOff | 37 | HM7.0 offset13 | 7590.24 | 27.72 | 31.56 | 35.97 |
|  |  | Proposal-1 offset13 | 7605.85 | 27.66 | 31.56 | 35.97 |
|  |  | Proposal-2-FIX offset13 | 7610.22 | 27.66 | 31.56 | 35.97 |
| ChinaSpeed | 37 | HM7.0 offset13 | 678.34 | 31.42 | 38.39 | 37.05 |
|  |  | Proposal-1 offset13 | 687.15 | 31.12 | 38.43 | 37.07 |
|  |  | Proposal-2-FIX offset13 | 702.31 | 30.91 | 38.42 | 36.98 |

Table 3-3: objective coding result@LB-Main, QP41

| Sequence | QP | Condition | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Riverbed | 41 | HM7.0 offset13 | 1555.01 | 29.54 | 37.78 | 40.63 |
|  |  | Proposal-1 offset13 | 1551.49 | 29.54 | 37.78 | 40.64 |
|  |  | Proposal-2-FIX offset13 | 1552.91 | 29.54 | 37.78 | 40.63 |
| DucksTakeOff | 41 | HM7.0 offset13 | 3713.40 | 25.98 | 30.94 | 35.67 |
|  |  | Proposal-1 offset13 | 3717.73 | 25.95 | 30.94 | 35.66 |
|  |  | Proposal-2-FIX offset13 | 3717.62 | 25.94 | 30.94 | 35.66 |
| ChinaSpeed | 41 | HM7.0 offset13 | 346.16 | 28.99 | 37.50 | 35.80 |
|  |  | Proposal-1 offset13 | 347.84 | 28.83 | 37.55 | 35.81 |
|  |  | Proposal-2-FIX offset13 | 351.63 | 28.67 | 37.52 | 35.75 |

## Subjective results of Proposal-1

Figure 3-1: Riverbed, HM7.0, LP-Main, QP37,

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13, Cropped from Frame 13th

Figure 3-2: Riverbed, Proposed-1, LP-Main, QP37,

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13, Cropped from Frame 13th

Figure 3-3: DucksTakeOff, HM7.0, LP-Main, QP37,

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13, Cropped from Frame 79th

Figure 3-4: DucksTakeOff, Proposed-1, LP-Main, QP37,

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13, Cropped from Frame 79th

## Subjective results of Proposal-2

Figure 3-5: Riverbed, Proposed-2, LP-Main, QP37,

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13, Cropped from Frame 13th

Figure 3-6: DucksTakeOff, Proposed-2, LP-Main, QP37,

LoopFilterBetaOffset_div2 = 13, LoopFilterTcOffset_div2 = 13, Cropped from Frame 79th

# Conclusion

The core motivation of proposed solution above is to expand value range of deblocking filter parameters where are not covered by current HM7.0. For the tested sequences ‘riverbed’ and ‘DucksTakeOff’ shown above, visible blocking noises can be reduced to satisfied extent. Simulation results show that comparing with HM7.0 the proposed methods have negligible influence on Bitrate/SNR.

# References

[1] “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003, April 2012

[2] “On deblocking filter parameter,” JCTVC-I0071, April 2012

[3] “On deblocking clipping table,” JCTVC-I0258, April 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: WD modification

Proposal1

Table 8-1 in the draft is replaced by Table 2-2 in Section2.

Proposal2

8.7.1.4.3 Decision process for luma block edge

……

The value of the variable β′ is determined as specified in Table 8-1 based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) )	(8-1)

where beta_offset_div2 is the value of the variable beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as:

β = β′*(1<<(BitDepthY − 8))	(8-2)

The value of the variable tC′ is determined as specified as Table 8-1 based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 53, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) )	(8-3)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))

The value of the variable β’ is derived as follow:

β' =  Max(0,β’ [qPL] + (beta_offset_div2 << beta_step))

where β’[qPL] is specified as Table 8-13 with qPL as input, and beta_offset_div2 is the value of the variable beta_offset_div2 for the slice that contains sample q0,0.

beta_step is determined as in Table xx below(for proposal 2.2.1)

or

beta_step is determined as beta_step_log2 (for proposal 2.2.2)

The variable β is derived as:

β = β’*(1<<(BitDepthY − 8))

The value of the variable tc’ is derived as follow:

tc' =  Max(0,tc’ [qPL + 2*( bS − 1)] + (tc_offset_div2 << tc_step))

where tc’[qPL] is specified as Table 8-13 with qPL as input and tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

tc_step is determined as in Table xx below(for proposal 2.2.1).

or

tc_stepis determined as tc_step_log2 (for proposal 2.2.2).

The variable tC is derived as:

tC = tC’ *(1<<(BitDepthY − 8))

Table xx: The beta_step and tc_step (for proposal 2.2.1)

|  | Q∊[0,16) | Q∊[16,27) | Q∊[27,37) | Q∊[37,53] |
| --- | --- | --- | --- | --- |
| beta_step | 0 | 1 | 2 | 3 |
| tc_step | 0 | 0 | 1 | 2 |

……

8.7.1.4.5  Filtering process for chroma block edge

……

The value of the variable tC′ is determined as specified as Table 8-1 based on the chroma quantization parameter Q derived as:

Q = Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2 << 1 ) )	(8-4)

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(8-5)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

The value of the variable tc’ is derived as follow:

tc' =  Max(0, tc’ [QPC + 2*( bS − 1)] + (tc_offset_div2 << tc_step))

where tc’[QPC] is specified as Table 8-13 with QPC as input and tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

tc_step is determined as in Table xx of 8.7.1.4.3(for proposal 2.2.1).

or

tc_step is determined as in tc_step_log2 (for proposal 2.2.2).

The variable tC is derived as:

tC = tC’*(1<<(BitDepthY − 8))

……