| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0182 |
| --- | --- |

| Title: | AHG 9 / AHG 13: On identical reference picture lists |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Yongjoon Jeon Seungwook Park Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In current HEVC specification, information related to reference picture list 0 (L0) and reference picture list 1 (L1) is signalled independently. Such signalling design can be made more efficient given the fact that the content of L0 and L1 are often identical. This contribution proposes to add a flag in slice header to indicate whether or not the content of L0 and L1 are identical. Having such indication would be helpful as it can avoid the necessity to signal all syntax elements related to L1 such as the elements listed in the following:

- num_ref_idx_l1_active_minus1

- ref_pic_list_modification_flag_l1

- list-entry_l1

- collocated_from_10_flag

Furthermore, identical reference list indication can be helpful to avoid constructing L1 as decoder can simply copies its contents from L0.

# Introduction and Problem Statement

In current HEVC specification, information related to reference picture list 0 (L0) and reference picture list 1 (L1) is signalled independently. Such signalling design can be made more efficient given the fact that the content of L0 and L1 are often identical. For example, in common test condition [1], for low delay B and for every 8 pictures, the contents of L0 and L1 are identical.

In current slice header signalling design, there is no indication whether the content of L0 and L1 are identical. Having such indication would be helpful as it can avoid the necessity to signal all syntax elements related to L1 such as the elements listed in the following:

- num_ref_idx_l1_active_minus1

- ref_pic_list_modification_flag_l1

- list-entry_l1

- collocated_from_10_flag

Furthermore, identical reference list indication can be helpful to avoid constructing L1 as decoder can simply copies its contents from L0.

Given the usefulness of having a flag to indicate identical reference list, we propose to add such flag in flag in slice header. The syntax and semantics of the propose flag are given in the following section.

# Proposed identical_lists_flag

The following tables show the modification after the introduction of the proposed identical reference list flag.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type = = B ) |  |
| identical_lists_flag | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| pic_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| if( !identical_lists_flag ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| … |  |
| if( pic_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| if(!identical_lists_flag ) |  |
| collocated_from_l0_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

identical_lists_flag equal to 1 indicates that the reference picture list 0 and reference picture list 1 are identical. Otherwise, the reference picture list 0 and reference picture list 1 are not identical.

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| } |  |
| if( slice_type = = B ) { |  |
| if(!identical_lists_flag ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

In addition to the modification to syntax tables, the reference picture construction process should also be updated as the following:

In section 8.3.4.2

…

- When the slice is a B slice,

If identical_lists_flag is set, then

- The variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(8-1)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
}

The list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx ≤ num_ref_idx_l1_active_minus1; rIdx++)			(8-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

Otherwise,

NumRpsCurrTempList1 = NumRpsCurrTempList0

Copy RefPicListTemp1 from RefPicListTemp0

Copy RefPicList1 from RefPicList0

…

# References

[1] F. Bossen, "Common test conditions and software reference configurations," JCTVC-I1100, 9th meeting: Geneva, CH, 27 April – 7 May, 2012.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).