| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0184 |
| --- | --- |

| Title: | On Transform Skip |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami, Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan Toshiyasu Sugio Hisao Sasai 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 50 3750 2741 Yoshitaka.Morigami@jp.sony.com Kazushi.Sato@jp.sony.com +81-50-3587-2047 { sugio.toshiyasu, sasai.hisao }@jp.panasonic.com |
| Source: | Sony Corp. Panasonic Corporation |  |  |

_____________________________

# Abstract

In the 9th JCTVC meeting Transform Skipping for Intra TUs has been proposed by JCTVC-I0408. It improves coding efficiency with synthesized images like the ones in Class F test sets.

This contribution claims 2 modifications as follow:

- Transmission of transform_skip_enable_flag in PPS or Slice Header rather than in SPS

- Applying flat quantization matrices for TransformSkip TUs

# Transmission of transform_skip_enable_flag in PPS or Slice Header rather than in SPS

## Problem Statement

Currently transform_skip_enable_flag can be specified in SPS (sequence parameter set) as shown in Table 1.1.1. However there may be cases that a user would like to apply TS only for part of pictures in a sequence.

For example, assume that a sequence contains both programme scenes and commercial scenes, and the content of the programme is cartoon while commercial is camera-captured video. In this case the user would like to apply TU only with the former parts of the sequence.

If the mode decision of the encoder is RDO basis, TS TUs are properly applied only when it provides best in coding efficiency among candidate modes. However encoders in the real-world (especially that works real-time) cannot afford RDO-based mode decision and only simple mode decision like low-complexity mode in JM are implemented. In such case having TS TU as a candidate mode may cause loss in coding efficiency.

Table 1.1.1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| sps_max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_temporal_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| } |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| nsrqt_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Solution

To solve the problem stated above, it is recommended to move transform_skip_enabled_flag from SPS either to PPS (Solution 1) or SliceHeader (Solution 2) as shown in the next section.

## Proposed Change in CD text

Modifications for Proposal 1 and 2 are shown in Table 1.3.1 and Table 1.3.2, respectively. The semantics remains unchanged.

Table 1.3.1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software ] |  |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| diff_cu_qp_delta_depth | ue(v) |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| dependent_slice_enabled_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_header_extension_present_flag | u(1) |
| slice_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 1.3.2

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | u(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag[ 0 ] | u(1) |
| if( slice_sample_adaptive_offset_flag[ 0 ] ) { |  |
| slice_sample_adaptive_offset_flag[ 1 ] | u(1) |
| slice_sample_adaptive_offset_flag[ 2 ] | u(1) |
| } |  |
| } |  |
| if(adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| pic_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( pic_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Applying flat quantization matrices for TransformSkip TUs

## Problem Statement

It is proposed by JCTVC-I0408 that flat quantization matrices be applied for Transform-Skip TUs. With Non-Transform-Skip TUs output data are in frequency domain, while with Transform-Skip TUs it is in spatial domain. Therefore applying a flat matrix for Transform-Skip TUs is reasonable and the authors support this idea.

However it has not been adopted for 2 reasons:

Reason 1: Switching quantization matrices in the unit of 4x4 requires increase in complexity

Reason 2: Transform Skipping is effective for ClassF-type sequences. It may be OK that flat quantization matrices be set for such sequences as an encoder issue.

So, in current HEVC specification, quantization matrices, which are sets of weighting factors in frequency domain, are applied also with Transform-Skip TUs, whose data is in spatial domain. It is theoretically funny and should be fixed.

There exists the sequences that contain both natural image areas and synthesized image areas (caption) like F: BasketballDrillText. For such sequences former portions should be coded with Non-Transform-Skip TUs while latter be with Transform-Skip TUs. Users would like to use non-flat quantization matrices only with former areas, but current HEVC specification does not allow this.

## Solution

The meaning of “flat quantization matrices” in JCTVC-I0408 is somewhat unclear. There might be 3 possible solutions as follow:

Solution 1: Does not apply quantization matrices to Transform-Skip TUs

Solution 2: Quantization factors (not matrices!) for Transform-Skip TUs are additionally transmitted in the scaling_list syntax

Solution 3: DC component of the quantization matrix are applied to all components in Transform-Skip TUs

It is recommended that one of the solutions above be adopted. Solution 1 is simple, and Solution 3 provides more flexibility.If solution 2 is adopted, not Table 1.3.2 but Table 1.3.1 should be adopted, as the scaling_list needs to refer to transform_skip_flag.

On Reason 1 stated in the previous sub-seciton, switching of quantization matrix may occur if intra and inter TUs are concatenated. It means that current HEVC specification already includes switching of QM by 8x8-level with luma and 4x4-level with chroma.

With solution 3, switching of QM loading by 4x4-level can be avoided: QM[4,4] are always loaded, regardless of TS- or Non-TS TUs. For non-TS TUs, each component qm[i,j] is applied for each of the TU element t[i,j], whereas for TS TUs, DC component qm[0,0] is applied for all of the TU elements t[i,j], thus switching of loading quantization matrix can be avoided. Only modification exists in de-quantization process.

On Reason 2 stated in the previous sub-section, there might happen that a picture contains both camera-captured areas as well as synthesized areas like Class F: BasketballDrillText. For the former areas non-TS TUs are more likely to be selected and users would like to apply QMs for quality control. On the contrary, TS TUs are more likely to be applied with the latter areas and for such areas users would like to apply flat QMs. Under current HEVC specification there does not exists such degree of freedom.

## Proposed Change in CD text

Changes associated with each of the solutions stated in the previous sub-section are as follow:

Solution 1:

8.6.3	Scaling process for transform coefficients

...

The scaling factor mij is derived as follows.

–	If scaling_list_enable_flag is equal to 0 or transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1,

mij = 16									(8-245)

–	Otherwise (scaling_list_enable_flag is equal to 1 and transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0 ),

mij = ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ]		(8 246)

...

Solution 2:

| scaling_list( scalingList, sizeID , matrixID ) { | Descriptor |
| --- | --- |
| nextCoef = 8 | u(1) |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| if( sizeID > 1 ) { |  |
| scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ sizeID – 2 ][ matrixID ] + 8 |  |
| } |  |
| for( i=0; i < coefNum; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| scalingList[ i ] = nextCoef |  |
| } |  |
| if( sizeID == 0&&matrixID < 3&&transform_skip_enabled_flag ) { |  |
| scaling_list_transform_skip_minus8[ matrixID ] | se(v) |
| nextCoef = scaling_list_transform_skip_minus8[ matrixID ] + 8 |  |
| scalingList[ coefNum ] = nextCoef |  |
| } |  |
| } |  |

scaling_list_transform_skip_minus8[ matrixID ] plus 8 specifies the value of the scaling list for transform unit coded with TransformSkip mode. It shall be applied for all components of TransformSkip TU.

8.6.3	Scaling process for transform coefficients

...

The scaling factor mij is derived as follows.

-	If scaling_list_enable_flag is equal to 0,

mij = 16										(8-245)

–	Else if scaling_list_enable_flag is equal to 1 and transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0,

mij = ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ]			(8-246)

-	Otherwise (scaling_list_enable_flag is equal to 1 and transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1),

mij = ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ 16 ]			(8-247)

...

Solution 3:

8.6.3	Scaling process for transform coefficients

...

The scaling factor mij is derived as follows.

-	If scaling_list_enable_flag is equal to 0,

mij = 16									(8-245)

–	Else if scaling_list_enable_flag is equal to 1 and transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0,

mij = ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ]		(8-246)

-	Otherwise (scaling_list_enable_flag is equal to 1 and transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1),

mij = ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ 0 ]			(8-247)

...

# Conclusion

This contribution claims 2 modifications on Transform Skipping as follow:

-	Transmission of transform_skip_enable_flag in PPS or Slice Header rather than in SPS

-	Applying flat quantization matrices for TransformSkip TUs

It is recommended that the first modification be adopted into HEVC DIS. With regard to the second modification there are three possible solutions, and it is recommended to adopt one of them into HEVC DIS.

Sony and Panasonic are willing to volunteer implementing the suggested modifications on HM-8.0.

# References

[1] “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003, April 2012

[2] “Intra Transform Skipping”, JCTVC-I0408, April 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).