| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0187r1 |
| --- | --- |

| Title: | On temporal_mvp_enable_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information / Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In SPS and slice header syntax element “temporal_mvp_enable_flag” is contained. This document provides result of comparison between temporal_mvp_enable_flag =1 and =0. In addition we propose to separate this flag into temporal_mvp_enable_flag_l0 and temporal_mvp_enable_flag_l1 to provide more degree of freedom for trade-off between coding efficiency and complexity.

# Introduction

In HEVC both spatial and temporal motion vector prediction can be applied for each of the Prediction Unit. Motion information of spatially surrounding PUs is stored in line buffer, but the one of the PUs in the reference frames are stored in memory. So extracting tmvp requires more complexity than smvp. On the other hand, using both tmvp and smvp provides better coding efficiency than just smvp only.

For trade-off between coding efficiency and complexity, syntax element “temporal_mvp_enable_flag” is contained in SPS and slice header.

This document provides result of comparison between temporal_mvp_enable_flag =1 and =0. In addition we propose to separate this flag into temporal_mvp_enable_flag_l0 and temporal_mvp_enable_flag_l1 to provide more degree of freedom for trade-off between coding efficiency and complexity.

# Simulation

## Simulation Condition

Two experiments have been conducted in this contribution as follow:

Experiment -1: Anchor is HM-7.0, Tested is HM-7.0 with TMVP=0

Experiment -2: Anchor is HM-7.0, Tested is TMVP_L0=1 and TMVP_L1=0

For experiment -2, HM-7.0 is modified so that TMVP can be separately defined with List0 and List1.

RA_Main, RA_HE10, LB_Main, LB_HE10 conditions are tested with Class A to F sequences.

Please note that there is an inconsistency between HM-7.0 and CD text: in HM-7.0 temporal_mvp_enable_flag is transmitted in SPS but according to CD text it should be transmitted in SliceHeader. In HM-7.1 this inconsistency has been resolved.

However, in common test condtions one picture contains only one slice so difference of overhead on temporal_mvp_enable_flag information between HM-7.0 and -7.1 can be ignored.

## Simulation Result

Summary of the result with experiment -1 is shown in Table 2.2.1. Complete result is shown in JCTVC-J0187_1.xls.

Summary of the result with experiment -1 is shown in Table 2.2.2. Complete result is shown in JCTVC-J0187_2.xls.

The author would like to thank ETRI for crosschecking. (JCTVC-J0361)

Table 2.2.1

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 2.6% | 2.1% | 2.4% | 2.7% | 2.1% | 2.4% |
| Class B | 2.4% | 1.6% | 1.7% | 2.3% | 1.5% | 1.5% |
| Class C | 2.4% | 2.2% | 2.4% | 2.4% | 2.2% | 2.2% |
| Class D | 2.8% | 2.2% | 2.4% | 2.8% | 2.2% | 2.1% |
| Class E |  |  |  |  |  |  |
| Overall | 2.5% | 2.0% | 2.2% | 2.5% | 2.0% | 2.0% |
|  | 2.5% | 2.0% | 2.2% | 2.5% | 2.0% | 2.0% |
| Class F | 1.0% | 1.2% | 1.1% | 1.1% | 1.2% | 1.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.7% | 1.7% | 1.6% | 2.5% | 1.6% | 1.7% |
| Class C | 2.8% | 2.6% | 2.9% | 2.8% | 2.8% | 2.9% |
| Class D | 2.5% | 1.8% | 2.5% | 2.5% | 2.2% | 2.3% |
| Class E | 2.2% | 2.1% | 1.8% | 2.4% | 2.2% | 2.4% |
| Overall | 2.6% | 2.0% | 2.2% | 2.6% | 2.2% | 2.3% |
|  | 2.6% | 2.0% | 2.2% | 2.5% | 2.2% | 2.3% |
| Class F | 2.3% | 2.4% | 2.6% | 1.9% | 2.0% | 3.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |

Table 2.2.2

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.6% | 1.2% | 1.5% | 1.6% | 1.3% | 1.4% |
| Class B | 1.8% | 1.2% | 1.2% | 1.7% | 1.2% | 1.1% |
| Class C | 1.7% | 1.6% | 1.7% | 1.7% | 1.6% | 1.6% |
| Class D | 2.0% | 1.6% | 1.6% | 2.1% | 1.6% | 1.6% |
| Class E |  |  |  |  |  |  |
| Overall | 1.8% | 1.4% | 1.5% | 1.8% | 1.4% | 1.4% |
|  | 1.8% | 1.4% | 1.5% | 1.8% | 1.4% | 1.4% |
| Class F | 0.7% | 0.8% | 0.8% | 0.6% | 0.7% | 0.8% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | 0.1% | 0.3% | 0.3% | 0.2% | 0.3% |
| Class C | 0.3% | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% |
| Class D | 0.2% | -0.1% | 0.7% | 0.2% | 0.6% | -0.1% |
| Class E | 0.2% | 0.1% | -0.3% | 0.4% | 0.1% | 0.1% |
| Overall | 0.3% | 0.1% | 0.3% | 0.3% | 0.3% | 0.2% |
|  | 0.3% | 0.1% | 0.3% | 0.3% | 0.3% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% | 1.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

As can be seen disabling of TMVP causes loss in coding efficiency by 2.5%, 2.5%, 2.6% and 2.6% with RA_Main, RA_HE10, LB_Main, and LB_HE10 conditions respectively. Just by disabling TMVP_L1, loss becomes 1.8%, 1.8%, 0.3% and 0.3% with each of the conditions.

If, for example, GOP structure is traditional m=3, setting TMVP as shown in Fig 2.2.1 would provide a good trade-off between coding efficiency and complexity.

Fig 2.2.1

# Proposed Change in CD text

Suggested modifications on SPS syntax are shown in Table 3.1.

Table 3.1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| sps_max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_temporal_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| } |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| nsrqt_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| sps_temporal_mvp_enable_flag_L0 | u(1) |
| sps_temporal_mvp_enable_flag_L1 | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Suggested modifications on the semantics associated with Table 3.1 are as follows:

sps_temporal_mvp_enable_flag_L0 equal to 1 specifies that pic_temporal_mvp_enable_flag_L0 may be present in the slice headers and temporal motion vector predictors may be used for list 0 motion vector prediction. sps_temporal_mvp_enable_flag_L0 equal to 0 specifies that pic_temporal_mvp_enable_flag_L0 shall not be present in the slice headers and temporal motion vector predictors shall not be used for list 0 motion vector prediction.

sps_temporal_mvp_enable_flag_L1 equal to 1 specifies that pic_temporal_mvp_enable_flag_L1 may be present in the slice headers and temporal motion vector predictors may be used for list 1 motion vector prediction. sps_temporal_mvp_enable_flag_L1 equal to 0 specifies that pic_temporal_mvp_enable_flag_L1 shall not be present in the slice headers and temporal motion vector predictors shall not be used for list 1 motion vector prediction.

Suggested modifications on SlideHeader syntax are shown in Table 3.2.

Table 3.2

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | u(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag[ 0 ] | u(1) |
| if( slice_sample_adaptive_offset_flag[ 0 ] ) { |  |
| slice_sample_adaptive_offset_flag[ 1 ] | u(1) |
| slice_sample_adaptive_offset_flag[ 2 ] | u(1) |
| } |  |
| } |  |
| if(adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag_L0 ) |  |
| pic_temporal_mvp_enable_flag_L0 | u(1) |
| if(slice_type = = B && sps_temporal_mvp_enable_flag_L1 ) |  |
| pic_temporal_mvp_enable_flag_L1 | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( pic_temporal_mvp_enable_flag_L0 ¦¦ pic_temporal_mvp_enable_flag_L1 ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

Suggested modifications on the semantics associated with Table 3.2 are as follows:

pic_temporal_mvp_enable_flag_L0 specifies whether temporal motion vector predictors can be used for list 0 inter prediction. If pic_temporal_mvp_enable_flag_L0 is equal to 0, the temporal motion vector predictors shall not be used in decoding of the current picture for list 0 prediction. If pic_temporal_mvp_enable_flag_L0 is equal to 1, temporal motion vector predictors may be used in decoding of the current picture for list 0 prediction. When not present, the value of pic_temporal_mvp_enable_flag_L0 shall be inferred to be equal to 0.

pic_temporal_mvp_enable_flag_L1 specifies whether temporal motion vector predictors can be used for list 1 inter prediction. If pic_temporal_mvp_enable_flag_L1 is equal to 0, the temporal motion vector predictors shall not be used in decoding of the current picture for list 1 prediction. If pic_temporal_mvp_enable_flag_L1 is equal to 1, temporal motion vector predictors may be used in decoding of the current picture for list 1 prediction. When not present, the value of pic_temporal_mvp_enable_flag_L1 shall be inferred to be equal to 0.

When both pic_temporal_mvp_enable_flag_L0 and temporal_id are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vector vectors for list 0 prediction from any picture that precedes the current picture in decoding order.

When both pic_temporal_mvp_enable_flag_L1 and temporal_id are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vector vectors for list 1 prediction from any picture that precedes the current picture in decoding order.

# Conclusion

In SPS and slice header syntax element “temporal_mvp_enable_flag” is contained. Disabling of TMVP causes loss in coding efficiency by 2.5%, 2.5%, 2.6% and 2.6% with RA_Main, RA_HE10, LB_Main, and LB_HE10 conditions respectively. Just by disabling TMVP_L1, loss becomes 1.8%, 1.8%, 0.3% and 0.3% with each of the conditions. It is proposed to split temporal_mvp_enable_flag into temporal_mvp_enable_flag_L0 and temporal_mvp_enable_flag_L1, for better trade-off between coding efficiency and complexity. It is recommended that this modification be adopted into HEVC DIS.

# References

[1] “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003, Apr 2012

[2] “Common HM test conditions and software reference configurations”, JCTVC-I1100, Apr 2012

[3] “Cross-check report of JCTVC-J0187 on temporal_mvp_enable_flag”, JCTVC-J0361, Jul 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).