| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0189 |
| --- | --- |

| Title: | Cross check report of IDSQ (J0076) proposed by BBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

The contribution provide a cross verification report of intensity dependant spatial quantisation (IDSQ). IDSQ modulates QP values in a TU according to average pixel intensity of each TU. IDSQ is precisely implemented according to the document and the coding results are identical to the proponent's results.

# Introduction

This document is cross verification document of J0076 [1]. The proposal is the QP control method by using masking phenomena. The Human Vision System is less sensitive to the distortion in the brighter and darker area. The base idea is to choose coarser QP at such areas because it is expected to be hard to perceive distortion. The proposed IDSQ method modulates the base QP depending on the average pixel intensity of each TU. This proposal applies new profile data and modified the implementation from I0257. We checked the implementation according to the documents and confirmed the experimental results.

# Implementation

The source code, of which IDSQ to be integrated into HM7.0, was provided from the proponents. IDSQ needs to compute the average pixel intensity of each TU by using predicted pixels. The process is implemented in TComTrQuant::computeIQPOffset. The implementation is clear and matches the document.

# Experimental results

All experiments were performed on Linux 64 bit OS with gcc-4.1.2 with the same computer systems. Default conditions were according to common test condition [2]. The experimental results were identical to the results provided by the proponents.

# Observation

This proposal is modified from previous proposal in the previous Geneva meeting [3]. In the last meeting, the latency for the parallel implementation was pointed out. The current implementation is improved and the latency is removed.

This method requires the default ID profiles for each QP value. Therefore, the decoder needs the additional ROM for storing ID profiles. However, these ID profiles are just only defined at QP values, 22, 27, 32, 37. The rest of ID profiles have not been defined. To define the profiles requires the additional subjective experiments.

The ID profiles are designed as shown in the following figures. The lower and higher average pixel intensities of TU are quantized by coarser QPs independent from sequences. Especially, the range of modulation is larger in darker side. Therefore the sequences likes "ChinaSpeed" and "SteamLocomotive" whose average pixel intensities are around/under 50 are degraded perceptively. Theoretically, such distortion is appeared in such sequences. IDSQ needs more flexibility for changing ID profile according to input signals.

Figure 1. Proposed ID profiles (copied from [1])

This proposal aims to reflect percept characteristics depend on the average pixel intensity to the QP control. This role is basically the same as the rate control of TM5 step3 which manages quantisation parameters depending on the activity of each macro block.

# Reference

[1] M. Naccari, M. Mrak, D. Flynn and A. Gabriellini, " Improving HEVC compression efficiency by intensity dependant spatial quantisation", JCTVC-J0076, 10th meeting, Stockholm, SE, 11–20 July 2012.

[2] F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, 9th meeting, Geneva, CH, April-May 2012.

[3] M. Naccari, M. Mrak, D. Flynn and A. Gabriellini, “On Intensity Dependent Quantization in the HEVC codec”, JCTVC-I0257, 9th meeting, Geneva, CH, April-May 2012.

# Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.