| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0191 |
| --- | --- |

| Title: | Extension of HM7 to Support Additional Chroma Formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Silcock, K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 paul.silcock@eu.sony.com karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Limited |  |  |

_____________________________

# Abstract

In this proposal, a model based upon HM7.0 that provides support for 4:2:2, 4:4:4 and 4:0:0 chroma formats is presented. In this model, 4:2:0 coding is also supported and output files for 4:2:0 are reported to match those provided by HM7.0 for the 8 standard test configurations and other non-standard configurations, with encoding/decoding times similar to HM7.0. The changes that have been made to extend HM7.0 to support 4:2:2/4:4:4 are described.

# Introduction

Chroma sampling formats other than 4:2:0 are currently not supported in the standard HEVC test model (HM7.0). However, it is understood that a model will eventually be needed to include other commonly used chroma formats, such as 4:2:2 and 4:4:4.

This contribution aims to support discussions on how to support 4:2:2 and 4:4:4 formats in HEVC. Interest in 4:2:2 and 4:4:4 support for HEVC has already been expressed in several previous contributions, such as JCTVC-I0521 and JCTVC-I0336, and therefore a committee-approved software model that supports non-4:2:0 formats would be beneficial for future development of compression tools.

The process of extending the tools in the HEVC model to include the additional formats will require cross-verification from independent contributors, and as the model presented here supports the additional 4:2:2 and 4:4:4 (and 4:0:0) formats, it might be useful for such a cross-verification purpose.

The model presented here also mirrors the behaviour of the HM7.0 encoder and decoder for the 4:2:0 chroma format. It is observed that there are no differences in the output files/bit-rates/SNRs/MD5 checksums for the standard test conditions, and that coding times are relatively unchanged.

# Extended Chroma Format Support

In this section the changes to the tools of HM7.0 to support 4:2:2/4:4:4 are presented. Most of these changes are configurable, and are presented here to highlight where the current HM7.0 model differs between the luma and chroma channels for the 4:2:0 processing.

| Tool / Feature | Support |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | 4:0:0 | 4:2:0 | 4:2:2 | 4:4:4 |  |
| Compatibility with HM7.0. | N/A | Yes | N/A | N/A | Standard configurations tested. |
| Input and output file formats. | Yes | Yes | Yes | Yes | New file formats for 4:0:0, 4:2:2 and 4:4:4. |
| Slices/CU/PU structures. | Yes | Yes | Yes | Yes | Slices, CU and PU structures are chroma-format independent. |
| TU structures. | Yes | Yes | Yes | Yes | Chroma TU structure for 4:4:4 may be configured to be the same as luma, or may follow the same splits as 4:2:0. Chroma TU structure for 4:2:2 for non-NSQT structures has twice the number of samples vertically than horizontally. |
| NSQT. | Yes | Yes | Yes | Yes | New shapes may be used for chroma in 4:2:2 and 4:4:4. |
| Intra-prediction angular prediction. | Yes | Yes | Yes | Yes | The 4:2:2 algorithm has been modified to account for the change in sample aspect ratio for the chroma channels. |
| Intra reference sample filtering. | Yes | Yes | Yes | Yes | Options are also available to configure filtering on the chroma channel, applicable to 4:2:2 and 4:4:4. |
| TU edge filtering for H, V and DC modes. | Yes | Yes | Yes | Yes | Options are also available to configure filtering on the chroma channel, applicable to 4:2:2 and 4:4:4. |
| LM_Chroma. | N/A | Yes | Yes | Yes | The samples used for 4:4:4 may optionally be from the neighbouring column. |
| MDDT. | Yes | Yes | Yes | Yes | MDDT may be optionally applied to chroma 4x4 TUs for 4:4:4. |
| Motion interpolation filters. | Yes | Yes | Yes | Yes | May use either the luma or a sub-sampled version of the chroma filter to achieve a ¼ sub-pel accuracy prediction for the chroma channel. |
| Quantizer. | Yes | Yes | Yes | Yes | Quantizer may modify QP for 4:2:2 where the chroma TUs sample width:height ratios of 1:2, 2:1 or 1:8 would otherwise cause incorrect coefficient magnitudes. |
| Scaling lists. | Yes | Yes | Yes | Yes | For 4:2:2, a larger scaling list is used and sub-sampled. The 32x32 chroma scaling list is undefined, and currently uses the same as luma. |
| Chroma QP clipping table. | N/A | Yes | Yes | Yes | Unchanged from HM7.0, but there is the option to use a different clipping table for 4:2:2 and 4:4:4. |
| Chroma CBF Context Variable (CV) derivation. | N/A | Yes | Yes | Yes | Unchanged from HM7.0, but the CBF derivation for chroma may optionally use a non-depth based selection criteria. |
| Chroma CV sets and rules. | N/A | Yes | Yes | Yes | Unchanged from HM7.0, but the rules for chroma CV derivation may be optionally forced to be the same as luma. The CVs for Cb and Cr may be separated, or just a single set of CVs for Y, Cb and Cr may be used. |
| MDCS. | Yes | Yes | Yes | Yes | MDCS is only used for luma in HM7.0, but it may be optionally applied to chroma. |
| Significance map coding. | Yes | Yes | Yes | Yes | The significance map given in JCTVC-I0521 may be used, instead of the default neighbour-based algorithm, for the 4x8, 8x4, 8x16 and 16x8 4:2:2 chroma TUs. |
| Deblocking filter. | Yes | Yes | Yes | Yes | No significant changes required. |
| ALF. | Yes | Yes | Yes | Yes | No significant changes required. |
| SAO. | Yes | Yes | Yes | Yes | No significant changes required. |

## General Software Modification

### Input/Output File Formats

The input format of a YUV sequence file is set by the per-sequence configuration files, using the parameter ‘InputChromaFormat’, which can be assigned the integer values of ‘400’, ‘420’ (default), ‘422’ or ‘444’.

The chroma format used for the compression process and output files is determined by the syntax element chroma_format_idc, which can be set at the command line with “cf XXX” or “ChromaFormatIDC=XXX”, where ‘XXX’ is one of the four values listed above, or ‘0’ (default). If the value is 0, the output chroma format is the same as the input chroma format; if the output chroma format is different to that of the input chroma format, the software will sample-repeat or sub-sample as required.

Until JCT-VC-approved 4:2:2 and 4:4:4 source material is agreed, the sample-repeat mechanism could be used to help groups compare results using the current 4:2:0 source material, although of course noting that the behaviour of sample repeated or sub-sampled data has its limitations.

## TU Structure and NSQT

The rules for the slice, CU and PU structures are unmodified relative to HM7.0, but some small adjustments to the TU structures have been made for 4:2:2 and 4:4:4.

The same splitting that is possible for 4:2:0 is used for 4:2:2, noting that in the current implementation the TUs for 4:2:2 have twice the number of samples vertically than their 4:2:0 counterparts.

There are exceptions for 4:2:2 NSQT because some TU configurations are possible that would enable the chroma TUs to align better to PU boundaries. For example, for 4:2:0, a 16x4 luma NSQT TU has a corresponding 4x4 chroma TU, although these do not cover the same image region; but, for 4:2:2, the same luma TU has a corresponding 8x4 chroma TU, and these do cover the same image region.

For 4:4:4, the chroma is allowed to split into the same sized units as luma. For example, for 4:2:0, an 8x8 luma TU can be split into four 4x4 luma TUs, but just one pair of chroma TUs; but for 4:4:4, the same four luma TUs would have four pairs of chroma TUs. This additional splitting may overly weight the chroma data since chroma would then have the same number of DC values as luma, and therefore the model has the ability to stop 4:4:4 chroma splitting at the same level as the corresponding 4:2:0 chroma. In addition, the HM7.0 rule that a CU using an NxN PU structure only has one chroma PU can be enforced for 4:4:4 (or disregarded for 4:2:2/4:2:0), even if the CU can have a multi-level chroma TU tree.

The control for the splitting rules to diverge from those in the standard HM7.0 4:2:0 is configurable, and in the proposed model, all of the splitting and recursive aspects of the TU structure have been aggregated into a new TComTU class.

The various TU sizes available for the standard HM configuration are illustrated in the following diagram, which also shows where the new options for 4:2:2 diverge from those for 4:2:0.

Figure 1 - The CU/PU/TU choices for the standard configuration of 4:2:0 HM7.0 and also the optional new shapes for 4:2:2 chroma formats.

## Intra Prediction

### Angular prediction algorithm

The intra prediction angular algorithm for 4:4:4 is the same as for the current HM7.0 implementation of 4:2:0. For 4:2:2, the proposed algorithm is as follows:

- The angle step and its inverse (currently referred to in the HM7.0 code as ‘intraPredAngle’ and ‘invAngle’) are derived according to the same process used in HM7.0, as shown in Figure 2.

  - As in HM7.0, for modes 18-34, the angle step intraPredAngle, when multiplied by the current sample’s y-coordinate gives an offset (relative to the current x-coordinate, in units of 1/32 of a sample) to the reference sample to be used. The calculation being:

    - refX=(y+1)*intraPredAngle/32+x

    - For directions 18 to 25, reference samples to the left of the top-left reference sample are required, and are interpolated from the left-most column of reference samples using the invAngle.

  - For modes 2-17, the transpose of the above algorithm is applied.

- For a vertical intra mode (18-34 inclusive), the derived angle step is halved and its inverse is doubled.

- Otherwise, for a horizontal intra mode (2-17 inclusive), the derived angle step is doubled and its inverse is halved.

This modification to the angle step is required in order that the projection from the current sample to be predicted points to a valid reference sample, as illustrated in Figure 3.

Figure 2 - Left: Intra prediction modes. Right: Derived angle step.

Figure 3 - Example of intra prediction algorithm modification for 4:2:2 chroma.

### Reference Sample Filtering

In HM7.0, luma intra reference samples may be subject to a low-pass filter, depending on a mapping between the current intra prediction mode and the current TU width, as illustrated in Figure 4.

In the proposed model the use of reference sample filtering for chroma 4:4:4 can be enabled, subject to the same conditions as for luma.

For 4:2:2, should the optional reference sample filter be enabled, the same mapping between TU width and intra mode is used to determine the use of the filter. Furthermore, the low-pass filter can be optionally applied only to the reference samples in the (non-sub-sampled) vertical column for 4:2:2.

Figure 4 - The use of reference sample filter for luma in HM7.0.

### TU edge filtering for H/V/DC modes

For Horizontal, Vertical and DC prediction modes in 4:2:0, luma TU edges are subject to filtering. For 4:4:4 chroma, the model presented may also apply the same filtering process, and for 4:2:2 chroma, the filter may be applied just in the vertical direction, or in both directions, or never.

### LM_Chroma Reference Sample Location for 4:4:4

LM_Chroma is a chroma only intra prediction mode, in which an array of chroma prediction samples Pc is derived as aPL + b, where coefficients a and b are derived by both encoder and decoder, and PL is the corresponding array of luma samples.

Previously coded luma and chroma reference samples are used to derive coefficients a and b. Given that 4:2:0 chroma is sub-sampled with respect to luma, the column of luma reference samples used in the derivation of {a, b} actually originate from the column two samples to the left of the current TU, rather than to the immediate left, since only the column two samples to the left has valid co-sited chroma samples. This is illustrated in Figure 5.

For 4:4:4, co-sited chroma samples are in the same position as luma samples (because chroma has not been sub-sampled). Hence the proposed model has the option to choose luma reference samples from the column to the immediate left of the current TU.

Figure 5 - Derivation of reference samples in 4:2:0 and 4:2:2 intra prediction.

## Mode-Dependent Directional Transform (MDDT) for 4:4:4 Chroma

In HM7.0, MDDT is only applied to 4x4 luma TUs, where the use of a DCT for each stage of the transformation process (i.e. row and/or column transform) may be replaced with a Discrete Sine Transform (DST), depending upon the intra prediction mode of the corresponding PU.

In the proposed model, MDDT may be used for 4x4 chroma TUs for 4:4:4 with the same intra mode mapping table as luma.

## Motion Interpolation Filters

Currently in HM7.0, for 4:2:0, separate interpolation filters are used for luma and chroma in order to generate inter predictions at sub-pixel accuracy. The luma channel is subject to 1/4-sample interpolation; the chroma channels undergo 1/8-sample interpolation, as demonstrated in Figure 6.

Figure 6 - Luma/chroma interpolation for 4:2:0.

For 4:2:2, in order that an interpolated region of chroma data be of the same spatial area as its luma counterpart, 1/8-sample interpolation is required horizontally while 1/4-sample interpolation is required vertically (as illustrated in Figure 7). Likewise for 4:4:4, 1/4-sample interpolation is required in both horizontal and vertical directions.

Figure 7 - Luma/chroma interpolation for 4:2:2.

In the presented model, for 4:2:2/4:4:4 formats, the chroma 1/4-sample interpolation process may use the existing “x8 4-tap” chroma filter defined in HM7.0. Note, however, that because only 1/4-sample accuracy is required, only sub-pixels at even fractional positions are calculated, i.e. the chroma filter is used to derive sub-pixels located at 2/8, 4/8 and 6/8 positions. As an alternative, the model may be configured to use the existing “x4 8-tap” luma filter for 1/4-sample chroma interpolation.

## QP modification for rectangular TUs

Section 2.6.1 is provided as background for the current HM7.0 transformation and quantization process for 4:2:0 TUs, whilst 2.6.2 shows the proposed changes for the new TU dimensions encountered in 4:2:2 processing.

### Processing of 4:2:0 TUs

HM7.0 transforms and quantizes square TUs and also NSQT TUs that have a width:height ratio of 1:4 or 4:1. The number of samples contained within a TU is always an integer power of 4.

As a summary of the processes in HM7.0, the Integer Cosine Transform (ICT) coefficient matrix Y of an NxN Transform Unit (TU) X is evaluated using the following equation:

Y=AXAT

where the elements of the transform matrices A are created from those of a true normalized NxN DCT A’ using:

Aij=int64×N×A'ij

Ignoring the differences due to rounding of Aij, and extending the problem to non-square matrices, then since X is multiplied by both A and AT, the resulting coefficients differ from those of a true normalized MxN (M=height; N=width) DCT by a factor of:

64×N64×M=4096NM

This scaling factor is equivalent to a shift operation, since in HM7.0 this results in a power of 2:

transformShift=12+0.5log2N+0.5log2M

To reduce the requirement on internal bit-precision, the coefficients are right-shifted (using positive rounding) twice during the transform process:

shift1=log2(N)+bitDepth−9shift2=log2(M)+6

As a result, the coefficients as they leave the forward transform process and enter the quantizer are effectively left-shifted by:

resultingShift=12+0.5log2NM−shift1+shift212+0.5log2N+0.5log2M−log2N+bitDepth−9+log2M+615−0.5log2NM+bitDepth

During the quantization process, the quantizer removes resultingShift from the coefficients by applying a right shift ‘quantTransformRightShift’ equal to resultingShift to the coefficients. This is only possible because resultingShift is an integer.

### Processing of 4:2:2 TUs

In the proposed system, the TUs for 4:2:2 chroma are not square, and nor do they have a width:height ratio of 1:4 or 4:1; instead they have a ratio of 1:2, 2:1 or even 1:8. Under these circumstances, the value of resultingShift in the above equation is not an integer, and therefore the shift of quantTransformRightShift cannot be implemented.

resultingShift will always have a fractional part of 0.5 for TUs with ratios of 1:2, 2:1 or 1:8. Therefore, quantTransformRightShift can be adjusted by either:

quantTransformRightShift=15−0.5log22NM+bitDepth		(Eq 1)

noting that the resulting quantized coefficients are a factor of √2 larger than they should be.

or

quantTransformRightShift=15−0.5log20.5NM+bitDepth		(Eq 2)

noting that the resulting quantized coefficients are a factor of √2 smaller than they should be.

These equations for quantTransformRightShift will result in integers, and therefore can be used in the bit-wise shift operation in the quantizer. However, the quantized values are not the correct magnitude and should be adjusted.

Fortunately, the HM7.0 quantizer uses a logarithmic QP parameter, whereby increasing QP by 6 doubles the quantization divisor and increasing QP by 3 multiplies the divisor by √2. Therefore, to account for the factor of √2 in the coefficients, the QP for the TU can be adjusted by 3 – either locally increasing the QP by 3 for the first case (Equation 1), or decreasing the QP by 3 for the second case (Equation 2).

In HM7.0, the QP is split into two parts: the power-of-2 part of the QP (i.e. QP/6), which is used in the quantizer to apply a bit-wise shift, and a fractional-power-of-2 part of the QP (i.e. QP%6) which is used to index a 6 element table for a multiplier to apply to the coefficients.

The QP adjustment could be applied either prior to the splitting of the QP into its two parts, in which case the 6 element table can be left unaltered. Alternatively, the fractional QP can be adjusted to index a 9 element array, such as in:

| Index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inverseQStep | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 | 12945 | 11523 | 10280 |
| scaledQStep | 40 | 45 | 51 | 57 | 64 | 72 | 81 | 91 | 102 |

Table 1 - An extended QStep table for QP+3; used in conjunction with Equation 1.

or:

| Index | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inverseQStep | 36158 | 32768 | 29127 | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| scaledQStep | 29 | 32 | 36 | 40 | 45 | 51 | 57 | 64 | 72 |

Table 2 - An extended QStep table for QP-3; used in conjunction with Equation 2.

The model presented supports the four combinations described (i.e. QP+/-3 and with/without extended tables), but conclusions regarding the preferred method cannot be drawn without agreed test material.

### Chroma Scaling Lists for 4:2:2/4:4:4

For 4:2:2 support, new chroma TU sizes have been added, but scaling lists for these are not defined. Currently the proposed model will subsample one of the available scaling lists used in 4:2:0 HM7.0 for a chroma TU. For example, an 8x16 chroma 4:2:2 TU will use values corresponding to every other column of a 16x16 chroma 4:2:0 scaling list. The only exception is for 16x32 chroma 4:2:2 TUs, which use scaling list entries corresponding to every other column of a 32x32 luma TU, since 32x32 scaling lists are not defined for chroma 4:2:0 in HM7.0.

For 4:4:4, an NxM chroma TU will use the same NxM scaling list used in 4:2:0 HM7.0 for chroma, with the only exception being for 32x32 chroma 4:4:4 TUs, which use the 32x32 scaling lists defined for luma (given that 32x32 scaling lists are not defined for chroma 4:2:0).

### Chroma QP Clipping Tables

In HM7.0, the QPs for the Cb and Cr channels are derived by using the luma QP, plus an offset for the corresponding chroma channel, as an index to the following table:

Table 3 - Chroma QP Derivation for 4:2:0.

In the proposed model, for 4:2:2, the following mapping table may be used instead:

Table 4 - Chroma QP Derivation for 4:2:2.

In the proposed model, for 4:4:4, the derived chroma QP may be the same as the index value (i.e. luma QP + Cb/Cr QP offset), subject to the usual clipping.

This area has been explored in other proposals (such as JCTVC-I0265), but although they are not currently included in the 4:2:0 profile, they could provide alternatives to the above mechanism.

## Entropy Coding

### Context Variable Selection

In general, the selection of context variables for entropy coding has not changed from HM7.0, however, some changes are required to support 4:2:2 and 4:4:4, as described below.

#### Context Variable Sets

In HM7.0, each syntax element (e.g. significance map, coded block flag) is typically encoded using one set of context variables for luma and another for chroma (both Cb and Cr). In most cases, coded block flag being the exception, the luma context set is larger than the chroma context set. A larger context variable set allows for more context-specific adaptation.

For 4:2:2 and 4:4:4, the proportion of information contained in the chroma channels increases. Hence the chroma context set for each syntax element could be increased to be the same size as the equivalent luma context set and/or split out into separate sets for Cb and Cr. The syntax elements for which such changes could apply are detailed in the following sections.

#### Significance Map Coding

For square TUs, the method used to select context variables is the same across all chroma formats. As in HM7.0, square TUs of size 8x8 or smaller use a fixed positional mapping, whereas larger TUs select a mapping for each coefficient group based on the number of neighbouring coefficient groups that contain non-zero coefficients, as illustrated in Figure 8.

Figure 8 - Significance map context variable mappings for coefficient groups based on neighbouring coefficient groups.

For non-square TUs, the neighbourhood-based method is used at all sizes. In the case of 4x8 TUs (that may use scan orders other than diagonal), mappings have been defined in the proposed model for non-square coefficient groups. An illustration of 4x8 TUs that use 2x8 coefficient groups (due to vertical scanning) is shown in Figure 9.

Figure 9 - Extension of neighbourhood context variable selection method to support vertically-scanned 4x8 TUs.

For 4x8, 8x4, 8x16 and 16x8 TUs, the above method in the proposed model can optionally be replaced with a vertically or horizontally extended (by position-repeat) version of the fixed positional mapping, as proposed in JCTVC-I0521 for HM6.0 and illustrated for HM7.0 in Figure 10.

Figure 10 - Fixed positional context variable mapping for 4x4 and 8x8 in HM7.0 and extension to 4x8 and 8x16 for 4:2:2.

#### Last Significant Coefficient Position Coding

No changes are included in this area to support the new chroma formats, however, the context variables used can be separated, as described in section 2.7.1.1.

#### Greater-than-1 Map and Greater-than-2 Map

No changes are included in this area to support the new chroma formats, however, the context variables used can be separated, as described in section 2.7.1.1.

#### Coded Block Flag

In HM7.0, coded block flags (CBFs) are used to indicate the presence of non-zero coefficients in the quad-tree TU structure. These flags are handled differently for luma and chroma.

In HM7.0, for luma, a CBF is coded only for the “leaf” TUs at the lowest level of the quad-tree. The fact that the tree is split down to a level is indicative of the presence of non-zero coefficients at that level. The CBF is needed only to designate which of the four leaf TUs contains the non-zero coefficients.

In HM7.0, for chroma, a CBF is coded at every level of the quad-tree; the aim being to signify at which level of the residual quad-tree (RQT) a chroma channel has no non-zero coefficients, and to avoid having to code chroma CBFs for the leaf TUs.

In the proposed model, the same context variable selection method that is used for luma CBFs can be used for chroma CBFs.

### Mode-Dependent Coefficient Scanning (MDCS)

In the proposed model, MDCS may be enabled for 4:2:2 and 4:4:4, with the mapping of scan directions to intra prediction modes remaining the same as in HM7.0. The maximum TU sizes for which MDCS is applied are as follows:

Table 5 - MDCS maximum TU size for all chroma formats.

Figure 11 - MDCS intra prediction mode-to-scan-order mapping.

In addition, for chroma, MDCS may optionally be disabled, restricted to 4x4 TUs only or limited to using only horizontal or only vertical scan.

# Results

## Benchmarks

### 4:2:0 format

### 4:2:2 format

Since no official 4:2:2 test material has been agreed upon, the proposed model has been tested using sequences provided by the EBU. However, the feasibility of suitable source material is currently being investigated.

Crowd Run	Flowers	Girl Flower 1	Girl Flower 2	Graphics

Horse	Kids Soccer	Rain Roses	Vegi Candle	Vegies

Waterfall	Water Rocks	Water Rocks Close

Figure 12 – First frames of EBU test material for 4:2:2. All are 1920x1080 50P, 500 frames.

### 4:4:4 format

The proposed model has been tested using limited-rights native 4:4:4 material.

# Conclusion

A model has been presented that provides support for 4:2:2, 4:4:4 and 4:0:0 chroma formats, and also matches the encoder/decoder behaviour of HM7.0 for 4:2:0, with the aim of continuing discussion on HEVC support for 4:2:2/4:4:4. The model is available to HEVC contributors who are also interested in 4:2:2/4:4:4 formats and who may wish to cross-examine their own implementations of 4:2:2/4:4:4 compatible models.

Furthermore, areas of divergence between luma and chroma for 4:2:0 in HM7.0 have been identified, and some methods of dealing with such divergence for 4:2:2 and 4:4:4 have been included.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).