| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0194r1 |
| --- | --- |

| Title: | AHG5: Max exponential golomb code for reducing number of bins |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Tokumichi Murakami Information Technology R&D Center 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes modifications to the CABAC binarization for ref_idx, cu_qp_delta and sao_offset. In HM-7.0, truncated unary binarization is used for these syntax, which introduces 15, 32 and 32 context coded bins in the worst case. To reduce the throughput of the CABAC, this contribution proposes using kth order Max Exp-Golomb coding. Max Exp-Golomb coding is similar to Exp-Golomb coding, but the longest code is one bit shorter than Exp-Golomb code by truncating the code using the maximum possible index.

The proposed method was implemented in HM-7.0, and it is reported that there is negligible coding efficiency impact with reducing the number of context coded bins per syntax element in the worst case down to 4, 5 and 5.

It is reported that bit rate increase in terms of BD-rate is 0.0% for ref_idx and 0.0-0.1% for sao_offset in CTC. The reported bit rate increase for cu_qp_delta is 0.2% in AHG5 test condition.

It is also proposed to apply 1st order Max Exp-Golomb code for mvd binarization. By signaling motion search range, maximum bins for mvd is also reduced.

# Problem Statement

At the last Geneva meeting, It was pointed out that ref_idx, cu_qp_delta and sao_offset can introduce prohibitively large number of context coded bins due to the use of Truncated unary binarization for these parameters. As CABAC can high potentially become a bottle neck of the video codec, and hardware is designed considering the worst case senario, it is important to reduce the number of coded bins in the worst case senario.

# Proposal

## Max Exp-Golomb Code

This contribution proposes using kth order Max Exp-Golomb coding. Only the bins of the prefix are context coded, while the bins of the suffix are bypass coded. This is similar to how coeff_abs_level_minus1 was coded in H.264/AVC and how motion vector difference (mvd) is currently coded in HEVC.

Exp-Golomb coding consists of prefix part and suffix part, and the prefix part uses unary code. On the other hand, Max Exp-Golomb coding uses truncated unary code for the prefix part by introducing a parameter indicating the maximum length of the prefix. Table 1 shows an example of kth order Max Exp-Golomb code.

When Max prefix = 0, one bit flag is applied instead of using Exp-Golomb code.

Table 1. An example of kth order Max Exp-Golomb code

| Value | 0th order Exp-Golomb | 0th order Max Exp-Golomb (Max prefix = 3) | 0th order Max Exp-Golomb (Max prefix = 4) | 1st order Max Exp-Golomb (Max prefix = 3) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 0 |
| 1 | 10 0 | 10 0 | 10 0 | 0 1 |
| 2 | 10 1 | 10 1 | 10 1 | 10 00 |
| 3 | 110 00 | 110 00 | 110 00 | 10 01 |
| 4 | 110 01 | 110 01 | 110 01 | 10 10 |
| 5 | 110 10 | 110 10 | 110 10 | 10 11 |
| 6 | 110 11 | 110 11 | 110 11 | 110 000 |
| 7 | 1110 000 | 111 000 | 1110 000 | 110 001 |
| 8 | 1110 001 | 111 001 | 1110 001 | 110 010 |
| 9 | 1110 010 | 111 010 | 1110 010 | 110 011 |
| 10 | 1110 011 | 111 011 | 1110 011 | 110 100 |
| 11 | 1110 100 | 111 100 | 1110 100 | 110 101 |
| 12 | 1110 101 | 111 101 | 1110 101 | 110 110 |
| 13 | 1110 110 | 111 110 | 1110 110 | 110 111 |
| 14 | 1110 111 | 111 111 | 1110 111 | 111 0000 |
| 15 | 11110 0000 |  | 1111 0000 | 111 0001 |
| 16 | 11110 0001 |  | 1111 0001 | 111 0010 |
| 17 | 11110 0010 |  | 1111 0010 | 111 0011 |
| 18 | 11110 0011 |  | 1111 0011 | 111 0100 |
| 19 | 11110 0100 |  | 1111 0100 | 111 0101 |
| 20 | 11110 0101 |  | 1111 0101 | 111 0110 |
| 21 | 11110 0110 |  | 1111 0110 | 111 0111 |
| 22 | 11110 0111 |  | 1111 0111 | 111 1000 |
| 23 | 11110 1000 |  | 1111 1000 | 111 1001 |
| 24 | 11110 1001 |  | 1111 1001 | 111 1010 |
| 25 | 11110 1010 |  | 1111 1010 | 111 1011 |
| 26 | 11110 1011 |  | 1111 1011 | 111 1100 |
| 27 | 11110 1100 |  | 1111 1100 | 111 1101 |
| 28 | 11110 1101 |  | 1111 1101 | 111 1110 |
| 29 | 11110 1110 |  | 1111 1110 | 111 1111 |
| 30 | 11110 1111 |  | 1111 1111 |  |
| 31 | 111110 00000 |  |  |  |
| : | : |  |  |  |
| : | : |  |  |  |

For Max Exp-Golomb code with max prefix, the number of context coded bins is shorter by 1 compared with Exp-Golomb code.

## Proposed binarization

### ref_index

The first bin indicates if ref_index is 0 or not. This bin is coded using ctxIdxOffset = 0.

The rest bins indicate ref_index - 1. 0th order Max Exp-Golomb code is used for the binarization with the Max prefix value as follows:

When num_ref_idx_lx_active_minus1 is 1, Max prefix = 0.

When num_ref_idx_lx_active_minus1 is 2 or 3, Max prefix = 1.

When num_ref_idx_lx_active_minus1 is 4 to 7, Max prefix = 2.

When num_ref_idx_lx_active_minus1 is larger than 8, Max prefix = 3.

Prefix part is coded using ctxIdxOffset = 1, and suffix part is coded using bypass mode.

Binarization of the proposed method with Max prefix = 1

| Value of ref_index | Bin string |  |  |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 1 | 1 | 0 |  |
| 2 | 1 | 1 | 0 |
| 3 | 1 | 1 | 1 |
| binIdx | 0 | 1 | 2 |
| ctxIdxOffset | 0 | 1 | bypass |

Binarization of the proposed method with bit depth = 2

| Value of ref_index | Bin string |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |
| 2 | 1 | 1 | 0 | 0 |  |  |  |
| 3 | 1 | 1 | 0 | 1 |  |  |  |
| 4 | 1 | 1 | 1 | 0 | 0 |  |  |
| 5 | 1 | 1 | 1 | 0 | 1 |  |  |
| 6 | 1 | 1 | 1 | 1 | 0 |  |  |
| 7 | 1 | 1 | 1 | 1 | 1 |  |  |
| binIdx | 0 | 1 | 2 | 3 | 4 |  |  |
| ctxIdxOffset | 0 | 1 | 1 | bypass |  |  |  |

Binarization of the proposed method with bit depth = 3

| Value of sao_offset | Bin string |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |
| 2 | 1 | 1 | 0 | 0 |  |  |  |
| 3 | 1 | 1 | 0 | 1 |  |  |  |
| 4 | 1 | 1 | 1 | 0 | 0 | 0 |  |
| 5 | 1 | 1 | 1 | 0 | 0 | 1 |  |
| 6 | 1 | 1 | 1 | 0 | 1 | 0 |  |
| 7 | 1 | 1 | 1 | 0 | 1 | 1 |  |
| 8 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 9 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 10 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 11 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 12 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |
| 13 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 14 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| binIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| ctxIdxOffset | 0 | 1 | 1 | 1 or bypass | bypass |  |  |

### cu_qp_delta

The first bin indicates if cu_qp_delta is 0 or not. This bin is coded using ctxIdxOffset = 0.

The second bin indicates the sign of cu_qp_delta using bypass code.

The rest indicates the absolute value of cu_qp_delta minus 1 using 0th order Max Exp-Golomb code.

Prefix part is coded using ctxIdxOffset = 1, and suffix part is coded using bypass mode.

When QpBdOffsetY is smaller than 13 (internal bit depth is less than 11bits), Max prefix = 4, otherwise Max prefix = 5.

### sao_offset

The first bin indicates if sao_offset is 0 or not. This bin is coded using ctxIdxOffset = 0.

The rest indicates the absolute value of sao_offset minus 1 using 0th order Max Exp-Golomb code.

Max prefix = 3 + bit increment.

Prefix part is coded using ctxIdxOffset = 1, and suffix part is coded using bypass mode.

Sign bin is separately coded using bypass mode same as HM7.

Binarization of the proposed method with bit depth = 8

| Value of sao_offset | Bin string |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |
| 2 | 1 | 1 | 0 | 0 |  |  |  |
| 3 | 1 | 1 | 0 | 1 |  |  |  |
| 4 | 1 | 1 | 1 | 0 | 0 |  |  |
| 5 | 1 | 1 | 1 | 0 | 1 |  |  |
| 6 | 1 | 1 | 1 | 1 | 0 |  |  |
| 7 | 1 | 1 | 1 | 1 | 1 |  |  |
| binIdx | 0 | 1 | 2 | 3 | 4 |  |  |
| ctxIdxOffset | 0 | 1 | 1 | bypass |  |  |  |

Binarization of the proposed method with bit depth = 10

| Value of sao_offset | Bin string |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |
| 3 | 1 | 1 | 0 | 1 |  |  |  |  |  |  |  |
| 4 | 1 | 1 | 1 | 0 | 0 | 0 |  |  |  |  |  |
| 5 | 1 | 1 | 1 | 0 | 0 | 1 |  |  |  |  |  |
| 6 | 1 | 1 | 1 | 0 | 1 | 0 |  |  |  |  |  |
| 7 | 1 | 1 | 1 | 0 | 1 | 1 |  |  |  |  |  |
| 8 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |  |  |  |
| 9 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| 10 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |  |  |  |
| : | : | : | : | : | : | : | : | : |  |  |  |
| 30 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |
| 31 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| binIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |
| ctxIdxOffset | 0 | 1 | 1 | 1 or bypass |  | bypass |  |  |  |  |  |

### abs_mvd_minus2

The relationship between motion search range and possible maximum abs_mvd_minus2 are shown below:

| Search range | Max abs_mvd_minus2 |
| --- | --- |
| -8.0 to 7.75 | 61 |
| -16.0 to 15.75 | 125 |
| -32.0 to 31.75 | 253 |
| -64.0 to 63.75 | 509 |
| -128.0 to 127.75 | 1021 |

Maximum index of 1st order Max Exp-Golomb code and Max prefix value are also shown below:

| Max prefix | Max index |
| --- | --- |
| 4 | 61 |
| 5 | 125 |
| 6 | 253 |
| 7 | 509 |
| 8 | 1021 |

According to the above two tables, number of bits to represent integer value of the motion search range plus 1 can be Max prefix for its 1st order Max Exp-Golomb code.

If encoder knows motion search range, it can be signalled in SPS so that Max prefix can be shared with decoder. When encoder does not know motion search range, large enough value for search range can be signalled. In this case, 1st order Max Exp-Golomb code becomes identical to 1st order Exp-Golomb code.

By applying Max Exp-Golomb code to ref_index, cu_qp_delta, sao_offset and abs_mvd_minus2, the normal Exp-Golomb code would not be necessary to be implemented.

### Worst case comparison

Max context bins, bypass bins and overall bins per syntax element of HM-7.0 and the proposed method are shown below.

# Simulation

The HM-7.0 software is used for the evaluation, the simulation platforms is Intel(R) Xeon(R) with 64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

Table 2: Coding efficiency results

|  | main |  |  |  | HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | LP | AI | RA | LB | LP |
| ref_idx | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| cu_qp_delta | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| sao_offset | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |

# Conclusion

Applying Max Exp-Golomb code for CABAC binarization of ref_idx, cu_qp_delta and sao_offset is proposed. The proposed method was implemented in HM-7.0, and it is reported that there is negligible coding efficiency impact with reducing the number of context coded bins per syntax element in the worst case down to 3, 4 and 4.

It is also proposed to apply Max Exp-Golomb code for abs_mvd_minus2 binarization with signalling motion search range at SPS.

We propose adopting the proposed method into HM-8 and the DIS.

# Reference

[1] “Draft requirements for the scalable enhancement of HEVC”, ISO/IEC JTC1/SC29/WG11/N12783, May 2012.

[2] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7”, JCTVC-I1003, May 2012.

# Patent rights declaration(s)

Mitsubishi Electric  Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Changes to the CD text

Add the following section to 9.3.2

#### k-th order Max Exp-Golomb (MEGk) binarization process

Input to this process is a request for an MEGk binarization for a syntax element and a maximum prefix length value maxPrefixLength.

Output of this process is the MEGk binarization of the syntax element.

The bin string of the MEGk binarization process MEGk( synVal, maxPrefixLength) of a syntax element synVal with a maximum prefix length maxPrefixLength is specified by a process equivalent to the following pseudo-code:

absV = Abs( synVal )
stopLoop = 0
prefixLength = 0
do {
	// prefix part
	if( absV  >=  ( 1 << k ) && prefixLength <= MaxPrefixLength) {
		put( 1 )
		absV = absV − ( 1 << k )
		k++
	} else {

if (prefixLength != MaxPrefixLength)
		  put( 0 ) 																				(9-1)
	// suffix part
		while( k− − ) 
			put( ( absV >> k )  &  1 )
		stopLoop = 1	}
} while( !stopLoop )

For the bins in suffix part, DecodeBypass process is applied.

・cu_qp_delta

The following portion of Table 9-4

"

| coding_unit() | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |

"

is replaced by

"

| coding_unit() | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..31 | 4..7 2..3 | 8..11 4..5 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |

"

The following Table

"

Table 9-1 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 |

"

is replaced by

"

Table 9-2 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 119 | 119 | 119 | 119 | 119 | 119 |

"

The following portion of Table 9-34

"

| cu_qp_delta | 0 | as specified in subclause | 2 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 2 | Error: Reference source not found | 3 |
|  | 2 |  | 2 | Error: Reference source not found | 6 |

"

is replaced by

"

| cu_qp_delta | 0 | as specified in subclause | 1 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 1 | Error: Reference source not found | 2 |
|  | 2 |  | 1 | Error: Reference source not found | 4 |

"

The "9.2.2.6	Binarization process for cu_qp_delta" is replaced by

"

#### 9.2.2.6		Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta.

Output of this process is the binarization of the syntax element.

The bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
	if (QpBdOffsetY<12) 
     MEG0( absV-1, 4) 
   else
     MEG0( absV-1, 5)
	cMax = 24 + ( QpBdOffsetY >> 1 ) + signV
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
	cNum = absV − 2
		while( cNum− − ) 
			put( 1 ) 
		if( cMax > absV − 1 )
			put( 0 )
	}
}

The MEG0 binarization as specified in subclause 9.3.2.5.

・ref_index

The following portion of Table 9-4

"

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  | 0..4 | 5..9 |
|  | ref_idx_l0, ref_idx_l1 | Table 9-3 |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

"

is replaced by

"

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  | 0..4 | 5..9 |
|  | ref_idx_l0, ref_idx_l1 | Table 9-3 |  | 0..21 | 3..52..3 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

"

The following Table

"

Table 9-3 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

"

is replaced by

"

Table 9-4 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | 102 | 118 | 118 | 134 |

"

The following portion of Table 9-34

"

| ref_idx_l0 | 1 | TU, cMax = num_ref_idx_l0_active_minus1 | 2 | Table 9-3 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 2 | Table 9-3 | 3 |
| ref_idx_l1 | 1 | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Table 9-3 | 0 |
|  | 2 |  | 2 | Table 9-3 | 3 |

"

is replaced by

"

| ref_idx_l0 | 1 | MEG0, maxPrefix = num_ref_idx_l0_active_minus1 | 1 | Table 9-3 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 1 | Table 9-3 | 2 |
| ref_idx_l1 | 1 | MEG0, maxPrefix = num_ref_idx_l1_active_minus1 | 1 | Table 9-3 | 0 |
|  | 2 |  | 1 | Table 9-3 | 2 |

"

・sao_offset

The following portion of Table 9-4

"

| sao_offset_abs | 0 | TU, cMax = ( 1<< (Min(bitDepth, 10) − 5)) − 1 | 1 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 1 | Error: Reference source not found | 2 |
|  | 2 |  | 1 | Error: Reference source not found | 4 |

"

is replaced by

"

| sao_offset_abs | 0 | MEG0, maxPrefixLength = bitDepth-5 | 1 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 1 | Error: Reference source not found | 2 |
|  | 2 |  | 1 | Error: Reference source not found | 4 |

"