| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0195 |
| --- | --- |

| Title: | AHG5: on CABAC initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a modification on initialization table selection for CABAC context. The proposed method uses only two initialization tables instead of three tables in HM7. In P-slices and B-slices, one of intra-oriented or inter-oriented tables can be used by signaling cabac_init_flag, while intra-oriented table is always used for I-slices same as in HM7. To verify the performance, not only common test sequences but also a sequence including several scene changes is also used. In the simulations, intra-oriented initialization table was used in each picture just after scene change. Simulation results show that performance gain is 　　% for all settings in CTC in multiple 1500 byte slice mode. It is also reported that the performance gain 　% for all settings for the sequence with frequent scene changes.

# Introduction

In the current HEVC design, cabac_init_flag and three different CABAC initialization tables are used. On the other hand, the benefit of average 0.2% coding gain by selecting from two tables in P and B slices can be obtained by using slice level 2-pass encoding according to [1]. This contribution proposes a modification on initialization table selection using only two tables.

# Proposal

We propose two changes on the cabac_init_flag syntax element used for CABAC context initialization:

- Reduce the number of tables for CABAC initialization to two instead of three in the current design.

- Modify the definition of cabac_init_flag to be a flag to allow for P and B slice types to be initialized with either the “Intra slice type” table or “Inter slice type” table.

A primary benefit of the proposed method is allowing to switch intra slice type initialization also for inter slices. When intra CU are majority in an inter slice such as in a scene changed frame, intra slice type initialization is preferable to be used instead of inter slice type initialization.

# Simulation and results

The proposed setting was evaluated using HM-7.0 with the settings described in JCTVC-I1100 [2] with multiple 1500byte slice mode. In addition to the common test sequences, a sequence with frequent scene changes is also used for the simulation, which is taken from CoSME.

Table 1: Test sequence from CoSME

| Start frame | Frame #1364 of CoSME |
| --- | --- |
| End frame | Frame #1863 of CoSME |
| Number of frames | 500 |
| Frame rate | 24 frames/sec |
| Size | 1920 x 1080 |
| Format | 8 bits, YUV4:2:0 converted from 10bits, RGB4:4:4 by imagemagic converter |
| Scene changes | Frame #1463, #1550, #1608, #1713 and #1789 in the original sequence |

Table 2: Performance of the proposed method on CTC with 1500 byte slices mode

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |

Table 3: Performance of the proposed method on CoSME

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Overall |  |  |  |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Overall |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Overall |  |  |  |  |  |  |

# Conclusion

In this contribution reduction of cabac initialization table and modification of the definition of cabac_init_flag is proposed. It is proposed to adopt the described method in this contribution to HM8 and DIS.

# Reference

- Kiran Misra, Andrew Segall, “CE1 subtest B4 – On CABAC Init IDC”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0540, 8th Meeting: San Jose, CA, US, February, 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, 9th Meeting: Geneva, CH, April-May, 2012.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).