| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0196 |
| --- | --- |

| Title: | AHG10: Proposed modification to video parameter set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes modifications to the specification of video parameter set, which was adopted at the last meeting. The video parameter set provides a normative way for encapsulation of multiple sub-bitstreams those constitutes a single coded video sequence. Here, the term “sub-bitstream” represents temporal layer within the context of current HEVC base specification, however, it also assumes support of (spatial) enhancement layers that refer to HEVC base layer stream for their decoding. As discussed in the requirement on scalable coding extension to the current HEVC spec, base layer for such scalable extension could be other coding standard such as AVC. Considering the requirement such that encoder producing bitstream conforming to HEVC spec shall output consistent syntax element of video parameter set, this document addresses the issue of video parameter set and proposes solution for that.

# Problem Statement

At the last Geneva meeting, video_parameter_set was adopted as a normative way for encapsulation of multiple sub-bitstreams those constitutes a single coded video sequence. From the Geneva meeting report, a base decoder could ignore decoding and interpretation of video_parameter_set, however, any base encoder shall multiplex it into bitstream to convey consistent information about bitstream content to base decoder. From the current requirement on scalable extension of HEVC [1], there may be a possibility to define HEVC scalable bitstream where its base layer is not “HEVC” but may be “AVC” (or other standard such as MPEG-2 as suggested in the requirement under consideration). In such scenario, base layer stream should be transmitted via other mechanism than encapsulation using the video_parameter_set, which means that base layer stream will not be described by the video_parameter_set. The current video_parameter_set does not consider bitstream content description for this kind of combination. The main issue would be the semantics of vps_max_layers_minus1, since the current Draft 7 [2] semantics specifies that vps_max_layers_minus1 equal to 0 represents that it is always HEVC stream. In order to allow description of bitstream content that may utilize non-HEVC base layer, we propose to insert a simple 1-bit signalling into video_parameter_set, which specifies whether the base layer is HEVC or non-HEVC stream.

# Proposed Changes to Draft 7

The following changes to syntax and semantics of video_parameter_set are proposed.

7.3.2.1  Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_max_temporal_layers_minus1 | u(3) |
| vps_base_layer_present_flag | u(1) |
| vps_max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| vps_temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= vps_max_temporal_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

7.4.2.1  Video parameter set RBSP semantics

vps_max_temporal_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream.

vps_base_layer_present_flag specifies whether the coded video sequence contains bitstream constituting base layer conforming to this Recommendation | International Standard. The value of vps_base_layer_present_flag shall be equal to 1 in bitstreams conforming to this Recommendation | International Standard. When the value of vps_base_layer_present_flag is equal to 0, the value of vps_num_layers_minus1 shall be greater than or equal to 1 and the base layer stream should be transmitted via proper system layer mechanism outside the scope of this Recommendation | International Standard.

vps_max_layers_minus1 plus 1 specifies the maximum number of layers that may be present in the coded video sequence. Decoders conforming to this Recommendation | International Standard should ignore vps_num_layers_minus1 not equal to 0 in a video parameter set NAL unit.

[Ed.(YK): Add a definition of layer (probably it should "scalable layer", as there is already a definition of layer for something else) in Clause 3.]

…..

# Reference

[1] “Draft requirements for the scalable enhancement of HEVC”, ISO/IEC JTC1/SC29/WG11/N12783, May 2012.

[2] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7”, JCTVC-I1003, May 2012.

# Patent rights declaration(s)

Mitsubishi Electric  Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).