| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0199 |
| --- | --- |

| Title: | AHG 9: Comments on DPB size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Davies A. Fuldseth | Email: | thdavies@cisco.com arilfuld@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

The document discusses the level constraints on DPB size in the context of error resilient operation, where long-term reference pictures are stored for error recovery purposes in the case of frame losses. It observes that the maximum size of the DPB is determined in picture units rather than total memory as in AVC, and that it can be substantially lower than can be achieved in AVC for lower resolutions within a level. It is asserted that: the additional reference frames available in AVC are used widely to provide error recovery; the current HEVC level limitations decrease the number and increase the average age of a recovery frame that can be used; and that this will have an adverse effect on resilience and performance. It is recommended that the maximum number of pictures in the DPB allowed within a given level be increased to 10 or preferably 16 pictures.

# Long-term references and error recovery

Within a network, even if loss rates of individual packets are low, there are several packets/slices in each frame, and therefore the frame loss rates can still be significant. Frame losses are therefore common and must be routinely addressed in a variety of application scenarios, notably video conferencing and telepresence but also surveillance or any application where errors are likely and retransmission or IDR frame insertion difficult.

Long-term references (LTRs) are used in such applications to provide error resilience. In order to use a reference for recovery an encoder must be sure that the decoder has received it. For that reason a return path is usually employed to report data as successfully received (ACK) or not received (NACK). The new structure of the Reference Picture Buffer in HEVC greatly improves the reliability with which a decoder can generate a NACK [1].

To be useful a LTR therefore should not be removed from the reference buffer until:

- The effective Round Trip Time (ERTT) incorporating all delays has expired

- The encoder can determine that a more current LTR has been received at the decoder

Therefore the minimum number of LTRs for error recovery in a feedback scheme is two, one at least of which must be older than the ERTT (ACK received) and the others younger (ACK/NACK not yet received).

The ERTT is determined not just by the 'basic' round-trip time of a packet from the encoder to the decoder, but also by encoding and decoding delay, and by the time taken to receive ACKs and NACKs reliably, as these themselves may be lost. This is so that error recovery starts at the right time using the right reference frames. More than one basic round-trip time may be needed to get reliable information.

The ERTT is also further increased in multicast scenarios (switched conferences). Here all participants receive the same stream and therefore the possibility that an ACK or NACK is lost from at least one recipient is high. Therefore the encoder must wait longer on average to ensure that a reference frame has definitely been lost by at least one participant and that it knows of a reference that all recipients have got.

Typically the ERTT is surprisingly high – of the order of 100ms for a simple switched video conference in a WAN. The worst-case ERTT is substantially greater – 0.5s or even more.

# Distributing LTRs

To recover from errors, LTRs can be distributed evenly across the ERTT period, so that the youngest possible frame can be used for recovery. The worst-case age of a LTR is therefore:

( 1 + 1/(N-1) ) * ERTT

where N is the number of LTRs.

Clearly, the larger N is the smaller the maximum age of the recovery frame. If the ERTT is small, the size of N may not matter very much. However, this is not the case as the ERTT grows.

The age of a LTR used for recovery substantially impacts on performance. One important case is when a video conference is starting up. To keep latency low, a very small, low quality initial I frame will be used, and quality increased as rapidly as possible afterwards. In this scenario, dropped frames are likely as bit rate adjustment is more difficult, yet a reference frame even one frame older could be substantially worse.

# Limits on Decoded Picture Buffer (DPB) size

The current draft specification [2] determines a limit of 6 frames for the DPB, including the current frame, set using the MaxDpbSize variable in the level table (Annex A). With 2 LTRs, this limits the number of short-term references to 3, already less than the common conditions [3]. Increasing the number of LTRs to reduce the recovery frame age reduces the number of STRs and thus coding efficiency.

By contrast, within AVC [4] it is possible to specify a higher level than is required for the picture size, and divide the memory allocation for the DPB into a larger number of pictures – up to 16. It is quite normal to encounter AVC systems which use all 16 DPB slots available. AVC is therefore substantially more functional for error resilience than the current HEVC draft while using less memory.

Our experience is that up to 12 LTRs may be needed to cover worst-case scenarios for 60Hz video. This gives a reference every 3 frames for 0.5s, or every 5 frames for 1s.

We therefore recommend that the specification be modified to support up to 16 pictures in the DPB, covering 3 STRs, 12 LTRs and the current picture. It is our view that a minimum of 10 pictures would be needed to provide acceptable error resilience across a range of scenarios.

There are at least two options for achieving this:

- Increase the value of MaxDpbSize for each level, or

- Adopt the AVC “memory bucket” approach

WD text has been prepared for each option, with DPB size equivalent to 5 pictures of maximum resolution in option 2.

# Other applications

We also observe that other (non-resilient) applications may make use of a higher number of reference frames than allowed for in the current draft or the common conditions. For example, it is not clear that a very small number of LTRs are sufficient for scene change applications. Consider:

- remote desktop applications where a number of windows are being minimized and maximized;

- flash photography obscuring a real scene change with a number of apparent changes

Interlace applications or high frame rate applications may also require higher-depth hierarchical GOP structures for greater efficiency.

# Summary

A description of the use of LTRs for error recovery has been presented, and an analysis given of how the effective round-trip time impacts the age and quality of recovery frames. It has been noted that AVC provides greater capability for use of LTRs for error recovery than the current draft. It is recommended to increase the number of reference frames that are available in the DPB above the current limits provided by MaxDpbSize, to provide error resilience and recovery capabilities similar to AVC. This may be achieved either by increasing MaxDpbSize or by reverting to the memory mechanism provided in AVC.

# References

[1]	JCTVC-F493, “Absolute signalling of reference pictures”, R. Sjöberg, J. Samuelsson (Ericsson), JCTVC 6th Meeting, Torino, 2011

[2] JCTVC-I1003, “High Efficiency Video Coding (HEVC) text specification draft 7”, B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand (Editors), JCTVC 9th Meeting, Geneva 27 April – 7 May 2012

[3]	JCTVC-I1100, “Common HM test conditions and software reference configurations”, F. Bossen, JCTVC 9th Meeting: Geneva, CH, 27 Apr – 7 May 2012

[4]	ITU-T Rec. H.264 | ISO/IEC 14496-10 Advanced Video Coding

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).