| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0200 |
| --- | --- |

| Title: | Cross verification of Refined significant map context derivation for large TU (JCTVC-J0068) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Toru Kumakura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com kumakura.toru@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross verification results of JCTVC-J0068. In the proposal, the context assignments for significant_coeff_flag for large transforms are changed from HM7.0.

It has confirmed that the proposed algorithm is appropriately implemented on HM7.0 software and the simulation results are completely matched with the results provided by the proponent.

# Introduction

This contribution reports cross verification results of JCTVC-J0068 [4] by Sharp. JCTVC-J0068 proposes refinement technique of the context derivation for significant_coeff_flag proposed in JCTVC-I0296 [3].

# Algorithm Description

In the HM7.0, the context indices of syntax elements significant_coeff_flag for large TU are derived from the syntax elements significant_coeff_group_flag of neighboring subsets and the position of decoded coefficient.

The context assignments for significant_coeff_flag for large transforms in HM-7.0 are shown in Figure 1.

Figure 1 context assignments for significant_coeff_flag for large transform in HM-7.0

The proposed context assignments for significant_coeff_flag for large transforms are changed as shown in Figure 2.

Figure 2 proposed context assignments for significant_coeff_flag for large transforms

The proposed algorithm is implemented on HM-7.0 software.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 presents the simulation results under common conditions.

Table 2 Simulation results under common conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class F | -0.1% | 0.0% | 0.0% | -0.1% | -0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.3% | 0.0% | 0.0% | 0.4% |
| Class B | -0.1% | -0.1% | -0.3% | -0.1%! | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.1% | -0.1% | 0.0% | 0.1% | -0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.2% | 0.0% | 0.3% | 0.0% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.3% |
| Class D | 0.0% | 0.2% | -0.1% | 0.0% | 0.6% | 0.5% |
| Class E | -0.1% | -0.1% | -0.6% | 0.1% | -1.1% | -0.7% |
| Overall | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | -0.3% | 0.0% | 0.1% | 0.1% |
| Class F | -0.2% | -0.6% | 0.3% | 0.1% | 0.3% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

Table 2 presents the simulation results under low QP conditions.

Table 3 Simulation results under low QP conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.1% |
| Class E | -0.1% | 0.3% | 0.4% | 0.0% | 0.2% | 0.3% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class F | -0.1% | -0.2% | -0.1% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

It has confirmed that the proposed algorithm is appropriately implemented on HM7.0 software and the simulation results are completely matched with the results provided by the proponent.

The context assignments for significant_coeff_flag for large transform are changed from HM-7.0. The proposed algorithm can provide average 0.1 % gain for all intra main, random access main and all intra HE 10 conditions.

# References

[1] “High efficiency video coding (HEVC) text specification draft 7”,

JCTVC-I1003, Geneva, April 2012

[2] “Common HM test conditions and software reference configurations”,

JCTVC-I1100, Geneva, April 2012

[3] “Non-CE3: Simplified context derivation for significance map” ,

JCTVC-I0296, Geneva, April 2012

[4] “Refined significant map context derivation for large TU” ,

JCTVC-J0068, Stockholm, September 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).