| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0201_r1 |
| --- | --- |

| Title: | QM Bypass for Transform Skip Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, Sung-Chang Lim *Kyung Yong Kim, *Gwang Hoon Park Jinho Lee, Jin Soo Choi ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University, 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {hykim5, sclim, jinosoul, jschoi} @etri.re.kr *{kimky, ghpark}@khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes to always use constant scaling factor for transform skipped residue regardless of whether scaling list is used or not. With the default scaling list applied and transform skipping is enabled, the experimental results reportedly show that the proposed method provides 2.8% and 2.6% luma BDR reduction for class F sequences in AI Main and AI HE10 configurations, respectively. It is also reported that roughly 3.6% and 2.0% luma BDR reduction in RA and LD configurations, respectively, could be achieved for class F sequences.

# Introduction

In the current HM, when non-flat scaling matrix is used for a slice, transform skipped residual blocks may undergo some position-dependent scaling, which is undesirable for spatial residue signal. This contribution proposes to bypass QM for transform skipped block to prevent such problem.

# Proposal

As shown in Fig. 2, we propose to always use constant (i.e., non-position-dependent) scaling factor for transform skipped residue regardless of the value of scaling_list_enable_flag. By doing this, the undesirable position-dependent scaling of spatial residue could be prevented. Also, optimization of scaling list for perceptual quantization could be done only for frequency-domain residue without being hampered by spatial-domain residue. Note that, as can be seen in the Annex of this document, minimal text change is involved for this modification.

Fig. 1	Scaling factor derivation process in HM 7

Fig. 2	Proposed scaling factor derivation process

# Experimental result

The proposed method was implemented on HM7.0 software. Table 1 shows the summary result of the proposed method compared to transform-skipping-enabled HM 7.0. The default scaling matrices are used instead of flat matrices for all TU sizes. All the other conditions are set as defined in the common test conditions [1]. Note that most gains are observed in the Class F sequences, where transform skip mode is selected more frequently than in the other classes.

Table 1. Summary result of the proposed method

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | 0.6% | 0.6% | -0.1% | 0.6% | 0.6% |
| Class C | -0.5% | 3.0% | 3.2% | -0.3% | 3.2% | 3.2% |
| Class D | -0.5% | 4.0% | 3.8% | -0.3% | 3.9% | 3.8% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Overall | -0.2% | 1.5% | 1.5% | -0.1% | 1.6% | 1.6% |
|  | -0.2% | 1.5% | 1.5% | -0.1% | 1.5% | 1.5% |
| Class F | -2.8% | 1.0% | -0.7% | -2.6% | 2.2% | 0.6% |
| Enc Time[%] | 108% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.4% | -0.3% | 0.0% | -0.1% | -0.4% |
| Class B | 0.2% | 0.6% | 0.4% | 0.1% | 0.4% | 0.3% |
| Class C | -0.7% | 0.5% | 0.6% | -0.7% | 0.7% | 0.9% |
| Class D | -0.4% | 1.6% | 1.1% | -0.5% | 1.5% | 1.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.6% | 0.4% | -0.2% | 0.6% | 0.5% |
|  | -0.2% | 0.5% | 0.4% | -0.2% | 0.6% | 0.4% |
| Class F | -3.6% | -2.6% | -3.9% | -3.6% | -1.3% | -2.6% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.2% | 0.2% | -0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.6% | 0.0% | 0.0% | 0.5% | 0.4% |
| Class E | 0.0% | 0.8% | 0.2% | 0.0% | -0.4% | 0.5% |
| Overall | 0.0% | 0.4% | 0.2% | 0.0% | 0.1% | 0.2% |
|  | 0.0% | 0.4% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class F | -2.1% | -1.2% | -2.2% | -1.9% | 0.1% | -1.9% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this contribution, we propose to always use constant (i.e., non-position-dependent) scaling factor for transform skipped residue regardless of the value of scaling_list_enable_flag. With the default scaling list applied and transform skipping is enabled, the experimental results show that the proposed method provides 2.8% and 2.6% luma BDR reduction for class F sequences in AI Main and AI HE10 configurations, respectively. It is also reported that roughly 3.6% and 2.0% luma BDR reduction in RA and LD configurations, respectively, could be achieved for class F sequences. It is recommended to adopt the proposed method into the DIS version of HM.

# Reference

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-I1100, Apr. 2012.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex:	Proposed text changes based on JCTVC-I1003_d5

### 8.6.3	Scaling process for transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array c of transform coefficients with elements cij,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable transSkipFlag specifying whether transform is applied to the current block or not,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled transform coefficients as a (nW)x(nH) array of d with elements dij.

The variable log2TrSize is derived as follows:

log2TrSize = ( Log2( nW ) + Log2( nH ) ) >> 1		(8-1)

The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = BitDepthY + log2TrSize − 5		(8-2)

–	Otherwise,

shift = BitDepthC + log2TrSize − 5		(8-3)

The scaling array levelScale[·] is specified as levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5.

For the derivation of the scaled transform coefficients dij with i = 0..nW − 1, j = 0..nH − 1 the following applies.

–	The scaling factor mij is derived as follows.

–	If scaling_list_enable_flag is equal to 0 or transSkipFlag is equal to 1,

mij = 16		(8-4)

–	Otherwise (scaling_list_enable_flag is equal to 1 and transSkipFlag is equal to 0),

mij = ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ]	(8-5)

Where the variables SizeID and RefMatrixID are specified in Error: Reference source not found and Equation Error: Reference source not found, respectively, and trafoType is derived by

trafoType = ( ( nW = = nH ) ? 0 : ( ( nW > nH ) ? 1 : 2 ) )	(8-6)

–	The scaled transform coefficient dij is derived as follows.

dij = Clip3( −32768, 32767, ( (cij * mij * levelScale[ qP%6 ] << (qP/6)) + (1 << (shift − 1 )) ) >> shift )	(8-7)