| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0202_r1 |
| --- | --- |

| Title: | Coefficient Scan for Transform Skip Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, Sung-Chang Lim *Kyung Yong Kim, *Gwang Hoon Park Jinho Lee, Jin Soo Choi ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University, 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {hykim5, sclim, jinosoul, jschoi} @etri.re.kr *{kimky, ghpark}@khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes coefficient scanning methods for transform skipped residue. In the first method, the scan type for transform skipped block is fixed to diagonal scanning. In the second method, the scan type for transform skipped block is determined in accordance with the intra-prediction direction; vertical scan is used for vertical-like prediction directions and horizontal scan is used for horizontal-like prediction directions. Experimental results reportedly show that average 0.3% and 0.9% luma BDR reductions for method 1 and 2, respectively, could be obtained for Class F sequences in both AI-Main and AI-HE10 configurations.

# Introduction

In the current HM, residual coefficient scan type is determined independently to the application of transform skipping. Fig. 1 summarizes the scan type derivation process of the HM 7.0, where vertical scan is used for horizontal-like prediction modes and horizontal scan is used for vertical-like prediction modes. However, since the directionality of spatial residue is different to that of transformed residue, it is more desirable to determine the scan type of the residual signal differently if transform skipping is applied. This contribution proposes two simple and natural ways to determine the scan types of transform skipped residue.

Fig. 1	Scan type derivation in HM 7

# Proposal

## Method 1

In the first method, the scan type for transform skipped block is fixed to diagonal scanning (scanIdx == 0). Fig. 2 illustrates the modified scan type derivation process, where just one condition is added to the HM 7 process.

Fig. 2	Method 1: diagonal scan for transform skipped residue

## Method 2

In the second method, the scan type for transform skipped block determined as opposite to the non-transform-skipped block. In other words, vertical scanning (scanIdx == 2) is used for vertical-like prediction directions (i.e., mode 22 ~ 30) and horizontal scanning (scanIdx == 1) is used for horizontal-like prediction directions (i.e, mode 6 ~ 14). Fig. 3 illustrates the modified scan type derivation process, where the scan type is “reversed” if transform_skip_flag is true.

Fig. 3	Method 2: prediction-direction aligned scan for transform skipped residue

# Experimental result

The proposed method was implemented on HM7.0 software. Table 1 and Table 2 show the summary result of the proposed method 1 and 2, respectively, compared to transform-skipping-enabled HM 7.0. All the other conditions are set as defined in the common test conditions [1]. Note that most gains are observed in the Class F sequences, where transform skip mode is selected more frequently than in the other classes.

Table 1. Summary result of the proposed method 1 (Diagonal scanning)

|  | All Intra Main |  |  |  |  |  | All Intra HE10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class B | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class C | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class D | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class E | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Overall | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
|  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class F | -0.3% |  | -0.2% |  | -0.3% |  | -0.3% |  | -0.2% |  | -0.1% |  |  |
| Enc Time[%] | 100% |  |  |  |  |  | 98% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  | 101% |  |  |  |  |  |  |

Table 2. Summary result of the proposed method 2 (Prediction-direction aligned scanning)

|  | All Intra Main |  |  |  |  |  | All Intra HE10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class B | 0.0% |  | 0.0% |  | -0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class C | -0.1% |  | -0.2% |  | -0.1% |  | -0.1% |  | -0.1% |  | -0.1% |  |  |
| Class D | -0.1% |  | -0.1% |  | -0.2% |  | -0.1% |  | -0.1% |  | -0.2% |  |  |
| Class E | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Overall | 0.0% |  | -0.1% |  | -0.1% |  | 0.0% |  | -0.1% |  | -0.1% |  |  |
|  | 0.0% |  | -0.1% |  | -0.1% |  | 0.0% |  | -0.1% |  | -0.1% |  |  |
| Class F | -0.9% |  | -0.9% |  | -1.0% |  | -0.9% |  | -0.8% |  | -0.8% |  |  |
| Enc Time[%] | 101% |  |  |  |  |  | 100% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |

# Conclusion

In this contribution, we propose to two simple natural ways to determine residual coefficient scan type for transform-skipped blocks. Experimental results show that average 0.3% and 0.9% luma BDR reductions for method 1 and 2, respectively, could be achieved for Class F sequences in both AI-Main and AI-HE10 configurations. It is recommended to adopt the proposed method into the DIS version of HM.

# Reference

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-I1100, Apr. 2012.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1:	Text changes for Method 1 (based on JCTVC-I1003_d5)

Variation #1

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| if( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| scanIdx = 0 |  |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| … |  |
| } |  |

Variation #2

### 7.3.9 Transform unit syntax

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| if( PredMode = = MODE_INTRA ) { |  |
| scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ log2TrafoSize − 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0 ) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| if( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( PredMode = = MODE_INTRA && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) { |  |
| if( cIdx == 0 ) |  |
| scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanType[ log2TrafoSize − 1 ][ IntraPredModeC ] |  |
| } else |  |
| scanIdx = 0 |  |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| … |  |
| } |  |

# Annex 2:	Text changes for Method 2 (based on JCTVC-I1003_d5)

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| if( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| scanIdx = ( scanIdx = = 1 ) ? 2 : ( ( scanIdx = = 2 ) ? 1 : 0 ) |  |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| … |  |
| } |  |