| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0203 |
| --- | --- |

| Title: | Simplification on zero merge candidate derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul, jschoi} @etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents a simplification on zero merge candidate derivation. In order to unify the zero merge candidate derivation process in HM7.0 and reduce average memory bandwidth of motion compensation, it is proposed to derive L0 uni-predictive zero merge candidate instead of bi-predictive zero merge candidate regardless of slice_type in the zero merge candidate derivation process. The experimental result reportedly shows that the proposed method introduces no coding loss on average in all test conditions.

# Introduction

In HM7.0, zero merge candidate addition process and zero merge candidate fill-up process are performed after the derivation of combined bi-predictive merge candidate, as shown in Fig. 1. In order to improve the coding efficiency, the zero merge candidate addition process adds a zero merge candidate with increasing reference picture index, r in Fig. 1, by 1 to the merge candidate list until the merge candidate list is full or the reference picture index is equal to the number of reference pictures in the reference picture list [1]. And, until the merge candidate list is full, zero merge candidate is filled up in the merge candidate list in order to avoid the potential mismatch between encoder and decoder [2, 3].

Fig. 1	Zero merge candidate derivation process in HM7.0

More specifically, as shown in Table. 1, when slice_type is equal to B-slice, the zero merge candidate addition process adds bi-predictive zero merge candidate by increasing the reference picture index, r in Table 1, by 1 to the merge candidate list and the zero merge candidate fill-up process fills in the merge candidate list with bi-predictive zero merge candidate with the reference picture index equal to 0. Otherwise (when slice_type is equal to P-slice), the zero merge candidate addition process adds L0 uni-predictive zero merge candidate by increasing the reference picture index by 1 to the merge candidate list and the zero merge candidate fill-up process fills in the merge candidate list with L0 uni-predictive zero merge candidate with the reference picture index equal to 0. For P-slice, the reference picture index is increased until the reference picture index is equal to the number of reference picture in L0 reference picture list, and for B-slice, the reference picture index is increased until the reference picture index is equal to the minimum number of reference pictures among L0 reference picture list and L1 reference picture list.

Table 1. Zero merge candidates in HM7.0

| Variable | Zero merge candidate addition process |  | Zero merge candidate fill-up process |  |
| --- | --- | --- | --- | --- |
|  | P-slice | B-slice | P-slice | B-slice |
| predFlagL0 | 1 | 1 | 1 | 1 |
| predFlagL1 | 0 | 1 | 0 | 1 |
| mvL0 | (0, 0) | (0, 0) | (0, 0) | (0, 0) |
| mvL1 | (0, 0) | (0, 0) | (0, 0) | (0, 0) |
| refIdxL0 | r | r | 0 | 0 |
| refIdxL1 | -1 | r | -1 | 0 |

Thus, in HM7.0, the zero merge candidate is derived in two ways according to the slice_type. And, average memory bandwidth of motion compensation can be increased since the bi-predictive zero merge candidate is generated in the zero merge candidate derivation process for B-slice.

# Proposal

In order to simplify and unify the zero merge candidate derivation process in HM7.0, zero merge candidate derivation process regardless of slice_type is proposed. In this proposal, L0 uni-predictive zero merge candidate instead of bi-predictive zero merge candidate is derived regardless of slice_type in both the zero merge candidate addition process and the zero merge candidate fill-up process, as shown in Fig. 2.

Fig. 2	Proposed zero merge candidate derivation process

Thus, the proposed zero merge candidate addition process adds zero merge candidate with increasing reference picture index by 1 to the merge candidate list until the merge candidate list is full or the reference picture index is equal to the number of reference pictures in L0 reference picture list. And the proposed zero merge candidate with the reference picture index equal to 0 is filled up in the merge candidate list until the merge candidate list is full. Note that the proposed method is identical to the zero merge candidate derivation process for P-slice in order to unify the zero merge candidate derivation processes for P-slice and B-slice. Table 2 shows the unified zero merge candidate which is derived regardless of slice_type. The average memory bandwidth of motion compensation can be decreased since the L0 uni-predictive merge candidate instead of the bi-predictive zero merge candidate is derived in the proposed zero merge candidate derivation process.

Table 2. Proposed zero merge candidates

| Variable | Zero merge candidate addition process | Zero merge candidate fill-up process |
| --- | --- | --- |
| predFlagL0 | 1 | 1 |
| predFlagL1 | 0 | 0 |
| mvL0 | (0, 0) | (0, 0) |
| mvL1 | (0, 0) | (0, 0) |
| refIdxL0 | r | 0 |
| refIdxL1 | -1 | -1 |

# Experimental result

The proposed method was implemented on HM7.0 software. Table 3 shows the summary result of the proposed method compared to HM 7.0 anchor under common test conditions [4].

Table 3. Summary result of the proposed method

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.1% | 0.2% | 0.0% |
| Class B | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.4% | 0.4% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | 0.0% | -0.4% | 0.0% | -0.3% | 0.4% |
| Overall | 0.0% | 0.2% | 0.1% | 0.0% | -0.2% | 0.1% |
|  | 0.0% | 0.2% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class F | -0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

During the source code checking by the cross-checker, the cross-checker kindly reported a bug in the source code. Thus, the experiment was re-conducted using the bug-fixed source code. And the source code patch file with bug-fix is included in zip file. Table 4 shows the summary result of the proposed method with the bug-fix. No loss is observed on average in all test conditions.

Table 4. Summary result of the proposed method with the bug-fix

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.4% |
| Class C | 0.0% | 0.3% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.0% | -0.1% | 0.0% | 0.2% |
| Class E | -0.1% | -0.2% | -0.5% | 0.2% | 0.1% | 0.3% |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.2% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% |
| Class F | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 1.1% |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this contribution, it is proposed to derive L0 uni-predictive zero merge candidate instead of bi-predictive zero merge candidate regardless of slice_type in the zero merge candidate derivation process. The experimental result reportedly shows that the proposed method introduces no coding loss on average in all test conditions. Therefore, it is recommended to adopt this proposal to clean up the current design.

# Reference

- T. Sugio and T. Nishi, "Parsing Robustness for Merge/AMVP," JCTVC-F470, Jul. 2011.

- T. Sugio and T. Nishi, "On MVP candidate list for AMVP/Merge," JCTVC-I0134, Apr. 2012.

- H. Nakamura, S. Fukushima, and H. Takehara, "Default value for skip/merge and AMVP," JCTVC-I0314, Apr. 2012.

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-I1100, Apr. 2012.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex:	Proposed text changes based on JCTVC-I1003_d8

- 8.5.2.1.4	Derivation process for zero motion vector merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0zeroCandm and refIdxL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0zeroCandm and predFlagL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- motion vectors mvL0zeroCandm and mvL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

The variable numRefIdx is derived as follows.

- If slice_type is equal to P, numRefIdx is set to num_ref_idx_l0_active_minus1 + 1

- Otherwise (slice_type is equal to B), numRefIdx is set to min(num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1)

When numMergeCand is less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variable zeroIdx is set to 0, and the following steps are repeated until numMergeCand is equal to MaxNumMergeCand.

- For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies.

    - If slice_type is equal to P, tThe candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are derived as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = (zeroIdx < numRefIdx) ? zeroIdx : 0	(8-1)

refIdxL1zeroCandm = −1	(8-2)

predFlagL0zeroCandm = 1	(8-3)

predFlagL1zeroCandm = 0	(8-4)

mvL0zeroCandm[ 0 ] = 0	(8-5)

mvL0zeroCandm[ 1 ] = 0	(8-6)

mvL1zeroCandm[ 0 ] = 0	(8-7)

mvL1zeroCandm[ 1 ] = 0	(8-8)

numMergeCand = numMergeCand + 1	(8-9)

    - Otherwise (slice_type is equal to B), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are derived as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = (zeroIdx < numRefIdx) ? zeroIdx : 0	(8-10)

refIdxL1zeroCandm = (zeroIdx < numRefIdx) ? zeroIdx : 0	(8-11)

predFlagL0zeroCandm = 1	(8-12)

predFlagL1zeroCandm = 1	(8-13)

mvL0zeroCandm[ 0 ] = 0	(8-14)

mvL0zeroCandm[ 1 ] = 0	(8-15)

mvL1zeroCandm[ 0 ] = 0	(8-16)

mvL1zeroCandm[ 1 ] = 0	(8-17)

numMergeCand = numMergeCand + 1	(8-18)

- The variable zeroIdx is incremented by 1.