| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0204 |
| --- | --- |

| Title: | Dependency removal of temporal merge candidate and combined bi-predictive merge candidate derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul, jschoi} @etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents a dependency removal between temporal merge candidate and combined bi-predictive merge candidate derivation. In order to improve the throughput of merge candidate list derivation, it is proposed to use only spatial merge candidates for combined bi-predictive merge candidate derivation process. The experimental result reportedly shows that the proposed method introduces maximum 0.2% of coding loss in HE10-LB test condition and 0.1% of coding loss on average in the other test conditions.

# Introduction

In general, the derivation of temporal merge candidate requires more cycles than the derivation of spatial merge candidate. However, in HM7.0, the derivation of combined bi-predictive merge candidate can starts only after the derivation of temporal merge candidate, as shown in Fig. 1, since the derivation of combined bi-predictive merge candidate depends on both the spatial merge candidates and the temporal merge candidate. Thus, the combined bi-predictive merge candidate derivation prevents the possibility of throughput improvement in the current merge candidate list derivation process. For high-throughput encoder and decoder, it is desirable to remove the dependency between the temporal merge candidate and the combined bi-predictive merge candidate derivation in order to reduce cycles for derivation of merge candidate list as much as possible.

Fig. 1	Merge candidate list derivation in HM7.0

# Proposal

In this contribution, the combined bi-predictive merge candidate derivation without the temporal merge candidate is proposed. As shown in Fig. 2, the proposed combined bi-predictive merge candidate derivation uses only the derived spatial merge candidates. By removing dependency between the temporal merge candidate and the combined bi-predictive merge candidate derivation, the throughput of merge candidate list derivation can be improved. Also, since the proposal does not use the temporal merge candidate in the combined bi-predictive merge candidate derivation, the combined bi-predictive merge candidate derivation processes for both the temporal motion vector on and off cases can be implemented as one unified logic.

Fig. 2	Proposed merge candidate list derivation

# Experimental result

The proposed method was implemented on HM7.0 software. Table 1 shows the summary result of the proposed method compared to HM 7.0 anchor under common test conditions [1]. The proposed method introduces maximum 0.2% of coding loss in HE10-LB test condition and 0.1% of coding loss on average in the other test conditions.

Table 1. Summary result of the proposed method

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 0.2% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% | 0.2% |
| Class C | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.4% |
| Class D | 0.1% | 0.1% | 0.4% | 0.2% | 0.5% | 0.2% |
| Class E | -0.1% | 0.3% | -0.1% | 0.1% | -0.2% | -0.2% |
| Overall | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% |
|  | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.3% |
| Class F | -0.1% | 0.3% | 0.2% | 0.2% | 0.4% | 1.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

In this contribution, it is proposed to use only spatial merge candidates for combined bi-predictive merge candidate derivation process in order to improve throughput of the merge candidate list derivation. The experimental result reportedly shows that the proposed method introduces maximum 0.2% of coding loss in HE10-LB test condition and 0.1% of coding loss on average in the other test conditions. And, it is recommended to adopt this proposal to improve the throughput of merge mode.

# Reference

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-I1100, Apr. 2012.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex:	Proposed text changes based on JCTVC-I1003_d8

- 8.5.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPSW and nPSH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variable spatialCandCnt is set equal to 0.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPSW and nPSH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the variable spatialCandCnt, the variable singleMCLFlag, the width and the height of the luma prediction block nPSW and nPSH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the variable spatialCandCnt, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPSW and nPSH, and refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand areis set to the number of merging candidates in the mergeCandList and the variable numOrigMergeCand is set equal to spatialCandCnt.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

- 8.5.2.1.2	Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable spatialCandCnt,

- a variable singleMCLFlag,

- variables specifying the width and the height of the luma prediction block, nPSW and nPSH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the variable spatialCandCnt,

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP − 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP − 1 ), ( xP + nPSW − 1,  yP − 1 ) or ( xP − 1,  yP − 1 ), the following applies.

–	When yP−1 is less than (( yC >> Log2CtbSize ) << Log2CtbSize), the following applies.

xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-7)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-8)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-9)

–	If one or more of the following conditions are true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction block covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- The prediction block covering luma location ( xN, yN ) is not available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ] or PredMode is MODE_INTRA.

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1 and N is equal to B1

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1 and N is equal to A1

- N is equal to B1 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B0 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to A0 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

–	Otherwise, spatialCandCnt is incremented by 1 and availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction block covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

- 8.5.2.1.3	Derivation process for combined bi-predictive merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

When numOrigMergeCand is greater than 1 and less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variable combIdx, the variable combStop is set to FALSE and the following steps are repeated until combStop is equal to TRUE.

- The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Error: Reference source not found.

- The following assignments are made with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList ( l0Cand = mergeCandList[ l0CandIdx ] , l1Cand = mergeCandList[ l1CandIdx ] ).

- When all of the following conditions are true,

    - predFlagL0l0Cand = = 1,

    - predFlagL1l1Cand = = 1,

    - PicOrderCnt( RefPicList0[ refIdxL0l0Cand ] ) != PicOrderCnt( RefPicList1[ refIdxL1l1Cand ] ) | | mvL0l0Cand != mvL1l1Cand,

the candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are derived as follows and numMergeCand is incremented by 1.

refIdxL0combCandk = refIdxL0l0Cand	(8-10)

refIdxL1combCandk = refIdxL1l1Cand	(8-11)

predFlagL0combCandk = 1	(8-12)

predFlagL1combCandk = 1	(8-13)

mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]	(8-14)

mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]	(8-15)

mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]	(8-16)

mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]	(8-17)

numMergeCand = numMergeCand + 1	(8-18)

- The variable combIdx is incremented by 1.

- When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to MaxNumMergeCand, combStop is set to TRUE.

Table 8-1 – Specification of l0CandIdx and l1CandIdx

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| l1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |