| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0205 |
| --- | --- |

| Title: | Cross-check report on removal of zigzag scan from scaling list coding (JCTVC-J0150) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul} @etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This document presents a cross-check report on removal of zigzag scan from scaling list coding (JCTVC-J0150) proposed by Canon. It is confirmed that the number of coded bits for quantization matrices are exactly matched with the results in JCTVC-J0150.

# Cross-check results

The source code patch file and text modification for JCTVC-J0150 [1] was provided by Canon. The provided patch file and text are matched with the description in the document.

Quantization matrix files for calculation of the number of coded bits for quantization matrices are provided by Canon at the last meeting [2]. Table I shows the verified results of the number of coded bits for quantization matrices using the up-right diagonal scan proposed in JCTVC-J0150 with the provided quantization matrix files. Note that the results are exactly matched with the results in JCTVC-J0150.

Table I. Comparison of the number of coded bits for each quantization matrix

| Matrix type | Zigzag scan in HM7.0 (bits) | Up-right diagonal scan in JCTVC-J0150 (bits) |
| --- | --- | --- |
| Symmetry1 | 2067 | 2067 |
| Symmetry2 | 1255 | 1255 |
| Symmetry3 | 2593 | 2593 |
| Symmetry4 | 2565 | 2565 |
| Symmetry5 | 1899 | 1899 |
| Symmetry6 | 2089 | 2089 |
| Asymmetry1 | 2377 | 2345 |
| Asymmetry2 | 1397 | 1371 |
| Asymmetry3 | 2873 | 2861 |
| Asymmetry4 | 3859 | 3829 |
| Asymmetry5 | 2165 | 2127 |
| Asymmetry6 | 2315 | 2293 |

# Comments from cross-checker

JCTVC-J0150 simplifies the current HEVC design as well as improves the coding efficiency of quantization matrix coding. If the up-right diagonal scan is used instead of zigzag scan for coding of quantization matrix, the zigzag scan can be removed from the current HEVC design. In fact, many lines in both text and S/W are reduced by removing the zigzag scan. Also, the up-right diagonal scan reduces the number of coded bits for quantization matrix.

# Conclusion

This document presents a cross-check report on removal of zigzag scan from scaling list coding (JCTVC-J0150) proposed by Canon. ETRI confirms that cross-cross results are exactly matched with the results of JCTVC-J0150.

# Reference

- M. Shima, "Removal of zigzag scan from scaling list coding," JCTVC-J0150, Jul. 2012.

- S.-C. Lim, H. Y. Kim, and J. Lee, "Cross-verification report on removal of zigzag scan from quantization matrices coding (JCTVC-I0370)," JCTVC-I0435, Apr. 2012.