| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0206 |
| --- | --- |

| Title: | AHG 4: Asynchronous Tile Output |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In current HEVC specification, when tile is used, coded data from each tile shall be written to output bitstream in the sequential order according to tile identification in increasing order from the first tile to the last one. It is assessed that, for parallel encoders, such design may cause delay in writing coded data into output bitstream since encoding of tile with lower tile_id is not always completed earlier due to possible processing speed of encoder resources or different texture complexity of regions in an input picture.

In this contribution proposes Asynchronous Tile Output (ATO). ATO allows encoding process that finishes faster / earlier writing its coded data into output bitstream without having to wait for process of lower tile_id to be written first so that delay can be reduced / avoided. To accommodate the proposed ATO with minimum changes to current specification, in this contribution, there are two technical options:

Option 1: When tile is used, signal tile_id (i.e., offset_tile_id [i]) for entry_point_offset [i].

Option 2: Extend option 1 with one flag in PPS to control ATO, that is, if the flag is PPS is not set, the ATO is not enabled and offset_tile_id [i] is not necessary to be signalled.

Experiment results show that the effect of the signalling offset_tile_id [i] to BD-rate performance is 0% for intra only, 0.1% for random access and low delay configurations.

# Introduction and Problem Statement

As per output of the 9th JCT-VC meeting, HEVC supports division of a picture into one or more tiles. When tile is used, each tile is given an identification / number starting from 0 to maximum number of tile in a picture minus 1 in the picture raster scan order as illustrated in the upper part of Figure 1. Furthermore, coded data from each tile shall be written to output bitstream in the sequential order according to tile identification in increasing order, that is, for the above example from tile 0 to tile 3 as shown in the below figure.

Figure 1 – A picture with 4 tiles (tile 0, 1, 2, and 3) and their order in bitstream

Mandating coded data of tiles be always written in sequential order into bitstream may have delaying output for process that complete it task faster / earlier. By requiring sequential writing process, coded data from a tile has to wait until coded data of previous tile with lower tile identification already written into the bitstream even if it is available earlier.

Consider the following example. Suppose that the above 4 tiles shown in Figure 1 are encoded by 4 CPU cores running in parallel. Due to several reasons such as different cores’ speed or texture complexity in each tile, the completion time of each CPU core may be different. Thus, it is difficult guarantee that core that encodes tile with lower identification number shall finish earlier. For example, if core #3 finishes earlier, it has to wait until all other cores finish and write their coded data before core #3 can write its coded data. This is clearly a delay that should be avoided.

# Proposed Asynchronous Tile Output (ATO)

To reduce output delay when tiles are encoded in parallel, we propose to relax order of coded tiles’ data in bitstream not necessary always in sequential order. With this relax order, coded data of tiles can be output / written asynchronously into bitstream according to its available order during encoding process.

To accommodate the proposed ATO concept, we propose technical changes in two options.

Option 1: Signal tile_id for each entry_point_offset [i] in Slice Header

In this option, it is assumed that ATO is by default always enabled for tile.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) { |  |
| entry_point_offset[ i ] | u(v) |
| if ( tiles_or_entropy_coding_sync_idc == 1) |  |
| offset_tile_id [i] | u(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

offset_tile_id [i] specifies tile_id that is associated with entry_point_offset [i]. offset_tile_id [i] shall be described by log2 ((num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1)). offset_tile_id [i] shall ranges from 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) – 1, inclusively.

The following figure illustrates the output order of coded data of tiles when ATO is used. For example, the order of completion tile is as follow: core #2 followed by core #0 followed by core #3, and finally core # 1.

Figure 2 – Example of order of writing coded data of tiles in bitstream when ATO is used.

Option 2: Option 1 + a flag in PPS to control (turn on/off) ATO

This option offer more flexibility to control whether or not ATO is enabled for tiles. Such control might be desired in profile such that for some profiles, ATO can be enabled whereas for other profiles, it can be turn off.

The syntax and semantic of the flag is as the following:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| asynchronous_tile_output_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| … |  |
| } |  |

asynchronous_tile_output_flag equals 0 indicates that coded data of tiles is in increasing order of tile id. asynchrous_tile_output_flag equals 1 indicates that coded data of tiles may be in arbitrary order. When not present, asynchronous_tile_output_flag shall be inferred as 0.

Accordingly, the syntax table in Slice Header should be adjusted as the following:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) { |  |
| entry_point_offset[ i ] | u(v) |
| if ( asynchronous_tile_output_flag == 1) |  |
| offset_tile_id [i] | u(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

# Discussion

When tile is used, loop filters can be applied in two options such as loop filters not across tile boundary and loop filters across tile boundary. For both loop filters options (i.e., loop filters across & not across tile boundary), ATO helps to reduce delay for writing encoding output. There may be concern that when loops filters are applied across boundary, delay is inevitable. However, in our assessment, loop filters can be applied after coded data is already written to the output stream. This is because loop filters only affect reconstructed frames while coded data in output bitstream itself is not changed. Thus, in any conditions of application of loop filters, ATO works well to reduce output delay.

# Experiment results

We have tested the effect of the proposed option 1 to the coding performance. The anchor of such test is the common test condition with tiles enabled.

The results show that the modification has little effect to BD-rate performance, which is 0% for Intra only and 0.1% for random access and low delay cases.

We thank Orange Labs for cross-checking our experiment results.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Conclusion

It is assessed that having coded data always in sequential order of tile_id in bitstream may cause delay in writing coded data when tiles are encoded in parallel. In this contribution, a concept called Asynchronous Tile Output (ATO) is proposed. ATO allows encoding process that finishes faster / earlier writing its coded data into output bitstream without having to wait for process of lower tile_id to be written first. There are two options covering technical solution to accommodate the proposed ATO concept with minimum changes to the HEVC specification.

Considering the benefit of ATO concept, we recommend the JCT-VC group to adopt it into the HEVC specification.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).