| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0208 WG11 Number: m25534 |
| --- | --- |

| Title: | AHG5: Bypass bins grouping on Intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Kengo Terada Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-6-6900-9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents a modification on intra mode coding parameters. It is proposed that the signalling order changes to improve the throughput of entropy coder by grouping bypass-coded bins. The proposed solution was implemented in HM7.0 and their coding efficiencies were evaluated. The results show no difference on coding performance.

# Introduction

It was adopted in the last meeting that the signalling order changes for luma intra mode which is that all the context based coded bins first and then all the bypass-coded bins for the case of INTRA_NxN [1].

However, in current draft text, several bins are still not signalled consecutively for intra mode coding syntax elements. Table 1 shows the current possible bins of syntax elements for intra mode coding in the case of INTRA_2Nx2N. The isolated bypass bin is used in case of "1 and 3", and the bypass bins can be grouped in case of 3, 4 and 6.

Table 1: current possible patterns for intra mode coding syntax elements for INTRA_2Nx2N

Note*1: In case of "chroma_pred_fromluma_enabled_flag=1", 2 context based bins are used for "intra_chroma_pred_mode = 0 to 4".

# Proposed Method

To improve throughput of the entropy coder for intra mode coding syntax elements, this proposal would introduce the syntax elements order changes. More specifically, the syntax element of chroma intra mode (intra_chroma_pred_mode) is separated into context coded part (intra_chroma_pred_prefix_mode) and bypass-coded part (rem_intra_chroma_pred_mode). Table 2 shows the proposed drat text for syntax elements. The possible bins of syntax elements for intra mode coding in the case of INTRA_2Nx2N by this proposal is shown in Table 3.

Table 2: proposed draft text

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| …. |  |
| } else if( PredMode = = MODE_INTRA ) { |  |
| …. |  |
| if( pcm_flag ) { |  |
| …. |  |
| } else { |  |
| d0 = 1 << log2CbSize |  |
| if ( !(x0 % d0) &&!(y0 % d0) ) { |  |
| d1 = (PartMode == PART_2Nx2N ? d0: (d0>>1)) |  |
| for( i = 0; i < d0 ; i += d1) |  |
| for( j = 0; j < d0 ; j+= d1) |  |
| prev_intra_luma_pred_flag[ x0+ j ][ y0 + i ] | ae(v) |
| intra_chroma_pred_prefix_mode[ x0 ][ y0 ] | ae(v) |
| for( i = 0; i < d0 ; i += d1){ |  |
| for( j = 0; j < d0 ; j+= d1){ |  |
| if( prev_intra_luma_pred_flag[ x0 + j ][ y0 + i ] ) |  |
| mpm_idx[ x0+ j ][ y0 + i] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + j ][ y0 + i ] | ae(v) |
| } |  |
| } |  |
| if ( intra_chroma_pred_prefix_mode[ x0 ][ y0 ] !=0 && intra_chroma_pred_prefix_mode[ x0 ][ y0 ] !=3 ) |  |
| rem_intra_chroma_pred_ mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| ….. |  |
| } |  |
| } |  |

Table 3: current possible patterns for intra mode coding syntax elements for INTRA_2Nx2N

Note*2: In case of "chroma_pred_fromluma_enabled_flag=1", 2 context based bins are used for "intra_chroma_pred_prefix_mode = 2 and 3".

# Experimental results

Proposed method was implemented on HM7.0 software and its performance relative to the HM7.0 software was evaluated under the common test condition described in JCTVC-I1100 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. Table 4 shows RD performance and execution time of the proposed method relative to HM7.0.

Table 4: experimental result of the proposed method

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 95% |  |  |

# Conclusion

In this contribution, it was proposed to change signalling order on intra mode coding. The bypass-coded bins are grouped to improve the entropy coder throughput. An experimental result shows that there is no coding performance changes compared to HM7.0. Based on the above result, we suggest the proposed method be adopted in the DIS of HEVC.

# References

- Wei-Jung Chien et al, “Intra mode coding for INTRA_NxN”, JCTVC-I0302, Geneva, April 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, Geneva, April 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Proposed changes on draft 7.

### 7.3.7	Prediction unit syntax

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| d0 = 1 << log2CbSize |  |
| if ( !(x0 % d0) &&!(y0 % d0) ) { |  |
| d1 = (PartMode == PART_2Nx2N ? d0: (d0>>1)) |  |
| for( i = 0; i < d0 ; i += d1) |  |
| for( j = 0; j < d0 ; j+= d1) |  |
| prev_intra_luma_pred_flag[ x0+ j ][ y0 + i ] | ae(v) |
| intra_chroma_pred_prefix_mode[ x0 ][ y0 ] | ae(v) |
| for( i = 0; i < d0 ; i += d1){ |  |
| for( j = 0; j < d0 ; j+= d1){ |  |
| if( prev_intra_luma_pred_flag[ x0 + j ][ y0 + i ] ) |  |
| mpm_idx[ x0+ j ][ y0 + i] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + j ][ y0 + i ] | ae(v) |
| } |  |
| } |  |
| if ( intra_chroma_pred_prefix_mode[ x0 ][ y0 ] !=0 && intra_chroma_pred_prefix_mode[ x0 ][ y0 ] !=3 ) |  |
| rem_intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

intra_chroma_pred_prefix_mode[ x0 ][ y0 ] and rem_intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

### 8.4.2  Derivation process for chroma intra prediction mode

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-2 or  with intra_chroma_pred_prefix_mode, rem_intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

Table 8-2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_prefix_mode, rem_intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_prefix_mode | rem_intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 26 | 10 | 1 |  | X ( 0 <= X < 35 ) |
| 3 | 0 | 34 | 0 | 0 | 0 | 0 |  |
| 3 | 1 | 26 | 34 | 26 | 26 | 26 |  |
| 3 | 2 | 10 | 10 | 34 | 10 | 10 |  |
| 3 | 3 | 1 | 1 | 1 | 34 | 1 |  |
| 2 | n/a | LM | LM | LM | LM | LM |  |
| 0 | n/a | 0 | 26 | 10 | 1 | X |  |

Table 8-3 – Specification of IntraPredModeC according to the values of intra_chroma_pred_prefix_mode, rem_intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_prefix_mode | rem_intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 26 | 10 | 1 | X ( 0 <= X < 35 ) |
| 1 | 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 1 | 26 | 34 | 26 | 26 | 26 |
| 1 | 2 | 10 | 10 | 34 | 10 | 10 |
| 1 | 3 | 1 | 1 | 1 | 34 | 1 |
| 0 | n/a | 0 | 26 | 10 | 1 | X |

#### 9.2.1.1	 	Initialization process for context variables

Table 9-4 – Association of ctxIdx and syntax elements for each initialisationType in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_prefix_mode |  | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-5 – Values of variable initValue for intra_chroma_pred_prefix_mode ctxIdx

| Initialization variable | intra_chroma_pred_prefix_mode ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 53 | 103 | 85 | 87 | 101 | 87 |

Table 9-6 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| rem_intra_luma_pred_mode | all | FL, cMax = 31 | na | na | na, (uses Decode Bypass) |
| --- | --- | --- | --- | --- | --- |
| intra_chroma_pred_prefix_mode | 0 | as specified in subclause | 1 |  | 0 |
|  | 1 |  | 1 |  | 2 |
|  | 2 |  | 1 |  | 4 |
| rem_intra_chroma_pred_mode | all | FL, cMax = 2 | na | na | na, (uses Decode Bypass) |
| merge_flag | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |

#### 9.2.2.9   Binarization process for intra_chroma_pred_prefix_mode

Input to this process is a request for a binarization for the syntax element intra_chroma_pred_prefix_mode and the chroma_pred_from_luma_enabled_flag.

Output of this process is the binarization of the syntax element.

Table 9-7 – Specifcation of intra_chroma_pred_prefix_mode depending on chroma_pred_from_luma_enabled_flag

| Value ofintra_chroma_pred_prefix_mode | chroma_pred_from_luma_enabled_flag = 1 | chroma_pred_from_luma_enabled_flag = 0 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | n/a | 1 |
| 2 | 10 | n/a |
| 3 | 11 | n/a |