| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0209_r1 |
| --- | --- |

| Title: | AHG 4: Constraint for slice and tile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Jungsun Kim Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com jungsun.kim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution identifies a possible need for clarification when one or more slice(s) contains multiple tiles, specifically, when there is a gap in tile_id of tiles that belong to the same slice. Based on the current text, the position of the next LCU to be processed be interpreted incorrectly.

To deal with the issue, it is proposed to add a new constraint governing slice and tile such that tiles that exist in a slice shall be sequential in raster scan order and such as there must be no discontinuity in tile_id.

After review in break-out group on high level parallelism and in Track A, it was suggested that it is only a matter of editorial clarification/correction. The following clarification text on definition of tile scan and slice is suggested by a group of experts during offline and email discussion.

tile scan: A specific sequential ordering of coding tree blocks partitioning a picture. The tile scan order traverses the coding tree blocks in coding tree block raster scan in within a tile and traverses tiles in raster scan order within a picture. Although a slice contains coding tree blocks that are consecutive in coding tree block raster scan of a tile, these coding tree blocks are not necessarily consecutive in coding tree block raster scan of the picture.

slice: An integer number of coding tree blocks ordered consecutively in the raster tile scan. The division of each picture into slices is a partitioning. The coding tree block addresses are derived from the first coding tree block address in a slice (as represented in the slice header).

# Introduction and Problem Statement

As per output of the 9th JCT-VC meeting, HEVC supports division of a picture into one or more slices and one or more tiles. However, relationship between slices and tiles might not be very clear yet. The only specified constraints governing how division of a picture into slices and tiles is as the following.

One or both of the following conditions shall be fulfilled for each slice and tile:

- All coded blocks in a slice belong to the same tile.

- All coded blocks in a tile belong to the same slice.

NOTE 2 – Within the same picture, there may be both slices that contain multiple tiles and tiles that contain multiple slices.

With only the above constraint, it is questionable whether configuration of tiles and slices in which one slices contains multiple tiles in vertical direction such as shown in Figure 1. In the figure, small black rectangle with black number denotes LCU and LCU index in the order of tile raster scan order, red rectangle with red number denotes tile and its tile_id, and different background color indicates different slices.

Figure 1 – Tile scan order of LCU. One picture has two slices and twelve tiles.

According to the specification, after processing LCU with CtbAddrTS n, the next LCU to be processed is LCU with CtbAddrTS n + 1. Thus, after processing LCU #7 (i.e., the last LCU in tile with tile_id = 1, in the first slice – gray color slice), the next LCU to be processed is LCU #8. However, LCU #8 is in other slice (i.e., slice with background color blue) so that there is mismatch since the decoder thinks it is processing LCU #8 while the actual coded data it reads from bitstream is LCU #16. This mismatch might be serious since it causes decoder to wrongly place reconstructed pixels of the LCU in wrong location.

# Proposed a new constraint for slices and tiles configuration

To deal with the above issue, we propose an additional constraint for governing slices and tiles as the following.

“Tiles that exist in a slice shall be sequential in raster scan order and such as there must be no discontinuity in tile_id”

The following figures show examples of tile & slice configuration that are OK and not OK.

Reason (c) is not OK: slice 0 (gray color) has Tile 0, 1, 4, 5, 8, 9. The tiles are not continuous. There is discontinuity in tile id such as from 1 to 4, from 5 to 8. Same goes for slice 1 (blue color).

Reason (e) and (f) are not OK: similar to the reason why (c) is not OK.

Figure 2 – Examples of allowed and not allowed configuration if the proposed constraint is used.

# Conclusion

This contribution identifies a possible decoding issue when one or more slice(s) contains multiple tiles, specifically, when there is a gap in tile_id of tiles that belong to the same slice. When such condition happens, decoder might misinterpret position of the next LCU it will process which may lead to incorrect reconstruction. To deal with that issue, it is proposed to add a new constraint governing slice and tile such that tiles that exist in a slice shall be sequential in raster scan order and such as there must be no discontinuity in tile_id.

We recommend the JCT-VC group to discuss identified possible issue and consider adopting the proposed constraint if the issue is valid.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).