| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0210_r2 |
| --- | --- |

| Title: | MaxDpbSize setting based on the picture size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fujibayashi TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +81-468-40-6833 fujibayashi@nttdocomo.co.jp +65 97993861 tktan.p@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc |  |  |

_____________________________

# Abstract

This contribution proposes to set the maximum number of DPB buffers supported by a decoder (MaxDpbSize) based on the decoded picture size. In current HM7.0, MaxDpbSize is defined as a fixed number regardless of the decoded picture size. Therefore, further improvement of coding performance using additional reference frames cannot be exploited. This contribution reports that further coding performance can be obtained for the random access coding structure when hierarchical GOP structures that require more reference picture buffers than MaxDpbSize – 1 is used. This contribution proposes a simple modification to double the number of reference picture buffers when the decoded picture size is equal to or smaller than half of maximum picture size (MaxLumaFS) for the level.  The maximum amount of memory needed for the level is not modified.

# Background

In the current specification of HEVC described in [1], MaxDpbSize which means maximum number of DPB buffers is constrained to be a fixed number= 6 for all levels as shown in Table1 below. This change was applied based on the JCTVC-H0168 [2].

On the other hand, MaxDpbSize was defined as a function of maximum picture size in the specification of H.264/AVC [3] as following.

MaxDpbSize =

Min( 1024 * MaxDPB / ( PicWidthInMbs * FrameHeightInMbs * 256 * ChromaFormatFactor ), 16 )

where PicWidthInMbs means the width of each decoded picture in units of macroblocks, FrameHeightInMbs means the height of a frame in units of macroblocks, ChromaFormatFactor is defined number depending on the chroma format sampling structure and MaxDPB value is defined in Table2.  Therefore, the number of storable decoded picture is changeable based on the decoded picture size. Additionally, in H.264/AVC, maximum number of storable decoded picture is restricted to 16.

Table 1: Level limits defined in HM7

| Level | Max luma pixel rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 |

- Table 2: Level limits defined in H.264/AVC

| Level number | Max macroblock processing rate MaxMBPS (MB/s) | Max frame size MaxFS (MBs) | Max decoded picture buffer size MaxDPB (1024 bytes) | Max video bit rate MaxBR (1000 bits/s or 1200 bits/s) | Max CPB size MaxCPB (1000 bits or 1200 bits) | Vertical MV component range MaxVmvR (luma frame samples) | Min compression ratio MinCR | Max number of motion vectors per two consecutive MBs MaxMvsPer2Mb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 485 | 99 | 148.5 | 64 | 175 | [-64,+63.75] | 2 | - |
| 1.1 | 3 000 | 396 | 337.5 | 192 | 500 | [-128,+127.75] | 2 | - |
| 1.2 | 6 000 | 396 | 891.0 | 384 | 1 000 | [-128,+127.75] | 2 | - |
| 1.3 | 11 880 | 396 | 891.0 | 768 | 2 000 | [-128,+127.75] | 2 | - |
| 2 | 11 880 | 396 | 891.0 | 2 000 | 2 000 | [-128,+127.75] | 2 | - |
| 2.1 | 19 800 | 792 | 1 782.0 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 2.2 | 20 250 | 1 620 | 3 037.5 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 3 | 40 500 | 1 620 | 3 037.5 | 10 000 | 10 000 | [-256,+255.75] | 2 | 32 |
| 3.1 | 108 000 | 3 600 | 6 750.0 | 14 000 | 14 000 | [-512,+511.75] | 4 | 16 |
| 3.2 | 216 000 | 5 120 | 7 680.0 | 20 000 | 20 000 | [-512,+511.75] | 4 | 16 |
| 4 | 245 760 | 8 192 | 12 288.0 | 20 000 | 25 000 | [-512,+511.75] | 4 | 16 |
| 4.1 | 245 760 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 4.2 | 491 520 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 5 | 589 824 | 22 080 | 41 310.0 | 135 000 | 135 000 | [-512,+511.75] | 2 | 16 |
| 5.1 | 983 040 | 36 864 | 69 120.0 | 240 000 | 240 000 | [-512,+511.75] | 2 | 16 |

# Possible problems

Constraining the maxDpbSize is an important issue for implementation. Memory allocation for DPB is preferable as small as possible from the viewpoint of implementation design of hardware/software.

On the other hand, current definition of maxDpbSize may be too small and further improvement of coding performance using additional reference frame may not be possible. Therefore, the impact of coding performance due to the number of reference frame was clarified in this proposal.

Coding performance using additional reference frame was observed under the random access (RA) coding conditions defined in JCTVC-I1100 [4] except for the setting of CRA period, GOP size and number of reference frames. In this simulation, HM7.0 with ticket #566 bugfix [5] was used and CRA picture was inserted to every 64 frames (RA64 setting) and was not to every 1 second for all sequences using coding structure as attached modified configuration files.In this test, GOP size was set to 8 using 4 reference frames (GOP8 RA64), set to 16 using 5 reference frames (GOP16 RA64) , set to 32 using 6 reference frames (GOP32 RA64) and set to 64 using 7 reference frames(GOP64 RA64).

Summary results of changed setting comparing current default setting defined in [4] are shown as Table3,4,5 and 6 as below. Table3 is the result of setting GOP 8 using 4 reference frames (RA64 GOP8) for clarifying the coding impact of changing RA point. Table4 is the result of setting GOP 16 using 5 reference frames (RA64 GOP16) ,Table5 shows the result of setting GOP 32 using 6 reference frames (RA64 GOP32) , Table6 shows the result of setting GOP 64 using 7 reference frames (RA64 GOP64). The full data is available in the excel sheets attached to this contribution.

Table 3: Summary results of BD-Rate comparing HM7.0 vs HM7.0 RA64 GOP8

| HM7.0 vs. HM7.0 RA64 GOP8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main |  |  | Random access HE10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.6% | -3.2% | -3.3% | -3.6% | -3.0% | -3.3% |
| Class B | -4.3% | -2.9% | -2.5% | -4.2% | -1.9% | -2.4% |
| Class C | -2.0% | -1.4% | -1.4% | -2.0% | -1.1% | -1.1% |
| Class D | -1.5% | -0.8% | -0.9% | -1.5% | -0.6% | -0.8% |
| Class E |  |  |  |  |  |  |
| Overall | -2.9% | -2.1% | -2.1% | -2.9% | -1.7% | -1.9% |
| Class F | -18.3% | -17.2% | -17.6% | -25.2% | -27.4% | -27.0% |
| Enc Time[%] | 101% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 103% |  |  |

Table 4: Summary results of BD-Rate comparing HM7.0 vs HM7.0 RA64 GOP16

| HM7.0 vs. HM7.0 RA64 GOP16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main |  |  | Random access HE10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.6% | -14.0% | -14.7% | -4.0% | -15.2% | -16.0% |
| Class B | -7.0% | -13.5% | -14.9% | -7.1% | -13.1% | -15.2% |
| Class C | -6.9% | -10.4% | -9.6% | -7.0% | -10.4% | -9.8% |
| Class D | -6.2% | -11.9% | -11.4% | -6.3% | -12.3% | -11.9% |
| Class E |  |  |  |  |  |  |
| Overall | -6.0% | -12.5% | -12.8% | -6.2% | -12.7% | -13.3% |
| Class F | -22.2% | -23.2% | -23.0% | -21.8% | -23.0% | -22.9% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  |

Table 5: Summary results of BD-Rate comparing HM7.0 vs HM7.0 RA64 GOP32

| HM7.0 vs. HM7.0 RA64 GOP32 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main |  |  | Random access HE10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -4.2% | -20.4% | -21.7% | -4.4% | -22.9% | -23.8% |
| Class B | -9.3% | -19.2% | -21.4% | -9.5% | -18.9% | -21.8% |
| Class C | -10.4% | -15.3% | -14.1% | -10.5% | -15.5% | -14.7% |
| Class D | -9.1% | -17.4% | -16.9% | -9.4% | -18.3% | -17.8% |
| Class E |  |  |  |  |  |  |
| Overall | -8.3% | -18.1% | -18.7% | -8.5% | -18.9% | -19.7% |
| Class F | -25.5% | -27.3% | -26.9% | -25.2% | -27.4% | -27.0% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 91% |  |  | 94% |  |  |

Table 6: Summary results of BD-Rate comparing HM7.0 vs HM7.0 RA64 GOP64

| HM7.0 vs. HM7.0 RA64 GOP64 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main |  |  | Random access HE10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.0% | -22.2% | -24.3% | -3.5% | -23.2% | -23.6% |
| Class B | -10.4% | -22.0% | -25.2% | -10.5% | -21.5% | -25.6% |
| Class C | -12.7% | -18.3% | -17.3% | -12.8% | -18.9% | -18.2% |
| Class D | -10.7% | -20.6% | -20.5% | -11.2% | -21.6% | -21.3% |
| Class E |  |  |  |  |  |  |
| Overall | -9.3% | -20.8% | -22.0% | -9.6% | -21.3% | -22.3% |
| Class F | -28.3% | -31.2% | -30.4% | -27.9% | -31.4% | -31.1% |
| Enc Time[%] | 85% |  |  | 85% |  |  |
| Dec Time[%] | 89% |  |  | 90% |  |  |

As can be seen in Table3, there is 2.9 % coding gain on average for both random access Main and random access HE10 due to the change of random access point. Further coding gain with larger GOP sized using additional reference frame can be observed to Table 4,5 and 6. To measure the value of coding gain due to the change of GOP sizes, coding performance of each larger GOP size comparing GOP size=8 using RA64 setting are shown in Table7,8 and 9.

Table 7: Summary results of BD-Rate comparing HM7.0 RA64 GOP8 vs HM7.0 RA64 GOP16

| HM7.0 RA64 GOP8 vs. HM7.0 RA64 GOP16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main |  |  | Random access HE10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1% | -11.1% | -11.8% | -0.6% | -12.4% | -13.1% |
| Class B | -2.9% | -11.0% | -12.8% | -3.1% | -11.4% | -13.3% |
| Class C | -4.9% | -9.0% | -8.3% | -5.0% | -9.3% | -8.7% |
| Class D | -4.7% | -11.1% | -10.5% | -4.9% | -11.7% | -11.2% |
| Class E |  |  |  |  |  |  |
| Overall | -3.1% | -10.6% | -11.0% | -3.4% | -11.3% | -11.7% |
| Class F | -4.3% | -6.9% | -6.1% | -4.5% | -7.9% | -7.1% |
| Enc Time[%] | 98% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 94% |  |  |

Table 8: Summary results of BD-Rate comparing HM7.0 RA64 GOP8 vs HM7.0 RA64 GOP32

| HM7.0 RA64 GOP8 vs. HM7.0 RA64 GOP32 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main |  |  | Random access HE10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7% | -17.7% | -19.0% | -1.0% | -20.4% | -21.1% |
| Class B | -5.3% | -17.0% | -19.6% | -5.5% | -17.5% | -20.0% |
| Class C | -8.5% | -14.0% | -12.7% | -8.6% | -14.5% | -13.7% |
| Class D | -7.7% | -16.6% | -16.0% | -8.0% | -17.8% | -17.1% |
| Class E |  |  |  |  |  |  |
| Overall | -5.5% | -16.4% | -17.0% | -5.8% | -17.5% | -18.1% |
| Class F | -7.9% | -11.5% | -10.6% | -8.2% | -12.8% | -11.7% |
| Enc Time[%] | 95% |  |  | 93% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  |

Table 9: Summary results of BD-Rate comparing HM7.0 RA64 GOP8 vs HM7.0 RA64 GOP64

| HM7.0 RA64 GOP8 vs. HM7.0 RA64 GOP64 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main |  |  | Random access HE10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4% | -19.7% | -21.7% | -0.2% | -20.8% | -21.0% |
| Class B | -6.4% | -19.8% | -23.5% | -6.5% | -20.1% | -24.0% |
| Class C | -10.8% | -17.0% | -15.9% | -10.9% | -17.9% | -17.1% |
| Class D | -9.2% | -19.7% | -19.6% | -9.8% | -21.0% | -20.6% |
| Class E |  |  |  |  |  |  |
| Overall | -6.5% | -19.1% | -20.4% | -6.8% | -20.0% | -20.9% |
| Class F | -11.3% | -16.1% | -14.7% | -11.5% | -17.6% | -16.6% |
| Enc Time[%] | 84% |  |  | 82% |  |  |
| Dec Time[%] | 91% |  |  | 87% |  |  |

As can be seen in Table 7,8 and 9, it is clarified that larger GOP size with additional reference frame allows further coding gain (3.1% for GOP size=16,5.5% for GOP size=32 and 6.5% for GOP size=64). Smaller resolution sequences such as class C and D sequences and class F sequences show larger coding gain.

# Proposal

Based on the observation shown in the previous section, MaxDpbSize setting as a function of picture size is proposed. In our proposal, if decoded picture size (calculated with pic_width_in_luma_samples and pic_height_in_luma_samples) is equal to or smaller than half of maximum picture size (MaxLumaFS), then MaxDpbSize is set to be doubled. Otherwise same size to current MaxDpbSize is used.

if (pic_width_in_luma_samples * pic_height_in_luma_samples <= (MaxLumaFS>>1))

MaxDpbSize= 2* MFSBuffer

else

MaxDpbSize= MFSBuffer

where MFSBuffer (MaxFrameSizeBuffer) is defined to be 6. This is the compromised idea between concern for implementation and argument for coding performance. MaxDpbSize is still fixed to 6 but only when decoded picture size is rather smaller than maximum picture size (MaxLumaSize) defined in each level, doubled MaxDpbSize is allowed to be used. This definition will allow more coding gain though the amount of maximum memory needed for the level is same to conventional definition.

# Conclusions

Based on the observation of coding gain using additional reference frame, MaxDpbSize setting as a function of decoded picture size was proposed. This proposal allows using twice as many decoded picture buffers only when decoded picture size is equal to or smaller than half the size of MaxLumaFS. It is recommended that the proposed MaxDpbSize setting method is adopted into the HEVC draft.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, Woo-Jin Han, Jens-Rainer Ohm, G. Sullivan, T.Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1003-d4, 9th Meeting: Geneva, CH, 27 April - 7 May, 2012.

- T.Suzuki, H.Sasai, T.Nishi, “Proposal of HEVC profile/level definitions”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0168-r3,8th Meeting: San José, CA, USA, 1–10 February, 2012

- “H.264: Advanced video coding for generic audiovisual services,” Joint Video Team of ITU-T VCEG and ISO/IEC MPEG, ITU-T Rec. H.264 and ISO/IEC 14496-10 (MPEG4 - Part 10), November 2007

- F.　Bossen., “Common HM test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, 9th Meeting: Geneva, CH, 27 April - 7 May, 2012.

- Bugfix for “Uninitialized LambdaModifier is used for large GOP sizes”

http://hevc.kw.bbc.co.uk/trac/attachment/ticket/566/FixFor566.patch

# Appendix: Proposed changes for HEVC WD7 (JCTVC-I1003_d4)

  - Levels

    - General

The following is specified for expressing the constraints in this annex.

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0.

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable fR be set to 1  300.

Bitstreams conforming to a profile at a specified level shall obey the following constraints:

- The nominal removal time of access unit n (with n > 0) from the CPB as specified in subclause Error: Reference source not found, satisfies the constraint that tr,n( n ) − tr( n − 1 ) is greater than or equal to Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Table 1 that applies to picture n − 1, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture n − 1.

- The difference between consecutive output times of pictures from the DPB as specified in subclause Error: Reference source not found, satisfies the constraint that to,dpb( n ) >= Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Table 1 for picture n, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples of picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- pic_width_in_luma_samples * pic_height_in_luma_samples <= MaxLumaFS, where MaxLumaFS is specified in Table 1

- pic_width_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

- pic_height_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

- sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] <= MaxDpbSize,

where

if (pic_width_in_luma_samples * pic_height_in_luma_samples <= (MaxLumaFS>>1))

MaxDpbSize= 2* MFSBuffer

else

MaxDpbSize= MFSBuffer

and where MFSBuffer is specified in Table 1.

Table 1 specifies the limits for each level. The use of the MinCR parameter column of Table 1 is specified in subclause Error: Reference source not found.

A level to which the bitstream conforms shall be indicated by the syntax element level_idc as follows.

–	level_idc shall be set equal to a value of 30 times the level number specified in Table 1.

[Ed. (YK): Putting level_idc 30 times instead of 10 times the level number can solve the level 1b problem occurred in AVC.]

Table A-1 – Level limits

| Level | Max luma sample rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MFSBuffer (picture storage buffers) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 |

Informative subclause Error: Reference source not found shows the effect of these limits on picture rates for several example picture formats.