| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0212 WG11 Number: m25538 |
| --- | --- |

| Title: | Modified scan method for intra transform skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, a modified scan method for intra transform skipping was proposed. In the proposed method, if the transform_skip_flag is equal to 1 and the scanIdx is equal to 2 (vertical scan), the scanIdx is modified to 1 (horizontal scan). Otherwise, if the transform_skip_flag is equal to 1 and the scanIdx is equal to 1 (horizontal scan), the scanIdx is modified to 2 (vertical scan). Experimental result reportedly showed 0.9% BR saving for All Intra scenario in class F sequences relative to HM7.0 with intra transform skipping.

# Introduction

Intra transform skipping was adopted in the last meeting [1]. It could be applied for a 4x4 intra TU (both luminance and chrominance). On the other hand, the scan method for coefficient coding is adaptively determined based on the intra prediction mode as shown Table 1 even for the block using intra transform skipping. For example, if the intra prediction is equal to the range of the value from 6 to 14 which are located nearby horizontal intra prediction mode as shown Figure 1, the vertical scan would be chosen as the coefficient scan method. This is because zero coefficients tend to be successive in a vertical direction after transforming the residual which is derived based on the horizontal intra prediction. In that case, the vertical scan would be effective (Figure 2(a)). However, if the transform were skipped, the vertical scan would not be effective since zero coefficients tend to be successive in horizontal direction (Figure 2(b)). The horizontal scan would be effective only for the combination of the vertical intra prediction and the transform.

Table 1: Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |
| --- | --- |
|  | 0 (TU4x4) |
| 6 – 14 | 2 (vertical scan) |
| 22 – 30 | 1 (horizontal scan) |
| others | 0 (diagonal scan) |

Figure 1: Intra prediction mode directions

Figure 2: example of the coefficients (a) with transform and (b) without transform in the case of horizontal intra prediction

# Proposed Method

To modify the scan method on intra transform skipping, this proposal would introduce the conversion of the scan method. Specifically, if the transform_skip_flag is equal to 1 and the scanIdx is equal to 2 (vertical scan), the scanIdx is modified to 1 (horizontal scan). Otherwise, if the transform_skip_flag is equal to 1 and the scanIdx is equal to 1 (horizontal scan), the scanIdx is modified to 2 (vertical scan). Table 2 shows the proposed drat text. Please note that this proposal would change only the assignment of scanIdx for intra transform skipping.

Table 2: proposed draft text

# Experimental results

Proposed method was implemented on HM7.0 software and its performance relative to the HM7.0 software was evaluated under the common test condition described in JCTVC-I1100 [1] with setting TS equal to 1 (intra transform skipping).

Table 3 shows RD performance and execution time of the proposed method relative to HM7.0 with intra transform skipping. In this case, proposed method shows 0.9% gain for All Intra, 0.7% gain for Random Access and 0.4% gain for Low delay B in class F sequence. The transform skipping would be especially effective to class F sequences, so it shows that the proposed method could strengthen its effectiveness. The difference in the encoding and decoding time is not obvious.

Table 3: experimental result of the proposed method 
(anchor: HM7.0 with intra transform skipping)

# Conclusion

In this contribution, it was proposed to modify the scan method for intra transform skipping. An experimental result shows that the proposed method saves 0.9% bits for All Intra scenario in class F sequences without obvious difference in encoding and decoding time. Based on the above result, we suggest the proposed method be adopted in the DIS of HEVC.

# References

- Cuiling Lan et al, “Intra transform skipping”, JCTVC-I0408, Geneva, April 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, Geneva, April 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Proposed changes on Committee Draft

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| If( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| scanIdx = ( scanIdx = = 1 ) ? 2 : ( ( scanIdx = = 2 ) ? 1 : scanIdx ) |  |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 0 ] >> 2 |  |
| yCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 1 ] >> 2 |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold && !cu_ transquant_bypass _flag) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |