| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0213 WG11 Number: m25539 |
| --- | --- |

| Title: | AHG6: A threshold for SAO edge offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Toru Matsunobu Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, matsunobu.toru, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, a threshold for SAO edge offset was proposed in order to control the categorization for a strong edge and a weak edge to improve the flexibility in terms of a subjective quality and an objective quality. This proposal could make it possible to apply edge offset only for a strong edge in LCU to prevent a blurring degradation due to the loss of an original weak edge, such as a camera noise or a film grain. Experimental result reportedly showed 0.1% BR saving for Random Access Main scenario and 0.2% BR saving for Low delay B Main scenario while improving the flexibility of SAO.

# Introduction

Sample adaptive offset (SAO) is a compensating method for the reconstructed image by adding offsets which is derived base on its characteristic. It includes two type of offsets, an edge offset (EO) and a band offset (BO). The type offset and values are determined for LCU according to its local characteristic. The EO has four categories with different features as shown Figure 1. In this categorization, there could be a possibility that both a strong edge and a weak edge are included in the same category. Figure 2 shows an example for that a reconstructed strong edge and a weak edge are included in the category 4 of EO. This is because there is no threshold to distinguish them and even if the difference values of target pixel and neighbour pixel are extremely different, they are treated as the same category. As the result, the averaged offset value could not be effective for both edges. Instead, there could be a possibility that those offset might cause a subjective degradation. In the case of Figure 2, an original weak edge, such as a camera noise or a film grain, would be lost and it might cause a blurring degradation. To improve the flexibility of SAO, it seems desirable to prepare an adjustable parameter in terms of a subjective quality and an objective quality.

Figure 1: edge offset categorization in HM7.0

Figure 2: example for the reconstructed pixel applied EO in HM7.0

# Proposed Method

To improve the flexibility of EO, this proposal would introduce a threshold to distinguish a strong edge and a weak edge. In HM7.0, a category index is calculated as follow.

category index = 2 + Sign( Px – Px-1 ) + Sign( Px+1 – Px )

Herein, Sign(x) in HM7.0 is calculated as follow.

Sign(x) = (x > 0) ? 1 : ((x < 0) ? –1 : 0)

In this proposal, a threshold would be added to above condition as follow.

Sign(x) = (x > 0 & x < threshold) ? 1 : ((x < 0 & x > –threshold) ? –1 : 0)

By setting a proper value to the threshold, it becomes possible to control the assignment of a strong edge and a weak edge with the different category as shown Figure 3. As the result, the effectiveness of EO could be strengthened as shown Figure 4. In Figure 4, applying EO would eliminate a ringing noise with keeping an original weak edge. Please note that the same categorization as HM7.0 could be possible by setting a maximum pixel value to the threshold.

Figure 3: edge offset categorization by using threshold in proposal

Figure 4: example for the reconstructed pixel applied EO in proposal

# Draft Text

This proposal introduces three EO thresholds for Y, Cb and Cr (ThresSaoEO[3]). To calculate those values for each slice, three parameters would be added in the slice header as follow. Highlighted parts with yellow line are modified ones in CD text.

----------------------------------------------------------------------------

### 7.3.3  Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag[ 0 ] | u(1) |
| if( slice_sample_adaptive_offset_flag[ 0 ] ) { |  |
| slice_sample_adaptive_offset_flag[ 1 ] | u(1) |
| slice_sample_adaptive_offset_flag[ 2 ] | u(1) |
| log2_bitdepth_minus_sao_edgeoffset_threshold[ 0 ] | ue(v) |
| if( slice_sample_adaptive_offset_flag[ 1 ] ) |  |
| log2_bitdepth_minus_sao_edgeoffset_threshold[ 1 ] | ue(v) |
| if( slice_sample_adaptive_offset_flag[ 2 ] ) |  |
| log2_bitdepth_minus_sao_edgeoffset_threshold[ 2 ] | ue(v) |
| } |  |
| } |  |
| … } |  |

### 7.4.3  Slice header semantics

log2_bitdepth_minus_sao_edgeoffset_threshold[ cIdx ] specifies the threshold value of the SAO edge offset for the colour component cIdx subtracted from bitdepth. The threshold value of the SAO edge offset, ThresSaoEO[ cIdx ] is computed as

ThresSaoEO[ cIdx ] = 1 << ( ( cIdx > 0 ? BitDepthC : BitDepthY ) – log2_bitdepth_minus_sao_edgeoffset_threshold[ cIdx ] ) 	(expression 1)

The value of log2_bitdepth_minus_sao_edgeoffset_threshold[ 0 ] shall be limited in the range of 0 to BitDepthY – 1, inclusive. The value of log2_bitdepth_minus_sao_edgeoffset_threshold[ 1 ] and log2_bitdepth_minus_sao_edgeoffset_threshold[ 2 ] shall be limited in the range of 0 to BitDepthC – 1, inclusive.

----------------------------------------------------------------------------

The thresholds ThresSaoEO[3] are calculated by using the above expression 1. Table 2 shows the pattern of ThresSaoEO[3].

Table 1: the value of ThresSaoEO[cIdx]

| log2_bitdepth_minus_sao_ edgeoffset_threshold[cIdx] | ThresSaoEO[cIdx] BitDepth=8 | ThresSaoEO[cIdx] BitDepth=10 |
| --- | --- | --- |
| 0 | 256 | 1024 |
| 1 | 128 | 512 |
| 2 | 64 | 256 |
| 3 | 32 | 128 |
| 4 | 16 | 64 |
| 5 | 8 | 32 |
| 6 | 4 | 16 |
| 7 | 2 | 8 |
| 8 | - | 4 |
| 9 | - | 2 |

ThresSaoEO[3] would be used in the following coding tree block modification process as specified 8.7.2.1 in CD text.

----------------------------------------------------------------------------

### 8.7.2.1  Coding tree block modification process

Inputs to this process are:

–	…

–	a threshold value of the SAO edge offset ThresSaoEO[ cIdx ].

- .

–	Otherwise, if saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k=0..1 are specified in Error: Reference source not found based on sao_type_idx[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows.

- If one or more of the following conditions for k=0..1 are true, edgeIdx is set equal to 0.

- …

- Otherwise, edgeIdx is dervied as follows.

edgeIdx = 2 + ∑k( Sign1( recPicture[ xC + i, yC + j ] – 
	recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-1)

where Sign1( x ) function is as follows,

Sign1( x ) 1−10; x>0∧x<ThresSaoEO[cIdx]; x<0∧x>−ThresSaoEO[cIdx]; otherwise

When edgeIdx is equal to 0, 1, or 2, it is modified as follows.

edgeIdx = (edgeIdx = = 2) ? 0 : (edgeIdx + 1)	(8-2)

- The modified picture buffer saoPicture[ xC + i, yC + j ] is derived as follows.

saoPicture[ xC + i, yC + j ] = Clip3( 0, (1 << bitDepth) – 1, recPicture[ xC + i, yC + j ] + 
		SaoOffsetVal[ cIdx ][ rx ][ ry ][ edgeIdx ] )	(8-3)

----------------------------------------------------------------------------

# Experimental results

Proposed method was implemented on HM7.0 software and its performance relative to the HM7.0 software was evaluated under the common test condition described in JCTVC-I1100 [1]. In this experiment, ThresSaoEO[0] were set according to the slice type and depth value of coding structure as shown Table 3. In general, a noise level trends to decrease in B slice because of bi-predictive motion compensation. Thus, ThresSaoEO[0] was set to lower value than I slice and P slice. Please note that the value 256 for Main and 1024 for HE10 are the maximum pixel value, so in this case, the categorization of EO is the same as HM7.0.

Table 2: ThresSaoEO[3] setting in this experiment

|  | depth | ThresSaoEO[0] (Y) | ThresSaoEO[1] (Cb) | ThresSaoEO[2] (Cr) |
| --- | --- | --- | --- | --- |
| Main | 0 | I slice, P-slice : 32 B slice : 16 | 256 | 256 |
|  | > 0 | 256 |  |  |
| HE10 | 0 | I slice, P-slice : 128 B slice : 64 | 1024 | 1024 |
|  | > 0 | 1024 |  |  |

Table 4 shows RD performance and execution time of the proposed method relative to HM7.0. In Main setting case, proposed method shows 0.0% gain for All Intra, 0.1% gain for Random Access, 0.2% gain for Low delay B and 0.1% loss for Low delay P on average without class F sequences. On the other hand, it shows 0.6% gain for All Intra, 0.8% gain for Random Access, 1.1% gain for Low delay B and 1.1% gain for Low delay P in class F sequence. The difference in the encoding and decoding time is not obvious.

Table 3: experimental result of the proposed method (anchor: HM7.0)

# Conclusion

In this contribution, it was proposed to introduce a threshold for SAO EO in order to control the categorization for a strong edge and a weak edge to improve the flexibility in terms of a subjective quality and an objective quality. For example, it becomes possible to apply EO only for a strong edge in LCU to prevent a blurring degradation due to the loss of an original weak edge, such as a camera noise or a film grain.

An experimental result shows that the proposed method saves 0.1% bits for Random Access Main case and 0.2% gain for Low delay B Main case while improving the flexibility of SAO.

Based on the above result and improving the flexibility of SAO, we suggest the proposed method be adopted in the DIS of HEVC.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, Geneva, April 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).