### 7.3.3  Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | u(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag[ 0 ] | u(1) |
| if( slice_sample_adaptive_offset_flag[ 0 ] ) { |  |
| slice_sample_adaptive_offset_flag[ 1 ] | u(1) |
| slice_sample_adaptive_offset_flag[ 2 ] | u(1) |
| log2_bitdepth_minus_sao_edgeoffset_threshold[ 0 ] | ue(v) |
| if( slice_sample_adaptive_offset_flag[ 1 ] ) |  |
| log2_bitdepth_minus_sao_edgeoffset_threshold[ 1 ] | ue(v) |
| if( slice_sample_adaptive_offset_flag[ 2 ] ) |  |
| log2_bitdepth_minus_sao_edgeoffset_threshold[ 2 ] | ue(v) |
| } |  |
| } |  |
| if(adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| pic_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( pic_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

## 7.4 Semantics

- 7.4.3 Slice header semantics

slice_sample_adaptive_offset_flag[ cIdx ] equal to 1 specifies that SAO is enabled for the colour component cIdx in the current slice; equal to 0 specifies that SAO is disabled for the colour component cIdx in the current slice. When slice_sample_adaptive_offset_flag[ cIdx ] is not present, slice_sample_adaptive_offset_flag[ cIdx ] is inferred to be equal to 0.

log2_bitdepth_minus_sao_edgeoffset_threshold[ cIdx ] specifies the threshold value of the SAO edge offset for the colour component cIdx subtracted from bitdepth. The threshold value of the SAO edge offset, ThresSaoEO[ cIdx ] is computed as

ThresSaoEO[ cIdx ] = 1 << ( ( cIdx > 0 ? BitDepthC : BitDepthY ) – log2_bitdepth_minus_sao_edgeoffset_threshold[ cIdx ] )

The value of log2_bitdepth_minus_sao_edgeoffset_threshold[ 0 ] shall be limited in the range of 0 to BitDepthY – 1, inclusive. The value of log2_bitdepth_minus_sao_edgeoffset_threshold[ 1 ] and log2_bitdepth_minus_sao_edgeoffset_threshold[ 2 ] shall be limited in the range of 0 to BitDepthC – 1, inclusive.

collocated_from_l0_flag equal to 1 specifies the picture that contains the collocated partition shall be derived from list 0, otherwise the picture shall be derived from list 1.When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

- 8.7.2 Sample adaptive offset process

Inputs of this process are the reconstructed picture buffers prior to sample adaptive offset recPictureL, recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture buffers after sample adaptive offset saoPictureL, saoPictureCb and saoPictureCr.

This process is performed on a coding tree block basis after the completion of the deblocking filter process for the decoded picture. This process is invoked when sample_adaptive_offset_enabled_flag is equal to 1.

The sample values in the modified reconstructed picture buffers after sample adaptive offset saoPictureL, saoPictureCb and saoPictureCr are initially set equal to the sample values in the reconstructed picture buffers prior to sample adaptive offset recPictureL, recPictureCb and recPictureCr.

For every coding tree unit with coding tree block location ( rx, ry ), where rx = 0..PicWidthInCtbs − 1 and ry = 0..PicHeightInCtbs − 1, the following applies:

–	When slice_sample_adaptive_offset_flag[ 0 ] of the current slice is equal to 1, the coding tree block modification process as specified in subclause  is invoked with recPicture set equal to recPictureL, cIdx set equal to 0, ( rx, ry ) and nS set equal to ( 1 << Log2CtbSize ) as inputs and the modified luma picture buffer saoPictureL as output.

–	When slice_sample_adaptive_offset_flag[ 1 ] of the current slice is equal to 1, the coding tree block modification process as specified in subclause  is invoked with recPicture set equal to recPictureCb, cIdx set equal to 1, ( rx, ry ) and nS set equal to ( 1 << (Log2CtbSize − 1) ) as inputs and the modified chroma picture buffer saoPictureCb as output.

–	When slice_sample_adaptive_offset_flag[ 2 ] of the current slice is equal to 1, the coding tree block modification process as specified in subclause  is invoked with recPicture set equal to recPictureCr, cIdx set equal to 2, ( rx, ry ) and nS set equal to ( 1 << (Log2CtbSize − 1) ) as inputs and the modified chroma picture buffer saoPictureCr as output.

#### 8.7.2.1 Coding tree block modification process

Inputs to this process are:

–	picture buffer recPicture for the colour component cIdx,

–	a variable cIdx specifying colour component index,

–	a pair of variables ( rx, ry ) specifying the coding tree block location,

–	a coding tree block size nS.

–	a threshold value of the SAO edge offset ThresSaoEO[ cIdx ].

Output of this process is a modified picture buffer saoPicture for the colour component cIdx.

The variable saoTypeIdx is set equal to sao_type_idx[ cIdx ][ rx ][ ry ].

The variable bitDepth is derived as follows.

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The variables xC and yC are set equal to rx*nS and ry*nS, respectively.

For i = 0..nS−1 and j = 0..nS−1, depending on the value of saoTypeIdx, pcm_loop_filter_disable_flag, pcm_flag, and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xC + i, yC + j ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_loop_filter_disable_flag and pcm_flag are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- saoTypeIdx is equal to 0.

–	Otherwise, if saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k=0..1 are specified in Table 8-1 based on sao_type_idx[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows.

- If one or more of the following conditions for k=0..1 are true, edgeIdx is set equal to 0.

- The sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) is outside picture boundary

- slice_loop_filter_across_slices_enabled_flag is equal to 0 and the sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to a different slice.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to a different tile.

- Otherwise, edgeIdx is dervied as follows.

edgeIdx = 2 + ∑k( Sign1( recPicture[ xC + i, yC + j ] – 
	recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-1)

where Sign1( x ) function is as follows,

Sign1( x ) 1−10; x>0∧x<ThresSaoEO[cIdx]; x<0∧x>−ThresSaoEO[cIdx]; otherwise

When edgeIdx is equal to 0, 1, or 2, it is modified as follows.

edgeIdx = (edgeIdx = = 2) ? 0 : (edgeIdx + 1)	(8-2)

- The modified picture buffer saoPicture[ xC + i, yC + j ] is derived as follows.

saoPicture[ xC + i, yC + j ] = Clip3( 0, (1 << bitDepth) – 1, recPicture[ xC + i, yC + j ] + 
		SaoOffsetVal[ cIdx ][ rx ][ ry ][ edgeIdx ] )	(8-3)

- Otherwise (saoTypeIdx is equal to 5), the following ordered steps apply:

- The variable bandShift is set equal to bitDepth − 5.

- The variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ].

- The vector bandTable is defined with 32 elements and all elements are initially set to 0. Then, four of its elements (indicating the starting position of bands for explicit offsets) are modified as follows.

for( k = 0; k < 4; k++ )
	bandTable[ (k + saoLeftClass) & 31 ] = k + 1	(8-4)

- The variable bandIdx is set equal to bandTable[ recPicture[ xC + i, yC + j ] >> bandShift ].

- The modified picture buffer saoPicture[ xC + i, yC + j ] is derived as follows.

saoPicture[ xC + i, yC + j ] = Clip3( 0, (1 << bitDepth) – 1, recPicture[ xC + i, yC + j ] + 
			SaoOffsetVal[ cIdx ][ rx ][ ry ][ bandIdx ] )	(8-5)

Table 8-1 – Specification of hPos and vPos according to the sample adaptive offset type

| sao_type_idx[ cIdx ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |