| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0214 WG11 Number: m25540 |
| --- | --- |

| Title: | AHG7: Cross-check report of disallow bi-predictive mode for 8x4 and 4x8 inter PUs (JCTVC-J0086) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result on disallow bi-predictive mode for 8x4 and 4x8 inter PUs proposed by TI in JCTVC-J0086. The experimental result perfectly matches with the one provided by TI in the rate-distortion performance.

# Introduction

JCTVC-J0086 [1] proposes to modify the CABAC binarization table of inter_pred_flag for 8x4 and 4x8 inter PUs to prohibit bi-predictive mode completely in the encoder and decoder side. In the proposed algorithm, the CABAC binarization table of inter_pred_flag for 8x4 and 4x8 inter PUs, and for inter PUs of size 8x8 and above is given in Table 1 and Table 2, respectively.

Table 1: CABAC binarization table of inter_pred_flag for 8x4 and 4x8 inter PUs

| inter_pred_flag | CABAC binarization table |
| --- | --- |
| Pred_L1 | 1 |
| Pred_L0 | 0 |

Table 2: CABAC binarization table of inter_pred_flag for inter PUs of size 8x8 and above (same as HM7.0)

| inter_pred_flag | CABAC binarization table |
| --- | --- |
| Pred_BI | 1 |
| Pred_L1 | 01 |
| Pred_L0 | 00 |

# Experimental Condition

The software to be tested was provided by TI. It was confirmed that the proposed method was implemented correctly. Its performance relative to the HM7.0 software was checked under the common test condition described in JCTVC-I1100 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

# Results

R-D performance and execution time of the experimental result is summarized in Table 3. Detailed result is included in the attached excel sheet. It was confirmed that the result perfectly matched with the one provided by TI.

Table 3: Experimental result of the proposed method in JCTVC-J0086

# Conclusion

The result of JCTVC-J0086 was confirmed. It seems desirable to clean up the current design with a modification such as described in JCTVC-J0086 in order to avoid a confusing design.

# References

- Minhua Zhou, “AHG7: Disallow bi-predictive mode for 8x4 and 4x8 inter PUs”, JCTVC-J0086, Stockholm, July 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, Geneva, April 2012.