| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0216r1 |
| --- | --- |

| Title: | AHG 9: Signalling slice index to detect lost slice earlier |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution discusses the possibility of detecting lost slices earlier when multiple slices are used. Since the second slice to the last slice are not easily distinguishable at parsing time, any lost that happen to these slices will not detected until they are all decoded and decoder realizes that the total number of decoded LCU does not match the number of LCU of a picture.

To improve current design, the following is proposed:

- Replace current syntax element first_slice_in_pic_flag (coded with u(1)) with slice_idx_in_pic (coded with ue(v)).

- Add a flag last_slice_in_pic_flag (coded with u(1))

The results show that the proposed modification affect BD-rate in average 0.1%Y, 0.1%U, 0.1%V for all intra and random access cases (AI-HE10, AI-Main, RAHE10, RA-Main), and 0%Y, 0%U, 0%V for low delay B cases.

# Introduction and Problem Statement

As per output of the 9th JCT-VC meeting, the following information is signalled in slice header:

- A flag called first_slice_in_pic_flag to indicate whether or not the slice is the first slice in a picture

- If the slice is not the first slice in a picture, then slice_address is signalled to specify the starting location of the slice within a picture.

The table of the above syntax elements is as the following:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| … |  |
| } |  |

Such design may have weakness in term of capability to detect lost of slices, particularly when slices are parsed and decoded in parallel. Detection of lost slice is possible only after all received slices in the frame have been parsed / decoded and the next received slice has different POC. Such issue may not be important in the past considering most decoders were running in sequential order. With the introduction of more processing cores which allows applications to parse and decode several slices at the same time, it may be desirable to improve slice lost detection capability at slice header parsing time.

Consider the following example. Suppose that a picture is coded into 4 slices as illustrated in the following figure. Note that each rectangle with number in the middle of it denotes LCU and different slices are marked with different lines’ color.

Figure 1 – Case example 1. A picture coded with 4 slices and the 2nd slice is lost.

With that configuration shown in the above figure, the signalling related to slice order is as follow:

- For the 1st slice: first_slice_in_pic_flag = 1

- For the 2nd slice: first_slice_in_pic_flag = 0; slice_address = 12

- For the 3rd slice: first_slice_in_pic_flag = 0; slice_address = 18

- For the 4th slice: first_slice_in_pic_flag = 0; slice_address = 30

Assume that the 2nd slice is lost during transmission so that decoder only receives 3 slices. These 3 slices should be the 1st slice, the 3rd slice, and the 4th slice. However, with current situation, it is difficult to know at parsing time that the 2nd slice is lost and the 2nd slice it receives is actually the 3rd slice, not the 2nd slice. The reason for that is because in slice header, there is no information about slice index so that it is difficult to tell which slice is not yet received.

Consider another example illustrated in Figure 2. Given the configuration shown in Figure 2, when the last slice of picture with POC 100 is lost, with current design, the lost can be detected only after both slice 0 and slice 1 of picture #100 have been decoded since after decoding, decoder then becomes aware that the total number of decoded LCU is less than the total number of LCU for a complete picture and the next received slice is from next picture.

Figure 2 – Case example 2. Two pictures coded with 4 slices and the last slice of the earlier picture is lost.

From both examples, it can be understood that the inability to detect lost at parsing time is because there is no information in slice header to indicate (1) slice index and (2) whether current slice is the last slice in a picture. If that information is available in slice header, it is possible to detect lost slice ealier, which is during parsing time, rather than having to wait until all received slices are decoded. Figure 3 illustrates how this early lost detection is possible. Since parsing process is much faster than actual decoding process, in many implementations, parser and decoder are separated as shown in the illustration. When lost occurs, by analyzing slice index and last slice indication, parser can detect which slice(s) not arrive yet / lost.

Figure 3 – Detecting lost slice ealier.

Having a feature to enable detecting lost slice earlier is beneficial in several ways such as:

- Decoder can decide whether or not to decode other received slices. In some application, sometime it is better to simply apply frame copy as strategy for error concealment. Thus, when lost slice can be detected earlier, there is not resource wasted for trying to decode other received slices.

- Decoder can try to conceal the lost slice earlier. For example, if the lost of the 2nd slice is detected earlier, error concealment may be started when decoders start decoding the second LCU row of the 1st slice (i.e., by Coder #1) and the first row of the 3rd slice (i.e., by Coder #2 or Coder #3).

# Proposal: Signalling slice index and indication of last slice

To improve slice header design to accommodate earlier detection of slice lost, we propose that:

- Instead of only signalling a flag to indicate whether or not a slice is the first slice of a picture, signal the order of the slice starting from 0 to the maximum number of slices in the picture minus 1.

- Signal one flag in slice header to indicate whether or not current slice is the last slice of a picture.

The necessary modification to accommodate the proposals is as the following:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| slice_idx_in_pic | ue(v) |
| last_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( slice_idx_in_pic > 0 ) |  |
| slice_address | u(v) |
| … |  |
| } |  |

slice_idx_in_pic specifies the index of current slice within a picture. Slice_idx_in_pic equals 0 indicates the first slice of a picture, slice_idx_in_pic equals n indicates the (n+1)th slice in a picture

last_slice_in_pic_flag indicates whether the slice is the last slice of the picture.

# Experiment results

We have tested the effect of the proposed modification to the coding performance. The anchor of such test is the common test condition with slice mode 2 and maximum 1500 bytes per slice. The results show that the proposed modification affect BD-rate in average 0.1%Y, 0.1%U, 0.1%V for all intra and random access cases (AI-HE10, AI-Main, RAHE10, RA-Main), and 0%Y, 0%U, 0%V for low delay B cases.

We thank Qualcomm. Inc. for cross-checking our experiment results.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Conclusion

When multiple slices are used, current slice header design can detect lost slice only after all slices of a picture have been decoded. In this contribution, a simple modification to current slice header can improve the ability of decoder to detect lost slice without having to wait until decoding process completed, instead detect it when parsing slice headers.

We recommend the JCT-VC group to discuss the described issue and consider adopting the proposed modification.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).