| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I1003_d5 |
| --- | --- |

| Title: | High efficiency video coding (HEVC) text specification draft 7 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Draft 7 of High efficiency video coding.

- CONTENTS

Page

Abstract	i

0	Introduction	1

0.1	Prologue	1

0.2	Purpose	1

0.3	Applications	1

0.4	Publication and versions of this Specification	1

0.5	Profiles and levels	2

0.6	Overview of the design characteristics	2

0.7	How to read this Specification	2

1	Scope	3

2	Normative references	3

2.1	General	3

2.2	Identical Recommendations | International Standards	3

2.3	Paired Recommendations | International Standards equivalent in technical content	3

2.4	Additional references	3

3	Definitions	3

4	Abbreviations	10

5	Conventions	10

5.1	Arithmetic operators	11

5.2	Logical operators	11

5.3	Relational operators	11

5.4	Bit-wise operators	11

5.5	Assignment operators	12

5.6	Range notation	12

5.7	Mathematical functions	12

5.8	Order of operation precedence	13

5.9	Variables, syntax elements, and tables	13

5.10	Text description of logical operations	14

5.11	Processes	15

6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	16

6.1	Bitstream formats	16

6.2	Source, decoded, and output picture formats	16

6.3	Spatial subdivision of pictures, slices and tiles	18

6.4	Derivation process for the availability of a coding block with a minimum coding block address	19

6.5	Scanning processes	20

6.5.1	Coding tree block raster and tile scanning conversation process	20

6.5.2	Z scanning array initialization process	21

6.5.3	Zig-zag scanning array initialization process	22

6.5.4	Up-right diagonal scanning array initialization process	22

6.5.5	Horizontal scanning array initialization process	23

6.5.6	Vertical scanning array initialization process	24

6.6	Luma inter transform block split direction derivation process	24

6.7	Chroma inter transform block split direction derivation process	25

7	Syntax and semantics	26

7.1	Method of specifying syntax in tabular form	26

7.2	Specification of syntax functions and descriptors	27

7.3	Syntax in tabular form	28

7.3.1	NAL unit syntax	28

7.3.2	Raw byte sequence payloads and RBSP trailing bits syntax	29

7.3.2.1	Video parameter set RBSP syntax	29

7.3.2.2	Sequence parameter set RBSP syntax	30

7.3.2.3	Picture parameter set RBSP syntax	32

7.3.2.4	Scaling list data syntax	33

7.3.2.5	Scaling list syntax	33

7.3.2.6	Adaptation parameter set RBSP syntax	34

7.3.2.7	Adaptive loop filter picture info syntax	34

7.3.2.8	Supplemental enhancement information RBSP syntax	34

7.3.2.9	Access unit delimiter RBSP syntax	35

7.3.2.10	Filler data RBSP syntax	35

7.3.2.11	Slice layer RBSP syntax	36

7.3.2.12	RBSP slice trailing bits syntax	36

7.3.2.13	RBSP trailing bits syntax	36

7.3.2.14	Byte alignment syntax	36

7.3.3	Slice header syntax	37

7.3.3.1	Short-term reference picture set syntax	39

7.3.3.2	Reference picture list modification syntax	40

7.3.3.3	Prediction weight table syntax	41

7.3.4	Slice data syntax	42

7.3.4.1	Sample adaptive offset parameter syntax	43

7.3.5	Coding tree syntax	44

7.3.6	Coding unit syntax	45

7.3.7	Prediction unit syntax	47

7.3.7.1	Motion vector difference coding syntax	48

7.3.7.2	PCM sample syntax	48

7.3.8	Transform tree syntax	49

7.3.9	Transform unit syntax	51

7.3.10	Residual coding syntax	52

7.4	Semantics	54

7.4.1	NAL unit semantics	54

7.4.1.1	Encapsulation of an SODB within an RBSP (informative)	57

7.4.1.2	Order of NAL units and association to coded pictures, access units, and video sequences	58

7.4.2	Raw byte sequence payloads and RBSP trailing bits semantics	61

7.4.2.1	Video parameter set RBSP semantics	61

7.4.2.2	Sequence parameter set RBSP semantics	62

7.4.2.3	Picture parameter set RBSP semantics	67

7.4.2.4	Scaling list data semantics	70

7.4.2.5	Scaling list semantics	71

7.4.2.6	Adaptation parameter set RBSP semantics	73

7.4.2.7	Adaptive loop filter picture info semantics	73

7.4.2.8	Supplemental enhancement information RBSP semantics	74

7.4.2.9	Access unit delimiter RBSP semantics	74

7.4.2.10	Filler data RBSP semantics	75

7.4.2.11	Slice layer RBSP semantics	75

7.4.2.12	RBSP slice trailing bits semantics	75

7.4.2.13	RBSP trailing bits semantics	75

7.4.2.14	Byte alignment semantics	75

7.4.3	Slice header semantics	75

7.4.3.1	Short-term reference picture set semantics	79

7.4.3.2	Reference picture list modification semantics	82

7.4.3.3	Weighted prediction parameters semantics	82

7.4.4	Slice data semantics	83

7.4.4.1	Sample adaptive offset parameter semantics	83

7.4.5	Coding tree semantics	84

7.4.6	Coding unit semantics	84

7.4.7	Prediction unit semantics	86

7.4.7.1	Motion vector difference coding semantics	87

7.4.8	Transform tree semantics	87

7.4.9	Transform coefficient semantics	88

7.4.10	Residual coding semantics	89

8	Decoding process	91

8.1	General [Ed. (GJS): Create similar subclauses for other places where there is "orphaned" text that is at the parent level of a section that contains subclauses.]	91

8.2	NAL unit decoding process	92

8.3	Slice decoding process	92

8.3.1	Decoding process for picture order count	92

8.3.2	Decoding process for reference picture set	93

8.3.4	Decoding process for reference picture lists construction	97

8.3.4.1	General	97

8.3.4.2	Initialization process for reference picture lists	97

8.3.5	Marking of the current picture after decoding	98

8.4	Decoding process for coding units coded in intra prediction mode	98

8.4.1	Derivation process for luma intra prediction mode	99

8.4.2	Derivation process for chroma intra prediction mode	101

8.4.3	Decoding process for intra blocks	102

8.4.3.1	Intra sample prediction	103

8.5	Decoding process for coding units coded in inter prediction mode	110

8.5.1	Inter prediction process	111

8.5.2	Decoding process for prediction units in inter prediction mode	113

8.5.2.1	Derivation process for motion vector components and reference indices	114

8.5.2.2	Decoding process for inter prediction samples	126

8.5.3	Decoding process for the residual signal of coding units coded in inter prediction mode	134

8.5.3.1	Decoding process for luma residual blocks	135

8.5.3.2	Decoding process for chroma residual blocks	136

8.6	Scaling, transformation and array construction process prior to deblocking filter process	138

8.6.1	Derivation process for quantization parameters	138

8.6.2	Scaling and transformation process	139

8.6.3	Scaling process for transform coefficients	140

8.6.4	Transformation process for scaled transform coefficients	140

8.6.4.1	Transformation process	142

8.6.5	Picture construction process prior to in-loop filter process	143

8.7	In-loop filter process	143

8.7.1	General	143

8.7.2	Deblocking filter process	144

8.7.2.1	Derivation process of transform block boundary	145

8.7.2.2	Derivation process of prediction block boundary	146

8.7.2.3	Derivation process of boundary filtering strength	147

8.7.2.4	Edge filtering process	148

8.7.3	Sample adaptive offset process	157

8.7.3.1	Coding tree block modification process	157

8.7.4	Adaptive loop filter process	158

8.7.4.1	Derivation process for luma filter coefficients	159

8.7.4.2	Derivation process for chroma filter coefficients	159

8.7.4.3	Coding tree block filtering process for luma samples	160

8.7.4.4	Coding tree block filtering process for chroma samples	161

8.7.4.5	Coding block boundary padding process	162

9	Parsing process	166

9.1	Parsing process for Truncated Unary codes	166

9.2	Parsing process for Exp-Golomb codes	166

9.2.1	Parsing process for 0-th order Exp-Golomb codes	166

9.2.1.1	Mapping process for signed Exp-Golomb codes	167

9.2.2	Parsing process for k-th order Exp-Golomb codes	168

9.3	CABAC parsing process for slice data	169

9.3.1	Initialization process	170

9.3.1.1	Initialization process for context variables	170

9.3.1.2	Memorization process for context variables	177

9.3.1.3	Synchronization process for context variables	177

9.3.1.4	Initialization process for the arithmetic decoding engine	177

9.3.2	Binarization process	178

9.3.2.1	Unary (U) binarization process	183

9.3.2.2	Truncated unary (TU) binarization process	184

9.3.2.3	Truncated Rice (TR) binarization process	184

9.3.2.4	k-th order Exp-Golomb (EGk) binarization process	184

9.3.2.5	Fixed-length (FL) binarization process	185

9.3.2.6	Binarization process for cu_qp_delta	185

9.3.2.7	Binarization process for part_mode	185

9.3.2.8	Binarization process for coeff_abs_level_remaining	186

9.3.2.9	Binarization process for intra_chroma_pred_mode	187

9.3.3	Decoding process flow	187

9.3.3.1	Derivation process for ctxIdx	188

9.3.3.2	Arithmetic decoding process	197

9.3.4	Arithmetic encoding process (informative)	204

9.3.4.1	Initialization process for the arithmetic encoding engine (informative)	204

9.3.4.2	Encoding process for a binary decision (informative)	205

9.3.4.3	Renormalization process in the arithmetic encoding engine (informative)	206

9.3.4.4	Bypass encoding process for binary decisions (informative)	208

9.3.4.5	Encoding process for a binary decision before termination (informative)	209

9.3.4.6	Byte stuffing process (informative)	211

10	Specification of bitstream subsets	211

10.1	Sub-bitstream extraction process	211

Annex A  Profiles and levels	212

A.1	Overview of profiles and levels	212

A.2	Requirements on video decoder capability	212

A.3	Profiles	212

A.3.1	General	212

A.3.2	Main profile	212

A.4	Levels	213

A.4.1	General	213

A.4.2	Profile-specific level limits for the Main profile	214

A.4.3	Effect of level limits on picture rate (informative)	215

Annex B  Byte stream format	218

B.1	Byte stream NAL unit syntax and semantics	218

B.1.1	Byte stream NAL unit syntax	218

B.1.2	Byte stream NAL unit semantics	218

B.2	Byte stream NAL unit decoding process	219

B.3	Decoder byte-alignment recovery (informative)	219

Annex C  Hypothetical reference decoder	220

C.1	General	220

C.2	Operation of coded picture buffer (CPB)	222

C.2.1	Timing of bitstream arrival	222

C.2.2	Timing of decoding unit removal and decoding of decoding unit	224

C.3	Operation of the decoded picture buffer (DPB)	224

C.4	Bitstream conformance	226

C.5	Decoder conformance	227

C.5.1	Operation of the output order DPB	228

C.5.2	Removal of pictures from the DPB	228

C.5.2.1	"Bumping" process	229

C.5.3	Picture decoding, marking and storage	229

Annex D  Supplemental enhancement information	230

D.1	SEI payload syntax	231

D.1.1	Buffering period SEI message syntax	232

D.1.2	Picture timing SEI message syntax	232

D.1.3	Pan-scan rectangle SEI message syntax	233

D.1.4	Filler payload SEI message syntax	233

D.1.5	User data registered by Rec. ITU-T T.35 SEI message syntax	233

D.1.6	User data unregistered SEI message syntax	233

D.1.7	Recovery point SEI message syntax	233

D.1.8	Scene information SEI message syntax	233

D.1.9	Full-frame snapshot SEI message syntax	233

D.1.10	Progressive refinement segment start SEI message syntax	233

D.1.11	Progressive refinement segment end SEI message syntax	233

D.1.12	Film grain characteristics SEI message syntax	233

D.1.13	Deblocking filter display preference SEI message syntax	233

D.1.14	Post-filter hint SEI message syntax	233

D.1.15	Tone mapping information SEI message syntax	233

D.1.16	Frame packing arrangement SEI message syntax	233

D.1.17	Display orientation SEI message syntax	234

D.1.18	SOP description SEI message syntax	234

D.1.19	Field indication SEI message syntax	234

D.1.20	Decoded picture hash SEI message syntax	235

D.1.21	Reserved SEI message syntax	235

D.2	SEI payload semantics	235

D.2.1	Buffering period SEI message semantics	235

D.2.2	Picture timing SEI message semantics	236

D.2.3	Pan-scan rectangle SEI message semantics	237

D.2.4	Filler payload SEI message semantics	237

D.2.5	User data registered by ITU-T Rec. T.35 SEI message semantics	237

D.2.6	User data unregistered SEI message semantics	237

D.2.7	Recovery point SEI message semantics	237

D.2.8	Scene information SEI message semantics	238

D.2.9	Full-frame snapshot SEI message semantics	238

D.2.10	Progressive refinement segment start SEI message semantics	238

D.2.11	Progressive refinement segment end SEI message semantics	238

D.2.12	Film grain characteristics SEI message semantics	238

D.2.13	Deblocking filter display preference SEI message semantics	238

D.2.14	Post-filter hint SEI message semantics	238

D.2.15	Tone mapping information SEI message semantics	238

D.2.16	Frame packing arrangement SEI message semantics	238

D.2.17	Display orientation SEI message semantics	239

D.2.18	SOP description SEI message semantics	240

D.2.19	Field indication SEI message semantics	240

D.2.20	Decoded picture hash SEI message semantics	243

D.2.21	Reserved SEI message semantics	244

Annex E  Video usability information	246

E.1	VUI syntax	247

E.1.1	VUI parameters syntax	247

E.1.2	HRD parameters syntax	248

E.2	VUI semantics	248

E.2.1	VUI parameters semantics	248

E.2.2	HRD parameters semantics	258

- List of figures

Figure 6-1 – Nominal vertical and horizontal locations of 4:2:0 luma and chroma samples in a picture [Ed. Re-draw figure]	17

Figure 6-2 – Nominal vertical and horizontal locations of 4:2:2 luma and chroma samples in a picture [Ed. Re-draw figure]	17

Figure 6-3 – Nominal vertical and horizontal locations of 4:4:4 luma and chroma samples in a picture [Ed. Re-draw figure]	18

Figure 6-4 – A picture with 11 by 9 coding tree blocks that is partitioned into two slices (informative) [Ed. Re-draw figure]	19

Figure 6-5 – A picture with 13 by 8 coding tree blocks that is partitioned into three tiles (informative)	19

Figure 7-1 – Structure of an access unit not containing any NAL units with nal_unit_type equal to 0, , or 30, or in the ranges of 9..24, 25..28, or 32..63	61

Figure 8-1 – Intra prediction mode directions (informative)	100

Figure 8-2 – Intra prediction angle definition (informative)	107

Figure 8-3 – Spatial motion vector neighbours	122

Figure 8-4 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation	128

Figure 8-5 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation	130

Figure 9-1 – Spatial neighbour T that is used to invoke the coding tree block availability derivation process relative to the current coding tree block (informative)	169

Figure 9-2 – Overview of the arithmetic decoding process for a single bin (informative)	198

Figure 9-3 – Flowchart for decoding a decision	200

Figure 9-4 – Flowchart of renormalization	203

Figure 9-5 – Flowchart of bypass decoding process	203

Figure 9-6 – Flowchart of decoding a decision before termination	204

Figure 9-7 – Flowchart for encoding a decision	206

Figure 9-8 – Flowchart of renormalization in the encoder	207

Figure 9-9 – Flowchart of PutBit(B)	208

Figure 9-10 – Flowchart of encoding bypass	209

Figure 9-11 – Flowchart of encoding a decision before termination	210

Figure 9-12 – Flowchart of flushing at termination	211

Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks [Ed. (GJS): Fix figure numbering in annex(es)]	220

Figure C-2 – HRD buffer model	221

Figure D-1 – Nominal vertical and horizontal sampling locations of 4:2:0 samples in top and bottom fields	242

Figure D-2 – Nominal vertical and horizontal sampling locations of 4:2:2 samples in top and bottom fields	242

Figure D-3 – Nominal vertical and horizontal sampling locations of 4:4:4 samples in top and bottom fields	243

Figure E-1 – Location of chroma samples for top and bottom fields for chroma_format_idc equal to 1 (4:2:0 chroma format) as a function of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field	255

- List of tables

Table 5-1 – Operation precedence from highest (at top of table) to lowest (at bottom of table)	13

Table 6-1 – SubWidthC, and SubHeightC values derived from  chroma_format_idc and separate_colour_plane_flag	16

Table 7-1 – NAL unit type codes and NAL unit type classes	54

Table 7-2 – Specification of SizeID	71

Table 7-3 – Specification of MatrixID according to SizeID, prediction type and colour component	71

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15	72

Table 7-5 – Specification of default values of ScalingList[ 1..3 ][ MatrixID ][ 0 ][ i ] with i=0..63	72

Table 7-6 – Meaning of pic_type	75

Table 7-7 – Name association to slice_type	76

Table 7-8 – used_by_curr_pic_flag[ j ] and use_delta_flag[ j ] operations for the reference picture set	80

Table 7-9 – Specification of the SAO type	83

Table 7-10 – Name association to prediction mode and partitioning type	86

Table 7-11 – Name association to inter prediction mode	86

Table 7-12 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]	89

Table 8-1 – Specification of intra prediction mode and associated names	99

Table 8-2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1	102

Table 8-3 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0	102

Table 8-4 – Specification of intraHorVerDistThres[ nS ] for various prediction unit sizes	105

Table 8-5 – Specification of intraPredAngle	107

Table 8-6 – Specification of invAngle	108

Table 8-7 – Specification of lmDiv	110

Table 8-8 – Specification of l0CandIdx and l1CandIdx	119

Table 8-9 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]	129

Table 8-10 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively	131

Table 8-11 – Specification of QPC as a function of qPI	139

Table 8-12 – Specification of horizTrType and vertTrType	141

Table 8-13 – Derivation of threshold variables β′ and tC′ from input Q	156

Table 8-14 – Specification of hPos and vPos according to the sample adaptive offset type	158

Table 8-15 – Specification of horPos[ i ] for adaptive loop filter process	161

Table 8-16 – Specification of verPos[ i ] for adaptive loop filter process	161

Table 9-1 – Bit strings with "prefix" and "suffix" bits and assignment to codeNum ranges (informative)	167

Table 9-2 – Exp-Golomb bit strings and codeNum in explicit form and used as ue(v) (informative)	167

Table 9-4 – Association of ctxIdx and syntax elements for each initialisationType in the initialization process	171

Table 9-5 – Values of variable initValue for sao_merge_left_flag ctxIdx	172

Table 9-6 – Values of variable initValue for sao_merge_up_flag ctxIdx	172

Table 9-7 – Values of variable initValue for sao_type_idx ctxIdx	172

Table 9-8 – Values of variable initValue for sao_offset_abs ctxIdx	172

Table 9-9 – Values of variable initValue for alf_cu_flag ctxIdx	172

Table 9-10 – Values of variable initValue for split_coding_unit_flag ctxIdx	172

Table 9-11 – Values of variable initValue for cu_transquant_bypass_flag ctxIdx	173

Table 9-12 – Values of variable initValue for skip_flag ctxIdx	173

Table 9-13 – Values of variable initValue for cu_qp_delta ctxIdx	173

Table 9-14 – Values of variable initValue for pred_mode and part_mode	173

Table 9-15 – Values of variable initValue for prev_intra_luma_pred_flag ctxIdx	173

Table 9-16 – Values of variable initValue for intra_chroma_pred_mode ctxIdx	173

Table 9-17 – Value of variable initValue for merge_flag ctxIdx	174

Table 9-18 – Values of variable initValue for merge_idx ctxIdx	174

Table 9-19 – Values of variable initValue for inter_pred_idc ctxIdx	174

Table 9-20 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx	174

Table 9-21 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx	174

Table 9-22 – Values of variable initValue for mvp_l0_flag, mvp_l1_flag ctxIdx	174

Table 9-23 – Values of variable initValue for no_residual_data_flag ctxIdx	175

Table 9-24 – Values of variable initValue for split_transform_flag ctxIdx	175

Table 9-25 – Values of variable initValue for cbf_luma ctxIdx	175

Table 9-26 – Values of variable initValue for cbf_cb and cbf_cr ctxIdx	175

Table 9-27 – Values of variable initValue for transform_skip_flag ctxIdx	175

Table 9-28 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx	175

Table 9-29 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx	176

Table 9-30 – Values of variable initValue for significant_coeff_group_flag ctxIdx	176

Table 9-31 – Values of variable initValue for significant_coeff_flag ctxIdx	176

Table 9-32 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx	177

Table 9-33 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx	177

Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset	178

Table 9-35 – Bin string of the unary binarization (informative)	184

Table 9-36 – Binarization for part_mode	186

Table 9-37 – Specifcation of cTRMax depending on cRiceParam	186

Table 9-38 – Specifcation of intra_chroma_pred_mode depending on chroma_pred_from_luma_enabled_flag	187

Table 9-39 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values	189

Table 9-40 – Specifcation of ctxIdxInc using left and above syntax elements	193

Table 9-41 – Specifcation of lastCtx[ i ]	193

Table 9-42 – Specifcation of ctxIdxMap4x4[ i ]	195

Table 9-43 – Specifcation of ctxIdxMap8x8[ i ]	195

Table 9-44 – Specification of rangeTabLPS depending on pStateIdx and qCodIRangeIdx	201

Table 9-45 – State transition table	202

Table A-10-1 – Level limits	214

Table A-10-2 – Specification of cpbBrVclFactor and cpbBrNalFactor	215

Table A-10-3 – SliceRate limits for Main profile	215

Table A-10-4 – Maximum picture rates (pictures per second) at level 1 to 4.3 for some example picture sizes	216

Table A-10-5 – Maximum picture rates (pictures per second) at level 5 to 6.2 for some example picture sizes	217

Table D-10-6 – Indicated interpretation of field_pic_flag and progressive_source_flag	241

Table D-10-7 – Interpretation of hash_type	244

Table E-10-8 – Meaning of sample aspect ratio indicator	249

Table E-10-9 – Colour primaries	250

Table E-10-10 – Transfer characteristics	251

Table E-10-11 – Matrix coefficients	254

Foreword

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardising telecommunications on a world-wide basis. The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups that, in turn, produce Recommendations on these topics. The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1. In some areas of information technology that fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialised system for world-wide standardization. National Bodies that are members of ISO and IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75% of the national bodies casting a vote.

This Recommendation | International Standard was prepared jointly by ITU-T SG 16 Q.6, also known as VCEG (Video Coding Experts Group), and by ISO/IEC JTC 1/SC 29/WG 11, also known as MPEG (Moving Picture Experts Group). VCEG was formed in 1997 to maintain prior ITU-T video coding standards and develop new video coding standard(s) appropriate for a wide range of conversational and non-conversational services. MPEG was formed in 1988 to establish standards for coding of moving pictures and associated audio for various applications such as digital storage media, distribution, and communication.

In this Recommendation | International Standard Annexes Error: Reference source not found through Error: Reference source not found contain normative requirements and are an integral part of this Recommendation | International Standard.

INTERNATIONAL  STANDARD

ITU-T  RECOMMENDATION

High efficiency video coding

# Introduction

This clause does not form an integral part of this Recommendation | International Standard.

## Prologue

This subclause does not form an integral part of this Recommendation | International Standard.

As the costs for both processing power and memory have reduced, network support for coded video data has diversified, and advances in video coding technology have progressed, the need has arisen for an industry standard for compressed video representation with substantially increased coding efficiency and enhanced robustness to network environments. Toward these ends the ITU-T Video Coding Experts Group (VCEG) and the ISO/IEC Moving Picture Experts Group (MPEG) formed a Joint Collaborative Team on Video Coding (JCT-VC) in 2010 for development of a new Recommendation | International Standard. This Recommendation | International Standard was developed in the JCT-VC.

## Purpose

This subclause does not form an integral part of this Recommendation | International Standard.

This Recommendation | International Standard was developed in response to the growing need for higher compression of moving pictures for various applications such as videoconferencing, digital storage media, television broadcasting, internet streaming, and communication. It is also designed to enable the use of the coded video representation in a flexible manner for a wide variety of network environments as well as to enable the use of multi-core parallel encoding and decoding devices. The use of this Recommendation | International Standard allows motion video to be manipulated as a form of computer data and to be stored on various storage media, transmitted and received over existing and future networks and distributed on existing and future broadcasting channels.

## Applications

This subclause does not form an integral part of this Recommendation | International Standard.

This Recommendation | International Standard is designed to cover a broad range of applications for video content including but not limited to the following:

CATV	Cable TV on optical networks, copper, etc.

DBS	Direct broadcast satellite video services

DSL 	Digital subscriber line video services

DTTB	Digital terrestrial television broadcasting

ISM	Interactive storage media (optical disks, etc.)

MMM	Multimedia mailing

MSPN	Multimedia services over packet networks (video streaming, etc.)

RTC	Real-time conversational services (videoconferencing, videophone, etc.)

RVS	Remote video surveillance

SSM	Serial storage media (digital VTR, etc.)

## Publication and versions of this Specification

This subclause does not form an integral part of this Recommendation | International Standard.

This Specification has been jointly developed by ITU-T Video Coding Experts Group (VCEG) and the ISO/IEC Moving Picture Experts Group (MPEG). It is published as technically-aligned twin text in both organizations ITU-T and ISO/IEC.

## Profiles and levels

This subclause does not form an integral part of this Recommendation | International Standard.

This Recommendation | International Standard is designed to be generic in the sense that it serves a wide range of applications, bit rates, resolutions, qualities, and services. Applications should cover, among other things, digital storage media, television broadcasting and real-time communications. In the course of creating this Specification, various requirements from typical applications have been considered, necessary algorithmic elements have been developed, and these have been integrated into a single syntax. Hence, this Specification will facilitate video data interchange among different applications.

Considering the practicality of implementing the full syntax of this Specification, however, a limited number of subsets of the syntax are also stipulated by means of "profiles" and "levels". These and other related terms are formally defined in clause 3.

A "profile" is a subset of the entire bitstream syntax that is specified by this Recommendation | International Standard. Within the bounds imposed by the syntax of a given profile it is still possible to require a very large variation in the performance of encoders and decoders depending upon the values taken by syntax elements in the bitstream such as the specified size of the decoded pictures. In many applications, it is currently neither practical nor economic to implement a decoder capable of dealing with all hypothetical uses of the syntax within a particular profile.

In order to deal with this problem, "levels" are specified within each profile. A level is a specified set of constraints imposed on values of the syntax elements in the bitstream. These constraints may be simple limits on values. Alternatively they may take the form of constraints on arithmetic combinations of values (e.g., picture width multiplied by picture height multiplied by number of pictures decoded per second).

Coded video content conforming to this Recommendation | International Standard uses a common syntax. In order to achieve a subset of the complete syntax, flags, parameters, and other syntax elements are included in the bitstream that signal the presence or absence of syntactic elements that occur later in the bitstream.

## Overview of the design characteristics

This subclause does not form an integral part of this Recommendation | International Standard.

The coded representation specified in the syntax is designed to enable a high compression capability for a desired image or video quality. The algorithm is typically not lossless, as the exact source sample values are typically not preserved through the encoding and decoding processes. A number of techniques may be used to achieve highly efficient compression. Encoding algorithms (not specified in this Recommendation | International Standard) may select between inter and intra coding for block-shaped regions of each picture. Inter coding uses motion vectors for block-based inter prediction to exploit temporal statistical dependencies between different pictures. Intra coding uses various spatial prediction modes to exploit spatial statistical dependencies in the source signal for a single picture. Motion vectors and intra prediction modes may be specified for a variety of block sizes in the picture. The prediction residual is then further compressed using a transform to remove spatial correlation inside the transform block before it is quantized, producing an irreversible process that typically discards less important visual information while forming a close approximation to the source samples. Finally, the motion vectors or intra prediction modes are combined with the quantized transform coefficient information and encoded using arithmetic coding.

## How to read this Specification

This subclause does not form an integral part of this Recommendation | International Standard.

It is suggested that the reader starts with clause 1 (Scope) and moves on to clause 3 (Definitions). Clause 6 should be read for the geometrical relationship of the source, input, and output of the decoder. Clause 7 (Syntax and semantics) specifies the order to parse syntax elements from the bitstream. See subclauses 7.1-7.3 for syntactical order and see subclause 7.3.10 for semantics; i.e., the scope, restrictions, and conditions that are imposed on the syntax elements. The actual parsing for most syntax elements is specified in clause 9 (Parsing process). Finally, clause 7.4.10 specifies how the syntax elements are mapped into decoded samples. Throughout reading this Specification, the reader should refer to clauses 2 (Normative references), 4 (Abbreviations), and 5 (Conventions) as needed. Annexes Error: Reference source not found through Error: Reference source not found also form an integral part of this Recommendation | International Standard.

Annex Error: Reference source not found specifies profiles each being tailored to certain application domains, and defines the so-called levels of the profiles. Annex Error: Reference source not found specifies syntax and semantics of a byte stream format for delivery of coded video as an ordered stream of bytes. Annex Error: Reference source not found specifies the hypothetical reference decoder and its use to check bitstream and decoder conformance. Annex Error: Reference source not found specifies syntax and semantics for supplemental enhancement information message payloads. Annex Error: Reference source not found specifies syntax and semantics of the video usability information parameters of the sequence parameter set.

Throughout this Specification, statements appearing with the preamble "NOTE –" are informative and are not an integral part of this Recommendation | International Standard.

# Scope

This document specifies High efficiency video coding.

# Normative references

## General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations. [Ed. TW: revise the following.]

## Identical Recommendations | International Standards

–	None.

## Paired Recommendations | International Standards equivalent in technical content

–	ITU-T H.264 (in force), Advanced video coding for generic audiovisual services.

ISO/IEC 14496-10: in force, Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding.

## Additional references

–	ITU-T Recommendation T.35 (2000), Procedure for the allocation of ITU-T defined codes for non-standard facilities.

–	ISO/IEC 11578:1996, Annex A, Universal Unique Identifier.

–	ISO/CIE 10527:2007, Colorimetric Observers.

# Definitions

[Ed. (TW) adapted definitions so far. Needs more work including turning them into 1 sentence each.]

For the purposes of this Recommendation | International Standard, the following definitions apply:

  - access unit: A set of NAL units that are consecutive in decoding order and contain exactly one coded picture. In addition to the coded slice NAL units of the coded picture, the access unit may also contain other NAL units not containing slices of the coded picture. The decoding of an access unit always results in a decoded picture.

  - AC transform coefficient: Any transform coefficient for which the frequency index in one or both dimensions is non-zero.

  - adaptive binary arithmetic decoding process: An entropy decoding process that derives the values of bins from a bitstream produced by an adaptive binary arithmetic encoding process.

  - adaptive binary arithmetic encoding process: An entropy encoding process, not normatively specified in this Recommendation | International Standard, that codes a sequence of bins and produces a bitstream that can be decoded using the adaptive binary arithmetic decoding process.

  - B slice: A slice that may be decoded using intra prediction or inter prediction using at most two motion vectors and reference indices to predict the sample values of each block.

  - bin: One bit of a bin string.

  - binarization: A set of bin strings for all possible values of a syntax element.

  - binarization process: A unique mapping process of all possible values of a syntax element onto a set of bin strings.

  - bin string: A string of bins. A bin string is an intermediate binary representation of values of syntax elements from the binarization of the syntax element.

  - bi-predictive slice: See B slice.

  - bitstream: A sequence of bits that forms the representation of coded pictures and associated data forming one or more coded video sequences. Bitstream is a collective term used to refer either to a NAL unit stream or a byte stream.

  - block: An MxN (M-column by N-row) array of samples, or an MxN array of transform coefficients.

  - broken link: A location in a bitstream at which it is indicated that some subsequent pictures in decoding order may contain serious visual artefacts due to unspecified operations performed in the generation of the bitstream.

  - broken link access (BLA) unit: An access unit in which the coded picture is a BLA picture.

  - broken link access (BLA) picture: A RAP picture for which each slice has nal_unit_type equal to 6 or 7.

  - byte: A sequence of 8 bits, written and read with the most significant bit on the left and the least significant bit on the right. When represented in a sequence of data bits, the most significant bit of a byte is first.

  - byte-aligned: A position in a bitstream is byte-aligned when the position is an integer multiple of 8 bits from the position of the first bit in the bitstream. A bit or byte or syntax element is said to be byte-aligned when the position at which it appears in a bitstream is byte-aligned.

  - byte stream: An encapsulation of a NAL unit stream containing start code prefixes and NAL units as specified in Annex Error: Reference source not found.

  - can: A term used to refer to behaviour that is allowed, but not necessarily required.

  - chroma: An adjective specifying that a sample array or single sample is representing one of the two colour difference signals related to the primary colours. The symbols used for a chroma array or sample are Cb and Cr.

NOTE – The term chroma is used rather than the term chrominance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term chrominance.

  - clean random access (CRA) access unit: An access unit in which the coded picture is a CRA picture.

  - clean random access (CRA) picture: A RAP picture for which each slice has nal_unit_type equal to 4 or 5.

  - coded picture: A coded representation of a picture containing all coding tree units of the picture.

  - coded picture buffer (CPB): A first-in first-out buffer containing access units in decoding order specified in the hypothetical reference decoder in Annex Error: Reference source not found.

  - coded representation: A data element as represented in its coded form.

  - coded slice NAL unit: A NAL unit containing a slice.

  - coded video sequence: A sequence of access units that consists, in decoding order, of a CRA access unit that is the first access unit in the bitstream, an IDR access unit or a BLA access unit, followed by zero or more non-IDR and non-BLA access units including all subsequent access units up to but not including any subsequent IDR or BLA access unit.

  - coding block: A square NxN block of samples. The division of a coding tree block into coding blocks is a partitioning.

  - coding tree block: A square NxN block of samples. The division of one of the arrays that make up a picture that has three sample arrays or of the array that make up a picture in monochrome format or a picture that is coded using three separate colour planes into coding tree blocks is a partitioning.

  - coding tree unit: A coding tree block of luma samples, two corresponding coding tree blocks of chroma samples of a picture that has three sample arrays, or a coding tree block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to code the samples. The division of a slice into coding tree units is a partitioning.

  - coding unit: A coding block of luma samples, two corresponding coding blocks of chroma samples of a picture that has three sample arrays, or a coding block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to code the samples. The division of a coding tree unit into coding units is a partitioning.

  - column: An integer number of coding tree blocks. Columns are delineated from one another by vertical boundaries that extend from the top boundary to the bottom boundary of the picture and are ordered consecutively from left to right in the picture. The division of each picture into columns is a partitioning.

  - component: An array or single sample from one of the three arrays (luma and two chroma) that make up a picture in 4:2:0, 4:2:2, or 4:4:4 colour format or the array or a single sample of the array that make up a picture in monochrome format.

  - context variable: A variable specified for the adaptive binary arithmetic decoding process of a bin by an equation containing recently decoded bins.

  - DC transform coefficient: A transform coefficient for which the frequency index is zero in all dimensions.

  - decoded picture: A decoded picture is derived by decoding a coded picture.

  - decoded picture buffer (DPB): A buffer holding decoded pictures for reference, output reordering, or output delay specified for the hypothetical reference decoder in Annex Error: Reference source not found.

  - decoder: An embodiment of a decoding process.

  - decoder under test (DUT): A decoder that is tested for conformance to this Recommendation | International Standard by operating the hypothetical stream scheduler to deliver a conforming bitstream to the decoder and to the hypothetical reference decoder and comparing the values and timing of the output of the two decoders.

  - decoding order: The order in which syntax elements are processed by the decoding process.

  - decoding process: The process specified in this Recommendation | International Standard that reads a bitstream and derives decoded pictures from it.

  - decoding unit: An access unit or a subset of an access unit. If SubPicCpbFlag is equal to 0, a decoding unit is an access unit. Otherwise, a decoding unit consists of one or more VCL NAL units in an access unit and the associated non-VCL NAL units. For the first VCL NAL unit in an access unit, the associated non-VCL NAL units are and the filler data NAL units, if any, immediately following the first VCL NAL unit and all non-VCL NAL units in the access unit that precede the first VCL NAL unit. For a VCL NAL unit that is not the first VCL NAL unit in an access unit, the associated non-VCL NAL units are the filler data NAL unit, if any, immediately following the VCL NAL unit.

  - display process: A process not specified in this Recommendation | International Standard having, as its input, the cropped decoded pictures that are the output of the decoding process.

  - emulation prevention byte: A byte equal to 0x03 that may be present within a NAL unit. The presence of emulation prevention bytes ensures that no sequence of consecutive byte-aligned bytes in the NAL unit contains a start code prefix.

  - encoder: An embodiment of an encoding process.

  - encoding process: A process, not specified in this Recommendation | International Standard, that produces a bitstream conforming to this Recommendation | International Standard.

  - flag: A variable that can take one of the two possible values 0 and 1.

  - frequency index: A one-dimensional or two-dimensional index associated with a transform coefficient prior to an inverse transform part of the decoding process.

  - hypothetical reference decoder (HRD): A hypothetical decoder model that specifies constraints on the variability of conforming NAL unit streams or conforming byte streams that an encoding process may produce.

  - hypothetical stream scheduler (HSS): A hypothetical delivery mechanism for the timing and data flow of the input of a bitstream into the hypothetical reference decoder. The HSS is used for checking the conformance of a bitstream or a decoder.

  - I slice: A slice that is decoded using intra prediction only.

  - informative: A term used to refer to content provided in this Recommendation | International Standard that is not an integral part of this Recommendation | International Standard. Informative content does not establish any mandatory requirements for conformance to this Recommendation | International Standard.

  - instantaneous decoding refresh (IDR) access unit: An access unit in which the coded picture is an IDR picture.

  - instantaneous decoding refresh (IDR) picture: A RAP picture for which each slice has nal_unit_type equal to 8.

  - inter coding: Coding of a block, slice, or picture that uses inter prediction.

  - inter prediction: A prediction derived from only data elements (e.g. sample value or motion vector) of reference pictures other than the current decoded picture.

  - intra coding: Coding of a block, slice, or picture that uses intra prediction.

  - intra prediction: A prediction derived from only data elements (e.g. sample value) of the same decoded slice.

  - intra slice: See I slice.

  - inverse transform: A part of the decoding process by which a set of transform coefficients are converted into spatial-domain values, or by which a set of transform coefficients are converted into DC transform coefficients.

  - layer: One of a set of syntactical structures in a non-branching hierarchical relationship. Higher layers contain lower layers. The coding layers are the coded video sequence, picture, slice, and coding tree layers.

  - leaf: A terminating node of a tree that is a root node of a tree of depth 0.

  - level: A defined set of constraints on the values that may be taken by the syntax elements and variables of this Recommendation | International Standard. The same set of levels is defined for all profiles, with most aspects of the definition of each level being in common across different profiles. Individual implementations may, within specified constraints, support a different level for each supported profile. In a different context, level is the value of a transform coefficient prior to scaling.

  - list 0 (list 1) motion vector: A motion vector associated with a reference index pointing into reference picture list 0 (list 1).

  - list 0 (list 1) prediction: Inter prediction of the content of a slice using a reference index pointing into reference picture list 0 (list 1).

  - luma: An adjective specifying that a sample array or single sample is representing the monochrome signal related to the primary colours. The symbol or subscript used for luma is Y or L.

NOTE – The term luma is used rather than the term luminance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term luminance. The symbol L is sometimes used instead of the symbol Y to avoid confusion with the symbol y as used for vertical location.

  - may: A term used to refer to behaviour that is allowed, but not necessarily required. In some places where the optional nature of the described behaviour is intended to be emphasized, the phrase "may or may not" is used to provide emphasis.

  - motion vector: A two-dimensional vector used for inter prediction that provides an offset from the coordinates in the decoded picture to the coordinates in a reference picture.

  - must: A term used in expressing an observation about a requirement or an implication of a requirement that is specified elsewhere in this Recommendation | International Standard. This term is used exclusively in an informative context.

  - NAL unit: A syntax structure containing an indication of the type of data to follow and bytes containing that data in the form of an RBSP interspersed as necessary with emulation prevention bytes.

  - NAL unit stream: A sequence of NAL units.

  - non-reference picture: A picture coded with nal_ref_flag equal to 0. A non-reference picture is not used for inter prediction of any other pictures.

  - note: A term used to prefix informative remarks. This term is used exclusively in an informative context.

  - output order: The order in which the decoded pictures are output from the decoded picture buffer in case the decoded pictures are to be output from the decoded picture buffer. The output order of a picture is specified by the picture order count value, regardless of whether the picture is to be output.

  - P slice: A slice that may be decoded using intra prediction or inter prediction using at most one motion vector and reference index to predict the sample values of each block.

  - parameter: A syntax element of a sequence parameter set or a picture parameter set. Parameter is also used as part of the defined term quantization parameter.

  - partitioning: The division of a set into subsets such that each element of the set is in exactly one of the subsets.

  - picture: A picture contains an array of luma samples in monochrome format or an array of luma samples and two corresponding arrays of chroma samples in 4:2:0, 4:2:2, and 4:4:4 colour format.

  - picture parameter set: A syntax structure containing syntax elements that apply to zero or more entire coded pictures as determined by the pic_parameter_set_id syntax element found in each slice header.

  - picture order count: A variable that is associated with each coded picture and has a value that is increasing with increasing picture position in output order relative to one of the following coded pictures:

- the previous IDR picture in decoding order, if any

- the previous BLA picture in decoding order, if any

- the previous CRA picture in decoding order, if any and if the previous CRA picture is the first coded picture in the bitstream in decoding order.

If more than one of the above coded pictures is present, the picture order count is relative to the last of such coded pictures in decoding order.

  - prediction: An embodiment of the prediction process.

  - prediction block: A rectangular MxN block of samples on which the same prediction is applied. The division of a coding block into prediction blocks is a partitioning.

  - prediction process: The use of a predictor to provide an estimate of the data element (e.g. sample value or motion vector) currently being decoded.

  - prediction unit: A prediction block of luma samples, two corresponding prediction blocks of chroma samples of a picture that has three sample arrays, or a prediction block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to predict the prediction block samples.

  - predictive slice: See P slice.

  - predictor: A combination of specified values or previously decoded data elements (e.g. sample value or motion vector) used in the decoding process of subsequent data elements.

  - profile: A specified subset of the syntax of this Recommendation | International Standard.

  - quadtree: A tree in which a parent node can be split into four child nodes. A child node may become parent node for another split into four child nodes.

  - quantization parameter: A variable used by the decoding process for scaling of transform coefficient levels.

  - random access: The act of starting the decoding process for a bitstream at a point other than the beginning of the stream.

  - random access point (RAP) access unit: An access unit in which the coded picture is a RAP picture.

  - random access point (RAP) picture: A coded picture containing only I slices and for which each slice has nal_unit_type in the range of 4 to 8, inclusive; a RAP picture may be a BLA picture, a CRA picture or an IDR picture; all coded pictures that follow the RAP picture both in decoding order and output order do not use inter prediction from any picture that precedes the RAP picture either in decoding order or output order; and any picture that precedes the RAP picture in decoding order also precedes the RAP picture in output order.

NOTE – If all preceding access units in decoding order are not present, provided that each parameter set referred to by a RAP picture and all subsequent coded pictures in decoding order is present before its activation, the RAP picture and all subsequent coded pictures in decoding order that also follow the RAP picture in output order can be correctly decoded. [Ed. (GJS/YK): The wording of this note may be improved.]

  - raster scan: A mapping of a rectangular two-dimensional pattern to a one-dimensional pattern such that the first entries in the one-dimensional pattern are from the first top row of the two-dimensional pattern scanned from left to right, followed similarly by the second, third, etc., rows of the pattern (going down) each scanned from left to right.

  - raw byte sequence payload (RBSP): A syntax structure containing an integer number of bytes that is encapsulated in a NAL unit. An RBSP is either empty or has the form of a string of data bits containing syntax elements followed by an RBSP stop bit and followed by zero or more subsequent bits equal to 0.

  - raw byte sequence payload (RBSP) stop bit: A bit equal to 1 present within a raw byte sequence payload (RBSP) after a string of data bits. The location of the end of the string of data bits within an RBSP can be identified by searching from the end of the RBSP for the RBSP stop bit, which is the last non-zero bit in the RBSP.

  - recovery point: A point in the bitstream at which the recovery of an exact or an approximate representation of the decoded pictures represented by the bitstream is achieved after a random access or broken link.

  - reference index: An index into a reference picture list.

  - reference picture: A picture with nal_ref_flag equal to 1. A reference picture contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

  - reference picture list: A list of reference pictures that is used for uni-prediction of a P or B slice. For the decoding process of a P slice, there is one reference picture list. For the decoding process of a B slice, there are two reference picture lists (list 0 and list 1).

  - reference picture list 0: A reference picture list used for inter prediction of a P or B slice. All inter prediction used for P slices uses reference picture list 0. Reference picture list 0 is one of two reference picture lists used for bi-prediction for a B slice, with the other being reference picture list 1.

  - reference picture list 1: A reference picture list used for bi-prediction of a B slice. Reference picture list 1 is one of two reference picture lists used for bi-prediction for a B slice, with the other being reference picture list 0.

  - reference picture set: A set of reference pictures associated with a picture, consisting of all reference pictures that are prior to the associated picture in decoding order, that may be used for inter prediction of the associated picture or any picture following the associated picture in decoding order.

  - reserved: The term reserved, when used in the clauses specifying some values of a particular syntax element, are for future use by ITU-T | ISO/IEC. These values shall not be used in bitstreams conforming to this Recommendation | International Standard, but may be used in future extensions of this Recommendation | International Standard by ITU-T | ISO/IEC.

  - residual: The decoded difference between a prediction of a sample or data element and its decoded value.

  - row: An integer number of coding tree blocks. Rows are delineated from one another by horizontal boundaries that extend from the left boundary to the right boundary of the picture and are ordered consecutively from top to bottom in the picture. The division of each picture into rows is a partitioning.

  - sample aspect ratio: Specifies, for assisting the display process, which is not specified in this Recommendation | International Standard, the ratio between the intended horizontal distance between the columns and the intended vertical distance between the rows of the luma sample array in a picture. Sample aspect ratio is expressed as h:v, where h is horizontal width and v is vertical height (in arbitrary units of spatial distance).

  - scaling: The process of multiplying transform coefficient levels by a factor, resulting in transform coefficients.

  - sequence parameter set: A syntax structure containing syntax elements that apply to zero or more entire coded video sequences as determined by the content of a seq_parameter_set_id syntax element found in the picture parameter set referred to by the pic_parameter_set_id syntax element found in each slice header.

  - shall: A term used to express mandatory requirements for conformance to this Recommendation | International Standard. When used to express a mandatory constraint on the values of syntax elements or on the results obtained by operation of the specified decoding process, it is the responsibility of the encoder to ensure that the constraint is fulfilled. When used in reference to operations performed by the decoding process, any decoding process that produces identical results to the decoding process described herein conforms to the decoding process requirements of this Recommendation | International Standard.

  - should: A term used to refer to behaviour of an implementation that is encouraged to be followed under anticipated ordinary circumstances, but is not a mandatory requirement for conformance to this Recommendation | International Standard.

  - slice: An integer number of coding tree blocks ordered consecutively in the raster scan. The division of each picture into slices is a partitioning. The coding tree block addresses are derived from the first coding tree block address in a slice (as represented in the slice header).

  - slice header: A part of a coded slice containing the data elements pertaining to the first or all coding blocks represented in the slice.

  - source: Term used to describe the video material or some of its attributes before encoding.

  - start code prefix: A unique sequence of three bytes equal to 0x000001 embedded in the byte stream as a prefix to each NAL unit. The location of a start code prefix can be used by a decoder to identify the beginning of a new NAL unit and the end of a previous NAL unit. Emulation of start code prefixes is prevented within NAL units by the inclusion of emulation prevention bytes.

  - string of data bits (SODB): A sequence of some number of bits representing syntax elements present within a raw byte sequence payload prior to the raw byte sequence payload stop bit. Within an SODB, the left-most bit is considered to be the first and most significant bit, and the right-most bit is considered to be the last and least significant bit.

  - syntax element: An element of data represented in the bitstream.

  - syntax structure: Zero or more syntax elements present together in the bitstream in a specified order.

  - tagged for discard (TFD) access unit: An access unit in which the coded picture is a TFD picture.

  - tagged for discard (TFD) picture: A coded picture for which each slice has nal_unit_type equal to 2; a TFD picture is associated with the previous CRA picture or BLA picture in decoding order and precedes the associated picture in output order; when the associated picture is a BLA picture, or when the associated picture is a CRA picture that is the first coded picture in the bitstream, the TFD picture may not be correctly decodable and is not output.

  - temporal layer access (TLA) unit: An access unit in which the coded picture is a TLA picture.

  - temporal layer access (TLA) picture: A coded picture for which each slice has nal_unit_type equal to 3.

  - tile: An integer number of coding tree blocks co-occurring in one column and one row, ordered consecutively in coding tree block raster scan of the tile. The division of each picture into tiles is a partitioning. Tiles in a picture are ordered consecutively in tile raster scan of the picture.

  - tile scan: A specific seqential ordering of coding tree blocks partitioning a picture. The tile scan order traverses the coding tree blocks in coding tree block raster scan in a tile. Although a slice contains coding tree blocks that are consecutive in coding tree block raster scan of a tile, these coding tree blocks are not necessarily consecutive in coding tree block raster scan of the picture.

  - transform block: A rectangular MxN block of samples on which the same transform is applied. The division of a coding block into transform blocks is a partitioning.

  - transform coefficient: A scalar quantity, considered to be in a frequency domain, that is associated with a particular one-dimensional or two-dimensional frequency index in an inverse transform part of the decoding process.

  - transform coefficient level: An integer quantity representing the value associated with a particular two-dimensional frequency index in the decoding process prior to scaling for computation of a transform coefficient value.

  - transform unit: A transform block of luma samples, two corresponding transform blocks of chroma samples of a picture that has three sample arrays, or a transform block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to transform the transform block samples.

  - tree: A tree is a finite set of nodes with a unique root node. A terminating node is called a leaf.

  - universal unique identifier (UUID): An identifier that is unique with respect to the space of all universal unique identifiers.

  - unspecified: The term unspecified, when used in the clauses specifying some values of a particular syntax element, indicates that the values have no specified meaning in this Recommendation | International Standard and will not have a specified meaning in the future as an integral part of this Recommendation | International Standard.

  - variable length coding (VLC): A reversible procedure for entropy coding that assigns shorter bit strings to symbols expected to be more frequent and longer bit strings to symbols expected to be less frequent.

  - VCL NAL unit: A collective term for coded slice NAL units.

  - z-scan: A specific seqential ordering of blocks partitioning a picture. When the blocks are of the same size as coding tree blocks, the z-scan order is identical to coding tree block raster scan of the picture. When the blocks are of a smaller size than coding tree blocks, i.e., coding tree blocks are further partitioned into smaller coding blocks, the z-scan order traverses from coding tree block to coding tree block in coding tree block raster scan of the picture, and inside each coding tree block, which may be divided into quadtrees hierachically to lower levels, the z-scan order traverses from quadree to quadree of a particular level in quadtree-of-the-particular-level raster scan of the quadtree of the immediately higher level.

  - zig-zag scan: A specific sequential ordering of transform coefficient levels from (approximately) the lowest spatial frequency to the highest.

# Abbreviations

For the purposes of this Recommendation | International Standard, the following abbreviations apply:

BLA	Broken Link Access

CABAC	Context-based Adaptive Binary Arithmetic Coding

CB	Coding Block

CBR	Constant Bit Rate

CRA	Clean Random Access

CPB	Coded Picture Buffer

CTB	Coding Tree Block

CTU	Coding Tree Unit

CU	Coding Unit

DPB	Decoded Picture Buffer

DUT	Decoder under test

FIFO	First-In, First-Out

HRD	Hypothetical Reference Decoder

HSS	Hypothetical Stream Scheduler

IDR	Instantaneous Decoding Refresh

LSB	Least Significant Bit

MSB	Most Significant Bit

NAL	Network Abstraction Layer

PB	Prediction Block

PU	Prediction Unit

RAP	Random Access Point

RBSP	Raw Byte Sequence Payload

SEI	Supplemental Enhancement Information

SODB	String Of Data Bits

TB	Transform Block

TFD	Tagged For Discard

TU	Transform Unit

UUID	Universal Unique Identifier

VBR	Variable Bit Rate

VCL	Video Coding Layer

VLC	Variable Length Coding

VUI	Video Usability Information

# Conventions

# Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

# Syntax and semantics

## Method of specifying syntax in tabular form

The syntax tables specify a superset of the syntax of all allowed bitstreams. Additional constraints on the syntax may be specified, either directly or indirectly, in other clauses.

NOTE – An actual decoder should implement means for identifying entry points into the bitstream and means to identify and handle non-conforming bitstreams. The methods for identifying and handling errors and other such situations are not specified here.

The following table lists examples of pseudo code used to describe the syntax. When syntax_element appears, it specifies that a syntax element is parsed from the bitstream and the bitstream pointer is advanced to the next position beyond the syntax element in the bitstream parsing process.

|  | C | Descriptor |
| --- | --- | --- |
| /* A statement can be a syntax element with an associated syntax category and descriptor or can be an expression used to specify conditions for the existence, type, and quantity of syntax elements, as in the following two examples */ |  |  |
| syntax_element | 3 | ue(v) |
| conditioning statement |  |  |
| /* A group of statements enclosed in curly brackets is a compound statement and is treated functionally as a single statement. */ |  |  |
| { |  |  |
| statement |  |  |
| statement |  |  |
| … |  |  |
| } |  |  |
| /* A "while" structure specifies a test of whether a condition is true, and if true, specifies evaluation of a statement (or compound statement) repeatedly until the condition is no longer true */ |  |  |
| while( condition ) |  |  |
| statement |  |  |
| /* A "do … while" structure specifies evaluation of a statement once, followed by a test of whether a condition is true, and if true, specifies repeated evaluation of the statement until the condition is no longer true */ |  |  |
| do |  |  |
| statement |  |  |
| while( condition ) |  |  |
| /* An "if … else" structure specifies a test of whether a condition is true, and if the condition is true, specifies evaluation of a primary statement, otherwise, specifies evaluation of an alternative statement. The "else" part of the structure and the associated alternative statement is omitted if no alternative statement evaluation is needed */ |  |  |
| if( condition ) |  |  |
| primary statement |  |  |
| else |  |  |
| alternative statement |  |  |
| /* A "for" structure specifies evaluation of an initial statement, followed by a test of a condition, and if the condition is true, specifies repeated evaluation of a primary statement followed by a subsequent statement until the condition is no longer true. */ |  |  |
| for( initial statement; condition; subsequent statement ) |  |  |
| primary statement |  |  |

## Specification of syntax functions and descriptors

The functions presented here are used in the syntactical description. These functions assume the existence of a bitstream pointer with an indication of the position of the next bit to be read by the decoding process from the bitstream.

byte_aligned( ) is specified as follows.

–	If the current position in the bitstream is on a byte boundary, i.e., the next bit in the bitstream is the first bit in a byte, the return value of byte_aligned( ) is equal to TRUE.

–	Otherwise, the return value of byte_aligned( ) is equal to FALSE.

more_data_in_byte_stream( ), which is used only in the byte stream NAL unit syntax structure specified in Annex Error: Reference source not found, is specified as follows.

–	If more data follow in the byte stream, the return value of more_data_in_byte_stream( ) is equal to TRUE.

–	Otherwise, the return value of more_data_in_byte_stream( ) is equal to FALSE.

more_rbsp_data( ) is specified as follows.

–	If there is no more data in the RBSP, the return value of more_rbsp_data( ) is equal to FALSE.

–	Otherwise, the RBSP data is searched for the last (least significant, right-most) bit equal to 1 that is present in the RBSP. Given the position of this bit, which is the first bit (rbsp_stop_one_bit) of the rbsp_trailing_bits( ) syntax structure, the following applies.

–	If there is more data in an RBSP before the rbsp_trailing_bits( ) syntax structure, the return value of more_rbsp_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_data( ) is equal to FALSE.

The method for enabling determination of whether there is more data in the RBSP is specified by the application (or in Annex Error: Reference source not found for applications that use the byte stream format).

more_rbsp_trailing_data( ) is specified as follows.

–	If there is more data in an RBSP, the return value of more_rbsp_trailing_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_trailing_data( ) is equal to FALSE.

next_bits( n ) provides the next bits in the bitstream for comparison purposes, without advancing the bitstream pointer. Provides a look at the next n bits in the bitstream with n being its argument. When used within the byte stream as specified in Annex Error: Reference source not found, next_bits( n ) returns a value of 0 if fewer than n bits remain within the byte stream.

read_bits( n ) reads the next n bits from the bitstream and advances the bitstream pointer by n bit positions. When n is equal to 0, read_bits( n ) is specified to return a value equal to 0 and to not advance the bitstream pointer.

The following descriptors specify the parsing process of each syntax element.

–	ae(v): context-adaptive arithmetic entropy-coded syntax element. The parsing process for this descriptor is specified in subclause 9.3.

–	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

–	f(n): fixed-pattern bit string using n bits written (from left to right) with the left bit first. The parsing process for this descriptor is specified by the return value of the function read_bits( n ).

–	i(n): signed integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a two's complement integer representation with most significant bit written first.

–	se(v): signed integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in subclause 9.2.

–	kes(v): signed syntax element using unsigned integer k-th order Exp-Golomb codes with the left bit first for the absolute value and one bit specifying the sign at the end. The parsing process for this descriptor is specified in subclause 9.2.2.

–	tu(n): truncated unary coded syntax element with left bit first. The parsing process for this descriptor is specified in subclause 9.1.

–	u(n): unsigned integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a binary representation of an unsigned integer with most significant bit written first.

–	ue(v): unsigned integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in subclause 9.2.1.

## Syntax in tabular form

### NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| NumBytesInRBSP = 0 |  |
| for( i = 2; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

### Raw byte sequence payloads and RBSP trailing bits syntax

#### Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_max_temporal_layers_minus1 | u(3) |
| vps_max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| vps_temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= vps_max_temporal_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| sps_max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_temporal_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| } |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| nsrqt_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software ] |  |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| diff_cu_qp_delta_depth | ue(v) |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_header_extension_present_flag | u(1) |
| slice_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 6; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| } |  |

#### Scaling list syntax

| scaling_list( scalingList, sizeID , matrixID ) { | Descriptor |
| --- | --- |
| nextCoef = 8 | u(1) |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| if( sizeID > 1 ) { |  |
| scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ sizeID – 2 ][ matrixID ] + 8 |  |
| } |  |
| for( i=0; i < coefNum; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| scalingList[ i ] = nextCoef |  |
| } |  |
| } |  |

#### Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| aps_alf_flag[ cIdx ] | u(1) |
| if( aps_alf_flag[ cIdx ] ) |  |
| alf_picture_info(cIdx) |  |
| } |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Adaptive loop filter picture info syntax

| alf_picture_info( cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_idc | ue(v) |
| if( alf_no_filters_idc = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_idc > 1 ) |  |
| for( i=0; i<15; i++ ) |  |
| alf_filter_pattern_flag[ i ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ i ][ j ] | kes(v) |
| } else |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ 0 ][ j ] | se(v) |
| } |  |

#### Supplemental enhancement information RBSP syntax

| sei_rbsp( ) { | Descriptor |
| --- | --- |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

##### Supplemental enhancement information message syntax

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadType += 255 |  |
| } |  |
| last_payload_type_byte | u(8) |
| payloadType += last_payload_type_byte |  |
| payloadSize = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadSize += 255 |  |
| } |  |
| last_payload_size_byte | u(8) |
| payloadSize += last_payload_size_byte |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

#### Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Filler data RBSP syntax

| filler_data_rbsp( ) { | Descriptor |
| --- | --- |
| while( next_bits( 8 ) = = 0xFF ) |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Slice layer RBSP syntax

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| if( slice_extension_present_flag ) { |  |
| slice_extention_flag | u(1) |
| if( slice_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| slice_extension_data_flag | u(1) |
| } |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

#### RBSP slice trailing bits syntax

| rbsp_slice_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_trailing_bits( ) |  |
| while( more_rbsp_trailing_data( ) ) |  |
| cabac_zero_word /* equal to 0x0000 */ | f(16) |
| } |  |

#### RBSP trailing bits syntax

| rbsp_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_stop_one_bit /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |

#### Byte alignment syntax

| byte_alignment( ) { | Descriptor |
| --- | --- |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |

[Ed. Note (YK): The byte_alignment() syntax structure and the rbsp_trailing_bits() syntax structure are essentially identical.]

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if((tiles_or_entropy_coding_sync_idc = = 2 ¦¦ tiles_or_entropy_coding_sync_idc = = 3)&& !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | u(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag[ 0 ] | u(1) |
| if( slice_sample_adaptive_offset_flag[ 0 ] ) { |  |
| slice_sample_adaptive_offset_flag[ 1 ] | u(1) |
| slice_sample_adaptive_offset_flag[ 2 ] | u(1) |
| } |  |
| } |  |
| if(adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| pic_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( pic_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| slice_alf_flag[ cIdx ] | u(1) |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_alf_flag[ 0 ] ¦ ¦ slice_alf_flag[ 1 ] ¦ ¦ slice_alf_flag[ 2 ] ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### Short-term reference picture set syntax

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

#### Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| } |  |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

#### Prediction weight table syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_param( xCtb >> Log2CtbSize, yCtb >> Log2CtbSize, cIdx ) |  |
| } |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| if( slice_alf_flag[ cIdx ] ) |  |
| ctb_alf_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

#### Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Coding tree syntax

| coding_tree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] >= SliceCbAddrZS && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| if( ( diff_cu_qp_delta_depth > 0 ) && log2CbSize >= Log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] > SliceCbAddrZS ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( MinCbAddrZS[ x0 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && x1 < pic_width_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( moreDataFlag && x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else { |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if( !( ( x0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && !( ( y0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && NumPCMBlock = = 0 ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| MaxTrafoDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Prediction unit syntax

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Motion vector difference coding syntax

| mvd_coding( mvd_x, mvd_y ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) { |  |
| if(abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag[ 1 ] ) { |  |
| if(abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| mvd_x = abs_mvd_greater0_flag[ 0 ] * ( abs_mvd_minus2[ 0 ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ 0 ] ) |  |
| mvd_y = abs_mvd_greater0_flag[ 1 ] * ( abs_mvd_minus2[ 1 ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ 1 ] ) |  |
| } |  |

#### PCM sample syntax

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock− − |  |
| } |  |

### Transform tree syntax

| transform_tree( x0L, y0L, x0C, y0C, xBase, yBase, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < MaxTrafoDepth && !(IntraSplitFlag && trafoDepth = = 0) ) |  |
| split_transform_flag[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0L ][ y0L ][ trafoDepth ] ) { |  |
| if( interTbSplitDirectionL = = 2 ) { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1L = y0L |  |
| x2L = x0L |  |
| y2L = y0L + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3L = x1L |  |
| y3L = y2L |  |
| } else { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y1L = y0L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| x2L = x1L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y2L = y1L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| x3L = x2L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y3L = y2L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| } |  |
| if( interTbSplitDirectionC = = 2 && log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| xC3 = xC1 |  |
| yC3 = yC2 |  |
| } else if ( log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y1C = y0C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| x2C = x1C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y2C = y1C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| x3C = x2C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y3C = y2C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| } else { |  |
| x1C = x0C |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C |  |
| x3C = x0C |  |
| y3C = y0C |  |
| } |  |
| if( interTbSplitDirectionL ! = 2 ) { |  |
| log2TrafoWidth = log2TrafoWidth − 2 * interTbSplitDirectionL + 1 |  |
| log2TrafoHeight = log2TrafoHeight + 2 * interTbSplitDirectionL − 1 |  |
| } |  |
| transform_tree( x0L, y0L, x0C, y0C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1L, y1L, x1C, y1C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2L, y2L, x2C, y2C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3L, y3L, x3C, y3C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| transform_unit (x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

### Transform unit syntax

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| if( PredMode = = MODE_INTRA ) { |  |
| scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ log2TrafoSize − 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0 ) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |

### Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 0 ] >> 2 |  |
| yCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 1 ] >> 2 |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold && !cu_ transquant_bypass _flag) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| TransCoeffLevel[x0][y0][cIdx][xC][yC] = − TransCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Semantics

Semantics associated with the syntax structures and with the syntax elements within these structures are specified in this subclause. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

### NAL unit semantics

NOTE 1 – The VCL is specified to efficiently represent the content of the video data. The NAL is specified to format that data and provide header information in a manner appropriate for conveyance on a variety of communication channels or storage media. All data are contained in NAL units, each of which contains an integer number of bytes. A NAL unit specifies a generic format for use in both packet-oriented and bitstream systems. The format of NAL units for both packet-oriented transport and byte stream is identical except that each NAL unit can be preceded by a start code prefix and extra padding bytes in the byte stream format.

NumBytesInNALunit specifies the size of the NAL unit in bytes. This value is required for decoding of the NAL unit. Some form of demarcation of NAL unit boundaries is necessary to enable inference of NumBytesInNALunit. One such demarcation method is specified in Annex Error: Reference source not found for the byte stream format. Other methods of demarcation may be specified outside of this Recommendation | International Standard.

forbidden_zero_bit shall be equal to 0.

nal_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, an adaptation parameter set or a slice of a reference picture.

nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture.

nal_ref_flag shall be equal to 1 for video parameter set, sequence parameter set, picture parameter set or adaptation parameter set NAL units. When nal_ref_flag is equal to 0 for one VCL NAL unit of a particular picture, it shall be equal to 0 for all VCL NAL units of the particular picture.

nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 4, 5, 6, 7, or 8.

nal_ref_flag shall be equal to 0 for all NAL units having nal_unit_type equal to 29, 30, or 31.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that use nal_unit_type equal to 0 or in the range of 48..63, inclusive, shall not affect the decoding process specified in this Recommendation | International Standard.

NOTE 2 – NAL unit types 0 and 48..63 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Recommendation | International Standard. Since different applications might use NAL unit types 0 and 48..63 for different purposes, particular care must be exercised in the design of encoders that generate NAL units with nal_unit_type equal to 0 or in the range of 48 to 63, inclusive, and in the design of decoders that interpret the content of NAL units with nal_unit_type equal to 0 or in the range of 48 to 63, inclusive.

Decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 3 – This requirement allows future definition of compatible extensions to this Recommendation | International Standard.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9..24 | Reserved | n/a |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

- NOTE 4 - A CRA picture having nal_unit_type equal to 4 may have associated TFD pictures present in the bitstream. A CRA picture having nal_unit_type equal to 5 does not have associated TFD pictures present in the bitstream. A BLA picture having nal_unit_type equal to 6 may have associated TFD pictures present in the bitstream. A BLA picture having nal_unit_type equal to 7 does not have associated TFD pictures present in the bitstream.

Coded slice NAL unit collectively refers to a VCL NAL unit, which has nal_unit_type in the range of 1 to 8, inclusive. The variable IdrPicFlag is specified as

IdrPicFlag = ( ( nal_unit_type  = =  8 )  ?  1  :  0 )		(7-1)

The variable RapPicFlag is specified as

RapPicFlag = ( ( nal_unit_type  >=  4  &&  nal_unit_type  <=  8  )  ?  1  :  0 )	(7-2)

When the value of nal_unit_type is equal to any particular value in the range of 1 to 8, inclusive, for a NAL unit of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to that particular value.

When the value of nal_unit_type is equal to 4 or 5 for all VCL NAL units of a particular picture, the particular picture is referred to as a CRA picture.

When the value of nal_unit_type is equal to 6 or 7 for all VCL NAL units of a particular picture, the particular picture is referred to as a BLA picture.

All coded pictures that follow a CRA or BLA picture both in decoding order and output order shall not use inter prediction from any picture that precedes the CRA or BLA picture either in decoding order or output order, and any picture that precedes the CRA or BLA picture in decoding order shall also precede the CRA or BLA picture in output order.

[Ed. Note (GJS/YK): The constraint on inter prediction should be expressed as a constraint on the reference picture set or the final reference picture list, whichever is easier to express.]

It is a requirement of bitstream conformance that no TFD pictures shall be present in the bitstream that are associated with a CRA picture having nal_unit_type equal to 5 or a BLA picture having nal_unit_type equal to 7.

When the value of nal_unit_type is equal to 8 for all VCL NAL units of a particular picture, that particular picture is referred to as an IDR picture. All coded pictures that follow an IDR picture in decoding order shall not use inter prediction from any picture that precedes the IDR picture in decoding order, and any picture that precedes the IDR picture in decoding order shall also precede the IDR picture in output order.

[Ed. Note (GJS/YK): The constraint on inter prediction should be expressed as a constraint on the reference picture set or the final reference picture list, whichever is easier to express.]

RAP picture collectively refers to a coded picture that is a CRA picture, a BLA picture or an IDR picture, and RAP access unit collectively refers to an access unit that is a CRA access unit, a BLA access unit or an IDR access unit.

- NOTE 5 - Any parameter set (video parameter set, sequence parameter set, picture parameter set, or adaptation parameter set) must be available before the activation of the parameter set. To be able to perform random access from any particular RAP picture by discarding all access units before the particular RAP access unit (and to correctly decode the particular RAP access unit and all the subsequent access units in both decoding and output order), the following condition must be satisfied: each parameter set that is activated during the decoding of the particular RAP access unit or during the decoding of any subsequent access unit in decoding order is either present or provided through external means at or subsequent to that particular RAP access unit and prior to any NAL unit activating that parameter set.

When the value of nal_unit_type is equal to 3 for all VCL NAL units of a particular picture, that particular picture is referred to as a TLA picture. A TLA picture and all coded pictures with temporal_id greater than or equal to the temporal_id of the TLA picture that follow the TLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the TLA picture that precedes the TLA picture in decoding order. A TLA picture shall not be TFD picture; hence TLA picture is also referred to as non-TFD TLA picture.

[Ed. Note (GJS/YK): The constraint on inter prediction should be expressed as a constraint on the reference picture set or the final reference picture list, whichever is easier to express.]

When temporal_id_nesting_flag is equal to 1 and temporal_id is greater than 0, nal_unit_type shall be equal to 3

temporal_id specifies a temporal identifier for the NAL unit. The value of temporal_id shall be the same for all VCL NAL units of an access unit. When an access unit is a RAP access unit, temporal_id for all VCL NAL units of the access unit shall be equal to 0. When nal_unit_type is equal to 3, temporal_id shall not be equal to 0.

The temporal_id of an access unit is derived as equal to the temporal_id value of the VCL NAL units in the access unit.

For a non-VCL NAL unit, the value of temporal_id shall be equal to the minimum value of the temporal_id values of all access units the non-VCL NAL unit applies to. When nal_unit_type is equal to 7 (sequence parameter set), temporal_id shall be equal to 0. When nal_unit_type is equal to 9 (access unit delimiter) or 12 (filler data), temporal_id shall be equal to the temporal_id of the access unit containing the non-VCL NAL unit. When nal_unit_type is equal to 8 (picture parameter set) or 14 (adaptation parameter set), temporal_id may be less than, equal to, or greater than the temporal_id of the containing access unit. When nal_unit_type is equal to 6 (SEI), temporal_id shall not be less than the temporal_id of the containing access unit.

NOTE 6 – When nal_unit_type is equal to 7 (sequence parameter set), temporal_id must be equal to 0, as a sequence parameter set applies at least to a RAP access unit. When nal_unit_type is equal to 9 (access unit delimiter) or 12 (filler data), temporal_id must be equal to the temporal_id of the access unit containing the non-VCL NAL unit, as access unit delimiter or filler data only applies to the containing access unit. When nal_unit_type is equal to 8 (picture parameter set) or 14 (adaptation parameter set), temporal_id may be less than, equal to, or greater than the temporal_id of the containing access unit, as a picture parameter set or adaptation parameter set may be repeated in access units not referring to the picture parameter set or adaptation parameter set for improved error resilience, and all picture parameter sets or adaptation parameter sets may be included in the beginning of a bitstream wherein the first coded picture has temporal_id equal to 0. When nal_unit_type is equal to 6 (SEI), temporal_id may be equal to or greater than the temporal_id of the containing access unit, as an SEI NAL unit may contain a picture buffering SEI message or a picture timing SEI message that applies to a bitstream subset including access units for which the temporal_id values are greater than the temporal_id of the access unit containing the SEI NAL unit.

reserved_one_5bits shall be equal to '00001'. Other values of reserved_one_5bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of reserved_one_5bits not equal to '00001'.

rbsp_byte[ i ] is the i-th byte of an RBSP. An RBSP is specified as an ordered sequence of bytes as follows.

The RBSP contains an SODB as follows.

–	If the SODB is empty (i.e., zero bits in length), the RBSP is also empty.

–	Otherwise, the RBSP contains the SODB as follows:

1)	The first byte of the RBSP contains the (most significant, left-most) eight bits of the SODB; the next byte of the RBSP contains the next eight bits of the SODB, etc., until fewer than eight bits of the SODB remain. [Ed. (GJS): Generally, there are way too many places where the word "shall" is used. If it is not a testable constraint on the content of a conforming bitstream and is not a testable constraint on the output behaviour of the decoder, it should not be expressed using a "shall".]

2)	rbsp_trailing_bits( ) are present after the SODB as follows:

i)	The first (most significant, left-most) bits of the final RBSP byte contains the remaining bits of the SODB (if any).

ii)	The next bit consists of a single rbsp_stop_one_bit equal to 1.

iii)	When the rbsp_stop_one_bit is not the last bit of a byte-aligned byte, one or more rbsp_alignment_zero_bit is present to result in byte alignment.

3)	One or more cabac_zero_word 16-bit syntax elements equal to 0x0000 may be present in some RBSPs after the rbsp_trailing_bits( ) at the end of the RBSP.

Syntax structures having these RBSP properties are denoted in the syntax tables using an "_rbsp" suffix. These structures shall be carried within NAL units as the content of the rbsp_byte[ i ] data bytes. The association of the RBSP syntax structures to the NAL units shall be as specified in Table 7-1.

NOTE 7 – When the boundaries of the RBSP are known, the decoder can extract the SODB from the RBSP by concatenating the bits of the bytes of the RBSP and discarding the rbsp_stop_one_bit, which is the last (least significant, right-most) bit equal to 1, and discarding any following (less significant, farther to the right) bits that follow it, which are equal to 0. The data necessary for the decoding process is contained in the SODB part of the RBSP.

emulation_prevention_three_byte is a byte equal to 0x03. When an emulation_prevention_three_byte is present in the NAL unit, it shall be discarded by the decoding process.

The last byte of the NAL unit shall not be equal to 0x00.

Within the NAL unit, the following three-byte sequences shall not occur at any byte-aligned position:

–	0x000000

–	0x000001

–	0x000002

Within the NAL unit, any four-byte sequence that starts with 0x000003 other than the following sequences shall not occur at any byte-aligned position:

–	0x00000300

–	0x00000301

–	0x00000302

–	0x00000303

NOTE 8 – When nal_unit_type is equal to 0, particular care must be exercised in the design of encoders to avoid the presence of the above-listed three-byte and four-byte patterns at the beginning of the NAL unit syntax structure, as the syntax element emulation_prevention_three_byte cannot be the third byte of a NAL unit.

#### Encapsulation of an SODB within an RBSP (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

The form of encapsulation of an SODB within an RBSP and the use of the emulation_prevention_three_byte for encapsulation of an RBSP within a NAL unit is described for the following purposes:

–	to prevent the emulation of start codes within NAL units while allowing any arbitrary SODB to be represented within a NAL unit,

–	to enable identification of the end of the SODB within the NAL unit by searching the RBSP for the rbsp_stop_one_bit starting at the end of the RBSP,

–	to enable a NAL unit to have a size larger than that of the SODB under some circumstances (using one or more cabac_zero_word syntax elements).

The encoder can produce a NAL unit from an RBSP by the following procedure:

- The RBSP data is searched for byte-aligned bits of the following binary patterns:

'00000000 00000000 000000xx'  (where xx represents any 2 bit pattern: 00, 01, 10, or 11),

and a byte equal to 0x03 is inserted to replace the bit pattern with the pattern:

'00000000 00000000 00000011 000000xx',

and finally, when the last byte of the RBSP data is equal to 0x00 (which can only occur when the RBSP ends in a cabac_zero_word), a final byte equal to 0x03 is appended to the end of the data. The last zero byte of a byte-aligned three-byte sequence 0x000000 in the RBSP (which is replaced by the four-byte sequence 0x00000300) is taken into account when searching the RBSP data for the next occurrence of byte-aligned bits with the binary patterns specified above.

- The resulting sequence of bytes is then prefixed as follows.

–	The sequence of bytes is prefixed with the first byte of the NAL unit containing the syntax elements forbidden_zero_bit, nal_ref_flag, and nal_unit_type, where nal_unit_type indicates the type of RBSP data structure the NAL unit contains.

The process specified above results in the construction of the entire NAL unit.

This process can allow any SODB to be represented in a NAL unit while ensuring that

–	no byte-aligned start code prefix is emulated within the NAL unit,

- no sequence of 8 zero-valued bits followed by a start code prefix, regardless of byte-alignment, is emulated within the NAL unit.

#### Order of NAL units and association to coded pictures, access units, and video sequences

This subclause specifies constraints on the order of NAL units in the bitstream.

Any order of NAL units in the bitstream obeying these constraints is referred to in the text as the decoding order of NAL units. Within a NAL unit, the syntax in subclauses 7.3, Error: Reference source not found, and Error: Reference source not found specifies the decoding order of syntax elements. Decoders shall be capable of receiving NAL units and their syntax elements in decoding order.

##### Order of video, sequence, picture and adaptation parameter set RBSPs and their activation

This subclause specifies the activation process of video, sequence, picture and adaptation parameter sets.

- NOTE 1 – The video sequence, picture and adaptation parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded block data. Video, sequence, picture and adaptation parameter sets may, in some applications, be conveyed "out-of-band".

- An adaptation parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures when at least one of sample_adaptive_offset_enabled_flag or adaptive_loop_filter_enabled_flag are equal to 1. Each adaptation parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one adaptation parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular adaptation parameter set RBSP results in the deactivation of the previously-active adaptation parameter set RBSP (if any).

- When an adaptation parameter set RBSP (with a particular value of aps_id) is not active and it is referred to by a coded slice NAL unit (using that value of aps_id), it is activated. This adaptation parameter set RBSP is called the active adaptation parameter set RBSP until it is deactivated by the activation of another adaptation parameter set RBSP. An adaptation parameter set RBSP, with that particular value of aps_id, shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to or less than the temporal_id of the adaptation parameter set NAL unit, unless the adaptation parameter set is provided through external means.

- A picture parameter set RBSP includes parameters that can be referred to by the coded slice NAL units or coded slice data partition A NAL units of one or more coded pictures. Each picture parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one picture parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular picture parameter set RBSP results in the deactivation of the previously-active picture parameter set RBSP (if any).

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not active and it is referred to by a coded slice NAL unit or coded slice data partition A NAL unit (using that value of pic_parameter_set_id), it is activated. This picture parameter set RBSP is called the active picture parameter set RBSP until it is deactivated by the activation of another picture parameter set RBSP. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to or less than the temporal_id of the picture parameter set NAL unit, unless the picture parameter set is provided through external means.

- Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active picture parameter set RBSP for a coded picture shall have the same content as that of the active picture parameter set RBSP for the coded picture unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP results in the deactivation of the previously-active sequence parameter set RBSP (if any).

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), it is activated. This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to 0, unless the sequence parameter set is provided through external means. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 2 – Because an IDR or BLA access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR or BLA access unit.

- Any sequence parameter set NAL unit containing the value of seq_parameter_set_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

- A video parameter set RBSP includes parameters that can be referred to by one or more sequence parameter set RBSPs. Each video parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one video parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular video parameter set RBSP results in the deactivation of the previously-active video parameter set RBSP (if any).

- When a video parameter set RBSP (with a particular value of video_parameter_set_id) is not already active and it is referred to by activation of a sequence parameter set RBSP (using that value of video_parameter_set_id), it is activated. This video parameter set RBSP is called the active video parameter set RBSP until it is deactivated by the activation of another video parameter set RBSP. A video parameter set RBSP, with that particular value of video_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to 0, unless the video parameter set is provided through external means. An activated video parameter set RBSP shall remain active for the entire coded video sequence.

- Any video parameter set NAL unit containing the value of video_parameter_set_id for the active video parameter set RBSP for a coded video sequence shall have the same content as that of the active video parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first sequence parameter set NAL unit of another coded video sequence.

- NOTE 3 – If video parameter set RBSP, sequence parameter set RBSP, picture parameter set RBSP or adaptation parameter set RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the video parameter set RBSP, sequence parameter set RBSP, picture parameter set RBSP or adaptation parameter set RBSP respectively. Otherwise (video parameter set RBSP, sequence parameter set RBSP, picture parameter set RBSP or adaptation parameter set RBSP are conveyed by other means not specified in this Recommendation | International Standard), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

All constraints that are expressed on the relationship between the values of the syntax elements (and the values of variables derived from those syntax elements) in video parameter sets, sequence parameter sets, picture parameter sets and adaptation parameter sets and other syntax elements are expressions of constraints that apply only to the active video parameter set, the active sequence parameter set, the active picture parameter set and the active adaptation parameter set. If any video parameter set RBSP, sequence parameter set RBSP, picture parameter set RBSP or adaptation parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

- During operation of the decoding process (see clause 8), the values of parameters of the active video parameter set, the active sequence parameter set, the active picture parameter set RBSP and the active adaptation parameter set RBSP shall be considered in effect. For interpretation of SEI messages, the values of the active video parameter set, the active sequence parameter set, the active picture parameter set RBSP and the active adaptation parameter set RBSP for the operation of the decoding process for the VCL NAL units of the coded picture in the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

##### Order of access units and association to coded video sequences

A bitstream conforming to this Recommendation | International Standard consists of one or more coded video sequences.

A coded video sequence consists of one or more access units. The order of NAL units and coded pictures and their association to access units is described in subclause 7.4.1.2.3.

The first access unit of each coded video sequence is an IDR or BLA access unit. All subsequent access units in the coded video sequence are non-IDR and non-BLA access units.

The values of picture order count for the coded pictures in consecutive access units in decoding order containing non-reference pictures shall be increasing.

When an SEI NAL unit contains data that pertain to more than one access unit (for example, when the SEI NAL unit has a coded video sequence as its scope), it shall be contained in the first access unit to which it applies.

##### Order of NAL units and coded pictures and association to access units

This subclause specifies the order of NAL units and coded pictures and association to access unit for coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses 2-9.

An access unit consists of one coded picture and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in subclause 7.4.1.2.5.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

The first of any of the following NAL units after the last VCL NAL unit of a coded picture specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	video parameter set NAL unit (when present),

–	sequence parameter set NAL unit (when present),

–	picture parameter set NAL unit (when present),

–	adaptation parameter set NAL unit (when present),

–	SEI NAL unit (when present),

–	NAL units with nal_unit_type in the range of 33 to 43, inclusive (when present),

–	first VCL NAL unit of a coded picture (always present).

The constraints for the detection of the first VCL NAL unit of a coded picture are specified in subclause 7.4.1.2.4.

The following constraints shall be obeyed by the order of the coded pictures and non-VCL NAL units within an access unit:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	When any SEI NAL units are present, they shall precede the coded picture.

–	When an SEI NAL unit containing a buffering period SEI message is present, the buffering period SEI message shall be the first SEI message payload of the first SEI NAL unit in the access unit.

–	NAL units having nal_unit_type equal to 0, 30, or in the range of 44 to 63, inclusive, shall not precede the first VCL NAL unit of the coded picture.

NOTE 1 – Video parameter set NAL units, sequence parameter set NAL units, picture parameter set NAL units or adaptation parameter set NAL units may be present in an access unit, but cannot follow the last VCL NAL unit of the coded picture within the access unit, as this condition would specify the start of a new access unit.

The structure of access units not containing any NAL units with nal_unit_type equal to 0, or 30, or in the ranges of 9..24, 25..28, or 32..63, is shown in Figure 7-1. [Ed. (TW/YK):adjust text and figure, replace "Primary coded picture" with "Coded picture", remove the "Redundant coded picture" box, remove the "Auxiliary coded picture" box, either remove "End of sequence" box and "End of stream" box or add the two NAL unit types into Table 7-1.]

Figure 7-1 – Structure of an access unit not containing any NAL units with nal_unit_type equal to 0, , or 30, or in the ranges of 9..24, 25..28, or 32..63

##### Detection of the first VCL NAL unit of a coded picture

- This subclause specifies constraints on VCL NAL unit syntax that are sufficient to enable the detection of the first VCL NAL unit of each coded picture.

- Any coded slice NAL unit or coded slice data partition A NAL unit of the coded picture of the current access unit shall be different from any coded slice NAL unit or coded slice data partition A NAL unit of the coded picture of the previous access unit in one or more of the following ways:

–	pic_parameter_set_id differs in value.

–	nal_ref_flag differs in value with one of the nal_ref_flag values being equal to 0.

–	pic_order_cnt_lsb differs in value.

–	IdrPicFlag differs in value.

–	RapPicFlag differs in value.

–	RapPicFlag is equal to 1 for both and rap_pic_id differs in value.

##### Order of VCL NAL units and association to coded pictures

[Ed. (TW): insert text]

### Raw byte sequence payloads and RBSP trailing bits semantics

#### Video parameter set RBSP semantics

vps_max_temporal_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream.

[Ed.(YK): Add a definition of sub-layer in Clause 3.]

vps_max_layers_minus1 plus 1 specifies the maximum number of layers that may be present in the coded video sequence. The value of vps_num_layers_minus1 shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. Other values for vps_num_layers_minus1 are reserved for future use by ITU-T | ISO/IEC. Decoders should ignore vps_num_layers_minus1 not equal to 0 in a video parameter set NAL unit.

[Ed.(YK): Add a definition of layer (probably it should "scalable layer", as there is already a definition of layer for something else) in Clause 3.]

video_parameter_set_id identifies a video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 15, inclusive.

vps_temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence.

Dependent on vps_temporal_id_nesting_flag, the following applies.

–	If vps_temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (vps_temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB that is less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with temporal_id equal to tIdC that is less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE 1 – The syntax element vps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM that is greater than tIdN, is always possible.

- vps_max_dec_pic_buffering[ i ] specifies the required size of the decoded picture buffer in units of picture storage buffers for the bitstream subset as specified in subclause Error: Reference source not found with i as input. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, vps_max_dec_pic_buffering[ i ] ) picture storage buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of vps_max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive.

- vps_num_reorder_pics[ i ] indicates the maximum allowed number of pictures preceding any picture in decoding order and succeeding that picture in output order for the bitstream subset as specified in subclause Error: Reference source not found with i as input. The value of vps_num_reorder_pics[ i ] shall be in the range of 0 to vps_max_dec_pic_buffering[ i ], inclusive.

- vps_max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by setting MaxLatencyPictures[ i ] to vps_num_reorder_pics[ i ] + vps_max_latency_increase[ i ] for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When vps_max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When vps_max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of vps_max_latency_increase[ i ] shall be in the range of 0 to 232 − 1, inclusive.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for vps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension_flag in a video parameter set NAL unit.

vps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

#### Sequence parameter set RBSP semantics

profile_space specifies the context for the intepretation of profile_idc and profile_combatibility_flag[ i ] for all possible values of i. The value of profile_space shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. Other values for for profile_space are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the coded video sequence if profile_space is not equal to 0.

When profile_space is equal to 0, profile_idc indicates the profile to which the coded video sequence conforms.

[Ed. (DS): We might prefer not to use the profile_idc value zero, or reserve it to mean "no profile signalled, bitstream is unconstrained"; this gives us one spare bit in the profile compatibility flags array.]

constraint_flags shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. Other values for for constraint_flags are reserved for future use by ITU-T | ISO/IEC. Decoders should ignore the value of constraint_flags.

level_idc indicates the level to which the coded video sequence conforms. A greater value of level_idc indicates a higher level.

When profile_space is equal to 0, profile_compatability_flag[ i ] equal to 1 indicates that the coded video sequence conforms to the profile indicated by profile_idc equal to i. When profile_space is equal to 0, profile_compatability_flag[ profile_idc ] shall be equal to 1.

NOTE 1 – The level_idc definitions need to be independent of profile_idc, as a decoder may not recognize the the profile_idc value but instead recognize, and be able to decode, one or more of the profiles indicated by the profile compatibility flags.

NOTE 2 – For bitstreams compatible with more than one profile, the value of profile_idc should be set to the "best viewed as" profile.

NOTE 3 – The constraint_flags can be used to indicate the respect of further constraints on the bitstream (e.g. that a selected tool, permitted by the profiles signalled, is nonetheless not used). These flags are ideally profile independent, but unlike level_idc, it is allowed to be contextual on profile_idc.

video_parameter_set_id refers to the active video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 15, inclusive.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause Error: Reference source not found. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive. When chroma_format_idc is not present, it is inferred to be equal to 1 (4:2:0 chroma format).

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it is inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb or Cr) that each use the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value.

NOTE 4 – There is no dependency in decoding processes between the colour planes having different colour_plane_id values. For example, the decoding process of a monochrome picture with one value of colour_plane_id does not use any data from monochrome pictures having different values of colour_plane_id for inter prediction.

Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

sps_max_temporal_layers_minus1 plus 1 specifies the maximum number of temporal layers that may be present in the sequence. The value of sps_max_temporal_layers_minus1 shall be in the range of 0 to 7, inclusive. The value of sps_max_temporal_layers_minus1 shall be equal to vps_max_temporal_layers_minus1.

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of ( 1 << Log2MinCbSize ).

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of ( 1 << Log2MinCbSize ).

pic_cropping_flag equal to 1 indicates that the picture cropping offset parameters follow next in the sequence parameter set. pic_cropping_flag equal to 0 indicates that the picture cropping offset parameters are not present.

pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset specify the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

The variables CropUnitX and CropUnitY are derived as follows:

–	If ChromaArrayType is equal to 0, CropUnitX and CropUnitY are derived as:

CropUnitX = 1		(7-3)
CropUnitY = 1		(7-4)

–	Otherwise (ChromaArrayType is equal to 1, 2, or 3), CropUnitX and CropUnitY are derived as:

CropUnitX = SubWidthC		(7-5)
CropUnitY = SubHeightC		(7-6)

The picture cropping rectangle contains luma samples with horizontal picture coordinates from CropUnitX * pic_crop_left_offset to pic_width_in_luma_samples − ( CropUnitX * pic_crop_right_offset + 1 ) and vertical picture coordinates from CropUnitY * pic_crop_top_offset to pic_height_in_luma_samples − ( CropUnitY * pic_crop_bottom_offset + 1 ), inclusive. The value of pic_crop_left_offset shall be in the range of 0 to ( pic_width_in_luma_samples / CropUnitX ) − ( pic_crop_right_offset + 1 ), inclusive; and the value of pic_crop_top_offset shall be in the range of 0 to ( pic_height_in_luma_samples / CropUnitY ) − ( pic_crop_bottom_offset + 1 ), inclusive.

When pic_cropping_flag is equal to 0, the values of pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset is inferred to be equal to 0.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE 5 – The picture cropping parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

bit_depth_luma_minus8 + 8 specifies the bit depth of the samples of the luma array and the value of the luma quantization parameter range offset QpBdOffsetY, as specified by

BitDepthY      = 8 + bit_depth_luma_minus8		(7-7)
QpBdOffsetY = 6 * bit_depth_luma_minus8		(7-8)

bit_depth_luma_minus8 shall be in the range of 0 to 6, inclusive.

bit_depth_chroma_minus8 + 8 specifies the bit depth of the samples of the chroma arrays and the value of the chroma quantization parameter range offset QpBdOffsetC, as specified by

BitDepthC      = 8 + bit_depth_chroma_minus8		(7-9)
QpBdOffsetC = 6 * bit_depth_chroma_minus8		(7-10)

bit_depth_chroma_minus8 shall be in the range of 0 to 6, inclusive.

pcm_enabled_flag equal to 0 specifies that PCM data shall not be present in the video sequence.

pcm_sample_bit_depth_luma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of luma component. The value of pcm_sample_bit_depth_luma_minus1 + 1 shall be less than or equal to the value of BitDepthY.

PCMBitDepthY = 1 + pcm_sample_bit_depth_luma_minus1	(7-11)

pcm_sample_bit_depth_chroma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of chroma components. The value of pcm_sample_bit_depth_chroma_minus1 + 1 shall be less than or equal to the value of BitDepthC.

PCMBitDepthC = 1 + pcm_sample_bit_depth_chroma_minus1	(7-12)

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )	(7-13)

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

- sps_max_dec_pic_buffering[ i ] specifies the required size of the decoded picture buffer in units of picture storage buffers for the bitstream subset as specified in subclause Error: Reference source not found with i as input. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, sps_max_dec_pic_buffering[ i ] ) picture storage buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of sps_max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive. The value of sps_max_dec_pic_buffering[ i ] shall be equal to vps_max_dec_pic_buffering[ i ] for each possible value of i.

- sps_num_reorder_pics[ i ] indicates the maximum allowed number of pictures preceding any picture in decoding order and succeeding that picture in output order for the bitstream subset as specified in subclause Error: Reference source not found with i as input. The value of sps_num_reorder_pics[ i ] shall be in the range of 0 to sps_max_dec_pic_buffering[ i ], inclusive. The value of sps_num_reorder_pics[ i ] shall be equal to vps_num_reorder_pics[ i ] for each possible value of i.

sps_max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by setting MaxLatencyPictures[ i ] equal to sps_num_reorder_pics[ i ] + sps_max_latency_increase[ i ] for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When sps_max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When sps_max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of sps_max_latency_increase[ i ] shall be in the range of 0 to 232 − 1, inclusive. The value of sps_max_latency_increase[ i ] shall be equal to vps_max_latency_increase[ i ] for each possible value of i.

restricted_ref_pic_lists_flag equal to 1 indicates that all slices having the same value of slice_type that belong to the same picture have identical reference picture lists. restricted_ref_pic_lists_flag equal to 0 indicates that there may be slices having the same value of slice_type that belong to the same picture that have different reference picture lists. num_ref_idx_active_override_flag, num_ref_idx_l0_active_minus1, and num_ref_idx_l1_active_minus1, when present , shall be identical for all slices having the same value of slice_type that belong to the same picture when restricted_ref_pic_lists_flag is equal to 1.

lists_modification_present_flag equal to 0 specifies that the syntax structure ref_pic_list_modification( ) is not present in the slice header. lists_modification_present_flag equal to 1 specifies that the syntax structure ref_pic_list_modification( ) is present in the slice header. When not present, the value of lists_modification_present_flag is inferred to be equal to 1.

log2_min_coding_block_size_minus3 specifies the minimum size of a coding block.

The variable Log2MinCbSize is set equal to log2_min_coding_block_size_minus3 + 3.

log2_diff_max_min_coding_block_size specifies the difference between the maximum and minimum coding block size.

The variables Log2CtbSize, PicWidthInCtbs, PicHeightInCtbs, PicWidthInMinCbs, and PicHeightInMinCbs are derived as follows.

Log2CtbSize = log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size	(7-14)
PicWidthInCtbs = Ceil( pic_width_in_luma_samples ÷ ( 1 << Log2CtbSize ) )	(7-15)
PicHeightInCtbs = Ceil( pic_height_in_luma_samples ÷ ( 1 << Log2CtbSize ) )	(7-16)
PicWidthInMinCbs = pic_width_in_luma_samples / ( 1 << Log2MinCbSize )	(7-17)
PicHeightInMinCbs = pic_height_in_luma_samples / ( 1 << Log2MinCbSize )	(7-18)

The variable CtbSize, which specifies the width and the height, of the square array for each luma coding tree block, is derived as follows.

CtbSize = 1 << Log2CtbSize		(7-19)

The variables CtbWidthC and CtbHeightC, which specify the width and height, respectively, of the array for each chroma coding tree block, are derived as follows.

–	If chroma_format_idc is equal to 0 (monochrome) or separate_colour_plane_flag is equal to 1, CtbWidthC and CtbHeightC are both equal to 0.

–	Otherwise, CtbWidthC and CtbHeightC are derived as

CtbWidthC = CtbSize / SubWidthC		(7-20)
CtbHeightC = CtbSize / SubHeightC		(7-21)

log2_min_transform_block_size_minus2 specifies the minimum transform size.

The variable Log2MinTrafoSize is set equal to log2_min_transform_block_size_minus2 + 2. The bitstream shall not contain data that result in Log2MinTrafoSize greater than or equal to Log2MinCbSize.

log2_diff_max_min_transform_block_size specifies the difference between the maximum and minimum transform size.

The variable Log2MaxTrafoSize is set equal to log2_min_transform_block_size_minus 2 + 2 + log2_diff_max_min_transform_block_size.

The bitstream shall not contain data that result in Log2MaxTrafoSize greater than Min( Log2CtbSize, 5).

The array ScanOrder[ log2BlockWidth ][ log2BlockHeight ][ scanIdx ][ sPos ][ sComp ] specifies the mapping of the scan position sPos, ranging from 0 to ( ( 1 << log2BlockWidth ) * ( 1 << log2BlockHeight ) ) − 1, inclusive, to horizontal and vertical components of the scan-order matrix. The array index scanIdx equal to 0 specifies an up-right diagonal scan, scanIdx equal to 1 specifies a horizontal scan, and scanIdx equal to 2 specifies a vertical scan. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

For the variables log2BlockWidth and log2BlockHeight ranging from Max( 2, Log2MinTrafoSize − 1) to Log2MaxTrafoSize, inclusive, the scanning order array ScanOrder is derived as follows.

–	The up-right diagonal scanning array initialization process as specified in Error: Reference source not found is invoked with 1 << log2BlockWidth and 1 << log2BlockHeight as input and the output is assigned to ScanOrder[ 0 ][ log2BlockWidth ][ log2BlockHeight ].

–	The horizontal scanning array initialization process as specified in Error: Reference source not found is invoked with 1 << log2BlockWidth and 1 << log2BlockHeight as input and the output is assigned to ScanOrder[ 1 ][ log2BlockWidth ][ log2BlockHeight ].

–	The vertical scanning array initialization process as specified in Error: Reference source not found is invoked with 1 << log2BlockWidth and 1 << log2BlockHeight as input and the output is assigned to ScanOrder[ 2 ][ log2BlockWidth ][ log2BlockHeight ].

log2_min_pcm_coding_block_size_minus3 + 3 specifies the minimum size of I_PCM coding blocks.

The variable Log2MinIPCMCUSize is set equal to log2_min_pcm_coding_block_size_minus3 + 3. The variable Log2MinIPCMCUSize shall be equal or less than Min( Log2CtbSize, 5 ).

log2_diff_max_min_pcm_coding_block_size specifies the difference between the maximum and minimum size of I_PCM coding blocks.

The variable Log2MaxIPCMCUSize is set equal to log2_min_pcm_coding_block_size_minus3 + 3 + log2_diff_max_min_pcm_coding_block_size. The variable Log2MaxIPCMCUSize shall be equal or less than Min( Log2CtbSize, 5 ).

max_transform_hierarchy_depth_intra specifies the maximum hierarchy depth for transform blocks of coding blocks coded in intra prediction mode.

max_transform_hierarchy_depth_inter specifies the maximum hierarchy depth for transform units of coding units coded in inter prediction mode.

scaling_list_enable_flag equal to 1 specifies that scaling list is used for scaling process for transform coefficients in Error: Reference source not found. scaling_list_enable_flag equal to 0 specifies that scaling list is not used for scaling process for transform coefficients in Error: Reference source not found.

sps_scaling_list_data_present_flag equal to 1 specifies that scaling list parameters are present in the sequence parameter set. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list parameters are not present in the sequence parameter set. When not present, the value of sps_scaling_list_data_present_flag is inferred to be 0.

chroma_pred_from_luma_enabled_flag equal to 1 specifies the intra chroma prediction process using the reconstructed luma block is applied according to the intra chroma prediction mode.

seq_loop_filter_across_slices_enabled_flag equal to 1 specifies that the slice_loop_filter_across_slices_enabled_flag determines whether in-loop filtering operations are performed across slice boundaries; otherwise, the in-loop filtering operations are slice-independent and not applied across slice boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter.

asymmetric_motion_partitions_enabled_flag equal to 1 specifies that asymmetric motion partitions may be used in coding tree blocks; asymmetric_motion_partitions_enabled_flag equal to 0 specifies that asymmetric motion partitions cannot be used in coding tree blocks.

nsrqt_enabled_flag equal to 1 specifies that non-square residual quadtree partitions may be used in coding tree blocks; nsrqt_enabled_flag equal to 0 specifies that non-square quadtree partitions cannot be used in coding tree blocks.

sample_adaptive_offset_enabled_flag equal to 1 specifies the sample adaptive offset process is applied to the reconstruced picture after the deblocking filter process.

adaptive_loop_filter_enabled_flag equal to 1 specifies the adaptive loop filter process is applied to the reconstructed picture after the sample adaptive offset process.

pcm_loop_filter_disable_flag specifies whether the loop filter process is disabled on reconstructed samples of I_PCM blocks. If the pcm_loop_filter_disable_flag value is equal to 1, deblocking filter, sample adaptive offset and adaptive loop filter processes on the reconstructed samples of I_PCM blocks are disabled; otherwise if the pcm_loop_filter_disable_flag value is equal to 0, deblocking filter, sample adaptive offset and adaptive loop filter processes on the reconstructed samples of I_PCM blocks are not disabled. When pcm_loop_filter_disable_flag is not present, it is inferred to be equal to 0.

[Ed. (WJ): select one expression – enabled_flag or disable_flag]

sps_temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence. sps_temporal_id_nesting_flag shall be equal to vps_temporal_id_nesting_flag.

Dependent on sps_temporal_id_nesting_flag, the following applies.

–	If sps_temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (sps_temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB that is less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with temporal_id equal to tIdC that is less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order. [Ed. Note (GJS/YK): It might be better to express the constraint on inter prediction to be a constraint on the final reference picture list.]

NOTE 6 – The syntax element sps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM that is greater than tIdN, is always possible.

inter_4x4_enabled_flag specifies whether inter prediction can be applied to blocks having the size of 4 by 4 luma samples.

num_short_term_ref_pic_sets specifies the number of short-term reference picture sets that are specified in the sequence parameter set. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE 7 – A decoder must allocate space for a total number of num_short_term_ref_pic_sets + 1 short-term reference picture sets since a coded video sequence may contain up to one short-term reference picture set explicitly signalled in the slice headers of a current picture. An explicitly signalled short-term reference picture set will always have an index equal to num_short_term_ref_pic_sets in the list of short-term reference picture sets.

long_term_ref_pics_present_flag equal to 0 specifies that no long-term reference picture is used for inter prediction of any coded picture in the coded video sequence. long_term_ref_pics_present_flag equal to 1 specifies that long-term reference pictures may be used for inter prediction of one or more coded pictures in the coded video sequence.

sps_temporal_mvp_enable_flag equal to 1 specifies that pic_temporal_mvp_enable_flag may be present in the slice headers and temporal motion vector predictors may be used. sps_temporal_mvp_enable_flag equal to 0 specifies that pic_temporal_mvp_enable_flag shall not be present in the slice headers and temporal motion vector predictors shall not be used.

sps_extension_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the sequence parameter set RBSP syntax structure. sps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for sps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for sps_extension_flag in a sequence parameter set NAL unit.

sps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

#### Picture parameter set RBSP semantics

pic_parameter_set_id identifies the picture parameter set that is referred to in the slice header. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

seq_parameter_set_id refers to the active sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

sign_data_hiding_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled

sign_hiding_threshold specifies a threshold to be compared with the number of coefficients that are between the last non-zero coefficient and the first non-zero coefficient along the scanning path within a coefficient group to determine whether the sign bit of the first nonzero coefficient is present in the bitstream or whether it will be inferred.

cabac_init_present_flag equal to 1 specifies that cabac_init_flag is present in the slice header.

num_ref_idx_l0_default_active_minus1 specifies how num_ref_idx_l0_active_minus1 is inferred for P and B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l0_default_active_minus1 shall be in the range of 0 to 15, inclusive. [Ed. (GJS): Constrain to the limit imposed by max DPB size instead of 15.]

num_ref_idx_l1_default_active_minus1 specifies how num_ref_idx_l1_active_minus1 is inferred for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l1_default_active_minus1 shall be in the range of 0 to 15, inclusive.

pic_init_qp_minus26 specifies the initial value minus 26 of SliceQPY for each slice. The initial value is modified at the slice layer when a non-zero value of slice_qp_delta is decoded, and is modified further when a non-zero value of cu_qp_delta is decoded at the coding unit layer. The value of pic_init_qp_minus26 shall be in the range of −(26 + QpBdOffsetY ) to +25, inclusive.

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring coding blockscoded using Inter prediction modes for the prediction of coding blockscoded using Intra prediction modes. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case prediction of coding blockscoded using Intra prediction modes only uses residual data and decoded samples from coding blocks coded using Intra prediction mdoes.

slice_granularity indicates the slice granularity within a picture. The value of slice_granularity shall not be larger than Min( Log2CtbSize − 4, log2_diff_max_min_coding_block_size ). The variable SliceGranularity is set to the value of ( slice_granularity << 1 ).

diff_cu_qp_delta_depth specifies the difference between the control granularity for QPY values for coding units within a picture and slice_granularity plus 1. The value of diff_cu_qp_delta_depth shall be in the range of 0 to log2_diff_max_min_coding_block_size − slice_granularity + 1, inclusive.

The variable Log2MinCUDQPSize specifying the minimum luma coding block size of coding units that convey cu_qp_delta, is derived as follows.

Log2MinCUDQPSize = Log2CtbSize − diff_cu_qp_delta_depth − slice_granularity + 1	(7-22)

cb_qp_offset, cr_qp_offset are specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr , respectively.

weighted_pred_flag equal to 0 specifies that weighted prediction shall not be applied to P slices. weighted_pred_flag equal to 1 specifies that weighted prediction shall be applied to P slices.

weighted_bipred_idc equal to 0 specifies that the default weighted prediction is applied to B slices. weighted_bipred_idc equal to 1 specifies that explicit weighted prediction is applied to B slices. weighted_bipred_idc equal to 2 specifies that implicit weighted prediction shall be applied to B slices.  The value of weighted_bipred_idc shall be in the range of 0 to 2, inclusive.

output_flag_present_flag equal to 1 indicates that the pic_output_flag syntax element is present in the associated slice headers. output_flag_present_flag equal to 0 indicates that the pic_output_flag syntax element is not present in the associated slice headers.

transquant_bypass_enable_flag specifies whether a cu_transquant_bypass_flag syntax element is coded or not. If transquant_bypass_enable_flag is equal to 1, cu_transquant_bypass_flag is coded, otherwise it is not coded.

tiles_or_entropy_coding_sync_idc equal to 0 specifies that there shall be only one tile in each picture referring to the picture parameter set, there shall be no specific synchronization process for context variables invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set, and the value of dependent_slice_flag for coded pictures referring to the picture parameter set shall not be equal to 1.

tiles_or_entropy_coding_sync_idc equal to 1 specifies that there may be more than one tile in each picture referring to the picture parameter set, there shall be no specific synchronization process for context variables invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set, and the value of dependent_slice_flag for coded pictures referring to the picture parameter set shall not be equal to 1.

tiles_or_entropy_coding_sync_idc equal to 2 specifies that there shall be only one tile in each picture referring to the picture parameter set, a specific synchronization process for context variables shall be invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set and a specific memorization process for context variables shall be invoked after decoding two coding tree blocks of a row of coding tree blocks in each picture referring to the picture parameter set, and the value of and dependent_slice_flag for coded pictures referring to the picture parameter set shall not be equal to 1.

tiles_or_entropy_coding_sync_idc equal to 3 specifies that there shall be only one tile in each picture referring to the picture parameter set, there shall be no specific synchronization process for context variables invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set, and the value of dependent_slice_flag for coded pictures referring to the picture parameter set may be equal to 1.

It's a requirement of bitstream conformance that the value of tiles_or_entropy_coding_sync_idc shall be the same for all picture parameter sets that are activated within a coded video sequence.

For each slice referring to the picture parameter set, when tiles_or_entropy_coding_sync_idc is equal to 2 and the first coding block in the slice is not the first coding block in the first coding tree block of a row of coding tree blocks, the last coding block in the slice shall belong to the same row of coding tree blocks as the first coding block in the slice slice.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture.

When num_tile_columns_minus1 is equal to 0, num_tile_rows_minus1 shall not be equal to 0.

uniform_spacing_flag equal to 1 specifies that column boundaries and likewise row boundaries are distributed uniformly across the picture. uniform_spacing_flag equal to 0 specifies that column boundaries and likewise row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements column_width[ i ] and row_height[ i ].

column_width[ i ] specifies the width of the i-th tile column in units of coding tree blocks.

row_height[ i ] specifies the height of the i-th tile row in units of coding tree blocks.

The list colWidth[ i ] specifies the width of the i-th tile column in units of CTBs with the column i ranging from 0 to num_tile_columns_minus1, inclusive.

The list CtbAddrRStoTS[ ctbAddrRS ] specifies the conversation from a CTB address in raster scan order to a CTB address in tile scan order with the index ctbAddrRS ranging from 0 to (picHeightInCtbs * picWidthInCtbs) − 1, inclusive.

The list CtbAddrTStoRS[ ctbAddrTS ] specifies the conversation from a CTB address in tile scan order to a CTB address in raster scan order with the index ctbAddrTS ranging from 0 to (picHeightInCtbs * picWidthInCtbs) − 1, inclusive.

The list TileId[ ctbAddrTS ] specifies the conversation from a CTB address in tile scan order to a tile id with ctbAddrTS ranging from 0 to (picHeightInCtbs * picWidthInCtbs) − 1, inclusive.

The values of colWidth, CtbAddrRStoTS, CtbAddrTStoRS and TileId are derived by invoking the CTB raster and tile scanning conversation process as specified in subclause Error: Reference source not found with PicHeightInCtbs and PicWidthInCtbs as inputs and the output is assigned to colWidth, CtbAddrRStoTS and TileId.

The values of ColumnWidthInLumaSamples[ i ], specifying the width of the i-th tile column in units of luma samples, are set equal to colWidth[ i ] << Log2CtbSize.

The array MinCbAddrZS[ x ][ y ], specifying the conversation from a location ( x, y ) in units of minimum CBs to a minimum CB address in z-scan order with x ranging from 0 to picWidthInMinCbs − 1, inclusive, and y ranging from 0 to picHeightInMinCbs − 1, inclusive, is dervied by invoking the Z scanning order array initialization process as specified in subclause Error: Reference source not found with Log2MinCbSize, Log2CtbSize, PicHeightInCtbs, PicWidthInCtbs, and the list CtbAddrRStoTS as inputs and the output is assigned to MinCbAddrZS.

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across tile boundaries. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

deblocking_filter_control_present_flag equal to 1 specifies the presence of deblocking filter control syntax elements in the picture parameter set and in the slice header for pictures referring to the picture parameter set. deblocking_filter_control_present_flag equal to 0 specifies the absence of deblocking filter control syntax elements in the picture parameter set and in the slice header for pictures referring to the picture parameter set.

deblocking_filter_override_enabled_flag equal to 1 specifies the presence of deblocking_filter_overriding_flag in the slice header for pictures referring to the picture parameter set. deblocking_filter_override_enabled_flag equal to 0 specifies the absence of deblocking_filter_overriding_flag in the slice header for pictures referring to the picture parameter set. When not present, the value of deblocking_filter_override_enabled_flag is inferred to be equal to 0.

pps_disable_deblocking_filter_flag equal to 1 specifies that the operation of deblocking filter shall not be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. pps_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. When not present, the value of pps_disable_deblocking_filter_flag is inferred to be equal to 0.

beta_offset_div2 and tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied for pictures referring to the picture parameter set unless the default deblocking parameter offsets are overriden by the deblocking parameter offsets present in the slice header for pictures referring to the picture parameter set.

pps_scaling_list_data_present_flag equal to 1 specifies that parameters are present to modify the scaling lists specified in the sequence parameter set. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling lists used for the pictures referring to the picture parameter set shall be inferred to be equal to those specified by the sequence parameter set. When scaling_list_enable_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0.

log2_parallel_merge_level_minus2 specifies the parallel processing level of merge/skip mode. The value of log2_parallel_merge_level_minus2 shall be in the range of 0 to log2_min_coding_block_size_minus3 + 1 + log2_diff_max_min_coding_block_size, inclusive.

slice_header_extension_present_flag equal to 0 specifies that no slice header extension syntax elements are present in the slice header for coded pictures referring to the picture parameter set. slice_header_extension_present_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for slice_header_extension_present_flag is reserved for future use by ITU-T | ISO/IEC.

slice_extension_present_flag equal to 0 specifies that no slice extension syntax elements are present in the slice layer RBSP for coded pictures referring to the picture parameter set. slice_extension_present_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for slice_extension_present_flag is reserved for future use by ITU-T | ISO/IEC.

pps_extension_flag equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. pps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for pps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for pps_extension_flag in a picture parameter set NAL unit.

pps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

#### Scaling list data semantics

scaling_list_pred_mode_flag equal to 0 specifies that the value of the scaling list is the same as the value of the scaling list already sent. The scaling list to copy the value is specified by scaling_list_pred_matrix_id_delta. scaling_list_pred_mode_flag equal to 1 speficies that the value of the scaling list is encoded by DPCM and exp-Golomb code.

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

RefMatrixID = MatrixID – scaling_list_pred_matrix_id_delta	(7-23)

where MatrixID is specified in Table 7-3. When scaling_list_pred_matrix_id_delta is equal to 0, i.e., RefMatrixID is equal to MatrixID, scalingList is inferred from the default scaling list ScalingList[ SizeID ][ MatrixID ] as specified in Table 7-4 and Table 7-5.

Table 7-2 – Specification of SizeID

| - Size of quantization matrix | - SizeID |
| --- | --- |
| - 4x4 | - 0 |
| - 8x8 (16x4, 4x16) | - 1 |
| - 16x16 (32x8, 8x32) | - 2 |
| - 32x32 | - 3 |

Table 7-3 – Specification of MatrixID according to SizeID, prediction type and colour component

| - SizeID | - Prediction type | - Colour component | - MatrixID |
| --- | --- | --- | --- |
| - 0, 1, 2 | - Intra | - Y | - 0 |
| - 0, 1, 2 | - Intra | - Cb | - 1 |
| - 0, 1, 2 | - Intra | - Cr | - 2 |
| - 0, 1, 2 | - Inter | - Y | - 3 |
| - 0, 1, 2 | - Inter | - Cb | - 4 |
| - 0, 1, 2 | - Inter | - Cr | - 5 |
| - 3 | - Intra | - Y | - 0 |
| - 3 | - Inter | - Y | - 1 |

#### Scaling list semantics

scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] plus 8 specifies the DC value of the scaling list for 16x16 size when sizeID is equal to 2 and specifies the DC value of the scaling list for 32x32 size when sizeID is equal to 3.

scaling_list_delta_coef specifies the difference of the matrix coefficient from the previous matrix coefficient, when pred_mode is equal to 1.

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |

Table 7-5 – Specification of default values of ScalingList[ 1..3 ][ MatrixID ][ 0 ][ i ] with i=0..63

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[1..2][0..2][i] - ScalingList[3][0][i] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 16 | - 17 | - 16 | - 17 | - 18 |
| - ScalingList[1..2][3..5][i] - ScalingList[3][1][i] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 17 | - 17 | - 18 |
| - i − 16 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][i] - ScalingList[3][0][i] | - 17 | - 18 | - 18 | - 17 | - 18 | - 21 | - 19 | - 20 | - 21 | - 20 | - 19 | - 21 | - 24 | - 22 | - 22 | - 24 |
| - ScalingList[1..2][3..5][i] - ScalingList[3][1][i] | - 18 | - 18 | - 18 | - 18 | - 18 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 24 | - 24 | - 24 | - 24 |
| - i − 32 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][i] - ScalingList[3][0][i] | - 24 | - 22 | - 22 | - 24 | - 25 | - 25 | - 27 | - 30 | - 27 | - 25 | - 25 | - 29 | - 31 | - 35 | - 35 | - 31 |
| - ScalingList[1..2][3..5][i] - ScalingList[3][1][i] | - 24 | - 24 | - 24 | - 24 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 28 | - 28 | - 28 | - 28 | - 28 |
| - i − 48 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][i] - ScalingList[3][0][i] | - 29 | - 36 | - 41 | - 44 | - 41 | - 36 | - 47 | - 54 | - 54 | - 47 | - 65 | - 70 | - 65 | - 88 | - 88 | - 115 |
| - ScalingList[1..2][3..5][i] - ScalingList[3][1][i] | - 28 | - 33 | - 33 | - 33 | - 33 | - 33 | - 41 | - 41 | - 41 | - 41 | - 54 | - 54 | - 54 | - 71 | - 71 | - 91 |

The four-dimensional array ScalingFactor[ SizeID ][ MatrixID ][ trafoType ][ i ], with i=0..( 1 << ( 4 + ( SizeID << 1 ) ) ) − 1 specifies the array of values of scaling factors according to the variables SizeID specified in Table 7-2, MatrixID specified in Table 7-3 and trafoType specified as follows:

- If the width of the quantization matrix is equal to the height of the quantization matrix, trafoType is set equal to 0,

- Otherwise, the width of the quantization matrix is greater than the height of the quantization matrix, trafoType is set equal to 1,

- Otherwise, trafoType is set equal to 2.

The elements of the quantization matrix of size 4x4, ScalingFactor[ 0 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 0 ][ MatrixID ][ 0 ][ y*4+x ] = ScalingList[ 0 ][ MatrixID ][ i ]	(7-24)
with i=0..15 and MatrixID=0..5

where zigZag is an array specified in subclause Error: Reference source not found with 4 as input, x=zigZag[ i ][ 0 ] and y=zigZag[ i ][ 1 ]

The elements of the quantization matrix of size 8x8, ScalingFactor[ 1 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 1 ][ MatrixID ][ 0 ][ y*8+x ] = ScalingList[ 1 ][ MatrixID ][ i ]	(7-25)
with i=0..63 and MatrixID=0..5

where zigZag is an array specified in subclause Error: Reference source not found with 8 as input, x=zigZag[ i ][ 0 ] and y=zigZag[ i ][ 1 ]

The elements of the quantization matrix of size 16x16, ScalingFactor[ 2 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ (y*2+j)*16+(x*2+k) ] = ScalingList[ 2 ][ MatrixID ][ i ]	(7-26)
with i=0..63, j=0..1, k=0..1 and MatrixID=0..5

where zigZag is an array specified in subclause Error: Reference source not found with 8 as input, x=zigZag[ i ][ 0 ] and y=zigZag[ i ][ 1 ]

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ MatrixID ] + 8	(7-27)
with MatrixID=0..5

The elements of the quantization matrix of size 32x32, ScalingFactor[ 3 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ (y*4+j)*32+(x*4+k) ] = ScalingList[ 3 ][ MatrixID ][ i ]	(7-28)
with i=0..63, j=0..3, k=0..3 and MatrixID=0..1

where zigZag is an array specified in subclause Error: Reference source not found with 8 as input, x=zigZag[ i ][ 0 ] and y=zigZag[ i ][ 1 ]

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ MatrixID ] + 8	(7-29)
with MatrixID=0..1

The elements of the quantization matrix of size 16x4, ScalingFactor[ 1 ][ MatrixID ][ 1 ][ ] is derived as follows:

ScalingFactor[ 1 ][ MatrixID ][ 1 ][ j*16+i ] = ScalingFactor[ 2 ][ MatrixID ][ 0 ][ (j*4)*16+i ]	(7-30)
with i=0..15, j=0..3 and MatrixID=0..5

The elements of the quantization matrix of size 4x16, ScalingFactor[ 1 ][ MatrixID ][ 2 ][ ] is derived as follows:

ScalingFactor[ 1 ][ MatrixID ][ 2 ][ j*4+i ] = ScalingFactor[ 2 ][ MatrixID ][ 0 ][ j*16+i*4 ]	(7-31)
with i=0..3, j=0..15 and MatrixID=0..5

The elements of the quantization matrix of size 32x8, ScalingFactor[ 2 ][ MatrixID ][ 1 ][ ] is derived as follows:

ScalingFactor[ 2 ][ MatrixID ][ 1 ][ j*32+i ] = ScalingFactor[ 3 ][ MatrixID ][ 0 ][ (j*4)*32+i ]	(7-32)
with i=0..31, j=0..7 and MatrixID=0..1

The elements of the quantization matrix of size 8x32, ScalingFactor[ 2 ][ MatrixID ][ 2 ][ ] is derived as follows:

ScalingFactor[ 2 ][ MatrixID ][ 2 ][ j*8+i ] = ScalingFactor[ 3 ][ MatrixID ][ 0 ][ j*32+i*4 ]	(7-33)
with i=0..7, j=0..31 and MatrixID=0..1

#### Adaptation parameter set RBSP semantics

aps_id identifies the adaptation parameter set that is referred to in the slice header. The value of aps_id shall be in the range of 0 to TBD, inclusive.

[Ed. (BB): Proponent suggests that this could be made level/profile dependend. ]

aps_alf_flag[ cIdx ] equal to 1 specifies that the ALF coefficients for the colour component cIdx are present in the current APS; equal 0 specifies that the ALF coefficients for the colour component cIdx are not present in the current APS.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

#### Adaptive loop filter picture info semantics

alf_no_filters_idc is used to derive the number of filters for the luma component in current picture. The values of alf_no_filters_idc shall be in the range of 0 to 2, inclusive.

alf_start_second_filter specifies the region adaptive (RA) mode index of luma samples for which the second filter is applied.

alf_filter_pattern_flag[ i ] equal to 1 specifies that the filter index for the (i+1)-th RA mode index is increased by 1: equal to 0 specifies that the filter index for the (i+1)-th RA mode index is the same as i-th RA mode index.

The number of filters AlfNumFilters is derived as follows.

- If alf_no_filters_idc is less than 2,

AlfNumFilters = alf_no_filters_idc + 1		(7-34)

- Otherwise (alf_no_filters_idc is equal to 2)

AlfNumFilters = (i alf_filter_pattern_flag[ i ]) + 1 with i = 0..14	(7-35)

The values of the vector MapFiltFidx[ i ] specifying the mapping between the i-th RA mode index for luma samples and the filter index are derived as follows.

- If AlfNumFilters is equal to 1,

MapFiltFidx[ i ] = 0 with i = 0..15		(7-36)

- Otherwise, if AlfNumFilters is equal to 2,

MapFiltFidx[ i ] = 0 with i = 0..alf_start_second_filter−1	(7-37)
MapFiltFidx[ i ] = 1 with i = alf_start_second_filter..15

- Otherwise (AlfNumFilters is greater than 2),

MapFiltFidx[ 0 ] = 0
MapFiltFidx[ i ] = MapFiltFidx[ i−1 ] + alf_filter_pattern[ i−1 ]	(7-38)
	with i = 1..15

alf_filt_coeff[ cIdx ][ i ][ j ] specifies the j-th filter coefficient of i-th filter used in the adaptive loop filtering process for the colour component cIdx. When cIdx is equal to 0, k-th order exponential-golomb (Exp-Golomb) code with a sign bit at the end is used where the order k for the j-th filter coefficient is equal to the value of alfkMap[ j ] which is specified as follows:

alfkMap[ j ] = {1, 2, 3, 4, 3, 1, 3, 3, 5, 0 } with j = 0..9	(7-39)

The restrictions apply for all values of cIdx=0..2 and i=0..15, whereas for the values of j it is different. When j=0..8, the values of alf_filt_coeff[cIdx][i][j] shall be in the range of −256 to 255, inclusive. With the restrictions for j=0..8, alf_filt_coeff[cIdx][i][9] is restricted by calculating centralCoeff and restricting the values thereof to be in the range of 0 to 511, inclusive.

centralCoeff = (alf_filt_coeff[ cIdx ][ i ][ 9 ] + (1<<8)−m(2*alf_filt_coeff[ cIdx] [ i ][ m ]) ), with m=0..8

#### Supplemental enhancement information RBSP semantics

Supplemental Enhancement Information (SEI) contains information that is not necessary to decode the samples of coded pictures from VCL NAL units.

##### Supplemental enhancement information message semantics

An SEI RBSP contains one or more SEI messages. Each SEI message consists of the variables specifying the type payloadType and size payloadSize of the SEI payload. SEI payloads are specified in Annex Error: Reference source not found. The derived SEI payload size payloadSize is specified in bytes and shall be equal to the number of RBSP bytes in the SEI payload.

NOTE – The NAL unit byte sequence containing the SEI message might include one or more emulation prevention bytes (represented by emulation_prevention_three_byte syntax elements). Since the payload size of an SEI message is specified in RBSP bytes, the quantity of emulation prevention bytes is not included in the size payloadSize of an SEI payload.

ff_byte is a byte equal to 0xFF identifying a need for a longer representation of the syntax structure that it is used within.

last_payload_type_byte is the last byte of the payload type of an SEI message.

last_payload_size_byte is the last byte of the payload size of an SEI message.

#### Access unit delimiter RBSP semantics

The access unit delimiter may be used to indicate the type of slices present in a coded picture and to simplify the detection of the boundary between access units. There is no normative decoding process associated with the access unit delimiter.

pic_type indicates that the slice_type values for all slices of the coded picture are members of the set listed in Table 7-6 for the given value of pic_type.

Table 7-6 – Meaning of pic_type

| pic_type | slice_type values that may be present in the coded picture |
| --- | --- |
| 0 | I |
| 1 | P, I |
| 2 | B, P, I |

#### Filler data RBSP semantics

The filler data RBSP contains bytes whose value shall be equal to 0xFF. No normative decoding process is specified for a filler data RBSP.

ff_byte is a byte equal to 0xFF.

#### Slice layer RBSP semantics

The slice layer RBSP consists of a slice header, slice data, and possibly slice extension data.

slice_extension_flag equal to 0 specifies that no slice_extension_data_flag syntax elements are present in the slice layer RBSP syntax structure. slice_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for slice_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for slice_extension_flag in a coded slice NAL unit.

slice_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

#### RBSP slice trailing bits semantics

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInVclNALunits be the sum of the values of NumBytesInNALunit for all VCL NAL units of a coded picture.

Let BinCountsInNALunits be the number of times that the parsing process function DecodeBin( ), specified in subclause Error: Reference source not found, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variables RawMinCUBits and PicSizeInMinCbs be derived as

RawMinCUBits = (1 << Log2MinCbSize ) * ( 1 << Log2MinCbSize ) * BitDepthY + 2 * (1 << Log2MinCbSize − 1) ) * ( 1 << Log2MinCbSize − 1 ) * BitDepthC, and

PicSizeInMinCbs = Ceil( pic_width_in_luma_samples ÷ ( 1 << Log2MinCbSize ) )* Ceil( pic_height_in_luma_samples ÷ ( 1 << Log2MinCbSize ) )

BinCountsInNALunits shall not exceed ( 32 ÷ 3 ) * NumBytesInVclNALunits + ( RawMinCUBits * PicSizeInMinCbs )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the slice layer NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNALunits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

#### RBSP trailing bits semantics

rbsp_stop_one_bit shall be equal to 1.

rbsp_alignment_zero_bit shall be equal to 0.

#### Byte alignment semantics

bit_equal_to_one shall be equal to 1.

bit_equal_to_zero shall be equal to 0.

### Slice header semantics

When present, the value of the slice header syntax elements pic_parameter_set_id, pic_output_flag, rap_pic_id, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx, num_long_term_pics and pic_temporal_mvp_enable_flag shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements poc_lsb_lt[ i ], delta_poc_msb_present_flag[ i ], delta_poc_msb_cycle_lt[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceCbAddrZS and SliceCtbAddrRS are both set to 0 and the decoding starts with the first coding tree block in the picture.

pic_parameter_set_id specifies the picture parameter set in use. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

slice_address specifies the address in slice granularity resolution in which the slice starts. The length of the slice_address syntax element is ( Ceil( Log2( PicWidthInCtbs * PicHeightInCtbs ) ) + SliceGranularity ) bits.

The variable SliceCtbAddrRS, specifying the coding tree block in which the slice starts in coding tree block raster scan order, is derived as follows.

SliceCtbAddrRS = ( slice_address >> SliceGranularity )	(7-40)

The variable SliceCbAddrZS, specifying the address of first coding block in the slice in minimum coding block granularity in z-scan order, is derived as follows.

SliceCbAddrZS = slice_address	<< ( ( log2_diff_max_min_coding_block_size − SliceGranularity ) <<1 )	(7-41)

[Ed. (BB): Change it that SliceCbAddrZS is in z-scan order in a coding tree block while the CTBs are in tiles scan order]

The slice decoding starts with the largest coding unit possible at the slice starting coordinate. [Ed. (BB): More precise description is needed here.]

dependent_slice_flag equal to 1 specifies that the value of each slice header syntax element not present is inferred to be equal to the value of corresponding slice header syntax element in the preceding slice containing the coding tree block for which the coding tree block address is SliceCtbAddrRS − 1. When not present, the value of dependent_slice_flag is inferred to be equal to 0. The value of dependent_slice_flag shall be equal to 0 when SliceCtbAddrRS equal to 0.

slice_type specifies the coding type of the slice according to Table 7-7.

Table 7-7 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - B (B slice) |
| - 1 | - P (P slice) |
| - 2 | - I (I slice) |

When nal_unit_type is equal to a value in the range of 4 to 8, inclusive (RAP picture), slice_type shall be equal to 2.

When sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] is equal to 0, slice_type shall be equal to 2.

colour_plane_id specifies the colour plane associated with the current slice RBSP when separate_colour_plane_flag is equal to 1. The value of colour_plane_id shall be in the range of 0 to 2, inclusive. colour_plane_id equal to 0, 1, and 2 correspond to the Y, Cb, and Cr planes, respectively.

NOTE 1 – There is no dependency between the decoding processes of pictures having different values of colour_plane_id.

pic_output_flag affects the decoded picture output and removal processes as specified in Annex Error: Reference source not found. When pic_output_flag is not present, it is inferred to be equal to 1.

rap_pic_id identifies a RAP picture. The values of rap_pic_id in all the slices of a RAP picture shall remain unchanged. When two consecutive access units in decoding order are both RAP access units, the value of rap_pic_id in the slices of the first such RAP access unit shall differ from the rap_pic_id in the second such RAP access unit. The value of rap_pic_id shall be in the range of 0 to 65535, inclusive.

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture. See Annex Error: Reference source not found. When the IDR or BLA picture is the first IDR or BLA picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the first IDR or BLA picture in the bitstream and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb is inferred to be equal to 0.

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active sequence parameter set. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active sequence parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active sequence parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_sps_flag )
	StRpsIdx = short_term_ref_pic_set_idx	(7-42)
else
	StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

poc_lsb_lt[ i ] specifies the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. The length of the poc_lsb_lt[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. For any values of j and k in the range of 0 to num_long_term_pics – 1, inclusive, if j is less than k, poc_lsb_lt[ j ] shall not be less than poc_lsb_lt[ k ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | |  poc_lsb_lt[ i − 1 ]  ! =  poc_lsb_lt[ i ] ) 
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]				
	else				(7-43)
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb – poc_lsb_lt[ i ] shall be in the range of 1 to 224 – 1, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

pic_temporal_mvp_enable_flag specifies whether temporal motion vector predictors can be used for inter prediction. If pic_temporal_mvp_enable_flag is equal to 0, the temporal motion vector predictors shall not be used in decoding of the current picture. If pic_temporal_mvp_enable_flag is equal to 1, temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of pic_temporal_mvp_enable_flag shall be inferred to be equal to 0.

When both pic_temporal_mvp_enable_flag and temporal_id are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vectors from any picture that precedes the current picture in decoding order.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_l0_active_minus1 is present for P and B slices and that the syntax element num_ref_idx_l1_active_minus1 is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are not present.

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that may be used to decode the slice. num_ref_idx_l0_active_minus1 shall be in the range of 0 to 15, inclusive. [Ed. (GJS): Constrain to the value imposed by the max DPB size limit instead of 15. (YK): It should be allowed to use the max number of active entries greater than the max DPB size limit as part of the level definition.]

When the current slice is a P or B slice and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 is inferred to be equal to num_ref_idx_l0_default_active_minus1.

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice.

When the current slice is a B slice and num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 is inferred to be equal to num_ref_idx_l1_default_active_minus1.

The range of num_ref_idx_l1_active_minus1 is constrained as specified in the semantics for num_ref_idx_l0_active_minus1 with l0 and list 0 replaced by l1 and list 1, respectively.

mvd_l1_zero_flag equal to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. mvd_l1_zero_flag equal to 0 indicates that mvd_l1[][][] is parsed.

cabac_init_flag specifies the method for determining the initialisation table used in the initialisation process for context variables. The value of cabac_init_flag shall be in the range of 0 to 1, inclusive. When cabac_init_flag is not present, it is inferred to be 0.

slice_qp_delta specifies the initial value of QPY to be used for all the coding blocks in the slice until modified by the value of cu_qp_delta in the coding unit layer. The initial QPY quantization parameter for the slice is computed as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-44)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the active picture parameter set shall be used for deblocking the current slice. deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the slice header shall be used for deblocking the current slice. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 1.

slice_header_disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall not be applied for the current slice. slice_header_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be applied for the current slice.

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice.

slice_sample_adaptive_offset_flag[ cIdx ] equal to 1 specifies that SAO is enabled for the colour component cIdx in the current slice; equal to 0 specifies that SAO is disabled for the colour component cIdx in the current slice. When slice_sample_adaptive_offset_flag[ cIdx ] is not present, slice_sample_adaptive_offset_flag[ cIdx ] is inferred to be equal to 0.

collocated_from_l0_flag equal to 1 specifies the picture that contains the collocated partition shall be derived from list 0, otherwise the picture shall be derived from list 1.When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

collocated_ref_idx specifies the reference index of the picture that contains the collocated partition. When the current slice is a P slice collocated_ref_idx refers to list 0. When the current slice is a B slice collocated_ref_idx refers to list 0 if collocated_from_l0 is 1, otherwise it refers to list 1. Collocated_ref_idx shall always refer to a valid list entry, and the resulting picture shall be the same for all slices of a coded picture. When collocated_ref_idx is not present, it is inferred to be equal to 0.

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 5 − five_minus_max_num_merge_cand	(7-45)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to 5, inclusive.

slice_alf_flag[ cIdx ] equal to 1 specifies that the ALF is enabled for the colour component cIdx in current slice; equal to 0 specifies that the ALF is disabled for the colour component cIdx in current slice. When there is an active APS and aps_alf_flag[ cIdx ] is equal to 0, the value of slice_alf_flag[ cIdx ] shall be equal to 0. When slice_alf_flag[ cIdx ] is not present, slice_alf_flag[ cIdx ] is inferred to be equal to 0.

slice_loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across slice boundaries; otherwise, the in-loop operations are slice-independent and not applied across slice boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter. When slice_loop_filter_across_slices_enabled_flag is not present, it is inferred to be equal to seq_loop_filter_across_slices_enabled_flag.

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to PicHeightInCtbs − 1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset[ i ] syntax elements.

entry_point_offset[ i ] specifies the i-th entry point offset, in bytes and shall be represented by offset_len_minus1 plus 1 bits. The coded slice data after the slice header consists of num_entry_point_offsets + 1 subsets, with subset index values ranging from 0 to num_entry_point_offsets, inclusive. Subset 0 consists of bytes 0 to entry_point_offset[ 0 ] − 1, inclusive, of the coded slice data, subset k, with k in the range of 1 to num_entry_point_offsets - 1, inclusive, consists of bytes entry_point_offset[ k − 1 ] to entry_point_offset[ k ] + entry_point_offset[ k − 1 ] − 1, inclusive, of the coded slice data, and the last subset (with subset index equal to num_entry_point_offsets) consists of the remaining bytes of the coded slice data.

When tiles_or_entropy_coding_sync_idc is equal to 1 and num_entry_point_offsets is greater than 0, each subset shall contain all coded bits of exactly one tile, and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to or less than the number of tiles in the slice.

NOTE 2 – When tiles_or_entropy_coding_sync_idc is equal to 1, each slice must include either a subset of one tile (in which case signalling of entry points is unnecessary) or an integer number of complete tiles.

When tiles_or_entropy_coding_sync_idc is equal to 2 and num_entry_point_offsets is greater than 0, each subset k with k in the range of 0 to num_entry_point_offsets − 1, inclusive, shall contain all coded bits of exactly one row of coding tree blocks, the last subset (with subset index equal to num_entry_point_offsets) shall contain all coded bits of the remaining coding blocks included in the slice, wherein the remaining coding blocks consist of either exactly one row of coding tree blocks or a subset of one row of coding tree blocks, and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to the number of rows of coding tree blocks in the slice, wherein a subset of one row of coding tree blocks in the slice is also counted..

NOTE 3 – When tiles_or_entropy_coding_sync_idc is equal to 2, a slice may include a number of rows of coding tree blocks and a subset of a row of coding tree blocks. For example, if a slice include two and a half rows of coding tree blocks, the the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to 3.

slice_header_extension_length specifies the length of the slice header extension data in bytes, not including the bits used for signalling slice_header_extension_length itself. The value of slice_header_extension_length shall be in the range of 0 to 256, inclusive.

slice_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

#### Short-term reference picture set semantics

A short-term reference picture set may be present in a sequence parameter set or in a slice header. If a short-term reference picture set is present in a slice header the content of the short-term reference picture set syntax structure shall be the same in all slice headers of a picture and the value of idx shall be equal to the syntax element num_ref_pic_sets from the active sequence parameter set.

inter_ref_pic_set_prediction_flag equal to 1 specifies that the reference picture set of the current picture shall be predicted using another reference picture set in the active sequence parameter set.

delta_idx_minus1 plus 1 specifies the difference between the index of the reference picture set of the current picture and the index of the reference picture set used for inter reference picture set prediction.

The variable RIdx shall be derived as follows.

RIdx = idx − (delta_idx_minus1+1)		(7-46)

delta_rps_sign specifies the sign of DiffPicOrderCnt( picA, picB ), where picA is associated with DeltaPoc[ RIdx ] and picB is associated with DeltaPoc[ idx ].  A value of 0 indicates that the sign is positive and a value of 1 indicates that the sign is negative.

abs_delta_rps_minus1 plus 1 specifies the absolute value of DiffPicOrderCnt( picA, picB ), where picA is associated with DeltaPoc[ RIdx ] and picB is associated with DeltaPoc[ idx ].

The variable DeltaRPS is derived as follows.

DeltaRPS = (1 – (delta_rps_sign << 1)) * (abs_delta_rps_minus1 + 1)	(7-47)

used_by_curr_pic_flag[ j ] indicates whether a picture is used for reference by the current picture.

use_delta_flag[ j ] specifies whether there is a corresponding i-th reference picture for determining the value of DeltaPoc[ Rdx ][ j ]. When use_delta_flag[ j ] is not present, it is inferred to be equal to 1.

Table 7-8 – used_by_curr_pic_flag[ j ] and use_delta_flag[ j ] operations for the reference picture set

| used_by_curr_pic_flag[ j ] | use_delta_flag[ j ] | Properties of i-th reference picture |
| --- | --- | --- |
| 1 | inferred | Picture is used for reference by the current picture DeltaPoc[ idx ][ i ] = DeltaPoc[ RIdx ][ j ] + DeltaRPS |
| 0 | 1 | Picture is not used for reference by the current picture (but used by future pictures) DeltaPoc[ idx ][ i ] = DeltaPoc[ RIdx ][ j ] + DeltaRPS |
| 0 | 0 | There is no corresponding i-th reference picture for determining the value of DeltaPoc[ RIdx ][ j ]. |

When inter_ref_pic_set_prediction_flag is equal to 1, the variables DeltaPocS0[ idx ][ i0 ], UsedByCurrPicS0[ idx ][ i0 ], DeltaPocS1[ idx ][ i1 ], UsedByCurrPicS1[ idx ][ i1], NumNegativePics[ idx ] and NumPositivePics[ idx ] are derived as follows.

for( j = 0, i0 = 0, i1 = 0; j <= NumDeltaPocs[ RIdx ]; j++ )
	if( used_by_curr_pic_flag[ j ]  | |  use_delta_flag[ j ] ) {
		DPoc = DeltaPoc[ RIdx ][ j ] + DeltaRPS
		if( DPoc < 0 ) {
			DeltaPocS0[ idx ][ i0 ] = DPoc	(7-48)
			UsedByCurrPicS0[ idx ][ i0 ] = used_by_curr_pic_flag[ j ]	(7-49)
			i0++
		} else {
			DeltaPocS1[ idx ][ i1 ] = DPoc	(7-50)
			UsedByCurrPicS1[ idx ][ i1 ] = used_by_curr_pic_flag[ j ]	(7-51)
			i1++
		}
	}
NumNegativePics[ idx ] = i0	(7-52)
NumPositivePics[ idx ] = i1	(7-53)

When DeltaPoc[ RIdx ][ j ] is unavailable, it is set to 0.

The elements of the lists DeltaPocS0[ idx ] and UsedByCurrPicS0[ idx ] are sorted together by the decoder, such that DeltaPocS0[ idx ][ i0 ] > DeltaPocS0[ idx ][ i0 + 1 ] for i0 in the range of 0 to NumNegativePics[ idx ] − 2.

The elements of the lists DeltaPocS1[ idx ] and UsedByCurrPicS1[ idx ] are sorted together by the decoder, such that DeltaPocS1[ idx ][ i1 ] < DeltaPocS1[ idx ][ i1 + 1 ] for i1 in the range of 0 to NumPositivePics[ idx ] − 2.

[Ed. (GJS): The reason for the language "sorted together" is that the list of flags needs to be reordered the same way that the list of delta pocs needs to be reordered. The reason for the language "by the decoder" is to make it clear that the sorting is an action, not just a property that will already be true at that point, and not an expression of a constraint on the encoder. This phrasing is not ideal, but it is an improvement.]

num_negative_pics specifies the number of the following delta_poc_s0_minus1[ i ] and used_by_curr_pic_s0_flag[ i ] syntax elements. The value of num_negative_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ], inclusive.

The variable NumNegativePics[ idx ] is derived as follows.

NumNegativePics[ idx ] = num_negative_pics		(7-54)

num_positive_pics specifies the number of the following delta_poc_s1_minus1[ i ] and used_by_curr_pic_s1_flag1[ i ] syntax elements. The value of num_positive_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] − num_negative_pics, inclusive.

The variable NumPositivePics[ idx ] is derived as follows.

NumPositivePics[ idx ] = num_positive_pics		(7-55)

delta_poc_s0_minus1[ i ] plus 1 specifies an absolute difference between two picture order count values. The value of delta_poc_s0_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

The variable DeltaPocS0[ idx ][ i ] is derived as follows.

if(i  = =  0) 
	DeltaPocS0[ idx ][ i ] = – ( delta_poc_s0_minus1[ i ] + 1 ) 	(7-56)
else
	DeltaPocS0[ idx ][ i ] = DeltaPocS0[ idx ][ i − 1 ] – ( delta_poc_s0_minus1[ i ] + 1 )

used_by_curr_pic_s0_flag[ i ] equal to 0 specifies that the i-th reference picture that has picture order count less than that of the current picture is not used for reference by the current picture.

The variable UsedByCurrPicS0[ idx ][ i ] is derived as follows.

UsedByCurrPicS0[ idx ][ i ] = used_by_curr_pic_s0_flag[ i ]	(7-57)

Let CurrPicOrderCntVal be the picture order count value of a current picture with RpsIdx equal to idx; If the reference picture with PicOrderCntVal equal to CurrPicOrderCntVal + DeltaPocS0[ idx ][ i ] has higher temporal_id than the current picture, UsedByCurrPicS0[ idx ][ i ] must be equal to 0.

delta_poc_s1_minus1[ i ] plus 1 specifies an absolute difference between two picture order count values. The value of delta_poc_s1_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

The variable DeltaPocS1[ idx ][ i ] is derived as follows.

if(i  = =  0) 
	DeltaPocS1[ idx ][ i ] = delta_poc_s1_minus1[ i ] + 1	(7-58)
else
	DeltaPocS1[ idx ][ i ] = DeltaPocS1[ idx ][ i − 1 ] + ( delta_poc_s1_minus1[ i ] + 1 )

used_by_curr_pic_s1_flag[ i ] equal to 0 specifies that the i-th reference picture that has picture order count greater than that of the current picture is not used for reference by the current picture.

The variable UsedByCurrPicS1[ idx ][ i ] is derived as follows.

UsedByCurrPicS1[ idx ][ i ] = used_by_curr_pic_s1_flag[ i ]	(7-59)

Let CurrPicOrderCntVal be the picture order count value of a current picture with RpsIdx equal to idx; If the reference picture with PicOrderCntVal equal to CurrPicOrderCntVal + DeltaPocS1[ idx ][ i ] has higher temporal_id than the current picture, UsedByCurrPicS1[ idx ][ i ] must be equal to 0.

The variables DeltaPoc[ idx ][ i ] and NumDeltaPocs[ idx ] are derived as follows.

for( i = 0; i < NumNegativePics[ idx ]; i++ )
	DeltaPoc[ idx ][ i ] = DeltaPocS0[ idx ][ i ]	(7-60)
for( i = 0; i < NumPositivePics[ idx ]; i++ )
	DeltaPoc[ idx ][ i + NumNegativePics[ idx ] ] = DeltaPocS1[ idx ][ i ]	(7-61)

NumDeltaPocs[ idx ] = NumNegativePics[ idx ] + NumPositivePics[ idx ]	(7-62)

#### Reference picture list modification semantics

ref_pic_list_modification_flag_lX (with X equal to 0 or 1) equal to 1 indicates that reference picture list X is specified explicitly as a list of list_entry_lX[ i ] values (with X being 0 or 1). ref_pic_list_modification_flag_lX equal to 0 indicates that reference picture list X is determined implicitly. When ref_pic_list_modification_flag_lX is not present, it is inferred to be equal to 0.

list_entry_lX[ i ] (with X equal to 0 or 1) specifies the index of the reference picture in RefPicSetCurrTempListX to be placed at the current position of reference picture list LX (with X being 0 or 1). The length of the list_entry_lX[ i ] syntax element is Ceil( Log2( NumPocTotalCurr) ) bits. The value of list_entry_lX[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive. If the syntax element list_entry_lX[ i ] is not present, it is inferred to be equal to 0. [Ed. (GJS): Unfortunately, NumPocTotalCurr is derived in the decoding process section. Properly, some restructuring of the text should be done to move its derivation to the semantics section, so that the parsing process does not need to reach into the decoding process in order to do its job.]

#### Weighted prediction parameters semantics

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag equal to 1 specifies that weighting factors for the luma component of list 0 prediction are present. luma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by (1 << luma_log2_weight_denom ) + delta_luma_weight_l0[ i ]. When luma_weight_l0_flag is equal to 1, the value of LumaWeightL0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, LumaWeightL0[ i ] is inferred to be equal to 2luma_log2_weight_denom for RefPicList0[ i ].

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction are present. chroma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2WeightDenom ) + delta_chroma_weight_l0[ i ][ j ]. When chroma_weight_l0_flag is equal to 1, the value of ChromaWeightL0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag is equal to 0, ChromaWeightL0[ i ][ j ] is inferred to be equal to 2ChromaLog2WeightDenom for RefPicList0[ i ].

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is specified as follows:

shift = 1 << ( BitDepthC − 1 )

ChromaOffsetL0[ i ][ j ] = (delta_chroma_offset_l0[i][j] – 
				( (shift*ChromaWeightL0[ i ][ j ]) >> ChromaLog2WeightDenom ) − shift )	 (7-63)

The variable ChromaOffsetL0[ i ][ j ] shall be in the range of −127 to 128, inclusive. When chroma_weight_l0_flag is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_l1_flag, delta_luma_weight_l1, luma_offset_l1, chroma_weight_l1_flag, delta_chroma_weight_l1, delta_chroma_offset_l1 have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

### Slice data semantics

ctb_alf_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] equal to 1 specifies that the adaptive loop filter is applied on current coding treeblock at location ( xCtb >> Log2CtbSize, yCtb >> Log2CtbSize ) for the colour component cIdx using filter coefficients alf_filt_coeff[ cIdx ]; equal to 0 specifies that the adaptive loop filter is not applied for the current coding treeblock at location ( xCtb >> Log2CtbSize, yCtb >> Log2CtbSize ) for the colour component cIdx. When ctb_alf_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] is not present, ctb_alf_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] is inferred to be 0.

#### Sample adaptive offset parameter semantics

sao_merge_left_flag equal to 1 specifies that the syntax elements sao_type_idx, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the left coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding tree block. When sao_merge_left_flag is not present, it is inferred to be equal to 0.

sao_merge_up_flag equal to 1 specifies that the syntax elements sao_type_idx, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the above coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the above coding tree block. When sao_merge_up_flag is not present, it is inferred to be equal to 0.

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-9 of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to 0.

Table 7-9 – Specification of the SAO type

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 | - 1D 0-degree edge offset |
| - 2 | - 1D 90-degree edge offset |
| - 3 | - 1D 135-degree edge offset |
| - 4 | - 1D 45-degree edge offset |
| - 5 | - Band offset |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the sample range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5.

When sao_band_position[ cIdx ][ rx ][ ry ] is not present it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx − 1 ][ ry ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to 0.

sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding tree block at position rx and ry for the colour component cIdx.

When sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_abs[ cIdx ][ rx − 1 ][ ry ][ i ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_abs[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to 0.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of sao_offset[ cIdx ][ rx ][ ry ][ i ] when sao_type_index is equal to 5.

When sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx − 1 ][ ry ][ i ]

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal 0.

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is less than 5 and i is greater than 1,offsetSign is set equal to −1.

- Otherwise, if sao_type_idx[ cIdx ][ rx ][ ry ] is less than 5 and i is less than 2, offsetSign is set equal to 1.

- Otherwise, if sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is equal to 0, offsetSign is set to equal to 1.

- Otherwise, offsetSign is set to equal to −1.

The array SaoOffsetVal is derived as follows.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-64)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			offsetSign*sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) ) 	(7-65)

### Coding tree semantics

split_coding_unit_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_coding_unit_flag[ x0 ][ y0 ] is not present, the following applies:

–	If log2CbSize is greater than Log2MinCbSize, the value of split_coding_unit_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	Otherwise (log2CbSize is equal to Log2MinCbSize), the value of split_coding_unit_flag[ x0 ][ y0 ] is inferred to be equal to 0.

end_of_slice_flag equal to 0 specifies that another coding block is following in the slice. end_of_slice_flag equal to 1 specifies the end of the slice and that no further coding block follows.

### Coding unit semantics

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.

skip_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ] are parsed after skip_flag[ x0 ][ y0 ]. skip_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not skipped. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode. The variable PredMode is derived as follows.

- If pred_mode_flag is equal to 0,

  - PredMode is set to MODE_INTER.

- Otherwise (pred_mode_flag is equal to 1),

  - PredMode is set to MODE_INTRA.

When pred_mode_flag is not present, the variables PredMode is derived as follows.

- If slice_type is equal to I,

  - PredMode is inferred to be equal to MODE_INTRA

- Otherwise (slice_type is equal to P or B), if skip_flag[ x0 ][ y0 ] is equal to 1,

  - PredMode is inferred to be equal to MODE_SKIP

part_mode specifies partitioning mode of the current coding unit. The semantics of part_mode depend on PredMode. The variables PartMode and IntraSplitFlag are derived from the value of part_mode as defined in Table 7-10.

The value of part_mode is restricted as follows.

- If PredMode is equal to MODE_INTRA, part_mode shall be equal to 0 or 1.

- Otherwise (PredMode is equal to MODE_INTER), the following applies

- If log2CbSize is greater than Log2MinCbSize, part_mode shall be in the range of 0 to 2, inclusive and in the range of 4 to 7, inclusive.

- Otherwise, if log2CbSize is equal to 3 and inter_4x4_enabled_flag is equal to 0, ,the value of part_mode shall be in the range of 0 to 2, inclusive.

- Otherwise (log2CbSize is greater than 3 or inter_4x4_enabled_flag is equal to q), the value of part_mode shall be in the range of 0 to 3, inclusive.

When part_mode is not present, the variables PartMode and IntraSplitFlag are derived as follows.

- PartMode is inferred to be equal to PART_2Nx2N,

- IntraSplitFlag is inferred to be equal to 0.

pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM. When the pcm_flag is not present, it shall be infered to be equal to 0.

num_subsequent_pcm specifies the number of subsequent I_PCM coding units with the current log2CbSize that successively follow the current I_PCM coding unit in the same depth of a TB. The values of pcm_flags of the subsequent coding units are set equal to 1. It is a requirement of bitstream conformance that the immediate roots of the current and subsequent I_PCM coding units are identical. The value num_subsequent_pcm shall be in the range of 0 to 3, inclusive.

pcm_alignment_zero_bit is a bit equal to 0.

pcm_sample_luma[ i ] represents a coded luma sample value in the raster scan within the coding unit. The number of bits used to represent each of these samples is PCMBitDepthY.

pcm_sample_chroma[ i ] represents a coded chroma sample value in the raster scan within the coding unit. The first half of the values represent coded Cb samples and the remaining half of the values represent coded Cr samples. The number of bits used to represent each of these samples is PCMBitDepthC.

Table 7-10 – Name association to prediction mode and partitioning type

| - PredMode | - part_mode | - IntraSplitFlag | PartMode |
| --- | --- | --- | --- |
| - MODE_INTRA | - 0 | - 0 | - PART_2Nx2N |
|  | - 1 | - 1 | - PART_NxN |
| - MODE_INTER | - 0 | - 0 | PART_2Nx2N |
|  | - 1 | - 0 | PART_2NxN |
|  | - 2 | - 0 | PART_Nx2N |
|  | - 3 | - 0 | PART_NxN |
|  | - 4 | - 0 | PART_2NxnU |
|  | - 5 | - 0 | PART_2NxnD |
|  | - 6 | - 0 | PART_nLx2N |
|  | - 7 | - 0 | PART_nRx2N |

no_residual_data_flag equal to 1 specifies that no residual data are present for the current coding unit. no_residual_data_flag equal to 0 specifies that residual data are present for the current coding unit.

When no_residual_data_flag is not present, its value is inferred to be equal to 0.

### Prediction unit semantics

mvp_l0_flag[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_l0_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvp_l1_flag[ x0 ][ y0 ] has the same semantics as mvp_l0_flag, with l0 and list 0 replaced by l1 and list 1, respectively.

prev_intra_luma_pred_flag[ x0 ][ y0 ], mpm_idx[ x0 ][ y0 ] and rem_intra_luma_pred_mode[ x0 ][ y0 ] specify the intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When prev_intra_luma_pred_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit according to subclause Error: Reference source not found.

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

merge_flag[ x0 ][ y0 ] specifies whether the inter prediction parameters for the current prediction unit are inferred from a neighbouring inter-predicted partition. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When merge_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1 or bi-prediction is used for the current prediction unit according to Table 7-11. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

Table 7-11 – Name association to inter prediction mode

| slice_type | inter_pred_idc | Name of inter_pred_idc |
| --- | --- | --- |
| P | inferred | Pred_L0 |
| B | 0 | Pred_L0 |
|  | 1 | Pred_L1 |
|  | 2 | Pred_BI |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to Pred_L0 when slice_type is equal to P and Pred_BI when slice_type is equal to B.

ref_idx_l0[ x0 ][ y0 ] specifies the list 0 reference picture index for the current prediction unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When ref_idx_l0[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

ref_idx_l1[ x0 ][ y0 ] has the same semantics as ref_idx_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

mvd_l0[ x0 ][ y0 ][ compIdx ], specifies the difference between a list 0 vector component to be used and its prediction. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When any of the two components is not present, the inferred value is 0.

mvd_l1[ x0 ][ y0 ][ compIdx ] has the same semantics as mvd_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

#### Motion vector difference coding semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus2[ compIdx ] is the absolute value of a motion vector component difference minus 2. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When abs_mvd_minus2[ compIdx ] is not present, it is inferred as follows.

- If abs_mvd_greater1_flag[ compIdx ] is equal to 0, abs_mvd_minus2[ compIdx ] is inferred to be equal to −1.

- Otherwise (abs_mvd_greater1_flag[ compIdx ] is equal to 1), abs_mvd_minus2[ compIdx ] is inferred to be equal to 0.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows.

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

### Transform tree semantics

The luma inter transform block split direction derivation process as specified in Error: Reference source not found is invoked with log2TrafoWidth, log2TrafoHeight and trafoDepth as inputs and the output is assigned to interTbSplitDirectionL.

The chroma inter transform block split direction derivation process as specified in Error: Reference source not found is invoked with log2TrafoWidth, log2TrafoHeight and trafoDepth as inputs and the output is assigned to interTbSplitDirectionC.

split_transform_flag[ x0L ][ y0L ][ trafoDepth ] specifies whether a block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0L, y0L specify the location ( x0L, y0L ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

The variable interSplitFlag is derived as follows.

- If max_transform_hierarchy_depth_inter is equal to 0 and PredMode is equal t oMODE_INTER and PartMode is not equal to PART_2Nx2N and trafoDepth is equal to 0, interSplitFlag is set to 1.

- Otherwise, interSplitFlag is set to 0.

When split_transform_flag[ x0L ][ y0L ][ trafoDepth ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, the value of split_transform_flag[ x0L ][ y0L ][ trafoDepth ] is inferred to be equal to 1.

- log2TrafoSize is greater than Log2MaxTrafoSize

- IntraSplitFlag is equal to 1 and trafoDepth is equal to 0

- interSplitFlag is equal to 1

- Otherwise, the value of split_transform_flag[ x0L ][ y0L ][ trafoDepth ] is inferred to be equal to 0.

cbf_luma[ x0L ][ y0L ][ trafoDepth ] equal to 1 specifies that the luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0L, y0L specify the location ( x0L, y0L ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_luma[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred to be equal to 1.

cbf_cb[ x0C ][ y0C ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0C, y0C specify the location ( x0C, y0C ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cb[ x0C ][ y0C ][ trafoDepth ] is not present, the value of cbf_cb[ x0C ][ y0C ][ trafoDepth ] is inferred as follows.

- If trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cb[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cb[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to 0.

cbf_cr[ x0C ][ y0C ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0C, y0C specify the location ( x0C, y0C ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cr[ x0C ][ y0C ][ trafoDepth ] is not present, the value of cbf_cr[ x0C ][ y0C ][ trafoDepth ] is inferred as follows.

- If trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cr[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cr[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to 0.

### Transform coefficient semantics

The transform coefficient levels are parsed into the arrays TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes and intra prediction modes, is derived as specified in Table 7-12.

Table 7-12 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 0 | 0 | 0 | 0 |
| 6 – 14 | 2 | 2 | 0 | 0 |
| 15 – 21 | 0 | 0 | 0 | 0 |
| 22 – 30 | 1 | 1 | 0 | 0 |
| 31 – 35 | 0 | 0 | 0 | 0 |

cu_qp_delta specifies the difference between a luma quantization parameter to be used and its prediction for a coding unit quantization group where the coding unit quantization group is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CbSize is greater than or equal to Log2MinCUDQPSize, the coding unit quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CbSize is equal to Log2MinCUDQPSize, the coding unit quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta is inferred to be equal to 0 when it is not present for any coding unit quantization group.

The luma quantization parameter QP’Y and the chroma quantization parameters QP’Cb and QP’Cr are derived by invoking the process specified in subclause Error: Reference source not found with ( x0 ,y0 ) as inputs.

### Residual coding semantics

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated transform block or not: The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1, specifies that no transform will be applied to the current transform block. When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

last_significant_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x_prefix shall be in the range from 0 to ( log2TrafoWidth << 1 ) − 1, inclusive.

last_significant_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_y_prefix shall be in the range from 0 to ( log2TrafoHeight << 1 ) − 1, inclusive.

last_significant_coeff_x_suffix specifies the suffix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x_suffix shall be in the range from 0 to ( 1 << ( ( last_significant_coeff_x_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The column position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffX is derived as follows.

- If last_significant_coeff_x_suffix is not present, the following applies.

LastSignificantCoeffX  =  last_significant_coeff_x_prefix	(7-66)

- Otherwise (last_significant_coeff_x_suffix is present), the following applies.

LastSignificantCoeffX  =  (1 << ((last_significant_coeff_x_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_x_prefix & 1))  +  	(7-67)
				last_significant_coeff_x_suffix

last_significant_coeff_y_suffix specifies the suffix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_y_suffix shall be in the range from 0 to ( 1 << (  ( last_significant_coeff_y_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The row position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffY is derived as follows.

- If last_significant_coeff_y_suffix is not present, the following applies.

LastSignificantCoeffY  =  last_significant_coeff_y_prefix	(7-68)

- Otherwise (last_significant_coeff_y_suffix is present), the following applies.

LastSignificantCoeffY  =  (1 << ((last_significant_coeff_y_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_y_prefix & 1))  +  	(7-69) 
				last_significant_coeff_y_suffix

When scanIdx is equal to 2, the coordinates are swapped as follows.

temp			=  LastSignificantCoeffX
LastSignificantCoeffX	=  LastSignificantCoeffY		(7-70)
LastSignificantCoeffY	=  temp

significant_coeff_group_flag[ xCG ][ yCG ] specifies for the coefficient group position ( xCG, yCG ) within the current transform block whether the corresponding coefficient group at location ( xCG, yCG ) has non-zero transform coefficient level. A coefficient group at location ( xCG, yCG ) is an array of 16 transform coefficient levels at locations ( xC, yC ).

- If significant_coeff_group_flag[ xCG ][ yCG ] is equal to 0, the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) are inferred to be equal to 0;

- Otherwise (significant_coeff_group_flag[ xCG ][ yCG ] is equal to 1), the following applies.

- If significant_coeff_group_flag[ xCG ][ yCG ] is present or ( xCG << 2, yCG << 2 ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ), at least one of the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) has a non zero value.

- Otherwise, at least one of the 16 significant_coeff_flag syntax elements is present for the coefficient group at location ( xCG, yCG )

When significant_coeff_group_flag[ xCG ][ yCG ] is not present, it is inferred as follows.

- If one or more of the following conditions are true, significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 1.

- ( xCG, yCG ) is equal to ( LastSignificantCoeffX >> 2, LastSignificantCoeffY >> 2 )

- ( xCG, yCG ) is equal to ( 0, 0 )

- Otherwise, significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 0.

significant_coeff_flag[ xC ][ yC ] specifies for the transform coefficient position ( xC, yC ) within the current transform block whether the corresponding transform coefficient level at location ( xC, yC ) is non-zero as follows.

- If significant_coeff_flag[ xC ][ yC ] is equal to 0, the transform coefficient level at location ( xC, yC ) is set equal to 0.

- Otherwise (significant_coeff_flag[ xC ][ yC ] is equal to 1), the transform coefficient level at location ( xC, yC ) has a non-zero value.

When significant_coeff_flag[ xC ][ yC ] is not present, it is inferred as follows.

- If ( xC, yC ) is the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order or both of the following conditions are true, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 1

- ( xC, yC ) is equal to ( xCG << 2, yCG << 2 )

- implicitNonZeroCoeff is equal to 1

- Otherwise, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.

coeff_abs_level_greater1_flag[ n ] specifies for the scanning position n whether there are transform coefficient levels greater than 1.

When coeff_abs_level_greater1_flag[ n ] is not present, it is inferred to be equal to 0.

coeff_abs_level_greater2_flag[ n ] specifies for the scanning position n whether there are transform coefficient levels greater than 2.

When coeff_abs_level_greater2_flag[ n ] is not present, it is inferred to be equal to 0.

coeff_sign_flag[ n ] specifies the sign of a transform coefficient level for the scanning position n as follows.

- If coeff_sign_flag[ n ] is equal to 0, the corresponding transform coefficient level has a positive value.

- Otherwise (coeff_sign_flag[ n ] is equal to 1), the corresponding transform coefficient level has a negative value.

When coeff_sign_flag[ n ] is not present, it is inferred to be equal to 0.

coeff_abs_level_remaining[ n ] is the remaining absolute value of a transform coefficient level that is coded with Golomb-Rice code at the scanning position n. The value of coeff_abs_level_remaining is constrained by the limits in subclause Error: Reference source not found.When coeff_abs_level_remaining [ n ] is not present, it is inferred as 0.

# Decoding process

# Parsing process

Inputs to this process are bits from the RBSP.

Outputs of this process are syntax element values.

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause 7.3 is equal to tu(n) (see subclause 9.1), ue(v), se(v) (see subclause 9.2.1), kes(v) (see subclause 9.2.2), or ae(v) (see subclause 9.3).

## Parsing process for Truncated Unary codes

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause 7.3 is equal to tu(n).

Inputs to this process are bits from the RBSP.

Outputs of this process are syntax element values.

Syntax elements coded as tu(n) are truncated unary coded. The range of possible values for the syntax element is determined first. The range of this syntax element may be between 0 and n, with n being greater than or equal to 1 and the range is used in the derivation of the value of the syntax element value. codeNum which is equal to the value of the syntax element is given by a process specified as follows:

codeNum = 0
keepGoing = 1
for(i = 0; i < n && keepGoing; i++){
	keepGoing = read_bits( 1 ) 	(9-71)
	if( keepGoing )
		codeNum ++
}

## Parsing process for Exp-Golomb codes

### Parsing process for 0-th order Exp-Golomb codes

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause 7.3 is equal to ue(v) or se(v).

Inputs to this process are bits from the RBSP.

Outputs of this process are syntax element values.

Syntax elements coded as ue(v) or se(v) are Exp-Golomb-coded. The parsing process for these syntax elements begins with reading the bits starting at the current location in the bitstream up to and including the first non-zero bit, and counting the number of leading bits that are equal to 0. This process is specified as follows:

leadingZeroBits = −1
for( b = 0; !b; leadingZeroBits++ )	(9-72)
	b = read_bits( 1 )

The variable codeNum is then assigned as follows:

codeNum = 2leadingZeroBits − 1 + read_bits( leadingZeroBits ) 	(9-73)

where the value returned from read_bits( leadingZeroBits ) is interpreted as a binary representation of an unsigned integer with most significant bit written first.

Table 9-13 illustrates the structure of the Exp-Golomb code by separating the bit string into "prefix" and "suffix" bits. The "prefix" bits are those bits that are parsed in the above pseudo-code for the computation of leadingZeroBits, and are shown as either 0 or 1 in the bit string column of Table 9-13. The "suffix" bits are those bits that are parsed in the computation of codeNum and are shown as xi in Table 9-13, with i being in the range 0 to leadingZeroBits − 1, inclusive. Each xi can take on values 0 or 1.

Table 9-13 – Bit strings with "prefix" and "suffix" bits and assignment to codeNum ranges (informative)

| Bit string form | Range of codeNum |
| --- | --- |
| 1 | 0 |
| 0 1 x0 | 1..2 |
| 0 0 1 x1 x0 | 3..6 |
| 0 0 0 1 x2 x1 x0 | 7..14 |
| 0 0 0 0 1 x3 x2 x1 x0 | 15..30 |
| 0 0 0 0 0 1 x4 x3 x2 x1 x0 | 31..62 |
| … | … |

Table 9-14 illustrates explicitly the assignment of bit strings to codeNum values.

Table 9-14 – Exp-Golomb bit strings and codeNum in explicit form and used as ue(v) (informative)

| Bit string | codeNum |
| --- | --- |
| 1 | 0 |
| 0 1 0 | 1 |
| 0 1 1 | 2 |
| 0 0 1 0 0 | 3 |
| 0 0 1 0 1 | 4 |
| 0 0 1 1 0 | 5 |
| 0 0 1 1 1 | 6 |
| 0 0 0 1 0 0 0 | 7 |
| 0 0 0 1 0 0 1 | 8 |
| 0 0 0 1 0 1 0 | 9 |
| … | … |

Depending on the descriptor, the value of a syntax element is derived as follows.

–	If the syntax element is coded as ue(v), the value of the syntax element is equal to codeNum.

–	Otherwise, if the syntax element is coded as se(v), the value of the syntax element is derived by invoking the mapping process for signed Exp-Golomb codes as specified in subclause 9.2.1.1 with codeNum as the input.

#### Mapping process for signed Exp-Golomb codes

Input to this process is codeNum as specified in subclause 9.2.1.

Output of this process is a value of a syntax element coded as se(v).

The syntax element is assigned to the codeNum by ordering the syntax element by its absolute value in increasing order and representing the positive value for a given absolute value with the lower codeNum. Table 9-15 provides the assignment rule.

Table 9-15 – Assignment of syntax element to codeNum for signed Exp-Golomb coded syntax elements se(v)

| codeNum | syntax element value |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | −1 |
| 3 | 2 |
| 4 | −2 |
| 5 | 3 |
| 6 | −3 |
| k | (−1)k+1 Ceil( k÷2 ) |

### Parsing process for k-th order Exp-Golomb codes

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause 7.3 is equal to kes(v).

Inputs to this process are bits from the RBSP.

Outputs of this process are syntax element values.

Syntax elements coded as kes(v) are k-th order Exp-Golomb-coded followed by a sign bit. The parsing process for these syntax elements begins with reading the bits starting at the current location in the bitstream up to and including the first zero bit, and counting the number of leading bits that are equal to 1. This process is specified as follows:

leadingOneBits = –1
for( b = 1; b!=0; leadingOneBits++ )	(9-74)
	b = read_bits( 1 )

The variable codeNum is then assigned as follows:

codeNum = (2leadingOneBits – 1) * 2k + read_bits( leadingOneBits+k )	(9-75)

where the value returned from read_bits( leadingOneBits+k ) is interpreted as a binary representation of an unsigned integer with most significant bit written first. When codeNum is not equal to zero, one signed bit is parsed as follows:

if ( codeNum != 0 ) {
	sign = read_bits( 1 )	(9-76)
	codeNum = (sign = = 1) ? codeNum : (−codeNum)
}

The value of a syntax element is equal to codeNum.

## CABAC parsing process for slice data

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses 7.3.3.3 and 7.3.4.1.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause 7.3.3.3, the initialization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.

Figure 9-2 – Spatial neighbour T that is used to invoke the coding tree block availability derivation process relative to the current coding tree block (informative)

The mininum coding block address of the coding tree block containing the spatial neighbor block T (Figure 9-2), ctbMinCbAddrT, is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows.

x = x0 + 2 << Log2CtbSize − 1
y = y0 − 1
ctbMinCbAddrT = MinCbAddrZS[ x >> Log2MinCbSize ][ y >> Log2MinCbSize ]	(9-77)

The variable availableFlagT is obtained by invoking the coding block availability derivation process as specified in subclause Error: Reference source not found with ctbMinCbAddrT as input.

When starting the parsing of a coding tree as specified in subclause 7.3.4.1, , the following ordered steps apply.

- The arithmetic decoding engine is initialised as follows.

  - If CtbAddrRS is equal to slice_address, dependent_slice_flag is equal to 1 and tiles_or_entropy_coding_sync_idc is equal to 2, the following applies.

  - The synchronization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.3 with TableStateIdxDS and TableMPSValDS as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause 9.3.1.4.

  - Otherwise if tiles_or_entropy_coding_sync_idc is equal to 2, and CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

  - When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.3 with TableStateIdxWPP and TableMPSValWPP as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause 9.3.1.4.

- When tiles_or_entropy_coding_sync_idc is equal to 2, the memorization process is applied as follows.

  - When CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.2 with TableStateIdxWPP and TableMPSValWPP as output.

  - When dependent_slice_flag is equal to 1, and end_of_slice_flag is equal to 1, the memorization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.2 with TableStateIdxDS and TableMPSValDS as output.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause Error: Reference source not found.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause Error: Reference source not found.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause Error: Reference source not found.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause Error: Reference source not found.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element pcm-flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit, num_subsequent_pcm, and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.3.1.2.

### Initialization process

Outputs of this process are initialised CABAC internal variables.

The processes in subclauses 9.3.1.1 and 9.3.1.2 are invoked when starting the parsing of the slice data of a slice in subclause 7.3.3.3 or when starting the parsing of the data of a coding tree in subclause 7.3.5 and the coding tree is the first coding tree in a tile.

#### Initialization process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdxTable and ctxIdx.

Table 9-22 to Table 9-45 contain the values of the 8 bit variable initValue used in the initialization of context variables that are assigned to all syntax elements in subclauses 7.3.3.3 to 7.3.10 except for the end-of-slice flag.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and intersecIdx are derived according to the following pseudo-code process:

slopeIdx = initValue >> 4
intersecIdx = initValue & 15

Slope m and Intersec n are derived from the indices as follows:

m = slopeIdx*5 − 45
n = ( intersecIdx << 3 ) − 16

The two values assigned to pStateIdx and valMPS for the initialization are derived from SliceQPY, which is derived in Equation 7-44. Given the variable m and n, the initialization is specified by the following pseudo-code process:

preCtxState = Clip3( 1, 126, ( ( m * Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )
valMPS = ( preCtxState  <=  63) ? 0 : 1
pStateIdx = valMPS ? (preCtxState − 64) : (63 – preCtxState)	(9-78)

The variable initType in  is derived as follows:

if( slice_type = = I )
	initType = 0
else if(slice_type = = P )
	initType = cabac_init_flag ? 2 : 1
else
	initType = cabac_init_flag ? 1 : 2

Table 9-16 – Association of ctxIdx and syntax elements for each initialisationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao_param() | sao_merge_left_flag | Table 9-17 | 0..2 | 3..5 | 6..8 |
|  | sao_merge_up_flag | Table 9-18 | 0 | 1 | 2 |
|  | sao_type_idx | Table 9-19 | 0..1 | 2..3 | 4..5 |
|  | sao_offset_abs | Table 9-20 | 0..1 | 2..3 | 4..5 |
| alf_cu_control_param() | alf_cu_flag | Table 9-21 | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Table 9-22 | 0..2 | 3..5 | 6..8 |
| coding_unit() | cu_transquant_bypass_flag | Table 9-23 | 0 | 1 | 2 |
|  | skip_flag | Table 9-24 |  | 0..2 | 3..5 |
|  | cu_qp_delta | Table 9-25 | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Table 9-26 |  | 0 | 1 |
|  | part_mode | Table 9-26 | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Table 9-27 | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Table 9-28 | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Table 9-29 |  | 0 | 1 |
|  | merge_idx | Table 9-30 |  | 0 | 1 |
|  | inter_pred_idc | Table 9-31 |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Table 9-32 |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Table 9-33 |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Table 9-33 |  | 2 | 3 |
|  | mvp_l0_flag, mvp_l1_flag | Table 9-34 |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Table 9-35 |  | 0 | 1 |
|  | split_transform_flag | Table 9-36 | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Table 9-37 | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Table 9-38 | 0..2 | 3..5 | 6..8 |
| residual_coding() | transform_skip_flag[ ][ ][ 0 ] | Table 9-39 | 0 | 1 | 2 |
|  | transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Table 9-39 | 3 | 4 | 5 |
|  | last_significant_coeff_x_prefix | Table 9-40 | 0..17 | 18..35 | 36..54 |
|  | last_significant_coeff_y_prefix | Table 9-41 | 0..17 | 18..35 | 36..54 |
|  | significant_coeff_group_flag | Table 9-42 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Table 9-43 | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Table 9-44 | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag | Table 9-45 | 0..5 | 6..11 | 12..17 |

NOTE 2 – ctxIdxTable equal to 0 and ctxIdx equal to 0 are associated with the end_of_slice_flag. The decoding process specified in subclause Error: Reference source not found applies to ctxIdxTable equal to 0 and ctxIdx equal to 0. This decoding process, however, may also be implemented by using the decoding process specified in subclause Error: Reference source not found. In this case, the initial values associated with ctxIdxTable equal to 0 and ctxIdx equal to 0 are specified to be pStateIdx = 63 and valMPS = 0, where pStateIdx = 63 represents a non-adapting probability state.

Table 9-17 – Values of variable initValue for sao_merge_left_flag ctxIdx

| Initialization variable | sao_merge_left_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 118 | 118 | 118 | 102 | 102 | 102 | 102 | 102 | 102 |

Table 9-18 – Values of variable initValue for sao_merge_up_flag ctxIdx

| Initialization variable | sao_merge_up_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-19 – Values of variable initValue for sao_type_idx ctxIdx

| Initialization variable | sao_type_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 61 | 104 | 168 | 120 | 184 | 120 |

Table 9-20 – Values of variable initValue for sao_offset_abs ctxIdx

| Initialization variable | sao_offset_abs ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 171 | 119 | 199 | 169 | 119 | 151 |

Table 9-21 – Values of variable initValue for alf_cu_flag ctxIdx

| Initialization variable | alf_cu_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-22 – Values of variable initValue for split_coding_unit_flag ctxIdx

| Initialization variable | split_coding_unit_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 87 | 74 | 107 | 84 | 103 | 105 | 84 | 103 | 105 |

Table 9-23 – Values of variable initValue for cu_transquant_bypass_flag ctxIdx

| Initialization variable | cu_transquant_bypass_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-24 – Values of variable initValue for skip_flag ctxIdx

| Initialization variable | skip_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 165 | 168 | 154 | 165 | 168 | 154 |

Table 9-25 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 |

Table 9-26 – Values of variable initValue for pred_mode and part_mode

| Initialization variable | pred_type ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 114 | 98 | 167 | 119 | 87 | 119 | 119 | 87 | 119 |

Table 9-27 – Values of variable initValue for prev_intra_luma_pred_flag ctxIdx

| Initialization variable | prev_intra_luma_pred_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 167 | 119 | 150 |

Table 9-28 – Values of variable initValue for intra_chroma_pred_mode ctxIdx

| Initialization variable | intra_chroma_pred_mode ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 53 | 103 | 85 | 87 | 101 | 87 |

Table 9-29 – Value of variable initValue for merge_flag ctxIdx

| Initialization variable | merge_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 72 | 119 |

Table 9-30 – Values of variable initValue for merge_idx ctxIdx

| Initialisation variable | merge_idx ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 100 | 116 |

Table 9-31 – Values of variable initValue for inter_pred_idc ctxIdx

| Initialisation variable | inter_pred_idc ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 |  |  |  |
| initValue | 95 | 79 | 63 | 31 | 31 |  |  |  |

Table 9-32 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

Table 9-33 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialization variable | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | 120 | 166 | 135 | 166 |

Table 9-34 – Values of variable initValue for mvp_l0_flag, mvp_l1_flag ctxIdx

| Initialization variable | mvp_l0_flag, mvp_l1_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 134 | 134 |

Table 9-35 – Values of variable initValue for no_residual_data_flag ctxIdx

| Initialization variable | no_residual_data_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 39 | 39 |

Table 9-36 – Values of variable initValue for split_transform_flag ctxIdx

| Initialization variable | split_transform_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 119 | 162 | 148 | 100 | 119 | 71 | 86 | 55 | 119 | 102 | 86 | 86 |

Table 9-37 – Values of variable initValue for cbf_luma ctxIdx

| Initialization variable | cbf_luma ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 73 | 74 | 102 | 89 | 102 | 89 |

Table 9-38 – Values of variable initValue for cbf_cb and cbf_cr ctxIdx

| Initialization variable | cbf_cb ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 55 | 86 | 133 | 114 | 84 | 117 | 114 | 68 | 117 |

Table 9-39 – Values of variable initValue for transform_skip_flag ctxIdx

| Initialization variable | transform_skip_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 109 | 102 | 102 | 102 | 102 | 102 |

Table 9-40 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 9-41 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 9-42 – Values of variable initValue for significant_coeff_group_flag ctxIdx

| Initialization variable | significant_coeff_group_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 83 | 122 | 98 | 121 | 99 | 120 | 67 | 119 | 99 | 120 | 67 | 119 |

Table 9-43 – Values of variable initValue for significant_coeff_flag ctxIdx

| Initialization variable | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 74 | 73 | 88 | 72 | 72 | 55 | 71 | 54 | 71 | 88 | 103 | 71 | 53 | 87 | 134 | 86 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 84 | 70 | 68 | 89 | 90 | 84 | 88 | 74 | 130 | 118 | 88 | 120 | 87 | 149 | 70 | 52 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 118 | 133 | 116 | 114 | 129 | 132 | 162 | 115 | 51 | 115 | 66 | 120 | 74 | 115 | 87 | 89 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 118 | 134 | 118 | 101 | 68 | 85 | 101 | 116 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initValue | 100 | 68 | 67 | 136 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 70 | 53 | 67 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initValue | 117 | 102 | 117 | 1158 | 114 | 84 | 115 | 99 | 100 | 83 | 114 | 152 | 168 | 131 | 150 | 120 |
|  | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 71 | 103 | 118 | 101 | 68 | 85 | 101 | 116 |
|  | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| initValue | 116 | 68 | 67 | 152 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 86 | 84 | 67 |
|  | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| initValue | 117 | 102 | 117 | 115 | 99 | 100 | 115 | 99 | 100 | 83 | 114 | 152 | 152 | 131 | 150 | 120 |

Table 9-44 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 104 | 68 | 116 | 86 | 104 | 132 | 86 | 87 | 102 | 66 | 114 | 68 | 87 | 84 | 100 | 101 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 104130 | 130 | 147 | 149 | 104 | 196 | 100 | 165 | 119 | 179 | 179 | 164 | 119 | 85 | 117 | 149 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 116 | 135 | 146 | 147 | 164 | 119 | 148 | 116 | 133 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 119 | 179 | 148 | 164 | 119 | 85 | 117 | 149 | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 100 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |  |  |  |  |  |  |  |  |
| initValue | 135 | 177 | 147 | 164 | 119 | 132 | 148 | 149 |  |  |  |  |  |  |  |  |

Table 9-45 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 86 | 102 | 115 | 117 | 101 | 101 | 84 | 117 | 83 | 100 | 84 | 117 |
|  | 12 | 13 | 14 | 15 | 16 | 17 |  |  |  |  |  |  |
| initValue | 84 | 117 | 83 | 84 | 84 | 117 |  |  |  |  |  |  |

#### Memorization process for context variables

Inputs of this process are the CABAC context variables indexed by ctxIdx.

Output of this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMPS used in the initialization process of context variables that are assigned to all syntax elements in subclauses 7.3.3.3 and 7.3.4.1 except for the end_of_slice_flag.

For each context variable, the corresponding entries pStateIdx and valMPS of tables tableStateSync and tableMPSSync are initialized to the corresponding pStateIdx and valMPS.

#### Synchronization process for context variables

Inputs of this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMPS used in the memorization process of context variables that are assigned to all syntax elements in subclauses 7.3.3.3 and 7.3.4.1 except for the end_of_slice_flag.

Outputs of this process are the CABAC context variables indexed by ctxIdx..

For each context variable, the corresponding context variables pStateIdx and valMPS are initialized to the corresponding entries pStateIdx and valMPS of tables tableStateSync and tableMPSSync.

#### Initialization process for the arithmetic decoding engine

This process is invoked before decoding the first coding tree block of a slice.

Outputs of this process are the initialised decoding engine registers codIRange and codIOffset both in 16 bit register precision.

The status of the arithmetic decoding engine is represented by the variables codIRange and codIOffset. In the initialization procedure of the arithmetic decoding process, codIRange is set equal to 510 and codIOffset is set equal to the value returned from read_bits( 9 ) interpreted as a 9 bit binary representation of an unsigned integer with most significant bit written first.

The bitstream shall not contain data that result in a value of codIOffset being equal to 510 or 511.

NOTE – The description of the arithmetic decoding engine in this Recommendation | International Standard utilizes 16 bit register precision. However, a minimum register precision of 9 bits is required for storing the values of the variables codIRange and codIOffset after invocation of the arithmetic decoding process (DecodeBin) as specified in subclause Error: Reference source not found. The arithmetic decoding process for a binary decision (DecodeDecision) as specified in subclause Error: Reference source not found and the decoding process for a binary decision before termination (DecodeTerminate) as specified in subclause Error: Reference source not found require a minimum register precision of 9 bits for the variables codIRange and codIOffset. The bypass decoding process for binary decisions (DecodeBypass) as specified in subclause Error: Reference source not found requires a minimum register precision of 10 bits for the variable codIOffset and a minimum register precision of 9 bits for the variable codIRange.