| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0217 |
| --- | --- |

| Title: | On dependent slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jeonghoon Park | Email: | tammy.lee@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the previous meeting, dependent slice is adopted. According to the specification, dependent slice cannot be used with entropy slice while it can be used with either WPP or tile within a set of pictures. Since entropy slice is regarded as similar tools to WPP or tile to enable parallel processing, it is proposed to enable dependent slice with entropy slice within a set of pictures for consistency. Or it is proposed to restrict the usage of dependent slice to be used only if WPP is used.

# Introduction

In the current specification[1], dependent slice is enabled only when dependent_slice_enabled_flag is equal to 1 and either of the following condition is satisfied:

- tiles_or_entropy_coding_sync_idc is in range of 0 ~ 2 (0: no parallel tools, 1: tiles, 2: WPP)

- tiles_or_entropy_coding_sync_idc is equal to 3 and cabac_independent_flag is equal to 0

where dependent_slice_enabled_flag, tiles_or_entropy_coding_sync_idc and cabac_independent_flag are coded in PPS and cabac_independent_flag is coded only if tiles_or_entropy_coding_sync_idc is equal to 3.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| dependent_slice_enabled_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| … | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| … |  |
| } |  |

dependent_slice_enabled_flag equal to 1 specifies the presence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. dependent_slice_enabled_flag equal to 0 specifies the absence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. When tiles_or_entropy_coding_sync_idc is equal to 3, the value of dependent_slice_enabled_flag shall be equal to 1.

cabac_independent_flag equal to 1 specifies that CABAC decoding of coding blocks in a slice is independent from any state of the previously decoded slice. cabac_independent_flag equal to 0 specifies that CABAC decoding of coding blocks in a slice is dependent from the states of the previously decoded slice. When not present, the value of cabac_independent_flag is inferred to be equal to 0.

On the other hand, Entropy slice is only enabled when dependent_slice_enabled_flag == 1, tiles_or_entropy_coding_sync_idc == 3 and cabac_independent_flag == 1.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| … |  |
| } |  |
| … |  |
| } |  |

dependent_slice_flag equal to 1 specifies that the value of each slice header syntax element not present is inferred to be equal to the value of corresponding slice header syntax element in the preceding slice containing the coding tree block for which the coding tree block address is SliceCtbAddrRS − 1. When not present, the value of dependent_slice_flag is inferred to be equal to 0. The value of dependent_slice_flag shall be equal to 0 when SliceCtbAddrRS equal to 0.

# Proposal

Proposed working draft text is included below using the following text conventions:

- Removed text is shown with strikethrough.

- Newly added text is shown in red.

## Conditional coding of dependent_slice_enabled_flag

According to semantics of dependent_slice_enabled_flag, dependent_slice_enabled_flag is required to be 1 when tiles_or_entropy_coding_sync_idc is equal to 3. It is proposed to modify dependent_slice_enabled_flag coding order to be signaled after checking tiles_or_entropy_coding_sync_idc to impose it implicitly.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| dependent_slice_enabled_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc ! = 3 ) |  |
| dependent_slice_enabled_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| … | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| … |  |
| } |  |

dependent_slice_enabled_flag equal to 1 specifies the presence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. dependent_slice_enabled_flag equal to 0 specifies the absence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. When tiles_or_entropy_coding_sync_idc is equal to 3, the value of dependent_slice_enabled_flag shall be equal to 1. When not present, the value of dependent_slice_enabled_flag is inferred to be equal to 1

## Extended usage of dependent slice

The current syntax means that

- Entropy slice is exclusively used with tiles, WPP and dependent slice

- Dependent slice is exclusively used with entropy slice

- Tiles is exclusively used with WPP and entropy slice

- WPP is exclusively used with Tiles and entropy slice

Since entropy slice is regarded as similar tools to WPP or tile to enable parallel processing, it was adopted to be used exclusively between entropy slice, WPP and tile within a set of pictures. Technically, dependent slice also can be used with entropy slice in a picture. For example, a picture can contain three slices where the first slice is normal slice, the second slice is entropy slice and the third slice is dependent slice. Therefore, for consistency, it is proposed to enable dependent slice extensively like

- Entropy slice, tile and WPP cannot be used concurrently

- Dependent slice can be used with either entropy slice, tile or WPP

The proposed syntax change is to move cabac_independent_flag from PPS to slice_header so as to enable slice to be entropy slice or dependent slice (or a normal slice) in a picture.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| dependent_slice_enabled_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| … | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( dependent_slice_flag && tiles_or_entropy_coding_sync_idc = = 3) |  |
| cabac_independent_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| … |  |
| } |  |
| … |  |
| } |  |

## Restricted usage of dependent slice

According to the proposal [2], the motivation of dependent slices is to enable low delay coding for WPP. Even though it can be used with tiles, it cannot be effective in the first slice of a tile because tiles are independent. Unless it is found that there are useful cases when dependent slice is enabled with tiles, it is proposed to restrict dependent slice on WPP by the proposed syntax modification.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| dependent_slice_enabled_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| … | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| if((tiles_or_entropy_coding_sync_idc = = 2 ¦¦ tiles_or_entropy_coding_sync_idc = = 3) && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| … |  |
| } |  |
| … |  |
| } |  |

In the proposed syntax, 
dependent slice = tiles_or_entropy_coding_sync_idc: 2 && dependent_slice_flag: 1
entropy slice  = tiles_or_entropy_coding_sync_idc: 3 && dependent_slice_flag: 1

In the current specification[1],

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found , the following ordered steps apply.

- The arithmetic decoding engine is initialised as follows.

  - If CtbAddrRS is equal to slice_address, dependent_slice_flag is equal to 1 and tiles_or_entropy_coding_sync_idc is equal to 2entropy_coding_reset_flag is equal to 0, the following applies.

  - The synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxDS and TableMPSValDS as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause Error: Reference source not found.

  - Otherwise if tiles_or_entropy_coding_sync_idc is equal to 2, and CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

  - When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxWPP and TableMPSValWPP as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause Error: Reference source not found.

- When cabac_independent_flag is equal to 0 and dependent_slice_flag is equal to 1, or when tiles_or_entropy_coding_sync_idc is equal to 2, the memorization process is applied as follows.

  - When tiles_or_entropy_coding_sync_idc is equal to 2 and CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxWPP and TableMPSValWPP as output.

  - When cabac_independent_flag is equal to 0, dependent_slice_flag is equal to 1, and end_of_slice_flag is equal to 1, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxDS and TableMPSValDS as output.

### Restricted starting position of dependent slice

If dependent slice is further restricted to be able to start only at the first CTB of CTB lines (i.e. CtbAddrRS % PicWidthInCtbs is equal to 0), the memorization process (or the initialization process) of the CABAC also can be simplified. Currently, the memorization process (or the initialization process) is unclear in the current specification[1]. In the original spirit of dependent slice, the memorization process is required for both normal slice and dependent slice in the arbitrary position of the end of slice when dependent_slice_enabled_flag is equal to 1 within coded pictures referring to the picture parameter set. It is because dependent slice inherits the CABAC status of the end status of the previous slice. For example, if the first slice is normal slice and the second slice is dependent slice, the synchronization process of the CABAC parsing process in the second slice requires TableStateIdxDS and TableMPSValDS as input. But according to the current specification, the memorization process to store TableStateIdxDS and TableMPSValDS is only invoked when dependent_slice_flag is equal to 1 which is not for the case of normal slice (the first slice in this example). The specification can be modified to store TableStateIdxDS and TableMPSValDS regardless of dependent_slice_flag values, which requires the memorization process of the CABAC of normal slice. However, it may not be used if the next slice is normal slice. Instead, if dependent slice is forced to be used only with WPP and to be started only at the first CTB of CTB lines, the memorization and the initialization process specialized for dependent slice are not necessary and it is enough to follow those process of WPP. So the memorization process of the CABAC is only needed in case of the second CTB (i.e. CtbAddrRS % PicWidthInCtbs is equal to 2).

…

Normal Slice

Dep. Slice

Dep. Slice

…

Normal Slice

Dep. Slice

…

Normal Slice

Dep. Slice

Normal Slice

The memorization process

The synchronization process

Figure 1. The memorization process at the end of slice is removed

In the current specification is simplified as follows,

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found , the following ordered steps apply.

- The arithmetic decoding engine is initialised as follows.

  - If CtbAddrRS is equal to slice_address, dependent_slice_flag is equal to 1 and entropy_coding_reset_flag is equal to 0, the following applies.

  - The synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxDS and TableMPSValDS as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause Error: Reference source not found.

  - Otherwise if tiles_or_entropy_coding_sync_idc is equal to 2, and CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

  - When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxWPP and TableMPSValWPP as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause Error: Reference source not found.

- When cabac_independent_flag is equal to 0 and dependent_slice_flag is equal to 1, or when tiles_or_entropy_coding_sync_idc is equal to 2, the memorization process is applied as follows.

  - When tiles_or_entropy_coding_sync_idc is equal to 2 and CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxWPP and TableMPSValWPP as output.

  - When cabac_independent_flag is equal to 0, dependent_slice_flag is equal to 1, and end_of_slice_flag is equal to 1, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxDS and TableMPSValDS as output.

# Conclusion

For newly adopted dependent slice, the syntax modification is proposed either to extend the usability or restrict the usability. In the case of 2.3, 2.1 is not necessary. On the other hand, 2.1 and 2.2 can be combined.

# Working draft modification

WD change on JCTVC-I1003_d5.doc is accompanied with this proposal in the zip file, in which ‘2.3 Restricted usage of dependent slice (without 2.3.1 starting position restriction)’ is enclosed.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand “High Efficiency Video Coding (HEVC) text specification draft 7”, JCTVC-I1003, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- T. Schierl, V. George, A. Henkel, and D. Marpe “Dependent Slices”, JCTVC-I0229, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).