| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0218 |
| --- | --- |

| Title: | Bi-pred merge restriction for small PUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jeonghoon Park | Email: | tammy.lee@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the previous meeting, bi-pred restriction for small PUs is adopted[2] where bi-predictive motion information for 8x4 and 4x8 is converted to uni-predictive List0 motion after motion information construction. Assuming that encoder restricts motion search on bi-prediction for such small PUs, the only place where bi-predictive motion is given is the merge mode. In this proposal, bi-predictive merge candidates are restricted during merge candidate derivation process. By removing or converting bi-predictive merge candidates, merge candidate derivation process is reduced in complexity and converting bi-predictive motion on motion compensation process is unnecessary assuming that encoder restricts motion search on bi-prediction for small PUs.

# Introduction

In the current HM, the bi-predictive motion for 8x4 and 4x8 PUs is restricted by force converting from bi-predictive to uni-predictive List0 motion compensation after motion information reconstruction to limit the maximum memory bandwidth of motion compensation. If the encoder restricts motion search on such cases correctly, bi-predictive motion is the only occurred in merge candidate derivation process which requires normative change. Therefore, it is proposed to restrict bi-predictive merge candidate during merge candidate derivation process not after derivation process to avoid making unnecessary candidates.

# Proposal

In merge mode, all the motion information including MVs, reference indices and inter prediction direction is derived.

Spatial candidates (5)

Partition redundancy removal

Duplicates removal

Combined bi-predictive candidates (5)

B-Slice

Zero motion merge candidates

Select 4

Final N merge candidates (N <= 5 and N is signaled)

Temporal candidate

Fig 1. The current merge candidate list construction

In case of spatial merge candidates, it is inherited from neighbours. But for temporal merge candidates, it is first trying to make bi-predictive motion by checking if both directional motions are available in the co-located picture by searching motion of the right-bottom and the center PU. And for combined merge candidates and zero merge candidates who are invoked when the derived merge candidates spatially or temporally are less than the pre-determined number of merge candidates, it is always to make bi-predictive motion artificially in B-slice. Since the bi-predictive merge candidates is converted to uni-predictive motion for small PUs, the process for making bi-predictive motion in temporal, combined and zero merge candidate derivation process is not necessary. So in this proposal, the following three designs for small PUs are tested.

## Proposal1: Uni-predictive merge candidate generation for temporal, combined and zero merge candidates

In this design, List1 motion in temporal merge candidate derivation process is not examined in B-slice and set by List0 co-located motion. Compared to the current design, this saves the unnecessary List1 co-located motion availability checking.

List0 co-located motion available?

B-Slice && !isSmallPU?

List1 co-located motion available?

Start

List0 uni-predictive

Bi-predictive

No temporal merge candidate

No

Yes

No

No

Yes

Yes

Fig 2. Temporal merge candidate derivation

Also, the combined merge candidate generation is completely disabled because the combined merge candidates are always bi-predictive by combining each List0 and List1 motions for existing merge candidates. By removing this redundant process, zero merge candidates have more chance to be included in merge candidate list.

Zero merge candidates generation method is modified to be uni-predictive motion as if it is in P-slice.

## Proposal2: Exclusion of bi-predictive spatial merge candidate

On top of Proposal1, bi-predictive spatial neighboring merge candidate is excluded by marking it as unavailable. With this modification, bi-predictive merge candidates are never generated for small PUs. And therefore, force converting from bi-predictive motion to uni-predictive motion in compensation stage is not needed assuming that encoder correctly restricts bi-predictive motion search for small PUs in non-merge mode. And this modification saves duplicates checking process by removing unnecessary candidates.

## Proposal3: Conversion of bi-predictive spatial merge candidate

On top of Proposal1, bi-predictive spatial neighboring merge candidate is converted to uni-predictive List0 motion during spatial merge candidate generation. Like in Proposal2, bi-predictive merge candidates are never generated for small PUs. This proposal increases chance to use spatial merge candidate compared to Proposal2 where such spatial candidates are excluded in the candidate list.

# Experiments

All the test results show no loss in average for each test configuration. Moreover, Proposal1 and Proposal3 show 0.1% gain in LD Main test configuration.

## Results of Proposal1

## Results of Proposal2

## Results of Proposal3

# Conclusion

By restricting bi-predictive merge candidates during derivation process, List1 motion generation for temporal merge candidates and zero merge candidates is removed and unnecessary process as combined bi-predictive merge candidate generation is removed. Moreover, by converting bi-predictive spatial merge candidates to uni-predictive or excluding those, it saves the effort to check duplicates when insertion. This proposed methods can be regarded as to move the existing converting process inside merge candidate derivation process while saving complexity in candidate list generation.

# Working draft modification

Proposed working draft text is included below using the following text conventions:

- Removed text is shown with strikethrough.

- Newly added text is shown in red.

## WD change of Proposal1

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPSW and nPSH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause 3.1.1.1.3 is invoked with luma location ( xP, yP ), the variable singleMCLFlag, the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows.

- If the following conditions are true, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available  [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ]

- the PartIdx is equal to 0

- PredMode is not MODE_INTRA

- (xP >> (log2_parallel_merge_level_minus2 + 2))  != 
((xP − 1) >> (log2_parallel_merge_level_minus2 + 2))	| |
(yP >> (log2_parallel_merge_level_minus2 + 2))  != 
((yP + nPSH − 1) >> (log2_parallel_merge_level_minus2 + 2)

- Otherwise, refIdxLX is set equal to 0.

- Temporal merging candidate is derived as follows.

- If slice_type is equal to P or either nPSW or nPSH is less than 8,

the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxL0 as the inputs and with the output being the availability flag availableFlagCol and the temporal motion vector mvL0Col. The prediction list utilization flag predFlagL0Col is set equal to availableFlagCol.

mvL1[ 0 ] = 0	(8-xx)

mvL1[ 1 ] = 0	(8-xx)

refIdxL1 = -1	(8-xx)

predFlagL1 = 0	(8-xx)

- Otherwise, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B and both nPSW and nPSH are greater than or equal to 8, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause 3.1.1.1.2 is invoked with the mergeCandList, the width and the height of the prediction unit nPSW and nPSH, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

        - Derivation process for zero motion vector merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0zeroCandm and refIdxL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0zeroCandm and predFlagL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- motion vectors mvL0zeroCandm and mvL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

The variable numRefIdx is derived as follows.

- If slice_type is equal to P or either nPSW or nPSH is less than 8, numRefIdx is set to num_ref_idx_l0_active_minus1 + 1

- Otherwise (slice_type is equal to B), numRefIdx is set to min(num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1)

When numMergeCand is less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variable zeroIdx is set to 0, the variable zeroStop is set to FALSE and the following steps are repeated until zeroStop is equal to TRUE.

- For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies.

    - If (slice_type is equal to P or either nPSW or nPSH is less than 8), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-7)

refIdxL1zeroCandm = −1	(8-8)

predFlagL0zeroCandm = 1	(8-9)

predFlagL1zeroCandm = 0	(8-10)

mvL0zeroCandm[ 0 ] = 0	(8-11)

mvL0zeroCandm[ 1 ] = 0	(8-12)

mvL1zeroCandm[ 0 ] = 0	(8-13)

mvL1zeroCandm[ 1 ] = 0	(8-14)

numMergeCand = numMergeCand + 1	(8-15)

    - Otherwise (slice_type is equal to B), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-16)

refIdxL1zeroCandm = zeroIdx	(8-17)

predFlagL0zeroCandm = 1	(8-18)

predFlagL1zeroCandm = 1	(8-19)

mvL0zeroCandm[ 0 ] = 0	(8-20)

mvL0zeroCandm[ 1 ] = 0	(8-21)

mvL1zeroCandm[ 0 ] = 0	(8-22)

mvL1zeroCandm[ 1 ] = 0	(8-23)

numMergeCand = numMergeCand + 1	(8-24)

- The variable zeroIdx is incremented by 1.

- When numMergeCand is equal to MaxNumMergeCand or zeroIdx is equal to or greater than numRefIdx, zeroStop is set to TRUE.

## WD change of Proposal2

        - Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- a variable singleMCLFlag,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP − 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP − 1 ), ( xP + nPSW − 1,  yP − 1 ) or ( xP − 1,  yP − 1 ), the following applies.

–	When yP−1 is less than (( yC >> Log2CtbSize ) << Log2CtbSize), the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-25)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-26)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-27)

–	If one or more of the following conditions are true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- The prediction unit covering luma location ( xN, yN ) is not available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ] or PredMode is MODE_INTRA.

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and N is equal to B1

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and N is equal to A1

- N is equal to B1 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B0 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to A0 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

When availableFlagN is equal to 1, predFlagL0 is equal to 1 and predFlagL1 is equal to 1 and either nPSW or nPSH is less than 8, the following applies:

mvL0[ 0 ] = 0		 (8-xx)
		mvL0[ 1 ] = 0		 (8-xx)
		refIdxL0 = -1		(8-xx)
		predFlagL0 = 0		(8-xx)
		mvL1[ 0 ] = 0		 (8-xx)
		mvL1[ 1 ] = 0		 (8-xx)
		refIdxL1 = -1		(8-xx)
		predFlagL1 = 0		(8-xx)

## WD change of Proposal3

        - Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- a variable singleMCLFlag,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP − 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP − 1 ), ( xP + nPSW − 1,  yP − 1 ) or ( xP − 1,  yP − 1 ), the following applies.

–	When yP−1 is less than (( yC >> Log2CtbSize ) << Log2CtbSize), the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-28)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-29)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-30)

–	If one or more of the following conditions are true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- The prediction unit covering luma location ( xN, yN ) is not available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ] or PredMode is MODE_INTRA.

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and N is equal to B1

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and N is equal to A1

- N is equal to B1 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B0 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to A0 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

When availableFlagN is equal to 1, predFlagL0 is equal to 1 and predFlagL1 is equal to 1 and either nPSW or nPSH is less than 8, the following applies:

mvL1[ 0 ] = 0		 (8-xx)
		mvL1[ 1 ] = 0		 (8-xx)
		refIdxL1 = -1		(8-xx)
		predFlagL1 = 0		(8-xx)

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand “High Efficiency Video Coding (HEVC) text specification draft 7”, JCTVC-I1003, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- S. Fukushima, M. Ueda, and H. Takehara, “AHG7: Bi-pred restriction for small PUs”, JCTVC-I0297, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).