| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0219 |
| --- | --- |

| Title: | On signalling the syntax of MVP flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongfang Shi Xingyu Zhang Oscar C. Au Feng Zou Chao Pang | Email: Email: Email: Email: Email: | yshiab@ust.hk eexyzhang@ust.hk eeau@ust.hk fengzou@ust.hk pcece@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

In current HM7.0, the motion information inside a reference list for a certain PU is signalled in the order: reference index, MVD information, and MVP flag. There are several syntaxes for MVD information, which are transmitted or parsed conditionally. At the decoder side, only after all MVD related syntaxes are parsed, the MVP flag can be parsed and then the selected MVP will be found. This contribution proposes to signal the MVP flag in front of the MVD related syntaxes. With such a change, decoding process of MV will be parallel-friendly, while no performance change is observed.

# Introduction

In current HM7.0, the motion information inside a reference list for a certain PU is signalled in the order: reference index, MVD information, and MVP flag. As Table 1 shows, the MVD information is complex, which related syntaxes conditionally include abs_mvd_greater0_flag, abs_mvd_greater1_flag, abs_mvd_minus2, and mvd_sign_flag for both the horizontal and vertical components of the MVD. As for the MVP flag (mvp_l0_flag or mvp_l1_flag), it indicates the selected MVP from two of the MVP candidates for the current PU. Those two MVP candidates are derived based on motion vectors of 5 PUs located in different spatial positions, and 2 PUs located in different temporal positions.

| mvd_coding( mvd_x, mvd_y ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) { |  |
| if(abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag[ 1 ] ) { |  |
| if(abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| mvd_x = abs_mvd_greater0_flag[ 0 ] * ( abs_mvd_minus2[ 0 ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ 0 ] ) |  |
| mvd_y = abs_mvd_greater0_flag[ 1 ] * ( abs_mvd_minus2[ 1 ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ 1 ] ) |  |
| } |  |

Table 1. Motion vector difference coding syntax

In addition, for MVD information, the first 4 syntaxes (i.e. abs_mvd_greater0_flag[0], abs_mvd_greater0_flag[1], abs_mvd_greater1_flag[0], and abs_mvd_greater1_flag[1]) are context coded, and the rest 4 syntaxes (i.e. abs_mvd_minus2[0], mvd_sign_flag[0], abs_mvd_minus2[1], and mvd_sign_flag[1]) are bypass coded. For the reference index and mvp flag, both of them are context coded. Figure 1 gives an illustration for the order of motion syntaxes in reference list 0 in HM7.0.

Figure 1. Order of motion syntaxes inside reference list 0 in HM7.0.

# Proposed Modification

This proposal proposes to signal MVP flag (mvp_l0_flag or mvp_l1_flag) in front of MVD related syntaxes.The main purpose of such a swap is to provide the flexibility to decode MVP while parsing the MVD related syntaxes.

Since the binarization of abs_minus_mvd[0] and abs_minus_mvd[1] uses 1st order Exp-Golomb code, the time used to parse mvd related syntaxes is closely related with the magnitude of abs_minus_mvd[0] and abs_minus_mvd[1]. When the magnitude of abs_minus_mvd[0] and abs_minus_mvd[1] is small, it’s easy to parse mvd related syntaxes. In this case, it makes little difference to parse MVP first or parse MVD related syntax first. But when the magnitude of abs_minus_mvd[0] and abs_minus_mvd[1] is large, it takes longer time to parse mvd related syntax. If the MVP flag is parsed in advance, the mvp can be completely decoded while parsing the mvd related syntaxes.

In addition, with the proposed modification, the context based syntaxes in the reference list are grouped together. The design of signalling the motion information is neater. Figure 2 illustrates the order of motion information in the proposed solution.

Figure 2. Order of motion syntaxes inside reference list 0 in the proposed solution.

.

# Simulation

Simulations are based on HM7.0 according to common test condition [1]. Since the proposed modification has no influence on intra condition, only RA_main/HE10bit, LB_ main/HE10bit, and LP_ main/HE10bit conditions are tested. Results show that there is no performance change for the proposed modification.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Conclusion

This contribution changes the signalling position of syntax mvp_l0_flag and mvp_l1_flag. It proposes to put the MVP flag in front of MVD related syntaxes, so that the decoding process of MV can be more parallel-friendly. No performance change is observed for proposed modification. It’s recommended to accept this modification into HM8.0.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, Geneva, April. 2012.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification changes

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| …. |  |
| } else if( PredMode = = MODE_INTRA ) { |  |
| …. |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] ! = Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] ! = Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] == Pred_BI) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |