| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0220 |
| --- | --- |

| Title: | cu qp delta enabling syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jeonghoon Park | Email: | tammy.lee@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

diff_cu_qp_delta_depth_slice_granularity is signalled as the increased value by 1 to involve the indication of cu_qp_delta coding, where diff_cu_qp_delta_depth_slice_granularity = 0 means no dQP signaling at CU-level. In this proposal, cu_qp_delta_enabled_flag is restored in PPS and diff_cu_qp_delta_depth_slice_granularity is signalled as the original delta qp granularity from slice granularity.

# Introduction

In the initial version of WD6[2], cu_qp_delta_enabled_flag was added in SPS according to software to enable cu_qp_delta coding. But later, it was removed and cu_qp_delta coding was enabled by setting max_cu_qp_delta_depth greater than 0 and the maximum cu qp delta depth was signalled as the increased value by 1 to involve the indication of cu_qp_delta coding. The current diff_cu_qp_delta_depth_slice_granularity also involves no dQP signalling as the value of 0 in the same sense and this makes hard to interpret the maximum cu qp delta depth intuitively. It is proposed to restore cu_qp_delta_enabled_flag in PPS and diff_cu_qp_delta_depth_slice_granularity is signalled as its original value.

# Proposal

Proposed working draft text is included below using the following text conventions:

- Removed text is shown with strikethrough.

- Newly added text is shown in red.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| … |  |
| slice_granularity | u(2) |
| cu_qp_delta_enabled_flag | u(1) |
| if ( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| … |  |
| } |  |

| coding_tree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] >= SliceCbAddrZS && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| if( (cu_qp_delta_enabled_flag diff_cu_qp_delta_depth > 0 ) && log2CbSize >= Log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| … |  |
| } |  |

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| if( (cu_qp_delta_enabled_flag diff_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| … |  |
| } |  |
| } |  |

cu_qp_delta_enabled_flag specifies whether a cu_qp_delta syntax element is coded or not. If cu_qp_delta_enable_flag is equal to 1, cu_qp_delta is coded, otherwise it is not coded.

diff_cu_qp_delta_depth specifies the difference between the control granularity for QPY values for coding units within a picture and slice_granularity plus 1. The value of diff_cu_qp_delta_depth shall be in the range of 0 to log2_diff_max_min_coding_block_size − slice_granularity + 1, inclusive.

The variable Log2MinCUDQPSize specifying the minimum luma coding block size of coding units that convey cu_qp_delta, is derived as follows.

Log2MinCUDQPSize = Log2CtbSize − diff_cu_qp_delta_depth − slice_granularity + 1	(7-1)

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand “High Efficiency Video Coding (HEVC) text specification draft 7”, JCTVC-I1003, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand “High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, 7th Meeting: Geneva, CH, 21–30 November, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).