| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0221 |
| --- | --- |

| Title: | Clean-up of semantics and decoding process on weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This document presents a clean-up text of the semantics and the decoding process on weighted prediction (WP) in the latest HEVC WD7. At first, the derivation processes of some variables on WP are described in the semantics. It is better to move them into an appropriate part of the decoding process. Secondary, the range limit of the signalling values of the weighting factor and the additive offset are not explicitly defined. This document proposes that the signalling value of the weighting factor is in the range of -128 to 127, inclusive and the signalling value of the additive offset is in the range of -512 to 511, inclusive. In addition, the reconstructed additive offset is clipped to the range of -128 to 127, which is same as H.264/MPEG-4 AVC specification. The related range limit issue in HM7.0 is fixed in this modification. The experimental results in HM software version 7.0 with weighted prediction under common test conditions are reported. The results show that BD-bitrate loss for both black/white-fade sequences are 0.0%, 0.0% and 0.0% for RA-MP, LDB-MP and LDP-MP compared with conventional WP method on average, respectively. It is reported that these schemes does not affect the encoding and decoding time.

# Problem statement

At 6th JCT-VC Torino meeting, AVC based weighted prediction (AVCWP) scheme [1] and [2] was adopted in HM4 and WD4 [3]. At 7th JCT-VC meeting, the WP parameter signalling scheme Error: Reference source not found to combine the WP parameters signalling for identical reference pictures in B slices was adopted. At 9th JCT-VC Geneva meeting, by removing the combined reference list scheme [5] from WD, the part of the combined WP parameter signalling is also removed and reverted to AVC-based WP parameter signalling scheme.

There are some ambiguous points to be refined in current HEVC WD7 text on the weighted prediction.

- The derivation processes of LumaWeightLX, ChromaWeightLX, ChromaOffsetLX are described in weighted prediction parameters semantics. It is better to move them into an appropriate part of the decoding process.

- The range limit of the signalling values of delta_luma_weight_l0[ i ], delta_chroma_weight_l0[ i ][ j ] and delta_chroma_offset_l0[ i ][ j ] are not explicitly defined. It means that the encoder cannot know whether the calculated values must be in the allowed range of the current specification before deriving the reconstructed values. Moreover, the weighting factor and the additive offset in current HM7.0 don’t check the range limit of the reconstructed values. It means that the encoder might signal an illegal stream to the decoder.

# Proposal

In order to fix these issues, this contribution proposes the clean-up text of weighted prediction. The detailed semantics and the decoding process are shown in Appendix.

To fix the point 1, the new decoding process in subclause 8.3.x is added in the WD7. LumaWeightLX, LumaOffsetLX, ChromaWeightLX, ChromaOffsetLX are derived in this process. This modification does not change any signal processing part and can clean the description in the weighted prediction parameters semantics.

To fix the point 2, the range of the signaling values of the weighing factor and the additive offset are newly defined. The signalling value of the weighting factor is in the range of -128 to 127 (signed 8bit), inclusive and the signalling value of the additive offset is in the range of -512 to 511 (signed 10bit), inclusive. In addition, the reconstructed additive offset is clipped to the range of -128 to 127 (signed 8bit), which is same as H.264/MPEG-4 AVC specification. When the weighting factor cannot satisfy the above range limit in HM7.0, the Log2Denom value is decremented by satisfying it.

# Syntax and decoding process

The detailed semantics and decoding process after clean-up are shown in appendix of this contribution.

# Experimental Results

HM software version 7.0 with the common configuration Error: Reference source not found is used in these experiments. Test sequences with black/white-fade are provided by WP AHG [7]. Table 1 and Table 2 show BD-bitrate and relative average encoding/decoding time for the proposed fix of point 2 compared with HM7.0 weighted prediction anchor. The negative value of BD-bitrate indicates gain. It observed that the loss of the proposed fix is 0.0%, 0.0% and 0.0% on average for RA-MP, LDB-MP and LDP-MP and 0.1%, 0.0% and 0.0% on average for RA-HE10, LDB-HE10 and LDP-HE10 in black-fade case, respectively. And the loss of the proposed fix is 0.0%, 0.0% and 0.0% on average for RA-MP, LDB-MP and LDP-MP and 0.1%, 0.0% and 0.0% on average for RA-HE10, LDB-HE10 and LDP-HE10 in white-fade case, respectively.

Table 1: BD-bitrate[%] and relative encoding/decoding time[%] of the fix of point 2

compared with HM7.0 WP anchor for black-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.1% | 0.1% | 0.2% | 0.3% | -0.9% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | -0.1% | 0.1% | -0.3% | -0.1% | -0.4% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | -0.4% | -0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 2: BD-bitrate[%] and relative encoding/decoding time[%] of the fix of point 2

compared with HM7.0 WP anchor for white-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.2% | 0.2% | 0.3% | 0.3% | -1.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | -0.3% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Additionally, we checked the change of range limit of the signaling value of the additive offset on point 2. We tried to test the range limit of signed 8 to 12bit. Table 3 shows the BD-bitrate of these conditions. It is suggest that the range limit of delta_chroma_offset_l0[ i ][ j ] is signed 10bit.

Table 3: Summary of BD-bitrate[%] on the range limit for delta_chroma_offset_l0[ i ][ j ]

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Sinaling range 8bit | 0.0% | 0.0% | 0.0% | 2.0% | 0.4% | 0.6% |
| Sinaling range 9bit | 0.0% | 0.0% | 0.0% | 0.3% | -0.2% | 0.1% |
| Sinaling range 10bit | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | -0.1% |
| Sinaling range 11bit | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | -0.1% |
| Sinaling range 12bit | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | -0.1% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Sinaling range 8bit | 0.0% | 0.0% | 0.0% | 0.2% | 0.7% | 0.6% |
| Sinaling range 9bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Sinaling range 10bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Sinaling range 11bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Sinaling range 12bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Sinaling range 8bit | 0.0% | 0.0% | 0.0% | 0.5% | 0.6% | 1.0% |
| Sinaling range 9bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Sinaling range 10bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Sinaling range 11bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Sinaling range 12bit | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

In this contribution, the clean-up text of the semantics and the decoding process on weighted prediction are provided. To introduce it, some ambiguous points on the current text of weighted prediction can be defined explicitly.

It is suggested that this clean-up text is reflected in the next version of HEVC draft specification.

# References

- P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-F265, Torino, July 2011.

- A. Tanizawa, "Explicit Weighted Prediction with simple WP parameter estimation," Joint Collaborative Team on Video Coding, JCTVC-F326, Torino, July 2011.

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, Document no JCTVC-I1003. May 2012.

- A. Tanizawa, T. Chujoh, T. Yamakage, “Redundancy removal of explicit weighted prediction syntax”, Document no JCTVC-G441, Nov 2011.

- Y. Suzuki and TK Tan, " Extension of uni-prediction simplification in B slices," Joint Collaborative Team on Video Coding, JCTVC-D421, Daegu, Jan. 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, Document no JCTVC-I1100. May 2012.

- AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## WD text for weighted prediction

#### 7.3.3.8 Prediction weight table syntax (Not changed)

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.3.8 Weighted prediction parameters semantics

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 0 prediction using RefPicList0[ i ] are present.  luma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by (1 << luma_log2_weight_denom ) + delta_luma_weight_l0[ i ]. invoking the decoding process in 8.3.x.  When luma_weight_l0_flag[ i ] is equal to 1, the value of LumaWeightL0[ i ] delta_luma_weight_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag[ i ] is equal to 0, LumaWeightL0[ i ] delta_luma_weight_l0[ i ] is inferred to be equal to 0 2luma_log2_weight_denom for RefPicList0[ i ].

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].  The variable LumaOffsetL0[ i ] is specified by invoking the decoding process in 8.3.x.  The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag[ i ] is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction using RefPicList0[ i ] are present.    chroma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present.

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2WeightDenom ) + delta_chroma_weight_l0[ i ][ j ].invoking the decoding process in 8.3.x.  When chroma_weight_l0_flag[ i ] is equal to 1, the value of ChromaWeightL0[ i ][ j ] delta_chroma_weight_l0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag[ i ] is equal to 0, delta_chroma_weight_l0[ i ][ j ]ChromaWeightL0[ i ][ j ] is inferred to be equal to 0 2ChromaLog2WeightDenom for RefPicList0[ i ].

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is specified by invoking the process in 8.3.x.  as follows:

shift = 1 << ( BitDepthC − 1 )

ChromaOffsetL0[ i ][ j ] = (delta_chroma_offset_l0[i][j] – 
				( (shift*ChromaWeightL0[ i ][ j ]) >> ChromaLog2WeightDenom ) − shift )	 (7-1)

The value of delta_chroma_offset_l0[ i ][ j ] shall be in the range of −516 to 515, inclusive.  When chroma_weight_l0_flag[ i ] is equal to 0, delta_chroma_offset_l0[ i ][ j ]ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_l1_flag, delta_luma_weight_l1, luma_offset_l1, chroma_weight_l1_flag, delta_chroma_weight_l1, delta_chroma_offset_l1 have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

#### 8.3.x Decoding process for weighted prediction parameters

This process is invoked at the beginning of the decoding process for each P slice when weighted_pred_flag is equal to 1 or at the beginning of the decoding process for each B slice when weighted_bipred_flag is equal to 1.

Inputs to this process are:

–	luma_log2_weight_denom and ChromaLog2WeightDenom,

–	for each entry in List 0, delta_luma_weight_l0, delta_luma_offset_l0, delta_chroma_weight_l0, and delta_chroma_offset_l0,

–	if the current slice is a B slice, for each entry in List 1, delta_luma_weight_l1, delta_luma_offset_l1, delta_chroma_weight_l1 and delta_chroma_offset_l1.

Outputs of this process are arrays of the weighting factors and the additive offsets for luma and chroma weighted sample prediction:

–	for each entry in List 0, LumaWeightL0, LumaOffsetL0, ChromaWeightL0, and ChromaOffsetL0.

–	if the current slice is a B slice, for each entry in List 1, LumaWeightL1, LumaOffsetL1, ChromaWeightL1, and ChromaOffsetL1.

The following ordered steps apply:

      - The array LumaWeightL0, LumaOffsetL0, ChromaWeightL0, and ChromaOffsetL0 are constructed as follows:

for( cIdx = 0; cIdx ≤ num_ref_idx_l0_active_minus1; cIdx++) {

LumaWeightL0[ cIdx ] = delta_luma_weight_l0[ cIdx ] + (1 << luma_log2_weight_denom ),

LumaOffsetL0[ cIdx ] = luma_offset_l0[ cIdx ],

ChromaWeightL0[ cIdx ][ 0 ] = delta_chroma_weight_l0[ cIdx ][ 0 ] + (1 << ChromaLog2WeightDenom ),

ChromaWeightL0[ cIdx ][ 1 ] = delta_chroma_weight_l0[ cIdx ][ 1 ] + (1 << ChromaLog2WeightDenom ),

ChromaOffsetL0[ cIdx ][ 0 ] = Clip3( –128, 127, ( delta_chroma_offset_l0[ cIdx ][ 0 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL0[ cIdx ][ 0 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

ChromaOffsetL0[ cIdx ][ 1 ] = Clip3( –128, 127, ( delta_chroma_offset_l0[ cIdx ][ 1 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL0[ cIdx ][ 1 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

}

      - The array LumaWeightL1, LumaOffsetL1, ChromaWeightL1, and ChromaOffsetL1 are constructed as follows:

for( cIdx = 0; cIdx ≤ num_ref_idx_l1_active_minus1; cIdx++) {

LumaWeightL1[ cIdx ] = delta_luma_weight_l1[ cIdx ] + (1 << luma_log2_weight_denom ),

LumaOffsetL1[ cIdx ] = luma_offset_l1[ cIdx ],

ChromaWeightL1[ cIdx ][ 0 ] = delta_chroma_weight_l1[ cIdx ][ 0 ] + (1 << ChromaLog2WeightDenom ),

ChromaWeightL1[ cIdx ][ 1 ] = delta_chroma_weight_l1[ cIdx ][ 1 ] + (1 << ChromaLog2WeightDenom ),

ChromaOffsetL1[ cIdx ][ 0 ] = Clip3( –128, 127, ( delta_chroma_offset_l1[ cIdx ][ 0 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL1[ cIdx ][ 0 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

ChromaOffsetL1[ cIdx ][ 1 ] = Clip3( –128, 127, ( delta_chroma_offset_l1[ cIdx ][ 1 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL1[ cIdx ][ 1 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

}