| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0222 |
| --- | --- |

| Title: | Improved weighted prediction parameter signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This document presents an improved method of weighted prediction (WP) parameter signaling. This scheme is to introduce reuse functionality to WP parameters for List 1 from WP parameters for List 0. WP parameters signaling for List 0 is not changed in the current WD7. A flag of delta_wp_param_signaling_flag is inserted in syntax elements of WP parameters for List 1. If this flag is equal to 1, the conventional WP parameter signaling for List 1 is used. Otherwise, an additional index of wp_delta_ref_idx is explicitly signaled in pred_weight_table syntax. This index shows that which WP parameters for List 0 are copied in WP parameters of the current reference picture for List 1. The experimental results in HM software version 7.0 with weighted prediction under common test conditions are reported. The results show that BD-bitrate gain is 0.2%, 1.3%, 0.3% and 2.1% for RA-MP, LDB-MP, RA-HE10 and LDB-HE10 compared with conventional WP method on average, respectively. It is reported that this scheme does not affect the encoding and decoding time.

# Problem statement

At the last JCT-VC 9th Geneva meeting, the combined reference list scheme [1] was removed in HM7 and WD7 [2] and the combined part of weighted prediction (WP) parameter signaling method [3] is reverted to AVC-based method. However, current WP parameter signaling in WD7 has still redundant representation for identical reference pictures, because same WP parameters for List 0 and List 1 are encoded twice and transmitted them to the decoder. In order to remove this redundancy, the improved WP parameter signaling method [4] has been proposed at the last meeting. This method is adding only one flag to pred_weight_table sytanx and the WP parameters for List 1 are predicted from WP parameters for List 0. It can reduce the overhead of redundant representation. It needs an additional decoding process to derive WP parameters for List 1, because it needs to calculate which reference picture for List 0 is identical to the current reference picture for List 1. It means that the decoder cannot know which WP parameters of each reference picture are identical or not at parsing process.

The current pred_weight_table syntax is shown in Table 1.

Table 1: conventional pred_weight_table syntax in WD7

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposal

WP parameter signaling for List 0 is not changed in the current WD7. In order to remove the redundant representation for identical reference pictures, a flag of delta_wp_param_signaling_flag is inserted in syntax elements of WP parameters for List 1. If this flag is equal to 1, the conventional WP parameter signaling for List 1 is used. Otherwise, an additional index of wp_delta_ref_idx is explicitly signaled in pred_weight_table syntax. This index shows that which WP parameters for List 0 are copied in WP parameters of the current reference picture for List 1. The proposed pred_weight_table syntax is shown in appendix. When reference pictures for both List 0 and List 1 are identical (for example, LDB case), delta_wp_param_signaling_flag is set to 0. wp_delta_ref_idx is calculated in the following equation.

delta_wp_ref_idx = ( curr_ref_idx_list1 – ref_idx_list0 ),

where curr_ref_idx_list1 shows the current ref_idx number of WP parameters for List 1 and ref_idx_list0 shows the ref_idx number of copied WP parameter for List0. WP parameters for List 0 have been already decoded in syntax elements, when WP parameters for List 1 are decoded. In LDB case, since reference pictures for both List 0 and List 1 are always identical, delta_wp_ref_idx might be always set to 0. Since the decoder can know a reuse information of WP parameters for List 0 at parsing process level by signaling delta_wp_ref_idx explicitly, it is not needed to have an additional calculation at decoding process level to recognize whether reference pictures are identical or not.

# Syntax and decoding process

The detailed syntax, semantics and decoding process for proposed weighted prediction are shown in appendix of this contribution. The proposed modifications are described on top of clean-up text in JCTVC-J0221 [5].

# Experimental Results

HM software version 7.0 with the common configuration Error: Reference source not found is used in these experiments. Test sequences with black/white-fade are provided by WP AHG [7]. Table 2 and Table 3 show BD-bitrate and relative average encoding/decoding time for the proposed method compared with HM7.0 weighted prediction anchor. The negative value of BD-bitrate indicates gain. It observed that the proposed method can achieve the gain of 0.1% and 1.2% on average for RA-MP and LDB-MP and the gain of 0.2% and 1.9% on average for RA-HE10 and LDB-HE10 in black-fade case, respectively. And the proposed method can achieve the gain of 0.2% and 1.5% on average for RA-MP and LDB-MP and the gain of 0.3% and 2.2% on average for RA-HE10 and LDB-HE10 in white-fade case, respectively.

Table 2: BD-bitrate[%] and relative encoding/decoding time[%] of the proposed method

compared with HM7.0 WP anchor for black-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| Class D | -0.4% | -0.4% | -0.4% | -0.7% | -0.6% | -0.6% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
|  | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.3% | -0.3% | -0.5% | -0.4% | -0.4% |
| Class C | -0.7% | -0.6% | -0.6% | -1.0% | -0.9% | -0.9% |
| Class D | -2.1% | -1.9% | -2.0% | -3.3% | -3.0% | -3.0% |
| Class E | -2.2% | -1.9% | -1.9% | -3.8% | -3.4% | -3.4% |
| Overall | -1.2% | -1.1% | -1.1% | -1.9% | -1.7% | -1.7% |
|  | -1.2% | -1.1% | -1.1% | -1.9% | -1.8% | -1.8% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3: BD-bitrate[%] and relative encoding/decoding time[%] of the proposed method

compared with HM7.0 WP anchor for white-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.2% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| Class D | -0.5% | -0.5% | -0.5% | -0.8% | -0.8% | -0.8% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% |
|  | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.3% | -0.3% | -0.6% | -0.5% | -0.5% |
| Class C | -0.8% | -0.8% | -0.8% | -1.2% | -1.1% | -1.1% |
| Class D | -2.7% | -2.4% | -2.5% | -3.8% | -3.4% | -3.5% |
| Class E | -2.9% | -2.4% | -2.5% | -4.3% | -3.9% | -4.0% |
| Overall | -1.5% | -1.4% | -1.4% | -2.2% | -2.0% | -2.1% |
|  | -1.5% | -1.4% | -1.4% | -2.3% | -2.0% | -2.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

# Conclusion

In this contribution, the improved method of the weighted prediction parameter signaling is proposed. Experimental results show that the proposed method can achieve the gain of 0.2% and 1.5% on average for RA-MP and LDB-MP and the gain of 0.3% and 2.3% on average for RA-HE10 and LDB-HE10 in black/white-fade case, respectively. These schemes do not affect the encoding and decoding time.

It’s suggested that the proposed method is introduced in the next version of HEVC draft specification.

# References

[1]	Y. Suzuki and TK Tan, " Extension of uni-prediction simplification in B slices," Joint Collaborative Team on Video Coding, JCTVC-D421, Daegu, Jan. 2011.

[2]	AHG report, "High efficiency video coding (HEVC) text specification draft 7," Joint Collaborative Team on Video Coding, JCTVC-I1003, Geneva, May 2012.

[3]	A. Tanizawa, T. Chujoh and T. Yamakage, “Redundancy removal of explicit weighted prediction syntax,” Joint Collaborative Team on Video Coding, JCTVC-G441, Geneva, Nov. 2011.

[4]	Y. He, J. Dong, Y. Ye, and E.S. Ryu, “On Weighted Prediction Parameter Signalling,” Joint Collaborative Team on Video Coding, JCTVC-I0260, Geneva, May. 2012.

[5]	A. Tanizawa, T. Chujoh and T. Yamakage, “AHG9: Clean-up of semantics and decoding process on weighted prediction,” Joint Collaborative Team on Video Coding, JCTVC-J0221, Stockholm, July 2012.

[6]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-I1100, Geneva, May 2012.

[7]	AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## WD text for weighted prediction

#### 7.3.3.8 Prediction weight table syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| delta_wp_param_signaling_flag | u(1) |
| if( delta_wp_param_signaling_flag ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } else { |  |
| delta_wp_ref_idx[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

#### 7.4.3.8 Weighted prediction parameters semantics

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 0 prediction are present.  luma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by invoking the decoding process in 8.3.7.  When luma_weight_l0_flag[ i ] is equal to 1, the value of delta_luma_weight_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag[ i ] is equal to 0, delta_luma_weight_l0[ i ] is inferred to be equal to 0 for RefPicList0[ i ].

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].  The variable LumaOffsetL0[ i ] is specified by invoking the decoding process in 8.3.7.  The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag[ i ] is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction are present.  chroma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present.

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by invoking the decoding process in 8.3.7.  When chroma_weight_l0_flag[ i ] is equal to 1, the value of delta_chroma_weight_l0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag[ i ] is equal to 0, delta_chroma_weight_l0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr. The variable ChromaOffsetL0[ i ][ j ] is specified by invoking the process in 8.3.7.  The value of delta_chroma_offset_l0[ i ][ j ] shall be in the range of −256 to 255, inclusive.  When chroma_weight_l0_flag[ i ] is equal to 0, delta_chroma_offset_l0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_l1_flag, delta_luma_weight_l1, luma_offset_l1, chroma_weight_l1_flag, delta_chroma_weight_l1, delta_chroma_offset_l1 have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

delta_wp_param_signaling_flag equal to 1 specifies that weighting parameters for the list 1 prediction are present. delta_wp_param_signaling_flag equal to 0 specifies that these weighting parameters are not present.

delta_wp_ref_idx[ i ] is the difference of the number of reference picture for list 0 prediction using RefPicList0[ i ] and the number of reference picture for list 1 prediction using RefPicList1[ i ]. The variable WpRefIdxL0 is specified by invoking the decoding process in 8.3.7. The value of delta_wp_ref_idx[ i ] shall be in the range of –( num_ref_idx_l1_active_minus1 + 1 ) to ( num_ref_idx_l0_active_minus1 + 1 ), inclusive.

#### 8.3.7 Decoding process for weighted prediction parameters

This process is invoked at the beginning of the decoding process for each P slice when weighted_pred_flag is equal to 1 or at the beginning of the decoding process for each B slice when weighted_bipred_flag is equal to 1.

Inputs to this process are:

–	luma_log2_weight_denom and ChromaLog2WeightDenom,

–	for each entry in List 0, delta_luma_weight_l0, delta_luma_offset_l0, delta_chroma_weight_l0, and delta_chroma_offset_l0,

–	if the current slice is a B slice, for each entry in List 1, delta_luma_weight_l1, delta_luma_offset_l1, delta_chroma_weight_l1,and delta_chroma_offset_l1, delta_wp_param_signaling_flag and delta_wp_ref_idx.

Outputs of this process are arrays of the weighting factors and the additive offsets for luma and chroma weighted sample prediction:

–	for each entry in List 0, LumaWeightL0, LumaOffsetL0, ChromaWeightL0, and ChromaOffsetL0.

–	if the current slice is a B slice, for each entry in List 1, LumaWeightL1, LumaOffsetL1, ChromaWeightL1, and ChromaOffsetL1.

The following ordered steps apply:

      - The array LumaWeightL0, LumaOffsetL0, ChromaWeightL0, and ChromaOffsetL0 are constructed as follows:

for( cIdx = 0; cIdx ≤ num_ref_idx_l0_active_minus1; cIdx++) {

LumaWeightL0[ cIdx ] = delta_luma_weight_l0[ cIdx ] + (1 << luma_log2_weight_denom ),

LumaOffsetL0[ cIdx ] = luma_offset_l0[ cIdx ],

ChromaWeightL0[ cIdx ][ 0 ] = delta_chroma_weight_l0[ cIdx ][ 0 ] + (1 << ChromaLog2WeightDenom ),

ChromaWeightL0[ cIdx ][ 1 ] = delta_chroma_weight_l0[ cIdx ][ 1 ] + (1 << ChromaLog2WeightDenom ),

ChromaOffsetL0[ cIdx ][ 0 ] = Clip3( –128, 127, ( delta_chroma_offset_l0[ cIdx ][ 0 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL0[ cIdx ][ 0 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

ChromaOffsetL0[ cIdx ][ 1 ] = Clip3( –128, 127, ( delta_chroma_offset_l0[ cIdx ][ 1 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL0[ cIdx ][ 1 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

}

      - The array LumaWeightL1, LumaOffsetL1, ChromaWeightL1, and ChromaOffsetL1 are constructed as follows:

for( cIdx = 0; cIdx ≤ num_ref_idx_l1_active_minus1; cIdx++) {

if( delta_wp_param_signaling_flag ) {

LumaWeightL1[ cIdx ] = delta_luma_weight_l1[ cIdx ] + (1 << luma_log2_weight_denom ),

LumaOffsetL1[ cIdx ] = luma_offset_l1[ cIdx ],

ChromaWeightL1[ cIdx ][ 0 ] = delta_chroma_weight_l1[ cIdx ][ 0 ] + (1 << ChromaLog2WeightDenom ),

ChromaWeightL1[ cIdx ][ 1 ] = delta_chroma_weight_l1[ cIdx ][ 1 ] + (1 << ChromaLog2WeightDenom ),

ChromaOffsetL1[ cIdx ][ 0 ] = Clip3( –128, 127, ( delta_chroma_offset_l1[ cIdx ][ 0 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL1[ cIdx ][ 0 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

ChromaOffsetL1[ cIdx ][ 1 ] = Clip3( –128, 127, ( delta_chroma_offset_l1[ cIdx ][ 1 ] + (1 << (BitDepthC – 1) ) – ( ChromaWeightL1[ cIdx ][ 1 ] << (BitDepthC – 1 – ChromaLog2WeightDenom ))))

} else {

WpRefIdxL0 = ( cIdx + delta_wp_ref_idx ),

LumaWeightL1[ cIdx ] = LumaWeightL0 [ WpRefIdxL0 ],

LumaOffsetL1 [ cIdx ] = LumaOffsetL0 [ WpRefIdxL0 ],

ChromaWeightL1 [ cIdx ][ 0 ] = ChromaWeightL0 [ WpRefIdxL0 ][ 0 ],

ChromaWeightL1 [ cIdx ][ 1 ] = ChromaWeightL0 [ WpRefIdxL0 ][ 1 ],

ChromaOffsetL1 [ cIdx ][ 0 ] = ChromaOffsetL0 [ WpRefIdxL0 ][ 0 ],

ChromaOffsetL1 [ cIdx ][ 1 ] = ChromaOffsetL0 [ WpRefIdxL0 ][ 1 ]

}

}