| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0223 |
| --- | --- |

| Title: | Cross-check of Weighted Prediction Parameter Signalling (JCTVC-J0055) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results on the Weighted Prediction Parameter Signalling proposed in JCTVC-J0055 by InterDigital. The proponents propose two methods. In method 1, the WP parameters for List 1 are derived from already coded WP parameters for List 0, when the reference pictures are identical. In method 2, the WP parameters for List 1 are predicted from already coded WP parameters for List 0 to reduce an overhead further when there are same entries in both Lists. The verification task using weighted prediction with black/white-fade sequences has been done successfully and the results match those provided by proponent. The encoding time and the decoding time of this cross-check are also similar to those provided by proponents. The cross-checkers confirmed that the proposed schemes are consistent with reference to JCTVC-J0055.

Experimental results show that BD-bitrate gain of the method 1 is 0.1%, 1.3%, 0.1% and 2.0% for black-fade sequences on RA-MP, LB-MP, RA-HE10 and LB-HE10 cases, respectively, and BD-bitrate gain of the method 1 is 0.1%, 1.6%, 0.2% and 2.3% for white-fade sequences on RA-MP, LB-MP, RA-HE10 and LB-HE10 cases, respectively. BD-bitrate gain of the method 2 is 0.2%, 1.3%, 0.3% and 2.0% for black-fade sequences on RA-MP, LB-MP, RA-HE10 and LB-HE10 cases, respectively, and BD-bitrate gain of the method 2 is 0.2%, 1.6%, 0.3% and 2.3% for white-fade sequences on RA-MP, LB-MP, RA-HE10 and LB-HE10 cases, respectively.

# Cross-checking

The software based on HM version 7.0 and the experimental results were provided on June 30, 2012 by InterDigital. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under common test conditions [1] for Main Profile (MP) conditions and high efficiency 10bit (HE10) conditions in the case of random access (RA) and low delay B (LB) coding structures using black/white-fade sequences provided by [2]. This scheme does not affect coding results in LP case.

Proponents add two macros in HM7.0 software as follows:

#define IDCC_WP_FLAG      1 // add weights_l1_present_flag

#define IDCC_WP_PRED      1 // predicting WP parameters of L1 from L0

The first macro is adding 1 flag in pred_weight_table() syntax to reuse WP parameters of List 0 for identical reference pictures. The second macro is introducing a method of predicting WP parameters for List 1 from WP parameters for List 0. The method 1 means that first macro is “on” and the second macro is “off” and the method 2 means that both macros are “on”.

# Results

Table 1 and Table 2 show the summary of BD-bitrate and relative encoding/decoding time of method 1 for black-fade and white-fade sequences, respectively and Table 3 and Table 4 show the summary of BD-bitrate and relative encoding/decoding time of method 2 for black-fade and white-fade sequences, respectively. Through the results, it turns out that the gain of method 2 is a little bigger than that of method 1 on average and both methods don’t affect the encoding/decoding time relative to the anchor weighted prediction signaling method. The test results for cross-checking match exactly those provided by proponents. The cross-checkers reviewed the source code the proponents provided and confirmed that the proposed schemes are consistent with reference to JCTVC-J0055.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] for method 1 (Black-fade sequences)

Table 2. BD-Rate[%] and relative encoding/decoding time[%]for method 1 (White-fade sequences)

Table 3. BD-Rate[%] and relative encoding/decoding time[%] for method 2 (Black-fade sequences)

Table 4. BD-Rate[%] and relative encoding/decoding time[%]for method 2 (White-fade sequences)

# Conclusion

We cross-checked the two weighted prediction parameter signaling methods proposed by InterDigital and confirmed that the test results of the proposed methods match exactly those provided by proponents. The source code of both methods is consistent with reference to JCTVC-J0055.

# References

[1]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-I1000, Geneva, May. 2012.

[2]	AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).