| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0224_r1 |
| --- | --- |

| Title: | AHG10: Hook for scalable extensions: Signalling TMVP reference index in slice header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici Miska M. Hannuksela Kemal Ugur Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Email: | ext-oguz.bici@nokia.com Miska.Hannuksela@nokia.com Kemal.ugur@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

Reference index of the temporal motion vector prediction (TMVP) candidate in the motion merge mode is always set as 0. However, for the envisioned scalable or multi-view extensions of HEVC, the picture at reference index=0 may be an inter-layer or inter-view reference picture with the same POC as current picture. In these cases, TMVP cannot be utilized for the enhancement layer or the non-base views, since POC based scaling is not possible. In this document, it is proposed to signal reference index of the TMVP candidate in the slice header instead of setting the reference index of TMVP to zero. This allows utilizing TMVP for scalable and multiview extensions of HEVC, the same way it is used in the base layer.

# Introduction

Reference index of the temporal motion vector prediction (TMVP) candidate in the motion merge mode is always set as 0. TMVP is generated by scaling the motion vector from collocated block according to the POC differences as illustrated in Figure 1. However, when an inter-layer or inter-view reference picture in envisioned scalable or multi-view extensions of HEVC has reference index 0, the picture at reference index 0 may result in invalid temporal motion vector predictor. In this case, the TMVP cannot be used and a loss in coding efficiency occurs. This case is illustrated in Figure 2.

Figure 1: TMVP generation in single view HEVC.

Figure 2: The case when a reference picture from another view/layer with same POC as current picture occupies the reference index=0. In this case, it is not possible to scale collocated MV to  generate TMVP.

More specifically, there may be several cases where reference index 0 may contain a picture with the same POC as the POC of the current picture. In this case, it is not possible to derive a TMVP based on POC scaling as illustrated in Figure 2. Reference index 0 may contain picture with the same POC in various cases:

- For scalable coding, an inter-layer reference picture may occupy reference index 0.

- For multiview coding, an inter-view reference picture may occupy reference index 0.

- For depth-enhanced multiview coding, a view synthesis reference picture may occupy reference index 0.

One of the motivations behind ordering the reference picture list as described is to make the codewords for reference picture index for AMVP as short as possible from the point of view of rate-distortion performance of AMVP.

The unavailability of TMVP in the merge mode with certain reference picture list orders leads to suboptimal rate-distortion performance.

# Proposed Solution

In order to avoid the described limitations in the envisioned HEVC extensions, it is proposed to signal reference index list0 and reference index list1 for TMVP merge candidate in the slice header. Corresponding changes in the WD are as follows (with respect to JCTVC-I1003_d6):

#### 7.3.2.3 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| merge_tmvp_ref_idx_present_flag | u(1) |
| … |  |
| } |  |

### 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pic_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| if( merge_tmvp_ref_idx_present_flag && ( slice_type = = P ¦ ¦ slice_type = = B ) && num_ref_idx_l0_active_minus1 > 0 ) |  |
| merge_tmvp_ref_idx_l0 | ue(v) |
| if( merge_tmvp_ref_idx_present_flag && slice_type = = B && num_ref_idx_l1_active_minus1 > 0 ) |  |
| merge_tmvp_ref_idx_l1 | ue(v) |
| } |  |
| … |  |
| } |  |

#### 7.4.2.3 Picture parameter set RBSP semantics

merge_tmvp_ref_idx_present_flag equal to 0 indicates that merge_tmvp_ref_idxl0 and merge_tmvp_ref_idxl1 are not present. merge_tmvp_ref_idx_present_flag equal to 1 indicates that merge_tmvp_ref_idxl0 and merge_tmvp_ref_idxl1 are present.

### 7.4.3 Slice header semantics

merge_tmvp_ref_idx_l0 specifies the reference index in list 0 for the temporal merging candidate in merge mode. merge_tmvp_ref_idx_l0 shall always refer to a valid list entry, and the resulting picture shall be the same for all slices of a coded picture. When merge_tmvp_ref_idx_l0 is not present, it is inferred to be equal to 0.

merge_tmvp_ref_idx_l1 specifies the reference index in list 1 for the temporal merging candidate in merge mode. merge_tmvp_ref_idx_l1 shall always refer to a valid list entry, and the resulting picture shall be the same for all slices of a coded picture. When merge_tmvp_ref_idx_l1 is not present, it is inferred to be equal to 0.

##### 8.5.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPSW and nPSH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPSW and nPSH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPSW and nPSH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate refIdxLX is set equal to 0 merge_tmvp_ref_idx_lX.

…

# Experiments

In order to observe coding efficiency impact on the base HEVC, the proposed solution is implemented on HM 7.0 and simulated under common test conditions. In the simulations, merge_tmvp_ref_idx_present_flag was set as 1 and merge_tmvp_ref_idx_l0 and merge_tmvp_ref_idx_l1 were always set as 0. Therefore, the slice header always contains the information that reference index for the TMVP merge candidate is to be set as 0. As mentioned before, the latest specification of HEVC sets reference indices of the TMVP merge candidate always as 0.

The results are summarized in Table 1. As shown in the table, coding efficiency impact is 0.0% BDR.

Table 1

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N\A |  |  |
| Dec Time[%] | N\A |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N\A |  |  |
| Dec Time[%] | N\A |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).