# Proposed Text for DIS

Based on JCTVC-I1003_d5.doc

9.2.2.6 Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta.

Output of this process is the binarization of the syntax element.

The bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
	cMax = 24 + ( QpBdOffsetY >> 1 ) + signV
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
		cNum = absV − 2
		while( cNum− − ) 
			put( 1 ) 
		if( cMax > absV − 1 )
			put( 0 )
	}
	cMax = 4
	cNum = min(absV – 1, cMax)
 	while( cNum–– )
		put( 1 ) 
	if( cMax > absV − 1 )
		put( 0 )
	if( absV – 1 >= cMax )
		put( 0th order Exp-Golomb( absV – 1 – cMax ) )
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
}

| Table 9-1 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 |  |  | 1 |  | 2 |  | 3 |  | >=4 |  |  |
| cu_qp_delta | Error: Reference source not found | 0 | 0 |  |  |  | na (uses Decode Bypass) 1 |  | 1 2 |  | 2 | 2 binIdx==4 : 2 binIdx>=5 : na (uses Decode Bypass) |  |  |
|  |  | 3 |  | 0 |  |  | na (uses Decode Bypass) 1 |  | 1 2 |  | 2 | 2 binIdx==4 : 2 binIdx>=5 : na (uses Decode Bypass) |  |  |
|  |  | 6 |  |  | 0 |  | na (uses Decode Bypass) 1 |  | 1 2 |  | 2 | 2 binIdx==4 : 2 binIdx>=5 : na (uses Decode Bypass) |  |  |