| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0226 WG11 Number: m25552 |
| --- | --- |

| Title: | AhG5: Reduction of context bins and bypass bins grouping for cu_qp_delta |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kengo Terada Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 { terada.kengo, sasai.hisao, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In the HEVC Draft 7, most of bins for the cu_qp_delta are arithmetic coded using the context models. This contribution proposes to use bypass coding instead of context based one for higher throughput and also proposes to concatenate bypass coded bins for parallel processing. The proposed modification was evaluated on top of HM7.0. Experimental result reportedly showed that the proposed modification can reduce context coded bins by 64% for cu_qp_delta and can reduce isolated bypass coded bins by 29% with less than 0.1% BD-rate loss.

# Introduction

In the HEVC draft 7, significant_flag, sign_flag and absVminus1 are coded for cu_qp_delta.

Table.1: cu_qp_delta coding

As the context based coding is used for most of bins for cu_qp_delta, the data dependency in the context selection makes parallel processing difficult for the parameter. Bypass coding is helpful for the parallelization in general.

On the other hand, concatenation of bypass coded bins can improve the throughput of the bypass coding as reported in [1]. However, bypass coded bins for sign_flag is isolated from other bypass coded bins in the Draft 7.

# Proposed Solution

## Swap of sign_flag and absVminus1 (STEP1)

In this proposal, dependency between sign_flag and cMax of TU for absVminus1 is removed same as in JCTVC-I0594 [2] and, in addition, the bin coding order for sign_flag and absVminus1 is swapped. Thus, the bypass coded bins for sign_flag is concatenated with that for suffix part of absVminus1 to improve throughput.

Fig.1: Syntax Order and Binarization Type

## Bypass coding for absVminus1 (STEP2)

In this proposal, prefix part of absVminus1 is coded using bypass coding instead of context based one. Three patterns are proposed (STEP2a / 2b / 2c) as shown in Table.2. The number of bypass coding is changed in STEP2a / 2b / 2c. In STEP2c, all bins in absVminus1 use bypass coding.

Table.2: Binarization Type and ctxIdxInc

## Benefit

The benefit is as follows.

- Reduction of context coded bins and total bins & bypass coded bins grouping

  - Worst Case (Max of cu_qp_dalta) : 26 context coded bins / 1 bypass coded bins / total 27 bins
-> STEP1	: 5 context coded bins / 10 concatenation bypass coded bins / total 15 bins
-> STEP2a	: 3 context coded bins / 12 concatenation bypass coded bins / total 15 bins
-> STEP2b	: 2 context coded bins / 13 concatenation bypass coded bins / total 15 bins
-> STEP2c	: 1 context coded bins / 14 concatenation bypass coded bins / total 15 bins

# Results

The proposed method was evaluated in HM-7.0 software under the conditions specified by CE4 on quantization in 8th meeting.

#======== Quantization ============= 
MaxDeltaQP 		: 0  	# CU-based multi-QP optimization 
MaxCuDQPDepth  	: 3 	# Max depth of a minimum CuDQP for sub-LCU-level delta QP 
DeltaQpRD         	: 0  	# Slice-based multi-QP optimization 
RDOQ              	: 1 	# RDOQ 
ChromaQpOffset    	: 0  	# ChromaQPOffset 
ChromaQpOffset2nd 	: 0  	# ChromaQPOffset2nd 
AdaptiveQP  		: 1  	# QP adaptation based on a psycho-visual model (0: None, 1: TM5Step3) 
MaxQPAdaptationRange	: 12	# dQP range of QP adaptation

The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

In addition, we investigated the ratio of non-serialized bypass coded bin, isolated bypass coded bin, for an evaluation of parallel processing of bypass coded.

Table.3: BD BR luminance for the proposed method

Table.4: Ratio of the number of context coded bins

Table.5: Ratio of the number of isolated bypass bins

# Conclusion

This contribution proposes to use bypass coding instead of context based one for higher throughput and bypass coded bins grouping is proposed for parallel processing of bypass coding. It was confirmed that proposed modification can reduce context coded bins and isolated bypass coded bins without significant impact on coding efficiency (in range of 0.1%). This contribution recommends STEP2c. It is suggested to consider the inclusion of this proposal in the DIS of HEVC.

# References

- B. Shi, W. Zheng, H.-S. Lee, D.-X. Li, and M. Zhang, “Pipelined Architecture Design of H.264/AVC CABAC Real-Time Decoding,” in IEEE International Conference on Circuits and Systems for Communications, May 2008, pp. 492 –496.

- V. Seregin et al, “Bypass bins for reference index and delta QP coding”, JCTVC-I0594, JCT-VC 9th Meeting, Geneva, May, 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Table.6: BD BR for STEP1

Table.7: BD BR for STEP2a

Table.8: BD BR for STEP2b

Table.9: BD BR for STEP2c