| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0230 |
| --- | --- |

| Title: | Improvement of HEVC lossless coding using transform coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min-Su Cheon Yumi Sohn Sunil Lee Il-Koo Kim Jeonghoon Park | Email: | minsu.cheon@samsung.com yumi.sohn@samsung.com sunil.lee@samsung.com ilkoo.kim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, a lossless coding method using the transform coefficient coding is proposed. Contrary to the current design of HEVC where lossless coding is simply implemented by bypassing transform, quantization, and in-loop filtering, in the proposed method, lossless coding is achieved by additionally signalling the information lost in the process of transform and quantization of the residual signal on top of the existing HEVC lossy coding framework. The simulation results show that the coding gain of the proposed method is (w/o Class F) 8.8% in AI Main and 8.3% in AI HE10 configurations, on average. The proposed method can be combined with other methods for the performance improvement of HEVC lossless coding, and the additional simulation results show that (w/o Class F) the coding gain of 10.8% and 11.2% are respectively achieved in AI Main and AI HE10 configurations, when combined with sample-based angular prediction in JCTVC-I0117.

# Introduction

Lossless coding has many applications where it is essential to maintain the integrity of an original image or video, e.g. medical image compression. In the current design of HEVC, lossless coding is simply implemented by bypassing transform, quantization, and all the in-loop filters (de-blocking, SAO, and ALF) [1]. Even though the current design is simple and straightforward, its coding performance is not satisfactory, and a few contributions have been proposed for the performance improvement, e.g. sample-based angular prediction [2]. This contribution proposes a lossless coding method using the transform coefficient coding. Contrary to the current design, in the proposed method, lossless coding is implemented on top of the lossy coding framework, and the information lost in the lossy coding process is additionally signalled to the decoder to achieve the lossless coding. The existing building blocks of HEVC are re-used to implement the proposed method, and the details of the algorithm and the simulation results will be provided in Section 2 and 3, respectively.

# Algorithm Description

Fig. 1 shows the encoder block diagram of the propose method. As the conventional coding process of HEVC lossy coding, a residual signal r is obtained by subtracting the intra or inter-predicted signal from an input signal (r=o−p). The quantized transform coefficient c is obtained by applying transform and quantization to the residual signal, and then it is coded by the entropy coding. The reconstructed residual r is obtained by applying inverse quantization and inverse transform to the quantized transform coefficients. Since the process of transform and quantization is lossy, the reconstructed residual r is not the same as the residual signal r, and this difference leads to the lossy coding unless the additional information is signalled to the decoder. In the proposed method, the remaining residual r, which is obtained as the difference between the residual r and the reconstructed residual signal r, is additionally coded and signalled to the decoder to achieve the lossless coding (r=r−r). The entropy coding process of the remaining residual, spatial_residual_coding, is described in Section 6 in the form of the pseudo code. Basically, it is a simplified form of the HEVC residual coding considering that the signal to be coded is not transform coefficient but spatial residual.

The proposed method is a natural extension of the HEVC design since it is implemented on top of the existing HEVC lossy coding, and no new coding tool has been added. Even though the entropy coding of the remaining residual is different from the HEVC residual coding, it can be also easily replaced with the HEVC residual coding method without much performance degradation. The proposed method is also a superset of the current design of HEVC lossless coding, since the remaining residual r can be the same as the residual signal r, when all the transform coefficients are determined to be zero in the rate-distortion optimization process at the encoder. This feature gives additional flexibility and possibility of the performance improvement to the encoder, since the encoder can determine how to distribute the total information for lossless coding between transform coefficients c in the frequency domain and spatial residual r in the spatial domain. The Qp value for the quantization also affects the coding gain and the best Qp value which leads to the best coding gain is different for each test sequence. However, the performance difference due to the Qp change is not large, and it is remained as an encoder option. Note that the fixed Qp values are used for all the tests in this contribution. The proposed method can be easily combined with other methods for the performance improvement of HEVC lossless coding, e.g. sample-based angular prediction [2].

Figure 1. Encoder block diagram of the proposed method.

# Simulation Results

The proposed lossless method is implemented on top of HM7.1. The test is done only for AI Main and AI HE10 configurations. For HM7.1 lossless coding mode, Qp is set as 0. The flags related to lossless coding, TransquantBypassEnableFlag and CUTransquantBypassFlagValue are set as 1. For the proposed method, those two flags are also set as 1. However, Qp is set as 22, since lossy coding is also performed as a part of lossless coding in the proposed method.

Table 1 and 2 summarize the coding performance of the proposed method and the proposed method combined with the sample-based angular prediction. The results show that the average coding gain of the proposed method is 8.8% in AI Main and 8.3% in AI HE10 configurations without Class F, and the sample-based prediction provides additional 2.0% in both configurations.

Table 3 also summarizes the coding gain of the proposed method combined with the sample-based angular prediction, however only 3 intra prediction modes (DC, Vertical, and Horizontal) are used in this experiment. The results show that the performance degradation due to the limited number of intra prediction modes is only 2.1% in AI Main and 1.3% in AI HE10 configurations, respectively. Considering the trade-off between the coding performance and the complexity, this combination can be considered as a reasonable compromise.

Table 1. Results of the proposed method

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A | 2.0 | 2.3 | -12.4% | 1.3 | 1.5 | -11.6% |
| Class B | 2.0 | 2.2 | -7.4% | 1.3 | 1.4 | -6.8% |
| Class C | 1.9 | 2.1 | -8.0% | 1.2 | 1.3 | -7.7% |
| Class D | 1.8 | 2.0 | -9.1% | 1.2 | 1.3 | -9.8% |
| Class E | 2.6 | 2.8 | -6.7% | 1.6 | 1.7 | -5.3% |
| Overall (w/o F) | 2.0 | 2.2 | -8.8% | 2.9 | 3.1 | -8.3% |
| Overall (w/ F) | 2.4 | 2.7 | -8.3% | 1.3 | 1.4 | -7.9% |
| Class F | 4.4 | 4.7 | -5.8% | 2.9 | 3.1 | -5.5% |
| Enc Time[%] | 193% |  |  | 158% |  |  |
| Dec Time[%] | 107% |  |  | 101% |  |  |

Table 2. Results of the proposed method combined with sample-based angular prediction

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A | 2.0 | 2.4 | -14.5% | 1.3 | 1.5 | -13.8% |
| Class B | 2.0 | 2.2 | -8.1% | 1.3 | 1.4 | -8.3% |
| Class C | 1.9 | 2.1 | -9.5% | 1.2 | 1.4 | -10.0% |
| Class D | 1.8 | 2.0 | -11.5% | 1.2 | 1.3 | -13.0% |
| Class E | 2.6 | 3.0 | -10.8% | 1.6 | 1.8 | -12.0% |
| Overall (w/o F) | 2.0 | 2.3 | -10.8% | 2.9 | 3.4 | -11.2% |
| Overall (w/ F) | 2.4 | 2.8 | -10.9% | 1.3 | 1.5 | -11.7% |
| Class F | 4.4 | 5.1 | -11.4% | 2.9 | 3.4 | -13.2% |
| Enc Time[%] | 195% |  |  | 156% |  |  |
| Dec Time[%] | 104% |  |  | 94% |  |  |

Table 3. Results of the proposed method combined with 
sample-based angular prediction (3 intra prediction modes)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A | 2.0 | 2.3 | -13.3% | 1.3 | 1.5 | -12.1% |
| Class B | 2.0 | 2.2 | -6.7% | 1.3 | 1.4 | -7.7% |
| Class C | 1.9 | 2.0 | -5.7% | 1.2 | 1.3 | -7.8% |
| Class D | 1.8 | 2.0 | -8.8% | 1.2 | 1.3 | -11.3% |
| Class E | 2.6 | 2.9 | -9.5% | 1.6 | 1.8 | -11.7% |
| Overall (w/o F) | 2.0 | 2.2 | -8.7% | 2.9 | 3.4 | -9.9% |
| Overall (w/ F) | 2.4 | 2.7 | -8.5% | 1.3 | 1.5 | -10.1% |
| Class F | 4.4 | 4.9 | -7.5% | 2.9 | 3.4 | -11.0% |
| Enc Time[%] | 117% |  |  | 90% |  |  |
| Dec Time[%] | 105% |  |  | 92% |  |  |

# Conclusion

HEVC lossless coding can be a good candidate for many applications which require the lossless coding feature. However, the performance of the HEVC lossless coding needs to be further improved for the wide adoption in the industry. Considering the performance improvement provided by the proposed method and its reasonable complexity, it is recommended to adopt the proposed method into HEVC lossless coding.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand  “High efficiency video coding (HEVC) text specification draft 7” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, Apr. 2012.

- M. Zhou, “AHG13: Sample-based angular intra prediction for HEVC lossless coding” Document of Joint Collaborative Team on Video Coding, JCTVC-I0117, Apr. 2012.

- Sunil Lee, Min-Su Cheon, Yumi Sohn, Il-Koo Kim, and Jeonghoon Park, “HEVC lossless coding for medical image compression” Document of Joint Collaborative Team on Video Coding, JCTVC-J0232, July 2012.

# Text for Working Draft

## Syntax table

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( lossless_enable_flag ) { |  |
| cu_lossless_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ){ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| if (lossless_enable_flag) |  |
| spatial_residual_tree_n( x0, y0, x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| MaxTrafoDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| if (lossless_enable_flag) |  |
| spatial_residual_tree_n( x0, y0, x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |

| spatial_residual_tree_n( x0L, y0L, x0C, y0C, xBase, yBase, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| if( trafoDepth = = 0 ¦ ¦ log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb_rem[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr_rem[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0L ][ y0L ][ trafoDepth ] ) { |  |
| if( interTbSplitDirectionL = = 2 ) { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1L = y0L |  |
| x2L = x0L |  |
| y2L = y0L + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3L = x1L |  |
| y3L = y2L |  |
| } else { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y1L = y0L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| x2L = x1L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y2L = y1L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| x3L = x2L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y3L = y2L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| } |  |
| if( interTbSplitDirectionC = = 2 && log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| xC3 = xC1 |  |
| yC3 = yC2 |  |
| } else if ( log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y1C = y0C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| x2C = x1C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y2C = y1C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| x3C = x2C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y3C = y2C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| } else { |  |
| x1C = x0C |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C |  |
| x3C = x0C |  |
| y3C = y0C |  |
| } |  |
| if( interTbSplitDirectionL ! = 2 ) { |  |
| log2TrafoWidth = log2TrafoWidth − 2 * interTbSplitDirectionL + 1 |  |
| log2TrafoHeight = log2TrafoHeight + 2 * interTbSplitDirectionL − 1 |  |
| } |  |
| remain_residual_tree_n( x0L, y0L, x0C, y0C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| remain_residual _tree_n( x1L, y1L, x1C, y1C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| remain_residual _tree_n( x2L, y2L, x2C, y2C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| remain_residual _tree_n( x3L, y3L, x3C, y3C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma_rem[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| spatial_residual_unit_n (x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

| spatial_residual_unit_n( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| if( PredMode = = MODE_INTRA ) { |  |
| scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ log2TrafoSize − 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if( remain_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| spatial_residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0 ) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( remain_cb[ x0L ][ y0L ][ trafoDepth ] ) |  |
| spatial_residual_coding( x0L, y0L, log2TrafoWidth-1, log2TrafoHeight-1, scanIdxC, 1 ) |  |
| if( remain_cr[ x0L ][ y0L ][ trafoDepth ] ) |  |
| spatial_residual_coding( x0L, y0L, log2TrafoWidth-1, log2TrafoHeight-1, scanIdxC, 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( remain_cb[ x0L ][ y0L ][ trafoDepth ] ) |  |
| spatial_residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1 ) |  |
| if( remain_cr[ x0L ][ y0L ][ trafoDepth ] ) |  |
| spatial_residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2 ) | Is it same?? |
| } |  |
| } |  |
| } |  |

| spatial_residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| n = 0 |  |
| maxNumCoeff = (1<<(log2TrafoWidth<<1))-1 |  |
| do { |  |
| residual_abs_level_greater1_flag[n] | ae(v) |
| if(residual_abs_level[n]) { |  |
| residual_sign_flag[n] | ae(v) |
| } |  |
| n++ |  |
| } while (n < maxNumCoeff) |  |
| n= 0 |  |
| do { |  |
| if(residual_abs_level[n]>=minCoeff) { |  |
| residual_abs_level_remaining[n] -- |  |
| residual_abs_level_remaining[n] | ae(v) |
| } | ae(v) |
| } while (n< maxNumCoeff) |  |

## Semantics

In HM7.1, if the transquant_bypass_enable_flag and cu_transquant_bypass_flag is 1, it means lossless coding with bypassing transform and quantization. In this proposed method, it is still lossless but transform and quantization processes are performed. If those flags are perceived as lossless coding syntax, name of the syntax should be changed to prevent confusion. Otherwise another flag is needed to specify transform coefficient coding for lossless in CU unit. We suggest changing those two flags’ name as lossless_enable_flag and cu_lossless_flag.

cbf_luma_rem[ x0L ][ y0L ][ trafoDepth ] equal to 1 specifies that the luma remain residual block contains one or more levels after subtracting reconstructed residual from original not equal to 0. The array indices x0L, y0L specify the location ( x0L, y0L ) of the top-left luma sample of the considered remain residual block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_luma_rem[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred to be equal to 1.

cbf_cb_rem[ x0C ][ y0C ][ trafoDepth ] equal to 1 specifies that the Cb remain residual block contains one or more levels after subtracting reconstructed residual from original not equal to 0. The array indices x0C, y0C specify the location ( x0C, y0C ) of the top-left luma sample of the considered remain residual block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cb_rem[ x0C ][ y0C ][ trafoDepth ] is not present, the value of cbf_cb_rem[ x0C ][ y0C ][ trafoDepth ] is inferred as follows.

- If trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cb_rem[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to cbf_cb_rem[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cb_rem[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to 0.

cbf_cr_rem[ x0C ][ y0C ][ trafoDepth ] equal to 1 specifies that the Cr remain residual block contains one or more levels after subtracting reconstructed residual from original not equal to 0. The array indices x0C, y0C specify the location ( x0C, y0C ) of the top-left luma sample of the considered remain residual block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cr_rem[ x0C ][ y0C ][ trafoDepth ] is not present, the value of cbf_cr_rem[ x0C ][ y0C ][ trafoDepth ] is inferred as follows.

- If trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cr_rem[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to cbf_cr_rem[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cr_rem[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to 0.

residual_abs_level_greater1_flag[ n ] specifies for the scanning position n whether there are remain residual levels greater than 1. When residual_abs_level_greater1_flag[ n ] is not present, it is inferred to be equal to 0.

residual_sign_flag[ n ] specifies the sign of a remain residual level for the scanning position n as follows.

- If residual_sign_flag[ n ] is equal to 0, the corresponding remain residual level has a positive value.

- Otherwise (residual_sign_flag[ n ] is equal to 1), the corresponding remain residual level has a negative value.

When residual_sign_flag[ n ] is not present, it is inferred to be equal to 0.

residual_abs_level_remaining[ n ] is the remaining absolute value of a remain residual level that is coded with Golomb-Rice code at the scanning position n. When residual_abs_level_remaining [ n ] is not present, it is inferred as 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ld. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).