| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0231r1 |
| --- | --- |

| Title: | On nal_ref_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | TK Tan Junya Takiue 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +65 9799 3861 tktan.p@rd.nttdocomo.co.jp takiue@nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution seeks to clarify the purpose of the nal_ref_flag.  The nal_ref_flag does not seem to have any use in the decoding process apart from the final marking of the picture as reference or non-reference picture.

This contribution proposes to remove the nal_ref_flag syntax element from the NAL unit header and create 3 new NAL unit types for coded slices that can be either used for reference or not..

# Introduction

With the exception of nal_unit_types 1, 2 and 3, all other nal_unit_types impose a restriction on the value of nal_ref_flag.  Hence for cases other that nal_unit_types 1, 2, and 3, the nal_ref_flag may be inferred from the nal_unit_types.

This contribution proposes to remove nal_ref_flag (or absorb the bit into the nal_unit_type) and duplicate nal_unit_types for 1, 2 and 3. Use nal_unit_types 1, 2 and 3 for non-reference coded slice and nal_unit_types 9, 10 and 11 for coded slice used for reference.

nal_ref_flag is inferred from the nal_unit_type (according to Table 7-1.

All specification relating to nal_ref_flag or sop_desc_nal_ref_flag may be removed with the exception of the following clause in the section on Marking of the current picture after decoding, where the nal_unit_type is checked instead of the nal_ref_idc.

### Marking of the current picture after decoding

This process is invoked after all slices of the current picture have been decoded.

–	If nal_unit_type of the current picture is equal to 1, 2 or 3, the current picture is marked as "unused for reference".

–	Otherwise, the current picture is marked as "used for short-term reference".

The syntax for the NAL unit header can be simplified as follows, and the nal_unit_type increased to 7 bits.

### NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(7) |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 2 |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

# Conclusion

This contribution proposed to remove the nal_ref_flag from the NAL unit header and infer if the picture is a reference picture or not using the nal_unit_type.

Currently half of the possible NAL unit type values have been assigned, leaving 32 out of 64 NAL unit types that are currently reserved and available for specifying new NAL unit types in the future.  By using 3 of these reserved NAL unit type values and expanding the number of bits for the NAL unit types to 7 bits, we will have 93 (128 – 32 – 3 = 93) more NAL unit types that can be specified in the future.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Category | Content of NAL unit and RBSP syntax structure | nal_ref_flag |
| --- | --- | --- | --- |
| 0 |  | Unspecified | - |
| 1 | Other slice | Non-reference coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | 0 |
| 2 | TFD slice | Non-reference coded slice of a TFD picture slice_layer_rbsp( ) | 0 |
| 3 | TLA slice | Non-reference coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | 0 |
| 4 | RAP slice | Reference coded slice of a CRAT picture slice_layer_rbsp( ) | 1 |
| 5 | RAP slice | Reference coded slice of an CRANT picture slice_layer_rbsp( ) | 1 |
| 6 | RAP slice | Reference coded slice of a BLCT picture slice_layer_rbsp( ) | 1 |
| 7 | RAP slice | Reference coded slice of a BLCNT picture slice_layer_rbsp( ) | 1 |
| 8 | RAP slice | Reference coded slice of an IDR picture slice_layer_rbsp( ) | 1 |
| 9 | Other slice | Reference coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | 1 |
| 10 | TFD slice | Reference coded slice of a TFD picture slice_layer_rbsp( ) | 1 |
| 11 | TLA slice | Reference coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | 1 |
| 12..24 |  | Reserved | - |
| 25 | Parameter Set | Video parameter set video_parameter_set_rbsp( ) | 1 |
| 26 | Parameter Set | Sequence parameter set seq_parameter_set_rbsp( ) | 1 |
| 27 | Parameter Set | Picture parameter set pic_parameter_set_rbsp( ) | 1 |
| 28 | Parameter Set | Adaptation parameter set aps_rbsp( ) | 1 |
| 29 | Information | Access unit delimiter access_unit_delimiter_rbsp( ) | 0 |
| 30 | Information | Filler data filler_data_rbsp( ) | 0 |
| 31 | Information | Supplemental enhancement information (SEI) sei_rbsp( ) | 0 |
| 32..47 |  | Reserved | - |
| 48..63 |  | Unspecified | - |
| 64..127 |  | Reserved | - |

- Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).