| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0232 |
| --- | --- |

| Title: | HEVC lossless coding for medical image compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sunil Lee Min-Su Cheon Yumi Sohn Il-Koo Kim Jeonghoon Park | Email: | sunil.lee@samsung.com minsu.cheon@samsung.com yumi.sohn@samsung.com ilkoo.kim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Medical image compression is one of the most promising applications of HEVC lossless coding. In this informative contribution, the coding performance of HM7.1 lossless coding and its improvement is compared with the existing lossless image codec including JPEG Lossless, JPEG-2000 Lossless, and JPEG-LS. The simulation results show that the current HM7.1 lossless coding is outperformed by JPEG-2000 Lossless (14.7%) and JPEG-LS (10.8). The results also show that the performance of HEVC lossless coding can be significantly improved by applying the method proposed in JCTVC-I0230 – coding gain of 17.2% on average when combined with the sample-based angular prediction proposed in JCTVC-I0117.

# Introduction

As medical imaging facilities move towards completely film-less imaging and 3-dimensional medical imaging data with high resolution and high sampling efficiency is widely available, efficient compression of medical images is receiving increased attention. Although high resolution medical image can be compressed efficiently using lossy compression methods, medical circle has been unwilling to adopt lossy compression since it can affect the accuracy of diagnosis and it can also cause legal issues associated with the clinical decision misled by the distortion from lossy compression. Currently, medical images are compressed by lossless image codec supported by Digital Imaging and Communications in Medicine (DICOM) standard, e.g. JPEG Lossless, JPEG-2000 Lossless, and JPEG-LS. Even though each sequence consists of multiple images scanned by Computerized Tomography (CT) or Magnetic Resonance (MR) devices, each image is individually encoded using lossless image codec to provide instantaneous random access to each image.

In this informative contribution, the performance of HEVC lossless coding [1] is evaluated for the medical image compression and compared with that of the existing lossless image codecs including JPEG Lossless, JPEG-2000 Lossless, and JPEG-LS. The improvement of HEVC lossless coding proposed in [2] is also applied and its performance is compared with the other methods.

# Algorithm Description

HEVC lossless coding is defined in [1], and the improvement of HEVC lossless coding is described in [2].

# Test Setting and Simulation Results

Total 9 MR scans (4 brain scans and 5 breast scans) were collected from The Cancer Imaging Archive (TCIA) site which is a public database provided by National Cancer Institute [3]. To enable the experiment in HM7.1, one channel gray scale image is extracted from DICOM files and converted to 4:2:0 YUV sequence by padding 0 in chroma. All the tests are performed using All Intra HE10 configuration.

Table 1 summarizes the properties (resolution, number of frames, and bit-depth) of 9 MR scans and the coding performance of each lossless coding method for each scan when the anchor is HM7.1 lossless coding. As shown in the table, the coding gain of HM7.1 lossless coding is higher than that of JPEG lossless, however, it is much lower than that of JPEG-2000 lossless and JPEG-LS standardized more than 10 years ago. The performance of HEVC lossless coding is significantly improved by applying the method proposed in [2], and its coding gain as compared with HM7.1 lossless coding is 14.7% on average. The performance can be further improved when the method proposed in [2] is combined with the sample-based angular prediction [4] which provides additional 2.5% coding gain on average. The results in the last column shows that when only 3 intra prediction modes (DC, Vertical, Horizontal) are used, the coding gain of 16.0% can be achieved, which means that the performance degradation introduced by disabling the other intra prediction modes is only 1.2%.

Table 1. Properties of MR scans and the coding performance of each lossless coding method
(Anchor: HM7.1 lossless coding)

| Sequence name | Width, Height | Number of Frames | Max Bit-depth | Bit-rate saving (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | JPEG lossless | J2K lossless | JPEG-LS | Proposed [2] | Proposed + SAP [2] | Proposed + SAP + 3 modes [2] |
| Medical_Brain_1 | 512, 512 | 22 | 10 | 24.8 | -15.6 | -10.9 | -17.4 | -20.7 | -19.2 |
| Medical_Brain_2 | 512, 512 | 60 | 9 | 25.7 | -14.7 | -10.8 | -13.2 | -15.8 | -14.0 |
| Medical_Brain_3 | 512, 512 | 60 | 10 | 26.6 | -14.5 | -10.3 | -11.9 | -15.2 | -13.5 |
| Medical_Brain_4 | 512, 512 | 60 | 9 | 26.4 | -14.2 | -10.0 | -12.0 | -14.8 | -12.5 |
| Medical_Breast_1 | 864, 864 | 84 | 9 | 42.4 | -19.8 | -15.2 | -22.9 | -26.7 | -26.1 |
| Medical_Breast_2 | 560, 560 | 84 | 9 | 35.3 | -17.2 | -13.0 | -17.7 | -20.4 | -18.9 |
| Medical_Breast_3 | 640, 640 | 90 | 9 | 41.9 | -12.5 | -9.0 | -13.6 | -15.8 | -14.6 |
| Medical_Breast_4 | 528, 528 | 41 | 10 | 23.3 | -13.0 | -10.3 | -13.3 | -14.3 | -13.7 |
| Medical_Breast_5 | 672, 672 | 60 | 10 | 30.3 | -10.5 | -7.7 | -10.3 | -11.4 | -10.8 |
| Average | - | - | - | 30.8 | -14.7 | -10.8 | -14.7 | -17.2 | -16.0 |

# Conclusion and Recommendation

The medical image compression is one of the applications where HEVC lossless coding can be widely adopted. However, the performance of the current HEVC lossless coding is not satisfactory and even worse than the existing lossless image codec. Fortunately, the simulation results show that the performance of HEVC lossless coding can be significantly improved by adopting the methods which have been proposed so far, e.g. [2]. It is strongly recommended that the coding performance of HEVC lossless coding should be further improved for the adoption in the industry, and considering the needs from the industry, the schedule related to lossless coding needs to be shortened.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand  “High efficiency video coding (HEVC) text specification draft 7” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, Apr. 2012.

- Min-Su Cheon, Yumi Sohn, Sunil Lee, and Jeonghoon Park, “Improvement of HEVC lossless coding using transform coefficient coding” Document of Joint Collaborative Team on Video Coding, JCTVC-J0230, July 2012.

[3] The Cancer Imaging Archive - https://www.cancerimagingarchive.net

- M. Zhou, “AHG13: Sample-based angular intra prediction for HEVC lossless coding” Document of Joint Collaborative Team on Video Coding, JCTVC-I0117, Apr. 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).