| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J 0234 m25561 |
| --- | --- |

| Title: | Inter-RPS complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström Rickard Sjöberg Jonatan Samuelsson Ericsson Research Färögatan 6 164 80 Stockholm | Tel: Email: | +46702672192 jacob.strom at ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents two changes to how inter-RPS parameters are encoded, and one change that removes an alleged underspecification. The first proposed change is to not transmit the variable delta_idx_minus1 when encoding inter-RPS parameters in the SPS, and instead infer that it has the value 0. The variable is still transmitted when encoding inter-RPS parameters from the slice header. The second proposal is to signal a flag called used_by_curr_present_in_inter_rps_flag in the SPS. If this flag is false, all inter-RPS coded reference pictures are assumed to have used_by_curr_pic_flag[ j ] = 1 and this means that the flag does not need to be transmitted.  Both proposals have been tested on the entire AHG13 set of configuration files as specified in JCTVC-I0608. The proposal claims to lower the average complexity slightly and reduce the number of bits spent on sending the RPS data in the SPS with -11.8%. Separately, the first and second proposals are reported to reduce the number of bits by -7.0% and -4.8% respectively. The third proposal is to add semantics for what happens if the RPS row to predict from does not exist; then the empty set will be used for prediction.

# Introduction and Problem Statement

Information about the reference picture sets (RPSes) to be used for a slice can either be sent explicitly in the slice header, or implicitly, by having the slice header index into a list of RPSes sent in the sequence parameter set (SPS). Both when sending the RPSes explicitly, and when building up the RPS list in the SPS, it is possible to transmit the RPS-values using intra-coding (intra-RPS coding), or relative to an RPS already in the SPS (inter-RPS coding).

The inter-RPS approach is of relative high complexity in relation to the bit savings made, due to the fact that the total amount of RPS data is a rather small percentage of the total for the video sequence. We therefore present two proposals for reducing the complexity. In addition to reducing complexity, both solutions also save bits.

# Proposal 1

During inter-RPS prediction, one of the parameters signalled is delta_idx_minus1, which describes which RPS row in the SPS the decoder should use for prediction. When inter-RPS is signalled explicitly, i.e., in the slice header, this is important information that needs to be signalled. However, when inter-RPS signalling is used to build up the RPS list in the SPS, it is most often the case that prediction should be done from the previous row in the list. In fact, the RPSes in all of the AHG13 set of configuration files specified in JCTVC-I0608, every single inter-RPS row is predicted from the previous row, i.e., delta_idx_minus1 is always zero. The reason for this is that it is always possible to arrange the rows such that prediction from the previous row is possible, and such an arrangement is also most often the most intuitive one as well as the one minimizing the cost for delta_idx_minus1, since it is UVLC-coded.

Hence, the proposed change is to remove the delta_idx_minus1 when using inter-RPS prediction for building up the list in the SPS. The variable is instead inferred to be equal to zero. In Appendix A we also describe why we believe why this will give a small complexity reduction on average compared to signalling delta_idx_minus1. We also give a detailed explanation on specification changes in Appendix A. For bit rate savings results, see section 4.

# Proposal 2

During inter-RPS prediction, a previously signalled RPS-row in the SPS is used to create candidate values. For each such candidate, it is necessary to state two things: 1) should the candidate be included in the current RPS, and, if the candidate is included, 2) what value should the flag used_by_curr_pic_flag have for this candidate. In the current specification this is signalled using

- ‘1’: Include candidate, set used_by_curr_pic_flag to 1.

- ‘01’: Include candidate, set used_by_curr_pic_flag to 0.

- ‘00’: Do not include candidate.

However, in the overwhelming number of cases in the AHG13 set of configuration files, used_by_curr_pic is always equal to 1. Hence we would like to simplify the signalling using

- ‘1’: Include candidate, set used_by_curr_pic_flag to 1.

- ‘0’: Do not include candidate.

For the few configuration files for which used_by_curr_pic_flag needs to be set to 0, the old signalling is used. Which signalling to use is controlled by a flag called used_by_curr_present_in_inter_rps_flag in the SPS. If used_by_curr_present_in_inter_rps_flag equals 0, the simplified signalling is used, and if it equals 1, the old signalling is used. As described in Appendix A, we believe this change will give a small complexity reduction on average compared to the present state of the standard. We also give a detailed explanation on specification changes in Appendix A. For bit rate savings results, see the next section.

# Proposal 3

The first RPS-row in the SPS can choose to do inter-RPS prediction by setting the inter_ref_pic_set_prediction_flag equal to 1. However, it is currently not described what should happen in the decoder in this case; since it is the first RPS-row there is no previous reference picture set to predict from. Therefore, we propose that prediction should be done from the empty set in this case. The semantic change needed is indicated below; new text in blue, removed text in overstrike:

delta_idx_minus1 plus 1 specifies the difference between the index of the reference picture set of the current picture and the index of the reference picture set used for inter reference picture set prediction.

The variable RIdx shall be derived as follows.

RIdx = idx − (delta_idx_minus1+1)

If RIdx is negative, prediction should be carried out from the empty set.

# Results

We have run the HM-7.0-dev-ahg13 software with and without the changes for all the AHG13 configuration files as specified in JCTVC-I0608. Then we have counted the number of bits used for the signalling for RPSes in the SPS. The results are shown in Table 1.

Table 1 – Results for proposal1, proposal 2 and proposal1 + proposal 2 combined

| configuration file | current standard (bits) | prop. 1 (bits) | prop. 2 (bits) | prop. 1 + 2 (bits) | flag |
| --- | --- | --- | --- | --- | --- |
| encoder_lowdelay_3_1 | 142 | 131 | 135 | 124 | 0 |
| encoder_lowdelay_3_5 | 142 | 131 | 135 | 124 | 0 |
| encoder_lowdelay_P_3_2 | 34 | 31 | 32 | 29 | 0 |
| encoder_lowdelay_P_3_3_1000msRTT | 10 | 10 | 11 | 11 | 0 |
| encoder_lowdelay_P_3_3_200msRTT | 10 | 10 | 11 | 11 | 0 |
| encoder_lowdelay_P_3_4 | 6 | 6 | 7 | 7 | 0 |
| encoder_randomaccess_2_1 | 149 | 140 | 143 | 134 | 0 |
| encoder_randomaccess_2_2_T1_32 | 91 | 84 | 84 | 79 | 0 |
| encoder_randomaccess_2_2_T2_32 | 103 | 96 | 98 | 91 | 0 |
| encoder_randomaccess_2_3_T3_32 | 93 | 86 | 89 | 82 | 0 |
| encoder_randomaccess_2_3_T4_32 | 106 | 99 | 107 | 100 | 1 |
| encoder_randomaccess_2_4_T5_64 | 337 | 314 | 319 | 296 | 0 |
| encoder_randomaccess_2_5_256 | 809 | 746 | 748 | 685 | 0 |
| encoder_randomaccess_2_6_10s_30s | 149 | 140 | 143 | 134 | 0 |
| encoder_randomaccess_2_6_20s_60s | 149 | 140 | 143 | 134 | 0 |
| encoder_randomaccess_2_7_32 | 171 | 160 | 172 | 161 | 1 |
| encoder_randomaccess_2_8 | 149 | 140 | 143 | 134 | 0 |
| sum | 2650 | 2464 | 2522 | 2336 |  |
| bit savings in percentage |  | -7.0% | -4.8% | -11.85% |  |

Hence the bit savings of the two proposals combined are -11.8%. If proposal 1 is used individually, it will yield 2464 bits, or a bit reduction of -7.0%. If proposal 2 is used individually, it yields 2522 bits which means a bit reduction of -4.8%. Note that these savings figures are for the RPS data in the SPS. They are not for the entire SPS, nor for the entire video sequence. The last column in Table 1 shows the value of the flag used_by_curr_present_in_inter_rps_flag, which is zero for all configuration files except two.

# Appendix A

In this appendix we describe the full changes to the specification needed to implement the proposals. The short-term reference picture set syntax specified in Section 7.3.3.1 would need to be changed according to Table 2, where new code is marked with blue and removed code is overstriked.

Table 2 – Proposed change to Short-term reference picture set

| 1 | short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- | --- |
| 2 | inter_ref_pic_set_prediction_flag | u(1) |
| 3 | if( inter_ref_pic_set_prediction_flag ) { |  |
| 4 | if( idx = = num_short_term_ref_pic_sets ) |  |
| 5 | delta_idx_minus1 | ue(v) |
| 6 | delta_rps_sign | u(1) |
| 7 | abs_delta_rps_minus1 | ue(v) |
| 8 | for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| 9 | if(used_by_curr_present_in_inter_rps_flag ) { |  |
| 10 | used_by_curr_pic_flag[ j ] | u(1) |
| 11 | if( !used_by_curr_pic_flag[ j ] ) |  |
| 12 | use_delta_flag[ j ] | u(1) |
| 13 | } |  |
| 14 | else |  |
| 15 | use_delta_flag[ j ] | u(1) |
| 16 | } |  |
| 17 | } |  |
| 18 | else { |  |
| 19 | num_negative_pics | ue(v) |
| 20 | num_positive_pics | ue(v) |
| 21 | for( i = 0; i < num_negative_pics; i++ ) { |  |
| 22 | delta_poc_s0_minus1[ i ] | ue(v) |
| 23 | used_by_curr_pic_s0_flag[ i ] | u(1) |
| 24 | } |  |
| 25 | for( i = 0; i < num_positive_pics; i++ ) { |  |
| 26 | delta_poc_s1_minus1[ i ] | ue(v) |
| 27 | used_by_curr_pic_s1_flag[ i ] | u(1) |
| 28 | } |  |
| 29 | } |  |
| 30 | } |  |

The inclusion of if(idx == num_short_term_ref_pic_sets) on line 4 is to check whether the function is called when constructing the RPS list in the SPS (in which case the statement would be false) or if called from the slice header (in which case the statement would be true). This corresponds to proposal 1. We believe that the most often taken path would be not to execute row 5, which would mean a small complexity gain on average, since executing the if-statement should be simpler than receiving the value of delta_idx_minus1. The worst case complexity would be slightly higher, since it would then execute both row 4 and 5.

We also need to make changes in Section 7.4.3.1 to describe the inferred value in the case when delta_idx_minus1 is not sent (additions in blue):

“delta_idx_minus1 plus 1 specifies the difference between the index of the reference picture set of the current picture and the index of the reference picture set used for inter reference picture set prediction. If delta_idx_minus1 is not present, it is inferred to be zero.”

The if-statement on line 9 in Table 2 corresponds to proposal 2; if the flag used_by_curr_present_in_inter_rps_flag is true, then the (old) code on lines 10 through 12 will be used. If it is true, the new code on line 15 will be used instead. We believe that line 15 will be the most often taken path, and that this is less complex than lines 10-12, hence producing a small complexity gain on average. Again, the worst case complexity may be slightly worse in the cases when 10-12 are executed due to the inclusion of the if-statement on line 9.

We also need to make changes in Section 7.4.3.1 to describe the inferred value in the case when used_by_curr_pic_flag[ j ] is not sent (additions in blue):

“used_by_curr_pic_flag[ j ] indicates whether a picture is used for reference by the current picture. If used_by_curr_pic_flag[ j ] is not present, it is inferred to be one.”

We also need to make changes in section 7.3.2.2 “Sequence parameter set RBSP syntax” where  used_by_curr_present_in_inter_rps_flag is signalled. We propose adding the line shown in blue in Table 3:

Table 3 – Proposed change to 7.3.2.2 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| . | . |
| . | . |
| . | . |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| used_by_curr_present_in_inter_rps_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| . |  |
| . |  |
| . |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Finally, we also need to add semantics to 7.4.2.2 “Sequence parameter set RBSP semantics”. We propose to add the following before the definition of  num_short_term_ref_pic_sets:

“used_by_curr_present_in_inter_rps_flag specifies whether used_by_curr_pic_flag will be signalled during inter-RPS prediction.”

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).