| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0235 |
| --- | --- |

| Title: | AHG4/AHG9: On tile size restrictions in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arild Fuldseth | Tel: Email: | +47 92 41 40 82 arild.fuldseth@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

In this contribution the effect on compression performance when using multiple tiles for encoding is analyzed. Using the HM software, the BDR loss for different number of tiles per frame is measured. It is asserted that for many real-time multi-core encoder implementations, the benefit from using more compute power through having a relatively large number of cores work in parallel is likely to be higher than the loss due to dependency breaks at tile boundaries. On the other hand, it has also been argued that having a large number of tiles per frame represents an implementation burden for single-core decoder designs. As a consequence, the current HEVC draft text mandates a minimum tile width of 384 luma samples. A desire to mandate a minimum tile height is also anticipated. This contribution proposes changes to the HEVC draft text to better satisfy the design requirements of both multi-core encoder and single-core decoder implementations. In particular, it is proposed to make the number of tiles a decoder capability parameter, where the maximum number of tiles vertically and horizontally applies only to the maximum resolution at each level. This is achieved by amending table A-1 to include a maximum number of tiles horizontally and vertically. This approach allows for the added flexibility for an encoder to use the same number of tiles independent of resolution and at the same time allows the decoder to express support for a maximum number of tiles though claiming conformance to a specific level.

# Introduction

Currently, there is increased interest in multi-core processing and several processors having a large number of cores (typically larger than 16) are available from various manufacturers. We believe the tiles feature in the HEVC standard is well suited for this type of architecture. To make maximum usage of these devices for HEVC encoding, it is desired to allow for a large number tiles per frame. Recognizing the fact that a large number of tiles might also impose a burden on some single-core decoder implementations, we propose to solve these apparent conflicting design requirements by allowing the decoder to express the maximum number of tiles as a decoder capability. Section 2 of this contribution presents experimental results showing that the impact on compression performance for a relatively large number of tiles is only small to moderate, while Section 3 discusses advantages of having a large number of tiles per frame. Finally, Section 4 proposes changes to the HEVC draft text to better accommodate various encoder and decoder design requirements.

# Experimental Results

The results in this section are based on the HM7.0 software but with tiles-related patches of revisions 2405 and 2445 from the HM-7.0-dev branch. Table 1 shows the number of tiles used in document JCTVC-F335. The same number of tiles is used Table 2 showing BDR results of single tile vs. multiple tiles using common coding conditions. Table 3 show results for class B&E and Main profile, low delay configurations using various numbers of tiles. As can be seen from the table, doubling the number of tiles (e.g. from 9 to 18 tiles or from 18 to 36 tiles for class B) increases the BDR loss with 1%-2% and 5%-6% for class B and class E respectively.  It should also be noted that the loss is highly qp/bit-rate dependent, especially for class E sequences. In particular, when operating at realistic bit rates for 720p60 videoconferencing material (512 kbps – 1 Mbps), the loss is significantly smaller than the average values given in Table 3.

Table 1 Number of tiles for various sequence classes (from JCVC-F335)

| Class | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
| --- | --- | --- | --- | --- |
|  | horizontally | vertically | horizontally | vertically |
| A | 5 | 5 | 8 | 5 |
| B | 6 | 3 | 5 | 6 |
| C | 2 | 2 | 7 | 4 |
| D | 1 | 1 | 7 | 4 |
| E | 4 | 3 | 5 | 4 |

Table 2  BDR results for single tile vs.  multiple tiles

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1,0% | 0,6% | 1,6% | 1,0% | 3,2% | 4,5% |
| Class B | 1,4% | 1,3% | 1,4% | 1,5% | 2,0% | 1,4% |
| Class C | 0,9% | 0,9% | 0,9% | 1,0% | 1,1% | 1,1% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 3,2% | 2,6% | 2,8% | 3,4% | 2,7% | 3,2% |
| Overall | 1,2% | 1,0% | 1,3% | 1,3% | 1,8% | 2,0% |
|  | 1,2% | 1,0% | 1,3% | 1,3% | 1,8% | 1,9% |
| Class F | 1,8% | 2,0% | 2,1% | 2,0% | 1,7% | 1,7% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 101 % |  |  | 101 % |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 3,3% | 3,9% | 4,9% | 3,1% | 7,4% | 6,9% |
| Class B | 3,4% | 3,7% | 3,4% | 3,1% | 4,0% | 3,5% |
| Class C | 1,8% | 2,1% | 2,0% | 1,8% | 2,3% | 2,1% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Overall | 2,2% | 2,5% | 2,6% | 2,1% | 3,5% | 3,1% |
|  | 2,3% | 2,5% | 2,7% | 2,1% | 3,5% | 3,1% |
| Class F | 4,7% | 5,0% | 5,0% | 4,7% | 5,1% | 4,4% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 101 % |  |  | 102 % |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2,9% | 4,0% | 3,7% | 2,6% | 4,3% | 4,0% |
| Class C | 1,2% | 1,9% | 1,8% | 1,2% | 1,8% | 2,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 9,4% | 8,4% | 7,3% | 9,5% | 8,7% | 7,9% |
| Overall | 3,0% | 3,3% | 3,0% | 2,9% | 3,4% | 3,2% |
|  | 2,9% | 3,3% | 3,0% | 2,9% | 3,4% | 3,2% |
| Class F | 5,5% | 5,2% | 6,3% | 5,6% | 5,6% | 7,1% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 99 % |  |  | 102 % |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2,6% | 3,6% | 3,1% | 2,3% | 3,8% | 3,4% |
| Class C | 1,2% | 1,4% | 1,5% | 1,2% | 1,9% | 2,1% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 9,0% | 7,5% | 7,4% | 8,9% | 8,7% | 7,9% |
| Overall | 2,8% | 2,9% | 2,7% | 2,7% | 3,3% | 3,1% |
|  | 2,8% | 2,9% | 2,7% | 2,7% | 3,3% | 3,1% |
| Class F | 5,2% | 5,2% | 6,0% | 5,3% | 5,7% | 5,5% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 99 % |  |  | 98 % |  |  |

Table 3  BDR results for single tile vs. multi tile – various numbers of tiles

| Class B |  |  | Class E |  |  |
| --- | --- | --- | --- | --- | --- |
| # tiles | LB Main | LP Main | # tiles | LB Main | LP Main |
| 9 | 1.6% | 1.4% | 6 | 4.2% | 4.1% |
| 18 | 2.9% | 2.6% | 12 | 9.4% | 9.0% |
| 36 | 4.9% | 4.5% | 24 | 15.4% | 14.8% |

# Multiple tiles for encoder parallelization

Based on the results above we believe that if we can allocate more compute resources (i.e. more cores) per frame by increasing the number of tiles, it is likely that one can get a net gain in compression performance by using an encoder that is configured at a higher complexity operating point.

In addition to the gain from allocating more compute resources per frame, we believe there is an additional benefit from having more tiles than cores (maybe by a factor of 2 or more). This is to achieve dynamic load balancing when there is unequal distribution of compute load across one frame.

# Proposed changes for adoption

The purpose of this contribution is to allow for a larger degree of flexibility in the HEVC standard accommodating various implementation constraints:

- Multi-core encoders benefiting from a large number of tiles.

- Decoders having limited tile processing capabilities.

- Decoders capable of processing a large number of tiles with marginal penalty on performance.

To allow this, we propose to make the maximum number of tiles (vertically and horizontally) a decoder capability parameter by adding two columns to table A-1 as shown below. This will allow an encoder to send the same number of tiles to a decoder independent of resolution. For example, when communicating with a decoder capable of level 4 (1080p), an encoder can use 9x5 tiles for all resolutions at 1080p and below.

An additional benefit is in cases where the each parameter of table A-1 is negotiated individually. In that case, a decoder can signal support for e.g. level 3.1, but additionally signal support for the number of tiles allowed at e.g. level 5.

Finally, we believe it is also beneficial to express the tile constraints in terms of maximum number of tiles rather than minimum tile size. This allows for more flexibility in terms of adaptive and non-uniform tile size distribution. For example, at 1080p resolution, a minimum tile width of 384 luma samples (i.e. current draft text) , allows for exactly 5 tile columns of 384 samples each (1920/384 = 5). However, if a maximum number of tile columns of 5 were specified for 1080p instead, one could choose e.g. one tile column equal to 320 samples and another tile column equal to 448 samples. This would be beneficial for dynamic load balancing and avoiding possible artefacts at static tile boundaries.

It is proposed to remove the sentence about ColumnWidthInLumaSamples[ i ] being less than or equal to 384 in Section A.3.2.

Table 4 – Proposed change to table A-1

| Level | Max luma pixel rate MaxLumaPR(samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) | Max number of ver. tiles | Max number of hor. tiles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 | 1 | 1 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 | 3 | 1 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 | 5 | 2 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 | 6 | 3 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 | 9 | 5 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 | 9 | 5 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 | 9 | 5 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 | 9 | 5 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 | 18 | 10 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 | 18 | 10 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 | 18 | 10 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 | 32 | 22 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 | 32 | 22 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 | 32 | 22 |

# Conclusions

To accommodate various encoder and decoder designs in a more flexible way, it is proposed to make the maximum number of tiles a decoder capability parameter. This is done by amending table A-1 to limit the maximum number of horizontal and vertical tiles for each level.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A

The following changes to the HEVC draft text are proposed.

    - Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have adaptive_loop_filter_enabled_flag equal to 0 only.

- Sequence parameter sets shall have chroma_pred_from_luma_enabled_flag equal to 0 only.

- Sequence parameter sets shall have asymmetric_motion_partitions_enabled_flag equal to 1 only.

[Ed. (KM): Confirm that this flag has been defined. (GJS): It is now in the SPS, but not yet connected to the decoding process.]

- Sequence parameter sets shall have nsrqt_enabled_flag equal to 0 only.

- Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

- Log2CtbSize shall be in the range from 4 to 6, inclusive.

- SliceGranularity shall be equal to 0.

- Picture parameter sets shall have tiles_or_entropy_coding_sync_idc in the range of 0 to 2, inclusive.

- Picture parameter sets shall have dependent_slice_enabled_flag equal to 0.

[Ed. (KM): Confirm that this is the best formulation]

- Picture parameter sets shall have pic_parameter_set_id in the range of 0 to 63, inclusive.

- Adaptation parameter sets shall have aps_id in the range of 0 to 63, inclusive.

- The level constraints specified for the Main profile in subclause A.2 shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by profile_idc being equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of level_idc) shall be capable of decoding all bitstreams in which the profile_idc is equal to 1 and level_idc represents a level lower than or equal to the specified level. For purposes of comparison of level capabilities, a particular level shall be considered to be a lower level than some other level if the level_idc of the particular level is less than the level_idc of the other level.

  - Levels

    - General

The following is specified for expressing the constraints in this annex.

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0.

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable fR be set to 1  300.

Bitstreams conforming to a profile at a specified level shall obey the following constraints:

- The nominal removal time of access unit n (with n > 0) from the CPB as specified in subclause Error: Reference source not found, satisfies the constraint that tr,n( n ) − tr( n − 1 ) is greater than or equal to Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Table A-10-1 that applies to picture n − 1, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture n − 1.

- The difference between consecutive output times of pictures from the DPB as specified in subclause Error: Reference source not found, satisfies the constraint that to,dpb( n ) >= Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Table A-10-1 for picture n, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples of picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- pic_width_in_luma_samples * pic_height_in_luma_samples <= MaxLumaFS, where MaxLumaFS is specified in Table A-10-1

- pic_width_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

- pic_height_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

- sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] <= MaxDpbSize, where MaxDpbSize is specified in Table A-10-1.

- When tiles_or_entropy_coding_sync_idc is equal to 1, num_tile_columns_minus1 + 1 <= MaxHorTiles, where MaxHorTiles is specified in Table A-10-1.

- When tiles_or_entropy_coding_sync_idc is equal to 1, num_tile_rows_minus1 + 1 <= MaxVerTiles, where MaxVerTiles is specified in Table A-10-1.

Table A-10-1 specifies the limits for each level. The use of the MinCR parameter column of Table A-10-1 is specified in subclause Error: Reference source not found.

A level to which the bitstream conforms shall be indicated by the syntax element level_idc as follows.

–	level_idc shall be set equal to a value of 30 times the level number specified in Table A-10-1.

[Ed. (YK): Putting level_idc 30 times instead of 10 times the level number can solve the level 1b problem occurred in AVC.]

Table A-10-1 – Level limits

| Level | Max luma pixel rate MaxLumaPR(samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) | Max number of ver. tiles MaxVer Tiles | Max number of hor. tiles maxHor Tiles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 | 1 | 1 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 | 3 | 1 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 | 5 | 2 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 | 6 | 3 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 | 9 | 5 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 | 9 | 5 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 | 9 | 5 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 | 9 | 5 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 | 18 | 10 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 | 18 | 10 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 | 18 | 10 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 | 32 | 22 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 | 32 | 22 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 | 32 | 22 |