| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0235 |
| --- | --- |

| Title: | On tile size restrictions in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arild Fuldseth | Tel: Email: | +47 92 41 40 82 arild.fuldseth@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

In this contribution the effect on compression performance when using multiple tiles for encoding is analyzed. Using the HM software, the BDR loss for different number of tiles per frame is measured. It is asserted that for many real-time multi-core encoder implementations, the benefit from using more compute power through having a relatively large number of cores work in parallel is likely to be higher than the loss due to dependency breaks at tile boundaries. On the other hand, it has also been argued that a large number of tiles per frame represent an implementation burden for single-core decoder designs. As a consequence, the current HEVC draft text mandates a minimum tile width of 384 luma samples. A desire to mandate a minimum tile height is also anticipated. This contribution proposes changes to the HEVC draft text to better satisfy the needs of both multi-core encoder and single-core decoder implementations. In particular, it is proposed to make the number of tiles a decoder capability parameter, where the maximum number of tiles vertically and horizontally applies only to the maximum resolution at each level. This is achieved by amending table A-1 to include a maximum number of tiles horizontally and vertically. This approach allows for the added flexibility for an encoder to use the same number of tiles independent of resolution and at the same time allows the decoder to express support for a maximum number of tiles though claiming conformance to a specific level.

# Introduction

Currently, there is increased interest in multi-core processing and several processors having a large number of cores (typically larger than 16) are available from various manufacturers. In particular, Cisco is currently shipping video conferencing equipment using a processing device having 64 cores and we believe the tiles feature in the HEVC standard is well suited for this type of architecture. To make maximum usage of such devices for video compression, it is desired to allow for a large number tiles per frame. Recognizing the fact that multiple tiles might impose a burden on some single-core decoder implementation, we propose to solve this by allowing the decoder to express maximum number of tiles as a decoder capability. Section 2 of this contribution presents experimental results showing the impact on the compression performance for various numbers of tiles while Section 3 discusses various advantages having a large number of tiles per frame. Finally, Section 4 proposes

# Experimental Results

The results in this section ae based on the HM7.0 software but with tiles-related patches of revisions 2405 and 2445 of the HM-7.0-dev branch. Table 1 shows the number of tiles used in document JCTVC-F335. The same number of tiles is used for the BDR results of Table 2 showing BDR results of single tile vs. multiple tiles using common coding conditions. Table 3 show results for class B&E and Main profile, low delay configurations using various numbers of tiles. As can be seen from the table, doubling the number of tiles increases the BDR loss with 1%-2% and 5%-6% for class B and class E respectively.

Table 1. Number of tiles for various sequence classes

| Class | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
| --- | --- | --- | --- | --- |
|  | horizontally | vertically | horizontally | vertically |
| A | 5 | 5 | 8 | 5 |
| B | 6 | 3 | 5 | 6 |
| C | 2 | 2 | 7 | 4 |
| D | 1 | 1 | 7 | 4 |
| E | 4 | 3 | 5 | 4 |

Table 2.  BDR results for single tile vs.  multiple tiles

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1,0% | 0,6% | 1,6% | 1,0% | 3,2% | 4,5% |
| Class B | 1,4% | 1,3% | 1,4% | 1,5% | 2,0% | 1,4% |
| Class C | 0,9% | 0,9% | 0,9% | 1,0% | 1,1% | 1,1% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 3,2% | 2,6% | 2,8% | 3,4% | 2,7% | 3,2% |
| Overall | 1,2% | 1,0% | 1,3% | 1,3% | 1,8% | 2,0% |
|  | 1,2% | 1,0% | 1,3% | 1,3% | 1,8% | 1,9% |
| Class F | 1,8% | 2,0% | 2,1% | 2,0% | 1,7% | 1,7% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 101 % |  |  | 101 % |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 3,3% | 3,9% | 4,9% | 3,1% | 7,4% | 6,9% |
| Class B | 3,4% | 3,7% | 3,4% | 3,1% | 4,0% | 3,5% |
| Class C | 1,8% | 2,1% | 2,0% | 1,8% | 2,3% | 2,1% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Overall | 2,2% | 2,5% | 2,6% | 2,1% | 3,5% | 3,1% |
|  | 2,3% | 2,5% | 2,7% | 2,1% | 3,5% | 3,1% |
| Class F | 4,7% | 5,0% | 5,0% | 4,7% | 5,1% | 4,4% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 101 % |  |  | 102 % |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2,9% | 4,0% | 3,7% | 2,6% | 4,3% | 4,0% |
| Class C | 1,2% | 1,9% | 1,8% | 1,2% | 1,8% | 2,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 9,4% | 8,4% | 7,3% | 9,5% | 8,7% | 7,9% |
| Overall | 3,0% | 3,3% | 3,0% | 2,9% | 3,4% | 3,2% |
|  | 2,9% | 3,3% | 3,0% | 2,9% | 3,4% | 3,2% |
| Class F | 5,5% | 5,2% | 6,3% | 5,6% | 5,6% | 7,1% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 99 % |  |  | 102 % |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2,6% | 3,6% | 3,1% | 2,3% | 3,8% | 3,4% |
| Class C | 1,2% | 1,4% | 1,5% | 1,2% | 1,9% | 2,1% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 9,0% | 7,5% | 7,4% | 8,9% | 8,7% | 7,9% |
| Overall | 2,8% | 2,9% | 2,7% | 2,7% | 3,3% | 3,1% |
|  | 2,8% | 2,9% | 2,7% | 2,7% | 3,3% | 3,1% |
| Class F | 5,2% | 5,2% | 6,0% | 5,3% | 5,7% | 5,5% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 99 % |  |  | 98 % |  |  |

Table 3. BDR results for single tile vs. multi tile – various numbers of tiles

| Class B |  |  | Class E |  |  |
| --- | --- | --- | --- | --- | --- |
| # tiles | LB Main | LP Main | # tiles | LB Main | LP Main |
| 9 | 1.6% | 1.4% | 6 | 4.2% | 4.1% |
| 18 | 2.9% | 2.6% | 12 | 9.4% | 9.0% |
| 36 | 4.9% | 4.5% | 24 | 15.4% | 14.8% |

# Multiple tiles for encoder parallelization

Based on the results above we believe that if we can allocate more compute resources (i.e. more cores) per frame by increasing the number of tiles, it is likely that one can get a net gain in compression performance by using an encoder that is configured at a higher complexity operating point.

In addition to the gain from allocating more compute resources per frame, we believe there is an additional benefit from having more tiles than cores (maybe by a factor of 2 or more). This is to achieve dynamic load balancing when there is unequal distribution of compute load across one frame.

# Proposed changes for adoption

In the current draft, the minimum tile width is defined as 384 luma samples. This limits the number of tile columns to e.g. 3 and 5 for 720p and 1080p resolutions respectively. However, we believe it is a reasonable assumption that a decoder that can support 5 tile columns at 1080p resolution can also support 5 tile columns at 720p resolution. Unfortunately, the current draft does not allow for this kind of flexibility.

The purpose of this contribution is to allow for a larger degree of flexibility in the HEVC standard accommodating various implementation constraints:

- Multi-core encoders benefiting from a large number of tiles.

- Decoders having limited tile processing capabilities.

- Decoders capable of processing a large number of tiles with marginal penalty on performance.

To allow this, we propose to make the maximum number of tiles (vertically and horizontally) a decoder capability parameter by adding two columns to table A-1 as shown below. This will allow an encoder to send the same number of tiles to a decoder independent of resolution. For example, when communicating with a decoder capable of level 4 (1080p), an encoder can use 9x5 tiles for all resolutions at 1080p and below.

An additional benefit is in cases where the each parameter of table A-1 is negotiated individually. In that case, a decoder can signal support for e.g. level 3.1, but additionally signal support for the number of tiles allowed at e.g. level 5.

It is proposed to remove the sentence about ColumnWidthInLumaSamples[ i ] being less than or equal to 384 in Section A.3.2.

Table 4 – Proposed change to table A-1

| Level | Max luma pixel rate MaxLumaPR(samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) | Max number of ver. tiles | Max number of hor. tiles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 | 1 | 1 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 | 3 | 1 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 | 5 | 2 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 | 6 | 3 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 | 9 | 5 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 | 9 | 5 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 | 9 | 5 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 | 9 | 5 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 | 18 | 10 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 | 18 | 10 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 | 18 | 10 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 | 32 | 22 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 | 32 | 22 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 | 32 | 22 |

# Conclusions

To accommodate various encoder and decoder designs in a more flexible way, it is proposed to make the maximum number of tiles a decoder capability parameter. This is done by amending table A-1 to limit the maximum number of horizontal and vertical tiles for each level.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).