| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0236_r1 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Gary J. Sullivan Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a further study of the relative objective (i.e. PSNR-based) compression performance of HEVC Main Profile and AVC High Profile. It builds upon the prior work reported in JCTVC-G399 JCTVC-H0360, JCTVC-I0409, updating the results by using the latest available reference software (JM-18.3 and HM-7.0). Relative to the results reported at the preceding meeting in JCTVC-I0409, there is nothing changed in the AVC anchor. Experimental results reportedly show that 1) for the Main Profile all-intra configuration, HM-7.0 can save about 22% in bit rate; 2) for the Main Profile random-access configuration, HM-7.0 can save about 34% in bit rate; and 3) for the Main Profile low-delay configuration, HM-7.0 can save about 36% in bit rate when compared with JM-18.3.

It is noted that these results are based on 6:1:1 weighted YUV PSNR measurements, which are an imperfect substitute for subjective quality assessment, and that the subjective performance of HEVC may actually be generally somewhat better than its objective (PSNR) performance may indicate.

This revision reports some additional tests, including lossless coding comparison between JM18.3 and HM-7 (r2461), individual HM “high efficiency” tool test, and the evaluation of the existing fast encoding algorithms in HM-7.0.

# Introduction

This contribution presents the results of objective (PSNR-based) compression comparison tests between the current state of the HEVC draft standard and the AVC High Profile as an anchor reference. Experimental results show that, when compared with JM-18.3, the following results are obtained:

- For the Main Profile all-intra configuration, HM-7.0 can save about 22% in bit rate;

- For the Main Profile random-access configuration, HM-7.0 can save about 34% in bit rate; and

- For the Main Profile low-delay configuration, HM-7.0 can save about 36% in bit rate.

Class F was excluded in these average results.

# Experimental results

The JM software was configured with the same coding structure and referencing structure as HM-7.0. The JM and HM used the same QP settings. Both the JM anchor and HM test encoding used 8-bit encoding and decoding. The detailed JM configuration is explained below.

The average YUV PSNR (shown in the right-most column of the tables below) was calculated as

PSNRavg=6×PSNRY+PSNRU+PSNRV8

It is noted that these results are based on PSNR measurements, which are an imperfect substitute for subjective quality assessment, and that the subjective performance of HEVC may actually be generally somewhat better than its objective (PSNR) performance may indicate.

## All Intra case

Nothing special was configured.

## Random access case

- Using 4 reference pictures

- Using the same referencing structure as HM-7.0 (by MMCO).

- Using B pictures with two identical lists at level 0.

## Low delay case

- Using 4 reference pictures

- Using a 1+3 referencing structure (by MMCO)

- Using B pictures

The summary results of performance comparison between AVC High Profile and HEVC Main Profile are shown in the Tables 1, 2, and 3. Details can be found in the attached file JCTVC-J0236_JMHM.xlsm. We do not provide the encoding time and decoding time ratio results, since the codebases are different and with different degrees of optimization; and neither of them is really speed-optimized.

The HEVC Main Profile is specified in [5], and we use the configuration files provided in [4] to generate the HEVC Main Profile bitstream.

Table 1 – Results for All Intra (AVC High Profile vs. HEVC Draft 7 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −23.7% | −21.6% | −20.4% | −23.4% |
| Class B | −22.8% | −22.5% | −22.1% | −22.8% |
| Class C | −19.9% | −21.1% | −21.3% | −20.3% |
| Class D | −16.7% | −16.9% | −17.6% | −16.9% |
| Class E | −29.1% | −27.4% | −27.1% | −28.8% |
| Class F | −22.8% | −19.5% | −20.1% | −22.7% |
| Average | −22.2% | −21.3% | −21.2% | −22.2% |
| Average without F | −22.1% | −21.7% | −21.4% | −22.1% |

Table 2 – Results for Random Access (AVC High Profile vs. HEVC Draft 7 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −36.7% | −37.8% | −39.4% | −36.9% |
| Class B | −39.7% | −38.0% | −37.2% | −39.5% |
| Class C | −30.0% | −32.2% | −32.2% | −30.4% |
| Class D | −27.2% | −31.5% | −33.2% | −28.2% |
| Class E |  |  |  |  |
| Class F | −25.6% | −25.7% | −27.0% | −26.0% |
| Average | −32.2% | −33.3% | −34.0% | −32.5% |
| Average without F | −33.7% | −35.1% | −35.6% | −34.1% |

Table 3 – Results for Low Delay (AVC High Profile vs. HEVC Draft 7 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −42.1% | −34.7% | −35.3% | −41.1% |
| Class C | −32.3% | −32.3% | −32.9% | −32.5% |
| Class D | −29.1% | −31.5% | −33.2% | −29.7% |
| Class E | −43.7% | −39.5% | −38.2% | −42.9% |
| Class F | −28.8% | −30.5% | −33.2% | −29.9% |
| Average | −35.1% | −33.5% | −34.4% | −35.1% |
| Average without F | −36.7% | −34.2% | −34.7% | −36.4% |

# Lossless encoding evaluation

To achieve lossless encoding, we configure JM according with the following setting:

- Use High 4:4:4 Predictive Profile (244)

- Set LosslessCoding to 1 in the cfg file of JM

- Set all QP to 0

- The other settings are identical to those described in Section 2.

There are still two other issues to be clarified:

- The JM encoder reports PSNR value in a strange way. When lossless encoding is applied, the output PSNR is reported as neither 99.99 nor 0.00. JM uses a strange number, and it seems that the number depends on the size of picture. So we manually compare the original sequence and the reconstructed sequence outputted at encoder side, and we can confirm that it is lossless encoding.

- We meet encoding/decoding mismatch when lossless encoding is applied. It seems that
https://ipbt.hhi.fraunhofer.de/mantis/view.php?id=228 reports the same (at least similar) bug. We applied the solution provided on that page and then the encoding/decoding mismatch disappears. But that modification (only) includes changes at decoder side, we are not sure whether that modification is proper or not.

HM software is configured as:

- The software version is HM7.0-dev-r2461.

- Set all Qp to 0

- LosslessCuEnabled        :  0

- TransquantBypassEnableFlag:  1

- CUTransquantBypassFlagValue: 1

Other parameters are same as those in the HM7.0 default configurations files.

The summary results are provided in Table 4 and the detailed results can be found in JCTVC-J0236_HMvsJM_lossless.xlsx.

Table 4 Summary results of lossless coding

|  | Average Compression ratio of AVC | Average Compression ratio of HEVC | Average delta bitrate (HEVC−AVC)/AVC% |
| --- | --- | --- | --- |
| AI | 3.29 | 3.14 | 4.7% |
| RA | 8.26 | 8.20 | −0.3% |
| LB | 12.03 | 12.27 | −0.0% |

It seems that although for normal lossy coding, HEVC outperforms than AVC, for lossless coding, the current design of HEVC may be even worse than that of AVC.

# Individual “High Efficiency” tool evaluation

We enable individual “high efficiency” coding tools based on the main profile specified in [4], including 10-bit encoding, ALF, Chroma LM, NSQT and TS. It should be noticed that when comparing the encoding/decoding time, the codecs (both the anchor and the test) include some degree of code level optimization, similar as proposed (but not adopted) in JCTVC-H0142 [6]. This kind of optimization saves about 10% encoding time and 7% decoding time for inter coding cases in our simulation. As both the anchor and the test include the same speedup, we think the comparison is fair.

The summary results are provided in Table 5 and the detailed results can be found in JCTVC-J0236_HEToolTest.xlsm. Class F is excluded in the average results.

Table 5 HEVC High Efficiency tool test

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| 10-bit | −1.1% | −1.3% | −1.6% | −1.1% | −1.7% | −4.4% | −5.6% | −2.2% | −1.9% | −3.5% | −4.3% | −2.2% |
|  | Enc./Dec. 100%/103% |  |  |  | Enc./Dec. 100%/106% |  |  |  | Enc./Dec. 100%/106% |  |  |  |
| ALF | −1.3% | −3.0% | −3.2% | −1.7% | −2.6% | −2.6% | −2.5% | −2.5% | −1.3% | −2.9% | −3.1% | −1.6% |
|  | Enc./Dec. 103%/113% |  |  |  | Enc./Dec. 101%/114% |  |  |  | Enc./Dec. 100%/113% |  |  |  |
| LM | −0.7% | −8.2% | −6.6% | −1.8% | −0.3% | −10.1% | −7.1% | −1.6% | −0.1% | −2.4% | −2.1% | −0.5% |
|  | Enc./Dec. 103%/101% |  |  |  | Enc./Dec. 100%/100% |  |  |  | Enc./Dec. 100%/101% |  |  |  |
| TS | 0.0% | 0.0% | −0.2% | 0.0% | 0.0% | −0.2% | −0.3% | −0.1% | 0.0% | 0.2% | 0.1% | 0.0% |
|  | Enc./Dec. 130%/100% |  |  |  | Enc./Dec. 102%/100% |  |  |  | Enc./Dec. 101%/100% |  |  |  |
| NSQT |  |  |  |  | −0.4% | −0.8% | −0.7% | −0.5% | −1.0% | −2.6% | −2.2% | −1.2% |
|  |  |  |  |  | Enc./Dec. 102%/100% |  |  |  | Enc./Dec. 102%/100% |  |  |  |

It should be noticed that although TS has no significant impact on the coding efficiency of the sequences in Class A to E, for Class F, TS brings about 7.5%, 5.5%, and 3.3% Y BD-Rate saving for AI, RA, and LB respectively.

# Fast encoding algorithms evaluation

There are already some fast algorithms existing in HM, such as Fast intra transform skipping, Early CU setting, Cbf fast mode setting, and Early SKIP detection setting (these names are from the help message of HM encoder). These fast algorithms are disabled in CTC to achieve better performance. Besides these existing fast algorithms, there are at least two straightforward fast mode decision algorithms for intra mode decision.

- Only check one mode with the smallest SATD cost, which can be implemented by setting every elements in g_aucIntraModeNumFast[] to 1.

- Only check DM and LM for chroma.

There are six fast algorithms mentioned above. We enable all of them to see the efficiency of the fast coding algorithms. The summary results are shown in Table 6, and the detailed results can be found in .

Table 6 Y BD-Rate of fast encoding algorithms in HM-7.0

|  | AI-Main | AI-HE10 | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | 1.2% | 1.0% | 2.0% | 1.9% |  |  |
| Class B | 1.3% | 1.1% | 2.5% | 2.4% | 1.7% | 1.8% |
| Class C | 2.0% | 1.8% | 2.8% | 2.8% | 1.5% | 1.6% |
| Class D | 2.4% | 2.1% | 2.9% | 2.8% | 1.6% | 1.8% |
| Class E | 2.0% | 1.7% |  |  | 2.2% | 2.3% |
| Overall | 1.8% | 1.5% | 2.5% | 2.5% | 1.7% | 1.8% |
| Enc. Time | 62% | 60% | 48% | 48% | 48% | 48% |
| Dec. Time | 100% | 100% | 99% | 99% | 99% | 99% |

From Table 6, we can know that

- When enabling all of the six fast encoding algorithms, about 40% encoding time saving for intra cases and 52% encoding time saving for inter cases can be achieved, with only about 1.5%~2.5% Y BD-Rate loss on average.

- The chroma BD-Rate losses for “Main” cases are somewhat larger than that of luma, which is about 4% for AI_Main and RA_Main. The possible reason is that we only check DM and LM when selecting intra chroma mode, and for “Main” cases, there is no LM. Thus, there is no chroma mode decision for those cases. For “HE10” cases and Low Delay cases, the chroma BD-Rate is similar to that of luma.

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, CA, USA, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-I1100, “Common HM test conditions and software reference configurations,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-I1003, “High Efficiency Video Coding (HEVC) text specification draft 7,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-I0142, “AHG3: Multiplication free interpolation filter implementation,” Geneva, CH, 27 April – 7 May 2012.