| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0237 |
| --- | --- |

| Title: | Inter transform skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiulian Peng Cuiling Lan Jizheng Xu Gary J. Sullivan | Email: | jzxu@microsoft.com |
| Source: | Microsoft Corp. Xidian Univ. |  |  |

_____________________________

# Abstract

This proposal extends the current transform skipping design in intra coding to inter coding. Similar to intra transform skipping, the proposed change is only on 4x4 TU level and does not change motion estimation/compensation, de-quantization, scaling, in-loop filtering and entropy coding modules, except for adding one flag to indicate if a 4x4 inter TU uses transform skipping or not. For class F, coding gains about 1.8% for random_access and 3.0~3.3% for low_delay are observed, with the encoding running increases about 5%. With both intra and inter transform skipping, about 7.5% (AI), 7.2% (RA) and 6.5% (LD) coding gains for class F are achieved compared with HM7 without transform skipping. It is a unification between intra and inter coding and also a text simplification. This document also provides text to change the default quantization matrices when transform_skip_enabled is equal to 1.

# Introduction

At the 9th JCT-VC meeting, it was decided to adopt intra transform skipping [1] design into the current draft, which has shown significant coding gains for screen and graphics contents. In this proposal, we unify the design for both intra and inter coding and test its performance. It is also a text simplification. We also provide text to change the default quantization matrices when transform_skip_enabled is equal to 1.

# Inter transform skipping

Similar to intra transform skipping, we only apply transform skipping to 4x4 TUs in inter coding. So for inter coding, if a TU is with size 4x4, it can choose to perform the current 4x4 transform or skip the transform at all. In such a case, a bit-shift procedure is performed to normalize the residuals’ magnitudes, which is same as that in intra transform skipping.Same as intra transform skipping, inter transform skipping does not change motion estimation/compensation, de-quantization and scaling modules. In entropy coding, a flag for each 4x4 inter TU with cbf = 1 is sent to indicate if transform is bypassed or not. The Y, U and V TUs use exactly the same two contexts in intra transform skipping to code the flag.

At the encoder, after the mode decision for each 4x4 Y/U/V TU on whether all block coefficients are reserved (cbf=1) or discarded (cbf=0) is finished, the transform skipping is further checked on the 4x4 TU to decide whether the transform is skipped or not. To reduce encoding time, transform skipping is only checked when there are nonzero coefficients. The Y，U and V TUs are separately checked, which is same as that in intra transform skipping.

# Coding performance

We compared the proposed scheme (both intra and inter transform skipping are turned on) with the current HM design. The results compared with HM7.0 are shown in table 1. Because intra transform skipping is off in the main configurations, the coding gains in the main configurations come from both intra and inter transform skipping. The coding gains in the HE10 configurations are purely from inter transform skipping, which are about 1.8% for random_access and 3.0% for low_delay.

Table 1. Comparison with HM7.0 Details can be found in JCTVC-J0237_1

_r1.xlsm.

Table 2. Comparison with HM7.0 w/ intra transform skipping on. Details can be found in JCTVC- J0237_2

_r1.xlsm.

Please note that we also update the results in the excel files because in the first version of this document and excel files, we used a slightly different version of HM other than HM7.0. The updated results are almost the same as those in the first version. Please also note that the running times are not accurate in JCTVC- J0237_2_r1.xlsm because different clusters were used.

To provide more results, in table 2, we compared the proposed scheme with HM7.0 w/ intra transform skipping on, in both the main and HE10 configurations. Then, all the coding gains are considered to be from inter transform skipping. It can be seen that the coding gains are consistent in both configurations. Minor coding gains can also be observed for the coding of sequences in class A to E.

The encoding time increase is about 5% and there is no obvious decoding time change.

# Proposed text modifications

7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| …… | …… |

# Suggestion on the 4x4 default quantization matrix when transform_skip_enabled is equal to 1.

At the previous JCT-VC meeting, there are discussions on quantization matrix for transform skipping TUs. It is clear that a flat quantization should be applied to transform skipping TUs. In [1], we suggested to change the default quantization matrix for 4x4 TUs to be a flat one.

Text modifications to change the default 4x4 quantization matrices for both intra and inter coding are show as follows.

7.4.2.3 Picture parameter set RBSP semantics

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

RefMatrixID = MatrixID – scaling_list_pred_matrix_id_delta	(7-1)

where MatrixID is specified in Error: Reference source not found. When scaling_list_pred_matrix_id_delta is equal to 0, i.e., RefMatrixID is equal to MatrixID, scalingList is inferred from the default scaling list ScalingList[ SizeID ][ MatrixID ] as specified in Table 7-1, Table 7-1 and Error: Reference source not found.

7.4.2.5 Scaling list semantics

Table 7-1 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15 when transform_skip_enabled is equal to 0.

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |

Table 7-5 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15 when transform_skip_enabled is equal to 1.

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 |

-

-

# References

[1] C. Lan, J. Xu, G. J. Sullivan, and F. Wu, “Intra transform skipping,” JCTVC-I0408. Apr. 2012. Geneva, Switzerland.

Microsoft Corp. and Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).