| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0238 |
| --- | --- |

| Title: | Lossless coding via transform skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cuiling Lan Jizheng Xu Dake He Xiang Yu | Email: | jzxu@microsoft.com |
| Source: | Xidian Univ. Microsoft Corp. Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This document shows that lossless coding can be achieved in the current design when QP=4 and both intra and inter transform skipping at 4x4 TU level are used. Besides inter transform skipping, the proposed scheme does not need other normative changes. Compared with the current lossless coding design, the average bit-increases of the proposed scheme are -2.3%, -0.4% and -0.2% for AI_main, RA_main and LD_main. Because an encoder can achieve zero distortion when QP=4, QP=0, 1, 2 or 3 may not be necessary. Thus, it is suggested to remove unnecessary QPs in the draft.

# Introduction

In the current HM design, there are two ways to achieve lossless coding. One is with PCM coding, which does not save bits compared with raw video data. The other is with transquant_bypass design, which enables a CU to skip transform, scaling, quantization and in-loop filter.

In this document, we show that lossless coding can also be achieved by transform skipping. Transform skipping was adopted at the previous JCT-VC meeting for intra 4x4 TUs [1]. In [2], transform skipping is simply extended to 4x4 inter TUs. We show that with these two kinds of TUs, i.e., intra and inter 4x4 TUs with transform skipping, we can achieve lossless coding when QP=4 and data hiding is disabled. The coding performance is similar to that  of the lossless mode in the current HM.

# Lossless coding via transform skipping

In HEVC, transforms/inverse-transforms are not exactly reversible, which may introduce loss during coding. Thus, a special design is required to support the lossless coding for HEVC. In this proposal, we show a non-normative way to enable lossless coding at slice level when both intra and inter transform skipping are used.

For the transform skipping, if the quantization parameter (QP) is 4, the equivalent quantization step size is 1. Therefore, lossless coding can be easily achieved without any change to the decoder when both intra and inter transform skipping are enabled. There are some required changes at the encoder.

Lossless coding can be achieved when an encoder applies following strategy:

- Set Qp to 4 for each picture.

- Turn off RDOQ.

- Turn off sign bit hiding.

- Allow only 4x4 intra or inter transform skipping TUs. The coding cost is only checked at 4x4 TU level during the RDO. To assure 4x4 TUs can be reached, the encoder will not choose 64x64 CU mode when the maximum quadtree depth is 4; both 32x32 and 64x64 CUs will not be used when the maximum quadtree depth is 3.

An encoder can generate ALF, SAO parameters to implicitly disable them. Deblocking has no effect when the average QP is not larger than 17. Thus, there is no need to disable deblocking, SAO and ALF.

Deblocking will not take effect on a boundary if the average of two blocks related to the boundary is not greater than 17. Thus, if all QPs are not greater than 30, blocks with QP=4 will not be influenced by deblocking. In another word, regionally lossless coding can be achieved when the current block is coded with QP=4 and all its neighboring blocks’ QPs are not greater than 30. An advantage of this solution is that it can achieve 4x4-level lossy-lossless control.

# Coding performance

We compared the proposed scheme to the HM7-dev lossless scheme. The HM7 software version we used is HM7.0-dev-r2461. To make the coding lossless, we set QP=0 and the following parameters.

LosslessCuEnabled:		 	0

TransquantBypassEnableFlag: 		1

CUTransquantBypassFlagValue: 	1

Table 1 show the performance comparisons, which can also be found in JCTVC-J0238_r1.xlsx.

Table 1. Performance comparison between the proposed scheme and HM7.0 lossless.

Table 2. Comparison between results w/ low QPs and the lossless result w/ QP=4

We also compare the bitrate when a QP (1, 2, 3 or 4) is used with the bitrate of lossless coding with the proposed scheme. Table 2 shows the results. Detail comparisons for all sequences can be found in JCTVC-J0238_r1.xlsx. To generate results for QP=1, 2, 3 and 4, we turn on both intraTS and interTS, and set QP constant for each picture. Detail RD results can be found in JCTVC-J0238_Q1-4.xlsm.

From table 2, we can see that compared to lossless coding, averagely QP=1 setting increases the bitrate by 2.8% to 5.3% while it also introduces distortions. Thus, if a encoder can generate lossless bitstreams with QP=4, QP=1 setting seems useless. Even for the case of QP=3, the average bitrate saving is from 2.6% to 4.5% while the distortion is larger than QP=1. The benefit for a encoder to use QP=3 is still marginal given that it can perform QP=4 lossless coding.

# Proposed modifications to the current draft

When both intra and inter transform skipping are enabled, an encoder can send a video to a decoder without any distortions using QP=4. However, most likely there are still distortions when QP=0, 1, 2 or 3, which is weird.

Thus, it is proposed to let QP be within the range of [4, 51] for 8-bit video. For a general case, where the bit-depth might be larger than 8, currently the QP range is [-QpBdOffsetY, 51]. We suggest that it should be changed to [-QpBdOffsetY+4, 51], because with transform skipping, an encoder can also achieve coding without any distortions when QP=-QpBdOffsetY+4.

Thus, changes to the current draft include:

QpBdOffsetY = 6 * bit_depth_luma_minus8 - 4		(7-1)

QpBdOffsetC = 6 * bit_depth_chroma_minus8 - 4		(7-2)

- Change pic_init_qp_minus26 to pic_init_qp_minus28, and replace the corresponding constant 26 with 28, because the median becomes 24 if we change the QP range for 8-bit input.

# References

[1] C. Lan, J. Xu, G. J. Sullivan, and F. Wu, “Intra transform skipping,” JCTVC-I0408. Apr. 2012. Geneva, Switzerland.

[2] X. Peng, C. Lan and J. Xu, “Inter transform skipping,” JCTVC-J0238. Jul. 2012. Stockholm, Sweden

Xidian University, Microsoft Corp. and Research In Motion Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).