| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0239 |
| --- | --- |

| Title: | AHG10: Selective inter-layer prediction signaling for HEVC scalable extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

Inter-layer prediction is expected to be used in the planned scalable extension of HEVC. However, inter layer prediction may not be used for every picture or every slice. Thus, if one decoder only needs to decode a high quality layer, some packets at the base layer can be discarded. This document present a solution to allow easily discarding unused base layer bits to save bits for decoding an enhancement layer. It also provides a unified scheme to handle both fully scalable bitstreams and simulcast bitstreams, or to any trade-off in-between. The concept can also be applied to indicate existence of inter-layer prediction in possible other HEVC extensions.

# Introduction and problem statement

Inter-layer prediction was a useful tool in AVC scalable extension to improve the coding performance at enhancement layers while backward compatibility can be maintained. Inter-layer prediction design is also expected to be used in the scalable extension of HEVC.

In inter-layer prediction, a picture at a higher quality layer, e.g., enhancement layer can use the information of its correspondence at a lower quality layer, e.g., base layer, as prediction. Motion information, intra prediction directions, partitioning, residuals and reconstructed pixels all can be used in inter-layer prediction depending on the design. Generally, all information mentioned above can be used in a multiple-loop decoding scalable extension design, where a decoder should maintain a prediction loop for each layer. However, in a single-loop decoding design, reconstructed pixels are generally not available.

Depending on the content and the current picture’s coding type, the efficiency of inter-layer prediction varies. For example, normally inter-layer prediction is much effective in intra coding than in inter coding, because an inter coded picture may have other references at the same layer, which are with higher quality than the reference at the base layer. For some pictures, inter-layer prediction may even have marginal or no contributions to the coding of a higher layer. In such a case, some bits at the base layer can be discarded if only a higher quality video is needed to be decoded.

Figure 1 shows an example, where B0 to B5 denote base layer pictures and E0 to E5 denote enhancement layer pictures. In the example, inter-layer prediction is used for picture 0, 2, 4, while there is no for picture 1 and 3. If a device just needs to decode the enhancement layer, i.e., E0 to E5, all it needs is bitstreams of E0 to E5, B0, B2 and B4. Bitstreams of B1 and B3 can be discarded. This can significantly reduce the bandwidth between the bitstream extractor and the device.

A good encoder can even optimize the RD performance for an enhancement layer by selectively enabling or disabling inter-layer prediction for some pictures. For example, it was demonstrated in [1] that the performance at an enhancement layer can be easily improved by 1dB when only the enhancement layer is needed by selectively using inter-layer prediction; while the performance penalty is marginal when all the layers are needed.

Figure 1. An illustration of selective inter-layer prediction.

Thus, it is suggested to enable a mechanism to indicate if a lower layer’s information will be used by a higher layer or not, which can let a scalable bitstream exactor easily reduce bitrates depending on different applications.

# Solution and proposed text modifications

We suggest using a flag to indicate if a base layer slice can be discarded or not when decoding only an enhancement layer, so that the bandwidth or storage size can be reduced. Given the high volume of video data, such a flag should be processed efficiently and easily by an extractor, which should be very simple. Thus, we suggest to add this flag at NAL unit header, so that an extractor can easily tell which NAL unit can be discarded without digging into video bitstreams.

For enhancement layers, it can be added during the development process of scalable extension of HEVC. But we also need to modify the syntax at base layer, i.e., HEVC, to support such a mechanism.

The proposed change to the current HEVC draft is as follows.

7.3.1 NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_sip_flag | u(1) |
| nal_unit_type | u(6) |
| temporal_id | u(3) |
| reserved_one_45bits | u(45) |
| …… | …… |

nal_sip_flag equal to 0 for a NAL unit containing a slice indicates that the slice is not used by the decoding of any other layer.

reserved_one_45bits shall be equal to '00001'. Other values of reserved_one_45bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of reserved_one_45bits not equal to '00001'.

By setting nal_sip_flag, it enables an extractor easily tell which NAL unit should be passed and which can be discarded to save bandwidth.

In an extreme case, such a mechanism can simulate a simulcast situation. An encoder can set all nal_sip_flag to be 0, which means that an enhancement layer will not use the base layer at all. In such a case, an exactor can dump all NAL units at the base layer if only the enhancement layer is to be decoded.

Thus, another benefit of the proposed scheme is to unify the cases of scalable coding and simulcast; and to enable the system to make a trade-off between a fully scalable case and a simulcast one.

# References

[1] K. Zhang, J. Xu and F. Wu, “Selective inter-layer prediction,” JVT-R064. Joint Video Team (JVT) of ISO/IEC MPEG & TU-T VCEG, Bangkok, Thailand. January 2006.

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).